| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0068 |
| --- | --- |

| Title: | Decoding Time and Energy Assessment of VTM-10.0 and VVdeC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Matthias Kränzler Christian Herglotz André Kaup | Email: | matthias.kraenzler@fau.de christian.herglotz@fau.de andre.kaup@fau.de |
| Source: | Multimedia Communications and Signal Processing, Friedrich-Alexander Universität Erlangen-Nürnberg (FAU) |  |  |

_____________________________

# Abstract

This contribution evaluates the decoding energy and time demand of VVC in relation to HEVC. The authors compare several implementations of both video codecs in terms of decoding energy and time. The evaluation reveals that the decoding energy demand of the VTM-10.0 decoder has a significant increase compared to HM-16.20, which depends on the coding configuration. For randomaccess, it is reported that the decoding energy is increased by 88.29% on average. Furthermore, the authors investigate one practical software decoder implementation for each video codec.

# Introduction

The energy efficiency of the VTM decoder was already evaluated on VTM-6.0 [1] and VTM-7.0 [2]. For this meeting, the VTM-10.0 and VVdeC decoder are further analyzed.

This document has the following contributions:

- Analysis of VTM-10.0 decoder in terms of decoding energy and time.

- With respect to the HM decoder, the energy increase of VTM is disproportionately high in comparison with the decoding time increase.

- Analysis of VVdeC in relation to VTM and openHEVC.

# Energy Measurement & Test Set

According to the descriptions in JVET-P0084 [1], decoding energies are measured with RAPL on an Intel i7-8700 CPU. For the decoding of HEVC bit streams, we use the HM-16.20 [3] reference software and the openHEVC implementation used in FFmpeg 4.3.1 [4]. For VVC bit streams, the VTM-10.0 [5] reference software and the optimized implementation Versatile Video Decoder (VVdeC) [6] are used. To have a certain accuracy for the measured decoding energy, multiple measurements are performed and evaluated with statistical tests (cf. JVET-P0084). For our evaluation, the bit streams are encoded as described in the CTC for SDR content [7] with HM-16.20 and VTM-10.0.

# Evaluation of HM-16.20 and VTM-10.0

This section compares the complexity of both video codecs with respect to decoding time and decoding energy. Therefore, we use the Bjøntegaard-Delta-Decoding-Energy (BDDE) [8] to describe the additional energy demand of the decoder and the Bjøntegaard-Delta-Decoding-Time (BDDT) for the additional decoding time, respectively. Furthermore, the Bjøntegaard-Delta-Rate (BDR) is calculated to evaluate the improved compression performance.

In Table 1, the results of the VTM decoder are shown compared to the HM decoder depending on the encoder configuration. For all-intra coding, the energy demand of the VTM decoder is increased by 81.70% for an equal objective visual quality and the time demand by 74.67%, respectively. For lowdelayB, the energy demand is increased by 76.70% and the time demand by 60.13%. For randomaccess configuration, the BDDE is 88.29% and BDDT is 77.19%. The results of Table 1 reveal that the increase in decoding energy demand is disproportionately high in comparison with the decoding time.

Table 1: Evaluation of BDR, BDDE, and BDDT for the comparison of several coding configurations with HM- and VTM-decoding.

|  | All Intra Over HM-16.20 |  |  |
| --- | --- | --- | --- |
|  | YUV-BDR [%] | BDDT [%] | BDDE [%] |
| Class A1 | -28.35 | 64.44 | 69.44 |
| Class A2 | -25.85 | 77.07 | 81.82 |
| Class B | -21.63 | 77.04 | 83.36 |
| Class C | -22.15 | 83.91 | 92.93 |
| Class D | -17.72 | 85.57 | 95.61 |
| Class E | -25.76 | 61.69 | 67.70 |
| Class F | -30.91 | 67.18 | 74.10 |
| Mean | -24.27 | 74.67 | 81.70 |

|  | Lowdelay B Over HM-16.20 |  |  |
| --- | --- | --- | --- |
|  | YUV-BDR [%] | BDDT [%] | BDDE [%] |
| Class A1 | / | / | / |
| Class A2 | / | / | / |
| Class B | -30.99 | 58.71 | 75.37 |
| Class C | -28.64 | 64.72 | 83.16 |
| Class D | -25.51 | 77.14 | 91.05 |
| Class E | -33.73 | 51.75 | 70.74 |
| Class F | -38.78 | 46.60 | 62.04 |
| Mean | -31.39 | 60.13 | 76.70 |

|  | Randomaccess Over HM-16.20 |  |  |
| --- | --- | --- | --- |
|  | YUV-BDR [%] | BDDT [%] | BDDE [%] |
| Class A1 | -38.42 | 70.74 | 82.39 |
| Class A2 | -41.91 | 89.91 | 101.48 |
| Class B | -34.92 | 72.83 | 86.38 |
| Class C | -29.82 | 76.50 | 85.94 |
| Class D | -27.47 | 83.16 | 89.44 |
| Class E | / | / | / |
| Class F | -35.24 | 72.69 | 86.40 |
| Mean | -34.16 | 77.19 | 88.29 |

# Evaluation of VVdeC and VTM-10.0

In Table 2, both VVC decoders are evaluated. By using optimizations like SIMD and multi-threading, VVdeC reduces the decoding time by over 86% on average in relation to VTM for each coding configuration. Therefore, the decoding time of VVdeC is reduced to approximately one-tenth of VTM. The decoding energy demand with VVdeC-decoding is reduced by 66.75% for all-intra, by 57.35% for lowdelayB, and by 62.11% for randomaccess. The decrease in decoding time demand is disproportionately high in comparison with the decoding energy, which can be explained by the usage of multiple threads.

Table 2: Evaluation of decoding time and energy for VVdec- and VTM-decoding for all coding configurations.

|  | All Intra Over VTM-10.0 |  | Lowdelay B Over VTM-10.0 |  | Randomaccess Over VTM-10.0 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BDDT [%] | BDDE [%] | BDDT [%] | BDDE [%] | BDDT [%] | BDDE [%] |
| Class A1 | -92.64 | -72.27 | / | / | -91.77 | -70.15 |
| Class A2 | -92.50 | -71.97 | / | / | -92.15 | -70.51 |
| Class B | -92.63 | -71.94 | -91.92 | -71.25 | -91.52 | -69.09 |
| Class C | -90.40 | -65.55 | -89.89 | -66.06 | -88.82 | -61.67 |
| Class D | -85.57 | -51.90 | -78.53 | -33.63 | -82.03 | -42.20 |
| Class E | -91.14 | -67.27 | -86.60 | -55.99 | / | / |
| Class F | -91.24 | -67.87 | -86.02 | -55.98 | -88.04 | -61.39 |
| Mean | -90.80 | -66.75 | -86.86 | -57.35 | -88.91 | -62.11 |

# Evaluation of VVdeC and FFmpeg

In this section, one practical software decoder implementation of HEVC and VVC is evaluated. In general, the time and energy demand of VVdeC is higher than of openHEVC. The evaluation of both codecs is shown in Table 3.

Table 3: Evaluation of BDDE and BDDT for the comparison of VVdeC and openHEVC.

|  | All Intra VVdeC Over FFmpeg 4.3.1 |  |  |
| --- | --- | --- | --- |
|  | YUV-BDR [%] | BDDT [%] | BDDE [%] |
| Class A1 | -28.35 | 158.92 | 181.96 |
| Class A2 | -25.85 | 184.43 | 191.34 |
| Class B | -21.63 | 192.78 | 197.74 |
| Class C | -22.15 | 251.22 | 257.87 |
| Class D | -17.72 | 334.37 | 372.00 |
| Class E | -25.76 | 247.78 | 247.30 |
| Class F | -30.91 | 208.48 | 216.04 |
| Mean | -24.27 | 227.45 | 239.77 |

|  | Lowdelay B VVdeC Over FFmpeg 4.3.1 |  |  |
| --- | --- | --- | --- |
|  | YUV-BDR [%] | BDDT [%] | BDDE [%] |
| Class A1 | / | / | / |
| Class A2 | / | / | / |
| Class B | -30.99 | 32.08 | 152.61 |
| Class C | -28.64 | -3.82 | 168.21 |
| Class D | -25.51 | 3.03 | 431.45 |
| Class E | -33.73 | 84.74 | 249.94 |
| Class F | -38.78 | 63.04 | 206.16 |
| Mean | -31.39 | 33.18 | 238.81 |

|  | Randomaccess VVdeC Over FFmpeg 4.3.1 |  |  |
| --- | --- | --- | --- |
|  | YUV-BDR [%] | BDDT [%] | BDDE [%] |
| Class A1 | -38.42 | 77.74 | 158.70 |
| Class A2 | -41.91 | 76.86 | 166.14 |
| Class B | -34.92 | 82.50 | 170.57 |
| Class C | -29.82 | 102.68 | 246.80 |
| Class D | -27.47 | 137.94 | 358.67 |
| Class E | / | / | / |
| Class F | -35.24 | 96.79 | 225.63 |
| Mean | -34.16 | 96.78 | 223.99 |

# Conclusion

This contribution showed that the VTM decoder has a significant increase in decoding energy demand. Furthermore, it could be shown that the additional decoding energy demand depends on the class and coding configuration of the bit stream. For randomaccess, the decoding time is increased by 77.19% and the decoding energy by 88.29%. With VVdeC, the decoding energy and time demand is reduced significantly in relation to VTM. The comparison of openHEVC and VVdeC shows that VVC has a higher computational complexity than HEVC.

# References

[1] M. Kränzler, C. Herglotz, A. Kaup, “Decoding Energy Assessment of VTM-6.0,” JVET-P0084, Oct. 2019.

[2] M. Kränzler, C. Herglotz, A. Kaup, “A Comparative Analysis of the Time and Energy Demand of Versatile Video Coding and High Efficiency Video Coding Reference Decoders,” in Proc. International Workshop on Multimedia Signal Processing (MMSP), Tampere, Finland, Sep. 2020.

[3] Joint Collaborative Team on Video Coding (JCT-VC). HEVC test model reference software (HM-16.20). https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.20/, 2019. Accessed 2019-07.

[4] Fast Forwards MPEG (FFmpeg). Accessed 2020-08. [Online]. Available: http://ffmpeg.org/

[5] Joint Video Exploration Team (JVET). VVC test model reference software (VTM-10.0). https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-7.0, 2019. Accessed 2020-08.

[6] Fraunhofer. Versatile Video Decoder (VVdeC). https://github.com/fraunhoferhhi/vvdec. Accessed 2020-09.

[7] F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010-v1, Mar. 2019.

[8] C. Herglotz, A. Heindel, and A. Kaup. Decoding-Energy-Rate-Distortion Optimization for Video Coding. IEEE Transactions on Circuits and Systems for Video Technology, Vol. 29(1), 2019.

# Patent rights declaration

The Friedrich-Alexander-Universität Erlangen-Nürnberg (FAU) does not have any current or pending patent rights relating to the technology described in this contribution.