| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting: by teleconference, 7 – 16 October 2020 | Document: JVET-T0069-v2 |
| --- | --- |

| Title: | AHG 11: SSIM based CNN model for in-loop filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tong Ouyang Feiyang Liu Han Zhu Zhenzhong Chen Wuhan University Xiaozhong Xu Shan Liu Tencent | Tel: Email: | oyjiyu@whu.edu.cn liufeiyang@whu.edu.cn zhuhanlyx@whu.edu.cn zzchen@whu.edu.cn xiaozhongxu@tencent.com shanl@tencent.com |
| Source: | Wuhan University, Tencent |  |  |

_____________________________

- Abstract

This contribution provides a CNNLF (convolutional neural network based in-loop filter) for VVC which focus on improving the subjective quality of encoded videos. The performance is evaluated using VMAF [1] in the VTM-10.0. Simulation results report BD-rate savings for luma, and both chroma components compared with VTM-10.0 under AI, RA and LDB configuration.

Introduction

With VVC v1 being completed, Neural Network (NN) based coding tools proposed in JVET AHG9 and coding performance improvement was demonstrated during the study. Most the networks are trained for improving the objective quality, however, there are some doubts as to whether a result with higher objective quality corresponds to higher subjective quality for human visual system or not. Experimental results also indicate that a higher PSNR score does not always denotes better visual experience. Motivated by this, a network trained for improving the subjective quality is proposed. The performance on BD-rate savings are compared with the network trained for improving the objective quality.

To verify the effectiveness of the proposed scheme, the network is trained and tested under different subjective metrics, considering VMAF is better in subjective quality but difficult to use for training network, the proposed network is trained on SSIM while tested on VMAF.

Technical description

The proposed CNNLF is introduced into VTM as an additional filter between debloking filter (DF) and sample adaptive offset (SAO), as shown in Fig. 1. The reconstructed frames are firstly filtered by DF before processed by the proposed network, the SAO is employed to further reduce the artifacts in the outputs of network. In this contribution, only one model is trained for different QPs. During the process of in-loop filtering, the decision of whether to apply proposed filter or not is give based on the rate-distortion optimization (RDO) in block level. To signal the employ of the proposed filter on luma and both chroma component, additional flags are signaled in the bitstream respectively.

Fig. 1 Proposed scheme in VTM.

Residual Unit

The residual unit consist of one 1×1convolutional layer, two 3×3 convolutional layers and a rectified linear unit (ReLU) between them. Besides, there are two shortcuts and a squeeze-and-excitation block (SE block) [2] in the unit. The architecture of SE block is shown in Fig.1. The feature maps are squeezed into channel-wise statistics, which is produced by aggregating feature maps across their spatial dimensions (H×W). The aggregation is followed by an excitation operation, which takes the form of a simple self-gating mechanism, it takes the channel descriptor as input and produces a collection of per-channel modulation weights. These weights are applied to the feature maps U the generate the output of the SE block which can be fed directly into subsequent layers of the network. SE block adaptively recalibrates channel-wise feature responses by explicitly modeling interdependencies between channels, which benefit the model in learning the compressed information.

Fig. 2 The architecture of SE block.

The inner shortcut performs the residual learning by adding up the outputs of the  convolutional layer with the last convolution layer. The outer shortcut directly passes the original input to the next unit. Let  be the input of the unit, the output of the residual unit can be formulated as

Fx=H([x, fx+g(f(x))])	(1)

Where H(⋅) denotes the operation of concatenation, f(⋅)represents the  convolutional layer, g(⋅) represents the residual learning.

Fig. 3 The residual unit.

Network architecture

The whole architecture of proposed network is shown in Fig. 3. The network mainly consists of 8 residual units with some 3x3 convolutional layers. Apart from the last convolution layer that generates a 3-channel output, other convolutional layers generate the same amount of feature maps (denoted as M=64). Two SE blocks and a 3×3 convolution layer is appended in the last of the network to adaptively fuse the preceding feature maps and generate the final reconstructed images. Note that 1x1 convolutional layer is not employed in the first unit. The global identity skip connection further facilitates the flow of shallow features.

Fig. 4 The architecture of proposed network.

Training stage

The proposed network is trained with DIV2K dataset, there are 800 images in its training set and 100 images in its validation set. All the images are compressed under the common test conditions (AI configuration and QP= {22,27,32,37}) with VTM-10.0. More information related with the network architecture are reported in Table 1.

Table 1. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | CPU: Intel® Xeon® CPU E5-2637 v4 @ 3.50GHz x8, 64GB GPU: GTX 1080ti x8 x 11GB |
|  | SW environment: | OS: CentOS Linux Release 7.4 GPU: CUDA 9.0, cuDNN 7.1.4, nvidia driver 396.37 |
|  | Framework: | Pytorch v0.4 |
|  | Epoch: | 300 |
|  | Batch size: | 2Kx16 |
|  | Training time: | 57h |
|  | Training data information: | DIV2K (800 images for training and 100 for validation) |
|  | Configurations for generating compressed training data (if different to VTM CTC): | QP= {22,27,32,37}, AI. |
| Optional | Patch size | 128x128 |
|  | Learning rate: | 5e-4 |
|  | Optimizer: | ADAM |
|  | Loss function: | w1×SSIM+w2×L1 |
|  | Preprocessing: | Mean shift |
|  | Other information: | NO |

L1 loss and SSIM is adopted as the loss function with different weights (w1, w2), LoΘ attach more importance on the objective metrics e.g. PSNR, the performance on SSIM is prioritized during the training phase when the LsΘ is adopted.

|  | LoΘ=i=1n0.2×SSIMFXi;Θ,Yi+0.8×FXi;Θ-Yi1, | (1) |
| --- | --- | --- |
|  | LsΘ=i=1n0.8×SSIMFXi;Θ,Yi+0.2×FXi;Θ-Yi1, | (2) |

Where Θ denotes the parameters of proposed network, Yi indicates the ground truth of the compressed image patch Xi.

Inference stage

The proposed model is tested with 4 different QPs= {22, 27, 32, 37} under AI, RA and LDB configuration. Considering the computational complexity of neural networks, only the first Intra Period of RA configuration is tested for both anchor VTM-10.0 and NN based VTM.

Table 2. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | CPU: Intel® Xeon® CPU E5-2637 v4 @ 3.50GHz x8, 64GB GPU: NO |
|  | SW environment: | OS: CentOS Linux Release 7.4 GPU: NO |
|  | Framework: | Pytorch v0.4 |
|  | Total Conv. Layers | 45 |
|  | Total FC Layers | 0 |
|  | Total Parameter Number | 131299 |
|  | Parameter Precision | 32(F) |
|  | Memory Parameter (MB) | 0.50 |
|  | Memory Temp (MB) | 32399.36 (input size: 3840x2160) |
|  | MAC (Giga) | 988 Giga (input size: 3840x2160) |
| Optional | Preprocessing: | Mean shift |
|  | Other information: | NO |

Simulation results

The model is tested under the conditions of section 4, except from the traditional BD-rate savings, an additional score of VMAF is also reported using the latest VMAF Development Kit (VDK) [1]. Considering the computational complexity of encoding process and the lack of graphics cards, the YUV sequences are encoded on computers without GPUs, the bitstream is decoded with CPUs and GPUs respectively. In addition, the bitstream encoded without GPUs could be decoded normally on a computer with GPUs.

Loss=0.8×SSIM+0.2×L1

The experimental results of improving the subjective quality are reported in the flowing tables.

Table 3 Experimental results (All Intra)

|  | All Intra Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | VMAF | Y | U | V | EncT-CPU | DecT-CPU | DecT-GPU |
| Class A1 | -5.84% | -1.39% | -3.72% | -4.11% | 170% | 41444% | 3363% |
| Class A2 | -7.22% | -1.56% | -7.54% | -8.19% | 137% | 21508% | 2276% |
| Class B | -6.51% | -0.67% | -7.97% | -6.85% | 131% | 21345% | 2043% |
| Class C | -8.11% | -2.03% | -8.53% | -7.34% | 117% | 21533% | 1852% |
| Class E | -10.29% | -2.86% | -4.65% | -5.29% | 139% | 37679% | 2804% |
| Overall | -7.50% | -1.61% | -6.76% | -6.46% | 136% | 26294% | 2331% |
| Class D | -9.26% | -2.70% | -7.64% | -8.16% | 115% | 19974% | 1984% |
| Class F | -3.08% | -0.34% | -4.70% | -3.50% | 115% | 20602% | 1826% |

Table 4 Experimental results (Random Access)

|  | Random access Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | VMAF | Y | U | V | EncT-CPU | DecT-CPU | DecT-GPU |
| Class A1 | -4.66% | -0.67% | 3.94% | 4.45% | 145% | 51787% | 4174% |
| Class A2 | -7.14% | -1.06% | -8.02% | -9.09% |  | 27635% | 2639% |
| Class B | -6.02% | -0.47% | -7.98% | -5.34% | 141% | 12545% | 1132% |
| Class C | -6.17% | -0.60% | -5.86% | -4.44% | 125% | 16030% | 1450% |
| Class E |  |  |  |  |  |  |  |
| Overall | -6.01% | -0.66% | -5.04% | -3.89% |  | 20826% | 1859% |
| Class D | -6.58% | -0.51% | -4.58% | -4.66% | 124% | 29333% | 2407% |
| Class F | -1.57% | 0.39% | -2.15% | -1.34% | 161% | 40522% | 3314% |

Table 5 Experimental results (Low Delay B)

|  | Low delay B Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | VMAF | Y | U | V | EncT-CPU | DecT-CPU | DecT-GPU |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -2.11% | 0.14% | -8.86% | -6.42% | 137% | 14937% | 1307% |
| Class C | -3.07% | 0.55% | -8.36% | -6.76% | 126% | 24803% | 2043% |
| Class E | -1.59% | -0.16% | -3.85% | -3.07% | 181% | 24373% | 2228% |
| Overall | -2.30% | 0.20% | -7.44% | -5.69% | 143% | 19991% | 1733% |
| Class D | -1.82% | 0.49% | -6.83% | -6.70% | 123% | 29599% | 2480% |
| Class F | -0.15% | 1.52% | -3.02% | -1.66% | 173% | 66822% | 4844% |

Loss=0.2×SSIM+0.8×L1

To prove the point that better objective quality doesn’t correspond to better subjective quality, a model trained with a loss function that puts more weight on L1 loss is tested under the same experimental conditions. The experimental results of improving the objective metrics are reported in the flowing tables.

Table 6 Experimental results (All Intra)

|  | All Intra Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | VMAF | Y | U | V | EncT-CPU | DecT-CPU | DecT-GPU |
| Class A1 | -4.68% | -1.79% | -5.88% | -6.16% | 156% | 22366% | 2423% |
| Class A2 | -4.41% | -1.94% | -8.81% | -7.77% | 130% | 33088% | 2789% |
| Class B | -3.96% | -1.48% | -8.85% | -8.31% | 124% | 23956% | 2163% |
| Class C | -4.48% | -2.40% | -8.47% | -8.37% | 115% | 20816% | 1829% |
| Class E | -6.68% | -3.28% | -5.47% | -7.27% | 130% | 34118% | 2790% |
| Overall | -4.72% | -2.11% | -7.70% | -7.70% | 129% | 25695% | 2312% |
| Class D | -6.69% | -3.14% | -7.24% | -9.38% | 112% | 19564% | 1939% |
| Class F | -1.82% | -0.78% | -4.59% | -3.92% | 114% | 9311% | 1190% |

Table 7 Experimental results (Random Access)

|  | Random access Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | VMAF | Y | U | V | EncT-CPU | DecT-CPU | DecT-GPU |
| Class A1 | -4.89% | -2.40% | -7.31% | -6.02% |  | 38157% | 3240% |
| Class A2 | -4.04% | -2.29% | -11.11% | -9.08% | 131% | 25447% | 2202% |
| Class B | -3.54% | -1.53% | -10.26% | -8.49% | 128% | 27498% | 2425% |
| Class C | -2.97% | -1.49% | -7.12% | -6.61% | 120% | 22800% | 1961% |
| Class E |  |  |  |  |  |  |  |
| Overall | -3.76% | -1.84% | -9.00% | -7.61% |  | 27499% | 2382% |
| Class D | -3.96% | -1.73% | -6.69% | -7.44% | 120% | 13263% | 1292% |
| Class F | -0.76% | -0.37% | -2.79% | -2.47% | 151% | 5186% | 886% |

Table 8 Experimental results (Low Delay B)

|  | Low delay B Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | VMAF | Y | U | V | EncT-CPU | DecT-CPU | DecT-GPU |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -2.61% | -1.03% | -12.13% | -9.81% | 127% | 32380% | 2847% |
| Class C | -2.43% | -0.91% | -10.84% | -9.41% | 120% | 33890% | 2804% |
| Class E | -2.56% | -2.55% | -5.33% | -7.24% | 171% | 18160% | 1616% |
| Overall | -2.54% | -1.37% | -10.00% | -9.04% | 134% | 28450% | 2459% |
| Class D | -3.09% | -1.17% | -10.34% | -10.32% | 118% | 20621% | 1812% |
| Class F | -0.77% | -0.31% | -4.95% | -3.77% | 154% | 11838% | 1326% |

The superiority of the subjective loss function is shown in the BD-Rate saving under the configuration of All Intra. It must be announced that the minor gain under the inter configuration is greatly contributed by the training process which adopts the images rather than the videos as training datasets.

Conclusion

This contribution provides a neural network based in-loop filter to improve the subjective quality for VTM-10.0. Simulation results report -7.50%, -6.01%, -2.30% BD-rate savings on VMAF under AI, RA and LDB configuration.

References

- Vmaf, https://github.com/Netflix/vmaf.

- J. Hu, L. Shen and G. Sun, “Squeeze-and-Excitation Networks,” 2018 IEEE/CVF Conference on Computer Vision and Pattern Recognition, Salt Lake City, UT, 2018, pp. 7132-7141.

- JVET-K0391, “AHG9: Dense Residual Convolutional Neural Network based In-Loop Filter”, Y. Wang, Z. Chen, Y. Li (Wuhan Univ.), L. Zhao (Tencent), Ljubljana, SI, 10–18 July 2018

- JVET-L0242, “AHG9: Dense Residual Convolutional Neural Network based In-Loop Filter”, Y. Wang, Z. Chen, Y. Li (Wuhan Univ.), L. Zhao (Tencent), Macao, CN, 03–12 Oct. 2018

- JVET-M0508, “AHG9: Test Results of Dense Residual Convolutional Neural Network based In-Loop Filter”, Y. Wang, Z. Chen, Y. Li (Wuhan Univ.), L. Zhao, S. Liu, X. Li (Tencent), Marrakech, MA, 9–18 Jan. 2019

- JVET-N0254, “CE13: Dense Residual Convolutional Neural Network based In-Loop Filter (Test 2.2 and 2.3)”, Y. Wang, Z. Chen, Y. Li, L. Zhao, S. Liu, X. Li, Geneva CH, 19–27 March 2019

- JVET-O0101, “CE10: Dense Residual Convolutional Neural Network based In-Loop Filter (Tests 2.5 and 2.7)”, Y. Wang, T. Ouyang, C. Zou, Y. Li, Z. Chen (Wuhan Univ.), L. Zhao, S. Liu, X. Li (Tencent), Gothenburg, SE, 3–12 July 2019

- JVET-T0041, “Methodology and reporting template for neural network coding tool testing”, S. Liu, A. Segall, E. Alshina, J. Boyce, M. Wien, D. Grois, teleconference, 7-16 Oct. 2020

Patent rights declaration(s)

Tencent does not have any current or pending patent rights relating to the technology described in this contribution.

Wuhan University does not have any current or pending patent rights relating to the technology described in this contribution.