| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0071-v1 |
| --- | --- |

| Title: | AHG9: SEI messages for support of cross RAP referencing based video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | +1 (908) 803-3888 yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following two aspects for VVC/VSEI:

- The DRAP indication SEI message is extended by adding a random access point (RAP) picture ID, where a RAP picture is either an IRAP picture or a DRAP picture.

- A new SEI message, named type 2 DRAP indication SEI message, is added, to indicates type 2 DRAP pictures and provide RAP picture IDs of the type 2 DRAP picture itself and the list of RAP pictures it depends on, where a type 2 DRAP picture is a picture that may use the associated IRAP picture and a few other (type 1 or type 2) DRAP pictures for inter prediction reference.

It is asserted that the proposed changes enable full support of the so-called cross RAP referencing based video coding approach.

# Introduction

Video coding approaches based on cross RAP referencing (CRR), also referred to as external decoding refresh (EDR), was proposed in JVET-M0360, JVET-N0119, JVET-O0149, and JVET-P0114.

The basic idea of such a video coding approach is as follows. Instead of coding random access points (RAPs) as intra-coded IRAP pictures (except for the very first picture in the bitstream), they are coded using inter prediction, to circumvent the unavailability of the earlier pictures if the RAPs are coded as IRAP pictures. The trick is to provide a limited number of the earlier pictures, typically representing different scenes of the video content, through a separate video bitstream, which can be referred to as an external means. Such earlier pictures are referred to as the external pictures. Consequently, each external picture can be used for inter prediction referencing by pictures across the random access points. The coding efficiency gain comes from having random access points coded as inter-predicted pictures and having more available reference pictures for pictures that follow EDR pictures in decoding order.

Such EDR picture are DRAP pictures per the proposals in this contribution.

As described in JVET-P0114, a bitstream coded with such a video coding approach can be used in applications based on ISOBMFF and DASH as described below (with the term EDR picture replaced with DRAP picture).

DASH content preparation operations

- A video content is encoded into one or more representations, each is of a particular spatial resolution, temporal resolution, and quality.

- Each particular representation of a video content is represented by a main stream and an external stream. The main stream contains coded pictures that include DRAP pictures and non-DRAP pictures. The external stream contains only RAP (IRAP or DRAP) pictures.

- Each main stream is carried in a Main Stream Representation (MSR). Each RAP picture in an MSR is the first picture of a Segment.

- Each external stream is carried in an External Stream Representation (ESR).

- For each Segment in an MSR starting with a DRAP picture, there is a Segment in the corresponding ESR having the same Segment start time derived from the MPD, carrying the external pictures needed for decoding of that DRAP picture and the subsequent pictures in decoding order in the main stream carried in the MSR.

- The MSRs of the same video content are included in one Adaptation Set (AS). The ESRs of the same video content are included in one AS.

DASH streaming operations

- A client gets the MPD of the DASH Media Presentation, parses the MPD, selects an MSR, and determines the starting presentation time from which the content is to be consumed.

- The client requests Segments of the MSR, starting from the Segment containing the picture having presentation time equal to (or close enough to) the starting presentation time.

  - If the first picture in the starting Segment is a DRAP picture, the corresponding Segment (having the same Segment start time derived from the MPD) in the associated ESR is also requested, preferably before requesting of the MSR Segments. Otherwise, no Segment of the associated ESR is requested.

- When switching to a different MSR, the client requests Segments of the switch-to MSR, starting from the first Segment having Segment start time greater than that of the last requested Segment of the switch-from MSR.

  - If the first picture in the starting Segment in the switch-to MSR is a DRAP picture, the corresponding Segment in the associated ESR is also requested, preferably before requesting of the MSR Segments. Otherwise, no Segment of the associated ESR is requested.

- When continuously operating at the same MSR (after decoding of the starting Segment after a seeking or stream switching operation), no Segment of the associated ESR needs to be requested, including when requesting any Segment starting with a DRAP picture.

The functionality of the DRAP indication SEI message can be considered as a subset of the CRR approach. For simplicity, a picture associated with a DRAP indication SEI message is referred to as type 1 DRAP picture.

From encoding point of view, although the CRR approach proposed in JVET-P0114 or earlier JVET contributions was not adopted into VVC, the encoder can still encoding the video bitstream in such a manner that certain pictures only rely on the associated IRAP picture for inter prediction reference (like type 1 DRAP pictures indicated by the DRAP SEI message), and certain other pictures (e.g., referred to as type 2 DRAP pictures) only rely on some pictures in a set of pictures consisting of the associated IRAP picture and some (type 1 or type 2) DRAP pictures earlier in decoding order.

However, given a VVC bitstream, it is unknown whether such type 2 DRAP pictures exist in the bitstream. Furthermore, even when it is known that such type 2 DRAP pictures exist in the bitstream, to compose a media file according to the an ISOBMFF and a DASH media presentation based on such a VVC bitstream to enable the CRR or EDR streaming operations, the file and DASH media presentation composer would need to parse and derive lots of information, including POC values and active entries in reference picture lists to figure out whether a particular picture is a type 2 DRAP picture, and, and if yes, which earlier IRAP or DRAP pictures are needed for random accessing from this particular picture, such that the proper set of pictures can be included in a separate, time-synchronized file track and DASH representation.

Therefore, to fully support CRR for VVC bitstreams, the following two changes are proposed:

- The DRAP indication SEI message is extended by adding a RAP picture ID.

- A new SEI message, named type 2 DRAP indication SEI message, is added, to indicates type 2 DRAP pictures and provide RAP picture IDs of the type 2 DRAP picture itself and the list of RAP pictures it depends on.

# Proposal

## An extension to the DRAP indication SEI message

The syntax and semantics of the DRAP indication SEI message are proposed to be changed as follows (aligned with the first option of the proposed fix for SEI message extension syntax in JVET-T0048):

### SEI payload syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| ExtensionBitsPresentFlag = 0 |  |
| ... |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| if( ExtensionBitsPresentFlag ¦ ¦ more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| reserved_payload_extension_data | u(v) |
| payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

### Dependent random access point indication SEI message syntax

| dependent_rap_indication( payloadSize ) { | Descriptor |
| --- | --- |
| if( more_data_in_payload( ) ) |  |
| if( payload_extension_present( ) ) { |  |
| ExtensionBitsPresentFlag = 1 |  |
| drap_rap_id_in_clvs | u(16) |
| } |  |
| } |  |

### Dependent random access point indication SEI message semantics

The picture associated with a dependent random access point (DRAP) indication SEI message is referred to as a type 1 DRAP picture.

Type 1 DRAP pictures and type 2 DRAP pictures (associated with a type 2 DRAP indication SEI message) are collectively referred to as DRAP pictures.

The presence of the DRAP indication SEI message indicates that the constraints on picture order and picture referencing specified in this subclause apply. These constraints can enable a decoder to properly decode the type 1 DRAP picture and the pictures that are in the same layer and follow it in both decoding order and output order without needing to decode any other pictures in the same layer except the associated IRAP picture of the type 1 DRAP picture.

The constraints indicated by the presence of the DRAP indication SEI message, which shall all apply, are as follows:

–	The type 1 DRAP picture is a trailing picture.

–	The type 1 DRAP picture has a temporal sublayer identifier equal to 0.

–	The type 1 DRAP picture does not include any pictures in the same layer in the active entries of its reference picture lists except the associated IRAP picture of the type 1 DRAP picture.

–	Any picture that is in the same layer and follows the type 1 DRAP picture in both decoding order and output order does not include, in the active entries of its reference picture lists, any picture that is in the same layer and precedes the type 1 DRAP picture in decoding order or output order, with the exception of the associated IRAP picture of the type 1 DRAP picture.

drap_rap_id_in_clvs specifies the RAP picture ID, denoted as RapPicId, of the type 1 DRAP picture.

Each IRAP or DRAP picture that is an IRAP or DRAP is associated with a RapPicId. The value of RapPicId for an IRAP picture is inferred to be equal to 0. The values of RapPicId shall be different for any two IRAP or DRAP pictures within a CLVS.

## A new SEI message, type 2 DRAP indication SEI message

The syntax and semantics of proposed type 2 DRAP indication SEI message are as follows:

### Type 2 DRAP indication SEI message syntax

| type2_drap_indication( payloadSize ) { | Descriptor |
| --- | --- |
| t2drap_rap_id_in_clvs | u(16) |
| t2drap_reserved_zero_13bits | u(13) |
| t2drap_num_ref_rap_pics_minus1 | u(3) |
| for( i = 0; i <= t2drap_num_ref_rap_pics_minus1; i++ ) |  |
| t2drap_ref_rap_id[ i ] | u(16) |
| } |  |

### Type 2 DRAP indication SEI message semantics

The picture associated with a type 2 DRAP indication SEI message is referred to as a type 2 DRAP picture.

Type 1 DRAP pictures (associated with a DRAP indication SEI message) and type 2 DRAP pictures are collectively referred to as DRAP pictures.

The presence of the type 2 DRAP indication SEI message indicates that the constraints on picture order and picture referencing specified in this subclause apply. These constraints can enable a decoder to properly decode the type 2 DRAP picture and the pictures that are in the same layer and follow it in both decoding order and output order without needing to decode any other pictures in the same layer except the list of pictures referenceablePictures, which consists of the list of IRAP or DRAP pictures in decoding order that are within the same CLVS and identified by the t2drap_ref_rap_id[ i ] syntax elements.

The constraints indicated by the presence of the type 2 DRAP indication SEI message, which shall all apply, are as follows:

–	The type 2 DRAP picture is a trailing picture.

–	The type 2 DRAP picture has a temporal sublayer identifier equal to 0.

–	The type 2 DRAP picture does not include any pictures in the same layer in the active entries of its reference picture lists except the referenceablePictures.

–	Any picture that is in the same layer and follows the type 2 DRAP picture in both decoding order and output order does not include, in the active entries of its reference picture lists, any picture that is in the same layer and precedes the type 2 DRAP picture in decoding order or output order, with the exception of the referenceablePictures.

–	Any picture in the list referenceablePictures does not include, in the active entries of its reference picture lists, any picture that is in the same layer and is not a picture at an earlier position in the list referenceablePictures.

NOTE – Consequenlty, the first picture in referenceablePictures, even when it is a DRAP picture instead of an IRAP picture, does not include any picture from the same layer in the active entries of its reference picture lists.

t2drap_rap_id_in_clvs specifies the RAP picture identifier, denoted as RapPicId, of the type 2 DRAP picture.

Each IRAP or DRAP picture that is an IRAP or DRAP is associated with a RapPicId. The value of RapPicId for an IRAP picture is inferred to be equal to 0. The values of RapPicId shall be different for any two IRAP or DRAP pictures within a CLVS.

t2drap_reserved_zero_13bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for t2drap_reserved_zero_13bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of t2drap_reserved_zero_13bits.

t2drap_num_ref_rap_pics_minus1 plus 1 indicates the number of IRAP or DRAP pictures that are within the same CLVS as the type 2 DRAP picture and may be included in the active entries of the reference picture lists of the type 2 DRAP picture.

t2drap_ref_rap_id[ i ] indicates RapPicId of the i-th IRAP or DRAP picture that is within the same CLVS as the type 2 DRAP picture and may be included in the active entries of the reference picture lists of the type 2 DRAP picture.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).