| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0072 |
| --- | --- |

| Title: | AHG12: Rice parameter selection for high bit depths |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adrian Browne Steve Keating Karl Sharman | Email: | adrian.browne@sony.com steve.keating@sony.com karl.sharman@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This proposal describes a modification to the method for the selection of Rice parameters for residual coding, for consideration in VVC version 2.  The aim of the modification is to offer BD-rate gains when residual coefficient values become larger, e.g. for high bit depths and low QPs, whilst offering negligible losses at VVC version 1 operating points.  It is suggested that the modification extends the operating point of the existing residual coding method to be applicable at all currently considered bit depths. Results show all intra gains of -26.09%/-21.90%/-20.57% for the proposed 16 bit CTC[1], -1.67%/-1.55%/-1.57% for the proposed 12 bit CTC[1], -0.14%/-0.19%/-0.22% for low QP[2] and 0.02%/-0.02%/0.01% for CTC[2]. A simplification was developed which reduces the coupling between the use and update of computed values within the modification, whilst aiming to maintain the efficiency gains. Results for the simplification are -26.60%/-22.07%/-20.74% for 16 bit, -1.50%/-1.37%/-1.38% for 12 bit and -0.03%/-0.08%/-0.11% for low QP.

# Introduction

In VVC version 1[3], the range of Rice parameters that may be used for coefficients during regular residual coding is limited to the range of 0 to 3 (inclusive). This range has been found to be suitable for both 8 and 10 bit data; however, as bit depths increase and QPs become lower, the size of each coded coefficient becomes larger. For high bit depth content at QPs below 0 it has previously been shown that VVC coding efficiency is below that of HEVC[4].

Previous work[5] has described approaches that increase the coding efficiency for high bit depths and low QPs to match or exceed HEVC.  This proposal takes one of the approaches, “Adding a Rice-based shift to existing VVC” and improves the implementation. This results in gains for 12 and 16-bit content without adversely affecting the operation at lower bit depths and higher QPs.

In addition to the basic implementation a further simplification to the technique is described which attempts to mitigate a potential latency in the original proposal.

# Algorithm description

The algorithm impacts entropy coding and in particular the selection of Rice parameters for the Golomb-Rice coding of residual coefficients. The algorithm can be considered as having four components as follows:

- A coefficient size estimation function which is backed by a series of counters and an index function to select which counter is used for each coefficient.

- An update function for the selected counter applied after each coefficient.

- A modification to the current algorithm for computing the clipped sum of adjacent nearby coefficients.

- A modification to the current algorithm of selecting Rice parameters.

Version 2 of this document added the draft text section.

## Coefficient size estimation

The technique requires an estimation of the log2 size of coefficients being coded. This is derived in a similar manner to Rice parameter estimation in HEVC (RExt) using a series of counters. For this proposal the counters are mapped to a two dimensional array, with the first dimension being the component, and the second being an index function coded as shown in Table 1 giving a total of 12 counters. This is a simplification of the technique described in [4] which used more counters and required the computation of the Manhattan distance from the top left of the transform block for each coefficient.

| Index | Description | 16x16 transform block |
| --- | --- | --- |
| 0 | Transform-skip |  |
| 1 (Red) | DC component of top left sub-TB |  |
| 2 (Blue) | Remainder of top left sub-TB |  |
| 3 (Yellow) | All other sub-TBs in transform unit |  |

Table 1. Index function derivation

## Counter update function

Each counter is set to 0 at the beginning of every slice.  For wavefront parallel coding each wavefront copies the counters before starting, treating them in a similar manner to CABAC contexts.

For each non-zero coefficient coded the counter is updated as follows:

if (abs(value) > 0) {

int idx = calcIndex(isTS, scanPos);    			// index function as defined in Table 1

int &count = riceCount[component][idx];		// riceCount is the array of counters, reset to 0 for each slice

int rice = count >> log2Divisor;				// log2Divisor = 4

if (abs(value) >= (3 << rice)) {      			    	// value larger than expected range

count += increment;    					// increment = 4

} else if ((abs(value) << 1) < (1 << rice)) {    	// value smaller than expected range

count = max(0, count - decrement);			// decrement = 1, clipped to be non-negative

}

}

Thus the value of rice provides an estimate of the log2 size of the average non-zero coefficient.  Mismatched increments and decrements have been found to be beneficial as slightly over-estimating the Rice parameter seems to offer the best gains.

## Modification to clipped sum of coefficients

For regular residual coding (RRC), VVC (section 9.3.3.2) defines locSumAbs as the sum of the absolute values of 5 previously coded coefficients below and right of the coefficient, less an offset which relates to the proportion of the coefficient that is coded with flags, and then clips this value to the range 0 to 31. This technique works well if clipping is only occasionally used, but as average coefficient values increase the number of times clipping is required increases.

In the modification described here, a shift right is introduced as below (modification in green).  The purpose of this shift is to reduce the magnitude of the sum and thus reduce the amount of clipping. The size of the shift is described in the following section.

locSumAbs = Clip3( 0, 31, ( locSumAbs − baseLevel * 5 ) >> shift )

For transform skip residual coding (TSRC) VTM currently does not use a clipped sum. The modification introduces a clipped sum for TSRC which is identical to that described for RRC but uses 5 coefficients above and left of the coefficient being coded.

## Rice parameter selection

In VTM, RRC selects the Rice parameter by using the value returned by the template function as an index into a lookup table which gives the Rice parameter.

In the proposed modification the selection of the Rice parameter can be described using the pseudo code below (modification is indicated in green)

int idx = calcIndex(isTS, scanPos); 				 // index function as defined in Table 1

int count = riceCount[component][idx];  			// select size estimation counters

int shift = std::max(0, (count >> log2Divisor) - 2); 	// compute template shift modifier

int idx = cctx.templateAbsSum(scanPos, absCoeff, baseLevel, shift);

// compute template fn using sum shifted right by shift modifier

int rice = g_auiGoRiceParsCoeff[idx] + shift;		// add shift modifier onto result from lookup table

Thus the selection of the Rice parameter may be seen as a modified version of the existing VVC technique rather than a new technique.  Experimentally for the 10 bit CTC the value of the shift modifier is rarely non-zero and thus the resultant coding is almost identical to VVC. But as average coefficient values increase and the shift modifier becomes greater than zero, the resultant Rice parameter is offset as below.

| Estimated value from counter 1 << (count >> log2Divisor) | Shift modifier | Range of possible Rice values |
| --- | --- | --- |
| 1 | 0 | 0..3 |
| 2 | 0 | 0..3 |
| 4 | 0 | 0..3 |
| 8 | 1 | 1..4 |
| 16 | 2 | 2..5 |

Table 2. Output Rice values for a range of estimated coefficient values

Thus when estimated coefficient values are larger this modification ensures a more appropriate range of Rice parameters is selectable. Unlike simply swapping to a technique based purely on size estimation, this modification introduces a hybrid technique which attempts to extend the operating point of the existing VVC technique.

# Simplification

One potential issue with the modification is that there is a close coupling between the update and use of the size estimation counters. This also prevents the caching/pre-calculation and reuse of derived values. This section describes a simplification which attempts to resolve this issue.

## Coefficient size estimation

The proposed simplification is to take a copy of the riceCount array at the beginning of coding each sub-TB as follows:

riceCopy = riceCount;

This copy could be further reduced to the copying of a maximum of two counters rather than the complete set of 12 if the proposed index function described in section 2.1 is used.

## Counter update function

The update function becomes:

if (abs(value) > 0) {

int idx = calcIndex(isTS, scanPos);

int &count = riceCount[component][idx];		// riceCount must be updated

int rice = riceCopy[component][idx];			// rice value for estimation uses copy

if (abs(value) >= (3 << rice)) {      			    	// use copied value

count += increment;    					// update original counter

} else if ((abs(value) << 1) < (1 << rice)) {    		// use copied value

count = max(0, count - decrement);			// update original counter

}

}

The update function could be further improved by caching/pre-calculating the values (3 << rice) and (1 << rice) and the reference count.

## Rice parameter selection

The Rice parameter selection function becomes:

int idx = calcIndex(isTS, scanPos);

int count = riceCopy[component][idx];  			// use copied value

int shift = std::max(0, (count >> log2Divisor) - 2);

int idx = cctx.templateAbsSum(scanPos, absCoeff, baseLevel, shift);

int rice = g_auiGoRiceParsCoeff[idx] + shift;

This function could be further improved by pre-computing the two possible values of shift per sub-TB.  It would also be possible to implement a version of the simplification which only uses the copied values for the rice parameter selection and not the counter update function.

# Results

## Without simplification

### 16 bit

|  |  |  | AI-16 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-10.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| SVT | -26.68% | -22.47% | -20.91% | 67% | 70% |
| Overall | -26.68% | -22.47% | -20.91% | 67% | 70% |

As QPs reduce the gains increase from -16.20%/-16.50%/-16.69% for QPs -8 to -23 to -65.26%/-60.83%/-63.82% for QPs -18 to -33.

The encoder/decoder timings were caused by a slow anchor due to processor loading and should be ignored

### 12 bit

|  |  |  | AI-12 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| SVT | -3.90% | -3.60% | -3.59% | 91% | 83% |
| PQ444 | -0.51% | -0.48% | -0.50% | 100% | 79% |
| PQ422 | -0.60% | -0.58% | -0.61% | 100% | 79% |
| Overall | -1.67% | -1.55% | -1.57% | 95% | 77% |

### 10 bit CTC

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM10.0 |  |  |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.02% | -0.06% | 0.03% | 0.02% | 100% | 100% |
| Class A2 | 0.02% | 0.00% | -0.02% | 0.01% | 98% | 99% |
| Class B | 0.02% | 0.03% | 0.01% | 0.02% | 100% | 100% |
| Class C | -0.01% | 0.00% | -0.06% | -0.02% | 100% | 100% |
| Class E | 0.05% | -0.08% | 0.11% | 0.04% | 100% | 99% |
| Overall | 0.02% | -0.02% | 0.01% | 0.01% | 100% | 100% |
| Class D | 0.00% | -0.04% | 0.13% | 0.01% | 101% | 100% |
| Class F | -0.04% | -0.07% | -0.09% | -0.04% | 99% | 99% |

### 10 bit low QP

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM10.0 LOWQP (JVET-N1010) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.04% | -0.03% | 105% | 107% |
| Class A2 | -0.07% | -0.12% | -0.12% | 106% | 108% |
| Class B | -0.12% | -0.16% | -0.20% | 105% | 102% |
| Class C | -0.24% | -0.20% | -0.24% | 107% | 100% |
| Class E | -0.19% | -0.46% | -0.52% | 104% | 105% |
| Overall | -0.14% | -0.19% | -0.22% | 105% | 104% |
| Class D | -0.30% | -0.36% | -0.38% | 107% | 103% |
| Class F | -1.55% | -1.29% | -1.35% | 104% | 101% |

### 10 bit lossless

| YUV 420 | All Intra |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-10.0 | Tested |  |
| Class A1 | 2.4 | 2.4 | -0.61% |
| Class A2 | 1.8 | 1.8 | -1.47% |
| Class B | 2.3 | 2.3 | -0.22% |
| Class C | 2.1 | 2.1 | -0.13% |
| Class D | 2.1 | 2.1 | -0.22% |
| Class E | 3.1 | 3.1 | 0.06% |
| Class F | 5.8 | 5.8 | -0.57% |
| TGM | 12.4 | 13.0 | -4.65% |
| Overall | 2.3 | 2.3 | -0.43% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 112% |  |  |

| RGB | All Intra |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-10.0 | Tested |  |
| TGM 1080p | 21.7 | 22.4 | -2.47% |
| TGM 720p | 10.1 | 10.1 | -0.26% |
| A | 6.0 | 6.0 | 0.05% |
| M | 6.0 | 6.0 | -0.57% |
| CC | 1.7 | 1.8 | -1.59% |
| Overall | 10.4 | 10.6 | -0.98% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 110% |  |  |

| YUV | All Intra |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-10.0 | Tested |  |
| TGM 1080p | 21.3 | 21.6 | -1.32% |
| TGM 720p | 12.7 | 12.8 | -0.31% |
| A | 7.1 | 7.1 | -0.03% |
| M | 7.8 | 7.8 | -0.77% |
| CC | 2.0 | 2.0 | -0.85% |
| Overall | 11.6 | 11.6 | -0.66% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 108% |  |  |

## With simplification

Please note that the encoder and decoder timings quoted here are for debug/experimental code without the caching/pre-calculation improvements suggested in section 3.  It is believed that by implementing these improvements the increase in encoder and decoder times will be much reduced.

### 16 bit

|  |  |  | AI-16 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-10.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| SVT | -26.60% | -22.07% | -20.74% | 67% | 74% |
| Overall | -26.60% | -22.07% | -20.74% | 67% | 74% |

### 12 bit

|  |  |  | AI-12 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| SVT | -3.72% | -3.37% | -3.37% | 89% | 84% |
| PQ444 | -0.36% | -0.33% | -0.35% | 99% | 79% |
| PQ422 | -0.42% | -0.41% | -0.43% | 99% | 79% |
| Overall | -1.50% | -1.37% | -1.38% | 96% | 81% |

### 10 bit low QP

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.01% | -0.01% | 105% | 108% |
| Class A2 | 0.04% | -0.02% | -0.01% | 107% | 109% |
| Class B | -0.03% | -0.06% | -0.12% | 106% | 103% |
| Class C | -0.05% | 0.01% | -0.02% | 106% | 101% |
| Class E | -0.09% | -0.37% | -0.42% | 104% | 105% |
| Overall | -0.03% | -0.08% | -0.11% | 106% | 105% |
| Class D | -0.06% | -0.09% | -0.08% | 107% | 104% |
| Class F | -1.44% | -1.17% | -1.24% | 104% | 102% |

### 10 bit lossless

| YUV 420 | All Intra |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-10.0 | Tested |  |
| Class A1 | 2.4 | 2.4 | -0.39% |
| Class A2 | 1.8 | 1.8 | -1.17% |
| Class B | 2.3 | 2.3 | 0.08% |
| Class C | 2.1 | 2.1 | 0.22% |
| Class D | 2.1 | 2.1 | 0.17% |
| Class E | 3.1 | 3.1 | 0.27% |
| Class F | 5.8 | 5.8 | -0.32% |
| TGM | 12.4 | 13.0 | -4.42% |
| Overall | 2.3 | 2.3 | -0.14% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 111% |  |  |

| RGB | All Intra |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-10.0 | Tested |  |
| TGM 1080p | 21.7 | 22.3 | -2.29% |
| TGM 720p | 10.1 | 10.1 | 0.08% |
| A | 6.0 | 6.0 | 0.29% |
| M | 6.0 | 6.0 | -0.25% |
| CC | 1.7 | 1.7 | -1.22% |
| Overall | 10.4 | 10.6 | -0.70% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 109% |  |  |

| YUV | All Intra |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-10.0 | Tested |  |
| TGM 1080p | 21.3 | 21.6 | -1.19% |
| TGM 720p | 12.7 | 12.7 | -0.14% |
| A | 7.1 | 7.1 | 0.15% |
| M | 7.8 | 7.8 | -0.60% |
| CC | 2.0 | 2.0 | -0.56% |
| Overall | 11.6 | 11.6 | -0.49% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 108% |  |  |

# Draft Text

#### 7.3.11.11	Residual coding syntax

| … |  |
| --- | --- |
| for( n = firstPosMode0; n > firstPosMode1; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( abs_level_gtx_flag[ n ][ 1 ] ) |  |
| abs_remainder[ n ] | ae(v) |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] +2 * abs_remainder[ n ] |  |
| if( AbsLevel[ xC ][ yC ] > 0 ) { |  |
| sbType = 4 * cIdx + 3 |  |
| if( xC < 4 && yC < 4 ) { |  |
| sbType-- |  |
| if( xC = 0 && yC = 0 ) |  |
| sbType-- |  |
| } |  |
| if( AbsLevel[xC][yC] >= ( 3 << ( StatCoeff[ sbType ] >> 4 ) ) ) |  |
| StatCoeff[ sbType ]+=4 |  |
| else if( 2 * AbsLevel[xC][yC] < ( 1 << ( StatCoeff[ sbType ] >> 4 ) ) ) { |  |
| if( StatCoeff[ sbType ] > 0 ) |  |
| StatCoeff[ sbType ]− − |  |
| } |  |
| } |  |
| } |  |

…

| residual_ts_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| … |  |  |
| for( n = 0; n <= numSbCoeff − 1; n++ ) { |  |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |  |
| if( ( n <= lastScanPosPass2 && AbsLevelPass2[ xC ][ yC ] >= 10 ) ¦ ¦ ( n > lastScanPosPass2 && n <= lastScanPosPass1 && AbsLevelPass1[ xC ][ yC ] >= 2 ) ¦ ¦ ( n > lastScanPosPass1 && sb_coded_flag[ xS ][ yS ] ) ) |  |  |
| abs_remainder[ n ] | ae(v) |  |
| if( n <= lastScanPosPass2 ) |  |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass2[ xC ][ yC ] + 2 * abs_remainder[ n ] |  |  |
| else if(n <= lastScanPosPass1 ) |  |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] + 2 * abs_remainder[ n ] |  |  |
| else { /* bypass */ |  |  |
| AbsLevel[ xC ][ yC ] = abs_remainder[ n ] |  |  |
| if( abs_remainder[ n ] ) |  |  |
| coeff_sign_flag[ n ] | ae(v) |  |
| } |  |  |
| if( AbsLevel[ xC ][ yC ] > 0 ) { |  |
| if( AbsLevel[xC][yC] >= ( 3 << ( StatCoeff[ cIdx * 4 ] >> 4 ) ) ) |  |
| StatCoeff[ cIdx * 4 ]+=4 |  |
| else if( 2 * AbsLevel[xC][yC] < ( 1 << ( StatCoeff[ cIdx * 4 ] >> 4 ) ) ) { |  |
| if( StatCoeff[ cIdx * 4 ] > 0 ) |  |
| StatCoeff[ cIdx * 4 ]− − |  |
| } |  |
| } |  |
| if( BdpcmFlag[ x0 ][ y0 ][ cIdx ] = = 0 && n <= lastScanPosPass1 ) { |  |  |
| absLeftCoeff = xC > 0 ? AbsLevel[ xC − 1 ][ yC ] : 0 |  |  |
| absAboveCoeff = yC > 0 ? AbsLevel[ xC ][ yC − 1 ] : 0 |  |  |
| predCoeff = Max( absLeftCoeff, absAboveCoeff ) |  |  |
| if( AbsLevel[ xC ][ yC ] = = 1 && predCoeff > 0 ) |  |  |
| AbsLevel[ xC ][ yC ] = predCoeff |  |  |
| else if( AbsLevel[ xC ][ yC ] > 0 && AbsLevel[ xC ][ yC ] <= predCoeff ) |  |  |
| AbsLevel[ xC ][ yC ]− − |  |  |
| } |  |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 1 − 2 * coeff_sign_flag[ n ] ) * AbsLevel[ xC ][ yC ] |  |  |
| } |  |  |

…

## 9.3	CABAC parsing process for slice data

### 9.3.1	General

Inputs to this process are a request for a value of a syntax element and values of prior parsed syntax elements.

Output of this process is the value of the syntax element.

The initialization process as specified in subclause 9.3.2 is invoked when starting the parsing of the CTU syntax specified in subclause 7.3.11.2 and one or more of the following conditions are true:

–	The CTU is the first CTU in a slice.

–	The CTU is the first CTU in a tile.

–	The value of sps_entropy_coding_sync_enabled_flag is equal to 1 and the CTU is the first CTU in a CTU row of a tile.

The parsing of syntax elements proceeds as follows:

For each requested value of a syntax element a binarization is derived as specified in subclause 9.3.3.

The binarization for the syntax element and the sequence of parsed bins determines the decoding process flow as described in subclause 9.3.4.

The storage process for context variables is applied as follows:

–	When ending the parsing of the CTU syntax in subclause 7.3.11.2, sps_entropy_coding_sync_enabled_flag is equal to 1, and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the storage process for context variables and Rice parameter initialisation states as specified in subclause 9.3.2.3 is invoked with TableStateIdx0Wpp, and TableStateIdx1Wpp and TableStatCoeffWpp as outputs.

When sps_palette_enabled_flag is equal to 1, the storage process for palette predictor is applied as follows:

–	When ending the parsing of the CTU syntax in subclause 7.3.11.2 and the decoding process of the last CU in the CTU in subclause 8.1.2, sps_entropy_coding_sync_enabled_flag is equal to 1 and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the storage process for palette predictor as speficied in subclause 9.3.2.6 is invoked.

…

### 9.3.2	Initialization process

#### 9.3.2.1	General

Outputs of this process are initialized CABAC internal variables and the initialized Rice parameter initialization states StatCoeff.

Figure 12 – Spatial neighbour T that is used to invoke the CTB availability derivation process relative to the current CTB (informative)

The context variables of the arithmetic decoding engine are initialized as follows:

–	If the CTU is the first CTU in a slice or tile, the following applies: the initialization process for context variables is invoked as specified in subclause 9.3.2.2 and the array PredictorPaletteSize[ chType ], with chType = 0, 1, is initialized to 0.

–	The initialization process for context variables is invoked as specified in subclause 9.3.2.2 and the array PredictorPaletteSize[ chType ], with chType = 0, 1, is initialized to 0.

–	The variables StatCoeff[ k ] are set equal to 0, for k in the range 0 to 11, inclusive.

–	Otherwise, if sps_entropy_coding_sync_enabled_flag is equal to 1 and CtbAddrX is equal to CtbToTileColBd[ CtbAddrX ], the following applies:

–	The location ( xNbT, yNbT ) of the top-left luma sample of the spatial neighbouring block T (Figure 12) is derived using the location ( x0, y0 ) of the top-left luma sample of the current CTB as follows:

( xNbT, yNbT ) = ( x0, y0 − CtbSizeY )	(1509)

–	The derivation process for neighbouring block availability as specified in subclause 6.4.4 is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ), the neighbouring location ( xNbY, yNbY ) set equal to ( xNbT, yNbT ), checkPredModeY set equal to FALSE, and cIdx set equal to 0 as inputs, and the output is assigned to availableFlagT.

–	The synchronization process for context variables is invoked as follows:

–	If availableFlagT is equal to 1, the following applies:

–	The synchronization process for context variables as specified in subclause 9.3.2.4 is invoked with TableStateIdx0Wpp, and TableStateIdx1Wpp and TableStatCoeffWpp as inputs.

–	When sps_palette_enabled_flag is equal to 1, the synchronization process for palette predictor as specified in subclause 9.3.2.7 is invoked.

–	Otherwise, the initialization process for context variables is invoked as specified in subclause 9.3.2.2, the variables StatCoeff[ k ] are set equal to 0, for k in the range 0 to 11, inclusive and the array PredictorPaletteSize[ chType ], with chType = 0, 1, is initialized to 0.

–	Otherwise, the initialization process for context variables is invoked as specified in subclause 9.3.2.2, the variables StatCoeff[ k ] are set equal to 0, for k in the range 0 to 11 inclusive, and the array PredictorPaletteSize[ chType ], with chType = 0, 1, is initialized to 0.

…

#### 9.3.2.3	Storage process for context variables and Rice parameter initialization states

Inputs to this process are:

–	The CABAC context variables indexed by ctxTable and ctxIdx.

–	The Rice parameter initialization states indexed by k.

Outputs of this process are:

–	The variables tableStateSync0 and tableStateSync1 containing the values of the variables pStateIdx0 and pStateIdx1 used in the initialization process of context variables that are assigned to all syntax elements in subclauses 7.3.11.1 through 7.3.11.11, except end_of_slice_one_bit, end_of_tile_one_bit, and end_of_subset_one_bit.

–	The variables tableStatCoeffSync containing the values of the variables StatCoeff[ k ] used in the initialization process of context variables and Rice parameter initialization states.

For each context variable, the corresponding entries pStateIdx0 and pStateIdx1 of tables tableStateSync0 and tableStateSync1 are initialized to the corresponding pStateIdx0 and pStateIdx1.

For each Rice parameter initialization state k, each entry of the table tableStatCoeffSync is initialized to the corresponding value of StatCoeff[ k ].

The storage process for context variables is illustrated in the flowchart of Figure 14.

…

#### 9.3.3.2	Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ]

Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the Rice parameter cRiceParam.

The variable sbType is derived as follows:

- If transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 0, and sh_ts_residual_coding_disabled_flag is equal to 0, the following applies:

sbType = 4 * cIdx

- Otherwise, the following applies if both xC and yC are 0

sbType = 4 * cIdx + 1

- Otherwise, the following applies if both xC and yC are less than 4

sbType = 4 * cIdx + 2

- Otherwise,

sbType = 4 * cIdx + 3

The variable initRiceValue is derived as follows:

initRiceValue = StatCoeff[ sbType ] >> 4

The variable shift is derived as follows:

shift = Max(0, initRiceValue – 2)

Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo-code process:

If transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 0:

locSumAbs = 0
if( xC < ( 1  <<  log2TbWidth ) − 1 ) {
	locSumAbs  +=  AbsLevel[ xC + 1 ][ yC ]
	if( xC < ( 1  <<  log2TbWidth ) − 2 )
		locSumAbs  +=  AbsLevel[ xC + 2 ][ yC ]
	if( yC < ( 1  <<  log2TbHeight ) − 1 )
		locSumAbs  +=  AbsLevel[ xC + 1 ][ yC + 1 ]	(1517)
}
if( yC < ( 1  <<  log2TbHeight ) − 1 ) {
	locSumAbs  +=  AbsLevel[ xC ][ yC + 1 ]
	if( yC < ( 1  <<  log2TbHeight ) − 2 )
		locSumAbs  +=  AbsLevel[ xC ][ yC + 2 ]
} 
locSumAbs = Clip3( 0, 31, (locSumAbs − baseLevel * 5) >> shift )

Otherwise,

locSumAbs = 0
if( xC > 0 ) {
	locSumAbs  +=  AbsLevel[ xC - 1 ][ yC ]
	if( xC > 1 )
		locSumAbs  +=  AbsLevel[ xC - 2 ][ yC ]
	if( yC > 0 )
		locSumAbs  +=  AbsLevel[ xC - 1 ][ yC - 1 ]	(1517)
}
if( yC > 0 ) {
	locSumAbs  +=  AbsLevel[ xC ][ yC - 1 ]
	if( yC > 1 )
		locSumAbs  +=  AbsLevel[ xC ][ yC - 2 ]
} 
locSumAbs = Clip3( 0, 31, (locSumAbs − baseLevel * 5) >> shift )

Given the variable locSumAbs, the Rice parameter cRiceParam is derived as the addition of the value of shift and the value specified in Table 128.

When baseLevel is equal to 0, the variable ZeroPos[ n ] is derived as follows:

ZeroPos[ n ] = ( QState < 2 ? 1 : 2 )  <<  cRiceParam	(1518)

Table 128 – Specification of cRiceParam based on locSumAbs

| locSumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |
| locSumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| cRiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |

…

#### 9.3.3.11	Binarization process for abs_remainder[ ]

Input to this process is a request for a binarization for the syntax element abs_remainder[ n ], the colour component cIdx, the current sub-block index i, and the luma location ( x0, y0 ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the binarization of the syntax element.

The variables lastAbsRemainder and lastRiceParam are derived as follows:

–	If this process is invoked for the first time for the current sub-block index i, lastAbsRemainder and lastRiceParam are both set equal to 0.

–	Otherwise (this process is not invoked for the first time for the current sub-block index i), lastAbsRemainder and lastRiceParam are set equal to the values of abs_remainder[ n ] and cRiceParam, respectively, that have been derived during the last invocation of the binarization process for the syntax element abs_remainder[ n ] as specified in this subclause.

The rice parameter cRiceParam is derived as follows:

–	If transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1 and sh_ts_residual_coding_disabled_flag is equal to 0, the Rice parameter cRiceParam is set equal to 1. the rice parameter cRiceParam is derived by invoking the rice parameter derivation process for abs_remainder[ ] as specified in subclause 9.3.3.2 with the variable baseLevel set equal to 0, the colour component index cIdx, the luma location ( x0, y0 ), the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as inputs.

–	Otherwise, the rice parameter cRiceParam is derived by invoking the rice parameter derivation process for abs_remainder[ ] as specified in subclause 9.3.3.2 with the variable baseLevel set equal to 4, the colour component index cIdx, the luma location ( x0, y0 ), the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as inputs.

…

# Conclusions

This proposal has described a modification to the method for the selection of Rice parameters for residual coding, for consideration in VVC version 2. This modification offers BD-rate gains when residual coefficient values become larger with negligible losses at VVC version 1 operating points.  It is suggested that the modification extends the operating point of the existing residual coding method to be applicable at all currently considered bit depths. A simplification of the technique is described which maintains most of the gains whilst reducing coupling in the modification between the use and update of computed values. This simplification also provides the potential for caching/pre-calculating values which should significantly reduce the run-time cost of the modification.

It is proposed that one of the two versions of the modification is adopted into VVC version 2 and VTM.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] “AHG12: Preliminary common test conditions for high bitdepth video coding”, JVET-T0047, T. Ikai, A. Browne and X. Xiu

[2] “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring

[3] “Versatile Video Coding (Draft 10)”, JVET-S2001-vH, B. Bross, J. Chen, S. Liu and Y-K. Wang

[4] “High bit depth coding”, JVET-R0351, A. Browne, S. Keating and K. Sharman

[5] “On operation beyond 10-bit”, JVET-S0243, A. Browne, S. Keating and K. Sharman