| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0072 |
| --- | --- |

| Title: | AHG12: Rice parameter selection for high bit depths |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adrian Browne Steve Keating Karl Sharman | Email: | adrian.browne@sony.com steve.keating@sony.com karl.sharman@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This proposal describes a modification to the method for the selection of Rice parameters for residual coding, for consideration in VVC version 2.  The aim of the modification is to offer BD-rate gains when residual coefficient values become larger, e.g. for high bit depths and low QPs, whilst offering negligible losses at VVC version 1 operating points.  It is suggested that the modification extends the operating point of the existing residual coding method to be applicable at all currently considered bit depths. Results show all intra gains of -26.09%/-21.90%/-20.57% for the proposed 16 bit CTC[1], -1.67%/-1.55%/-1.57% for the proposed 12 bit CTC[1], -0.14%/-0.19%/-0.22% for low QP[2] and 0.02%/-0.02%/0.01% for CTC[2]. A simplification was developed which reduces the coupling between the use and update of computed values within the modification, whilst aiming to maintain the efficiency gains. Results for the simplification are -26.60%/-22.07%/-20.74% for 16 bit, -1.50%/-1.37%/-1.38% for 12 bit and -0.03%/-0.08%/-0.11% for low QP.

# Introduction

In VVC version 1[3], the range of Rice parameters that may be used for coefficients during regular residual coding is limited to the range of 0 to 3 (inclusive). This range has been found to be suitable for both 8 and 10 bit data; however, as bit depths increase and QPs become lower, the size of each coded coefficient becomes larger. For high bit depth content at QPs below 0 it has previously been shown that VVC coding efficiency is below that of HEVC[4].

Previous work[5] has described approaches that increase the coding efficiency for high bit depths and low QPs to match or exceed HEVC.  This proposal takes one of the approaches, “Adding a Rice-based shift to existing VVC” and improves the implementation. This results in gains for 12 and 16-bit content without adversely affecting the operation at lower bit depths and higher QPs.

In addition to the basic implementation a further simplification to the technique is described which attempts to mitigate a potential latency in the original proposal.

# Algorithm description

The algorithm impacts entropy coding and in particular the selection of Rice parameters for the Golomb-Rice coding of residual coefficients. The algorithm can be considered as having four components as follows:

- A coefficient size estimation function which is backed by a series of counters and an index function to select which counter is used for each coefficient.

- An update function for the selected counter applied after each coefficient.

- A modification to the current algorithm for computing the clipped sum of adjacent nearby coefficients.

- A modification to the current algorithm of selecting Rice parameters.

## Coefficient size estimation

The technique requires an estimation of the log2 size of coefficients being coded. This is derived in a similar manner to Rice parameter estimation in HEVC (RExt) using a series of counters. For this proposal the counters are mapped to a two dimensional array, with the first dimension being the component, and the second being an index function coded as shown in Table 1 giving a total of 12 counters. This is a simplification of the technique described in [4] which used more counters and required the computation of the Manhattan distance from the top left of the transform block for each coefficient.

| Index | Description | 16x16 transform block |
| --- | --- | --- |
| 0 | Transform-skip |  |
| 1 (Red) | DC component of top left sub-TB |  |
| 2 (Blue) | Remainder of top left sub-TB |  |
| 3 (Yellow) | All other sub-TBs in transform unit |  |

Table 1. Index function derivation

## Counter update function

Each counter is set to 0 at the beginning of every slice.  For wavefront parallel coding each wavefront copies the counters before starting, treating them in a similar manner to CABAC contexts.

For each non-zero coefficient coded the counter is updated as follows:

if (abs(value) > 0) {

int idx = calcIndex(isTS, scanPos);    			// index function as defined in Table 1

int &count = riceCount[component][idx];		// riceCount is the array of counters, reset to 0 for each slice

int rice = count >> log2Divisor;				// log2Divisor = 4

if (abs(value) >= (3 << rice)) {      			    	// value larger than expected range

count += increment;    					// increment = 4

} else if ((abs(value) << 1) < (1 << rice)) {    	// value smaller than expected range

count = max(0, count - decrement);			// decrement = 1, clipped to be non-negative

}

}

Thus the value of rice provides an estimate of the log2 size of the average non-zero coefficient.  Mismatched increments and decrements have been found to be beneficial as slightly over-estimating the Rice parameter seems to offer the best gains.

## Modification to clipped sum of coefficients

For regular residual coding (RRC), VVC (section 9.3.3.2) defines locSumAbs as the sum of the absolute values of 5 previously coded coefficients below and right of the coefficient, less an offset which relates to the proportion of the coefficient that is coded with flags, and then clips this value to the range 0 to 31. This technique works well if clipping is only occasionally used, but as average coefficient values increase the number of times clipping is required increases.

In the modification described here, a shift right is introduced as below (modification in green).  The purpose of this shift is to reduce the magnitude of the sum and thus reduce the amount of clipping. The size of the shift is described in the following section.

locSumAbs = Clip3( 0, 31, ( locSumAbs − baseLevel * 5 ) >> shift )

For transform skip residual coding (TSRC) VTM currently does not use a clipped sum. The modification introduces a clipped sum for TSRC which is identical to that described for RRC but uses 5 coefficients above and left of the coefficient being coded.

## Rice parameter selection

In VTM, RRC selects the Rice parameter by using the value returned by the template function as an index into a lookup table which gives the Rice parameter.

In the proposed modification the selection of the Rice parameter can be described using the pseudo code below (modification is indicated in green)

int idx = calcIndex(isTS, scanPos); 				 // index function as defined in Table 1

int count = riceCount[component][idx];  			// select size estimation counters

int shift = std::max(0, (count >> log2Divisor) - 2); 	// compute template shift modifier

int idx = cctx.templateAbsSum(scanPos, absCoeff, baseLevel, shift);

// compute template fn using sum shifted right by shift modifier

int rice = g_auiGoRiceParsCoeff[idx] + shift;		// add shift modifier onto result from lookup table

Thus the selection of the Rice parameter may be seen as a modified version of the existing VVC technique rather than a new technique.  Experimentally for the 10 bit CTC the value of the shift modifier is rarely non-zero and thus the resultant coding is almost identical to VVC. But as average coefficient values increase and the shift modifier becomes greater than zero, the resultant Rice parameter is offset as below.

| Estimated value from counter 1 << (count >> log2Divisor) | Shift modifier | Range of possible Rice values |
| --- | --- | --- |
| 1 | 0 | 0..3 |
| 2 | 0 | 0..3 |
| 4 | 0 | 0..3 |
| 8 | 1 | 1..4 |
| 16 | 2 | 2..5 |

Table 2. Output Rice values for a range of estimated coefficient values

Thus when estimated coefficient values are larger this modification ensures a more appropriate range of Rice parameters is selectable. Unlike simply swapping to a technique based purely on size estimation, this modification introduces a hybrid technique which attempts to extend the operating point of the existing VVC technique.

# Simplification

One potential issue with the modification is that there is a close coupling between the update and use of the size estimation counters. This also prevents the caching/pre-calculation and reuse of derived values. This section describes a simplification which attempts to resolve this issue.

## Coefficient size estimation

The proposed simplification is to take a copy of the riceCount array at the beginning of coding each sub-TB as follows:

riceCopy = riceCount;

This copy could be further reduced to the copying of a maximum of two counters rather than the complete set of 12 if the proposed index function described in section 2.1 is used.

## Counter update function

The update function becomes:

if (abs(value) > 0) {

int idx = calcIndex(isTS, scanPos);

int &count = riceCount[component][idx];		// riceCount must be updated

int rice = riceCopy[component][idx];			// rice value for estimation uses copy

if (abs(value) >= (3 << rice)) {      			    	// use copied value

count += increment;    					// update original counter

} else if ((abs(value) << 1) < (1 << rice)) {    		// use copied value

count = max(0, count - decrement);			// update original counter

}

}

The update function could be further improved by caching/pre-calculating the values (3 << rice) and (1 << rice) and the reference count.

## Rice parameter selection

The Rice parameter selection function becomes:

int idx = calcIndex(isTS, scanPos);

int count = riceCopy[component][idx];  			// use copied value

int shift = std::max(0, (count >> log2Divisor) - 2);

int idx = cctx.templateAbsSum(scanPos, absCoeff, baseLevel, shift);

int rice = g_auiGoRiceParsCoeff[idx] + shift;

This function could be further improved by pre-computing the two possible values of shift per sub-TB.

# Results

## Without simplification

### 16 bit

|  |  |  | AI-16 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-10.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| SVT | -26.68% | -22.47% | -20.91% | 67% | 70% |
| Overall | -26.68% | -22.47% | -20.91% | 67% | 70% |

As QPs reduce the gains increase from -16.20%/-16.50%/-16.69% for QPs -8 to -23 to -65.26%/-60.83%/-63.82% for QPs -18 to -33.

The encoder/decoder timings were caused by a slow anchor due to processor loading and should be ignored

### 12 bit

|  |  |  | AI-12 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| SVT | -3.90% | -3.60% | -3.59% | 91% | 83% |
| PQ444 | -0.51% | -0.48% | -0.50% | 100% | 79% |
| PQ422 | -0.60% | -0.58% | -0.61% | 100% | 79% |
| Overall | -1.67% | -1.55% | -1.57% | 95% | 77% |

### 10 bit CTC

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM10.0 |  |  |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.02% | -0.06% | 0.03% | 0.02% | 100% | 100% |
| Class A2 | 0.02% | 0.00% | -0.02% | 0.01% | 98% | 99% |
| Class B | 0.02% | 0.03% | 0.01% | 0.02% | 100% | 100% |
| Class C | -0.01% | 0.00% | -0.06% | -0.02% | 100% | 100% |
| Class E | 0.05% | -0.08% | 0.11% | 0.04% | 100% | 99% |
| Overall | 0.02% | -0.02% | 0.01% | 0.01% | 100% | 100% |
| Class D | 0.00% | -0.04% | 0.13% | 0.01% | 101% | 100% |
| Class F | -0.04% | -0.07% | -0.09% | -0.04% | 99% | 99% |

### 10 bit low QP

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM10.0 LOWQP (JVET-N1010) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.04% | -0.03% | 105% | 107% |
| Class A2 | -0.07% | -0.12% | -0.12% | 106% | 108% |
| Class B | -0.12% | -0.16% | -0.20% | 105% | 102% |
| Class C | -0.24% | -0.20% | -0.24% | 107% | 100% |
| Class E | -0.19% | -0.46% | -0.52% | 104% | 105% |
| Overall | -0.14% | -0.19% | -0.22% | 105% | 104% |
| Class D | -0.30% | -0.36% | -0.38% | 107% | 103% |
| Class F | -1.55% | -1.29% | -1.35% | 104% | 101% |

### 10 bit lossless

| YUV 420 | All Intra |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-10.0 | Tested |  |
| Class A1 | 2.4 | 2.4 | -0.61% |
| Class A2 | 1.8 | 1.8 | -1.47% |
| Class B | 2.3 | 2.3 | -0.22% |
| Class C | 2.1 | 2.1 | -0.13% |
| Class D | 2.1 | 2.1 | -0.22% |
| Class E | 3.1 | 3.1 | 0.06% |
| Class F | 5.8 | 5.8 | -0.57% |
| TGM | 12.4 | 13.0 | -4.65% |
| Overall | 2.3 | 2.3 | -0.43% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 112% |  |  |

| RGB | All Intra |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-10.0 | Tested |  |
| TGM 1080p | 21.7 | 22.4 | -2.47% |
| TGM 720p | 10.1 | 10.1 | -0.26% |
| A | 6.0 | 6.0 | 0.05% |
| M | 6.0 | 6.0 | -0.57% |
| CC | 1.7 | 1.8 | -1.59% |
| Overall | 10.4 | 10.6 | -0.98% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 110% |  |  |

| YUV | All Intra |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-10.0 | Tested |  |
| TGM 1080p | 21.3 | 21.6 | -1.32% |
| TGM 720p | 12.7 | 12.8 | -0.31% |
| A | 7.1 | 7.1 | -0.03% |
| M | 7.8 | 7.8 | -0.77% |
| CC | 2.0 | 2.0 | -0.85% |
| Overall | 11.6 | 11.6 | -0.66% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 108% |  |  |

## With simplification

Please note that the encoder and decoder timings quoted here are for debug/experimental code without the caching/pre-calculation improvements suggested in section 3.  It is believed that by implementing these improvements the increase in encoder and decoder times will be much reduced.

### 16 bit

|  |  |  | AI-16 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-10.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| SVT | -26.60% | -22.07% | -20.74% | 67% | 74% |
| Overall | -26.60% | -22.07% | -20.74% | 67% | 74% |

### 12 bit

|  |  |  | AI-12 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| SVT | -3.72% | -3.37% | -3.37% | 89% | 84% |
| PQ444 | -0.36% | -0.33% | -0.35% | 99% | 79% |
| PQ422 | -0.42% | -0.41% | -0.43% | 99% | 79% |
| Overall | -1.50% | -1.37% | -1.38% | 96% | 81% |

### 10 bit low QP

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.01% | -0.01% | 105% | 108% |
| Class A2 | 0.04% | -0.02% | -0.01% | 107% | 109% |
| Class B | -0.03% | -0.06% | -0.12% | 106% | 103% |
| Class C | -0.05% | 0.01% | -0.02% | 106% | 101% |
| Class E | -0.09% | -0.37% | -0.42% | 104% | 105% |
| Overall | -0.03% | -0.08% | -0.11% | 106% | 105% |
| Class D | -0.06% | -0.09% | -0.08% | 107% | 104% |
| Class F | -1.44% | -1.17% | -1.24% | 104% | 102% |

### 10 bit lossless

| YUV 420 | All Intra |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-10.0 | Tested |  |
| Class A1 | 2.4 | 2.4 | -0.39% |
| Class A2 | 1.8 | 1.8 | -1.17% |
| Class B | 2.3 | 2.3 | 0.08% |
| Class C | 2.1 | 2.1 | 0.22% |
| Class D | 2.1 | 2.1 | 0.17% |
| Class E | 3.1 | 3.1 | 0.27% |
| Class F | 5.8 | 5.8 | -0.32% |
| TGM | 12.4 | 13.0 | -4.42% |
| Overall | 2.3 | 2.3 | -0.14% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 111% |  |  |

| RGB | All Intra |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-10.0 | Tested |  |
| TGM 1080p | 21.7 | 22.3 | -2.29% |
| TGM 720p | 10.1 | 10.1 | 0.08% |
| A | 6.0 | 6.0 | 0.29% |
| M | 6.0 | 6.0 | -0.25% |
| CC | 1.7 | 1.7 | -1.22% |
| Overall | 10.4 | 10.6 | -0.70% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 109% |  |  |

| YUV | All Intra |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-10.0 | Tested |  |
| TGM 1080p | 21.3 | 21.6 | -1.19% |
| TGM 720p | 12.7 | 12.7 | -0.14% |
| A | 7.1 | 7.1 | 0.15% |
| M | 7.8 | 7.8 | -0.60% |
| CC | 2.0 | 2.0 | -0.56% |
| Overall | 11.6 | 11.6 | -0.49% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 108% |  |  |

# Conclusions

This proposal has described a modification to the method for the selection of Rice parameters for residual coding, for consideration in VVC version 2. This modification offers BD-rate gains when residual coefficient values become larger with negligible losses at VVC version 1 operating points.  It is suggested that the modification extends the operating point of the existing residual coding method to be applicable at all currently considered bit depths. A simplification of the technique is described which maintains most of the gains whilst reducing coupling in the modification between the use and update of computed values. This simplification also provides the potential for caching/pre-calculating values which should significantly reduce the run-time cost of the modification.

It is proposed that the simplified version of the modification is adopted into VVC version 2 and VTM.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] “AHG12: Preliminary common test conditions for high bitdepth video coding”, JVET-T0047, T. Ikai, A. Browne and X. Xiu

[2] “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring

[3] “Versatile Video Coding (Draft 10)”, JVET-S2001-vH, B. Bross, J. Chen, S. Liu and Y-K. Wang

[4] “High bit depth coding”, JVET-R0351, A. Browne, S. Keating and K. Sharman

[5] “On operation beyond 10-bit”, JVET-S0243, A. Browne, S. Keating and K. Sharman