| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0073 |
| --- | --- |

| Title: | AHG11: neural network-based intra prediction with transform selection in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Thierry Dumas Franck Galpin Philippe Bordes Fabrice Le Léannec | Tel: Email: | Thierry.dumas@interdigital.com |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

This document describes a new neural network-based intra prediction mode inside VVC. This neural network-based intra prediction mode predicts a given block from neighboring reference samples located above and on the left side of this block. Besides, it infers both the LFNST kernel index and whether the primary transform coefficients resulting from the application of the DCT-2 horizontally and the DCT-2 vertically to the residue of the neural network prediction are transposed, which completely defines the LFNST in case LFNST is selected. When integrated into VTM-8.0, for the all-intra configuration, -3.36%, -2.95%, and -2.97% of Y BD-rate gain, U BD-rate gain, and V BD-rate gain respectively are reported. In this configuration, the encoding and decoding runtimes of VTM-8.0 with the neural network-based intra prediction mode with respect to VTM-8.0 are 395% and 3575% respectively. For the random-access configuration, -1.52%, -1.00%, and -1.26% of Y BD-rate gain, U BD-rate gain, and V BD-rate gain respectively are reported. In this configuration, the encoding and decoding runtimes of VTM-8.0 with the neural network-based intra prediction mode with respect to VTM-8.0 are 159% and 723% respectively.

# Composition of the neural network-based intra prediction mode

The neural network-based intra prediction mode contains 8 neural networks, each predicting blocks of a different size in 4×4, 8×4, 16×4, 32×4, 8×8, 16×8, 16×16, 32×32. The neural network predicting blocks of size w×h is denoted fh, w . , θh,w where θh,w gathers its parameters. For a given w×h block Y, fh, w takes a preprocessed version X of the context X made of na rows of nl+2w reference samples located above this block and nl columns of 2h reference samples on its left side to provide Y. The application of a postprocessing to Y yields a prediction Y of Y, see Figure 1. Besides, fh, w . , θh,w gives two indices grpIdx1 and grpIdx2, grpIdxi denoting the index characterizing the LFNST kernel index and whether the primary transform coefficients resulting from the application of the DCT-2 horizontally and the DCT-2 vertically to the residue of the neural network prediction are transposed when lfnstIdx=i, i∈1, 2, see Figure 1.

Figure 1: prediction of the current w×h block Y from its context X of neighboring reference samples via the neural network fh, w . , θh,w belonging to the new neural network-based intra prediction mode. Here, w=8, h=4, and na=nl=4.

If minh,w≤8, na=nl=minh,w. Otherwise, na=h2 and nl=w2.

This document first details the neural network prediction of the current block, see Section 2.1. Then, it focuses on the inference of the two indices characterizing the LFNST to be applied to the primary transform coefficients resulting from the application of the DCT-2 horizontally and the DCT-2 vertically to the residue of the neural network prediction when lfnstIdx=1 and lfnstIdx=2 respectively, see Section 2.2.

## Neural network-based intra prediction

### Preprocessing of the context of the current block

The “preprocessing” illustrated in Figure 1 consists in the four following successive steps.

- The reference samples in the context X of the current block Y are divided by 2b-8, b being the internal bitdepth, i.e. 10 in VVC.

- The mean μ of the available reference samples X in X, see Figure 2, is subtracted from X.

- All the unavailable reference samples Xu in X, see Figure 2, are set to 255.

- If minh,w≤8, the context of reference samples resulting from the last step is flattened. This is because the neural network predicting blocks of size w×h, minh,w≤8, is fully-connected, see Section 2.2. If minh,w>8, the context of reference samples resulting from the last step is split into the portion X0 containing the na rows of nl+2w reference samples located above Y and the portion X1 containing the nl columns of 2h reference samples on its left side. This is because the neural network predicting blocks of size w×h, minh,w>8, is convolutional, see Section 2.2, and two rectangles are efficiently processed by a convolutional neural network.

Thus, if minh,w≤8, X is a vector of size nanl+2w +2hnl. Otherwise, X=X0∪X1.

Figure 2: decomposition of the context X of reference samples surrounding the current w×h block Y into the available reference samples X and the unavailable reference samples Xu. Here, w=8, h=4, na=nl=4, and the number of unavailable reference samples reaches its maximum value, i.e. hnl+wna.

### Neural network architectures

The neural network predicting the current w×h block is fully-connected if minh,w≤8. In this case, the neural network architecture is described in Table 1.

| layer index | input | layer type | number of neurons | non-linearity |
| --- | --- | --- | --- | --- |
| 1 | X | fully-connected | 1200 | LeakyReLU |
| 2 | 1 | fully-connected | 1200 | LeakyReLU |
| 3 | 2 | fully-connected | hw | - |
| 4 | 2 | fully-connected | 14 | - |

Table 1: architecture of the neural network predicting the current w×h block, minh,w≤8. In the column “input”, a number refers to the index of the layer whose output is the input to the current layer. The layer of index 3 returns Y.

The neural network f16, 16 . , θ16,16 predicting 16×16 blocks is convolutional. More precisely, f16, 16 . , θ16,16 is made of three sub-neural networks f16, 160 . , θ016,16 with parameters θ016,16, f16, 161 . , θ116,16 with parameters θ116,16, and f16, 16f . , θf16,16 with parameters θf16,16, see Figure 3. The architectures of these three sub-neural networks are defined in Table 2, Table 3, and Table 4.

Figure 3: architecture of the neural network predicting the current w×h block, minh,w>8. The “flattening” operation flattens its input stack of feature maps. The “concatenation” operation concatenates its two input vectors into a single vector.

| layer index | input | layer type | filter size | number of filters | stride | non-linearity |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | X0 | convolutional | 3×3×1 | 32 | 2,2 | LeakyReLU |
| 2 | 1 | convolutional | 3×3×32 | 64 | 2,2 | LeakyReLU |
| 3 | 2 | convolutional | 3×3×64 | 128 | 1,2 | LeakyReLU |
| 4 | 3 | convolutional | 3×3×128 | 128 | 1,2 | LeakyReLU |

Table 2: architecture of f16, 160 . , θ016,16. In the column “stride”, each tuple refers to the pair of the vertical stride and the horizontal stride of a convolution. In the column “input”, a number refers to the index of the layer whose output is the input to the current layer.

| layer index | input | layer type | filter size | number of filters | stride | non-linearity |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | X1 | convolutional | 3×3×1 | 32 | 2,2 | LeakyReLU |
| 2 | 1 | convolutional | 3×3×32 | 64 | 2,2 | LeakyReLU |
| 3 | 2 | convolutional | 3×3×64 | 128 | 2,1 | LeakyReLU |
| 4 | 3 | convolutional | 3×3×128 | 128 | 2,1 | LeakyReLU |

Table 3: architecture of f16, 161 . , θ116,16. In the column “stride”, each tuple refers to the pair of the vertical stride and the horizontal stride of a convolution. In the column “input”, a number refers to the index of the layer whose output is the input to the current layer.

| layer index | input | layer type | number of neurons | non-linearity |
| --- | --- | --- | --- | --- |
| 1 | result of the concatenation | fully-connected | 500 | LeakyReLU |
| 2 | 1 | fully-connected | 256 | - |
| 3 | 1 | fully-connected | 14 | - |

Table 4: architecture of f16, 16f . , θf16,16. The layer of index 2 returns Y.

The neural network f32, 32 . , θ32,32 predicting 32×32 blocks is convolutional. Specifically, f32, 32 . , θ32,32 is made of three sub-neural networks f32, 320 . , θ032,32 with parameters θ032,32, f32 321 . , θ132,32 with parameters θ132,32, and f32, 32f . , θf32,32 with parameters θf32,32, see Figure 3. The architectures of these three sub-neural networks are defined in Table 5, Table 6, and Table 7.

| layer index | input | layer type | filter size | number of filters | stride | non-linearity |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | X0 | convolutional | 3×3×1 | 32 | 2,2 | LeakyReLU |
| 2 | 1 | convolutional | 3×3×32 | 64 | 2,2 | LeakyReLU |
| 3 | 2 | convolutional | 3×3×64 | 128 | 2,2 | LeakyReLU |
| 4 | 3 | convolutional | 3×3×128 | 128 | 1,2 | LeakyReLU |

Table 5: architecture of f32, 320 . , θ032,32.

| layer index | input | layer type | filter size | number of filters | stride | non-linearity |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | X1 | convolutional | 3×3×1 | 32 | 2,2 | LeakyReLU |
| 2 | 1 | convolutional | 3×3×32 | 64 | 2,2 | LeakyReLU |
| 3 | 2 | convolutional | 3×3×64 | 128 | 2,2 | LeakyReLU |
| 4 | 3 | convolutional | 3×3×128 | 128 | 2,1 | LeakyReLU |

Table 6: architecture of f32, 321 . , θ132,32.

| layer index | input | layer type | number of neurons | non-linearity |
| --- | --- | --- | --- | --- |
| 1 | result of the concatenation | fully-connected | 1024 | LeakyReLU |
| 2 | 1 | fully-connected | 1024 | - |
| 3 | 1 | fully-connected | 14 | - |

Table 7: architecture of f32, 32f . , θf32,32. The layer of index 2 returns Y.

### Postprocessing of the neural network prediction

The “postprocessing” illustrated in Figure 1 consists in reshaping the vector Y of size hw into a rectangle of height h and width w, adding to the result of the reshape the mean μ of the available reference samples in the context of the current block, multiplying by 2b-8, and clipping to 0, 2b-1. Therefore,

Y=minmax2b-8reshapeY+μ, 0, 2b-1.

## LFNST selection

For a given w×h block to be predicted, fh, w . , θh,w contains a fully-connected layer that computes a vector P of size 14 from an intermediate neuronal representation of the context of this block, c.f. the layer of index 4 in Table 1, the layer of index 3 in Table 4, and the layer of index 3 in Table 7.

### Inference of the LFNST to be applied

grpIdx1=argmaxP0:7 and grpIdx2=argmaxP7:14. Finally, if lfnstIdx∈1,2 for this block, grpIdxlfnstIdx completely defines the LFNST to be applied to the primary transform coefficients resulting from the application of the DCT-2 horizontally and the DCT-2 vertically to the residue of the neural network prediction by following Table 8.

| grpIdxi, i∈1, 2 | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| transposition of the primary transform coefficients resulting from the application of the DCT-2 horizontally and the DCT-2 vertically to the residue of the neural network prediction | no | no | no | no | yes | yes | yes |
| LFNST kernel index | 0 | 1 | 2 | 3 | 3 | 2 | 1 |

Table 8: definition of the LFNST from grpIdxi, , i∈1, 2.

### Prediction of the LFNST to be applied

Instead of inferring the LFNST to be applied, see Section 2.2.1, the neural network can provide a prediction of the grpIdxlfnstIdx selected by the encoder, lfnstIdx∈1,2. This time, grpPredIdx1=argmaxP0:7 and grpPredIdx2=argmaxP7:14. Then, if lfnstIdx∈1,2, the grpIdxlfnstIdx selected by the encoder is predictively coded with respect to grpPredIdxlfnstIdx, see Section 3.2.

# Signaling in VVC

## Signaling of the neural network-based intra prediction mode

### Signaling of the neural network-based intra prediction mode in luminance

The neural network-based intra prediction mode is signaled in VVC via the nnFLag. For the current w×h luminance CB whose top-left pixel is at position y,x in the current frame, if h,w∈T and the context of the current luminance CB does not go out of the bounds of the current frame, i.e. x≥nl && y≥na, nnFlag appears in the intra prediction mode signaling. Otherwise, the regular intra prediction mode signaling in luminance in VVC, denoted S⊂, is used. When nnFlag appears in the intra prediction mode signaling, nnFLag =1 means that the neural network-based intra prediction mode is selected to predict the current luminance CB and END. Otherwise, nnFLag =0 and the regular intra prediction mode signaling in luminance in VVC is used, see Figure 4.

T=4,4, 4,8, 8,4, 4,16, 16,4, 4,32, 32,4, 8,8, 8,16, 16,8, 16,16, 16,32,

32,16, 32,32, 64,64

Figure 4: intra prediction mode signaling for the current w×h luminance CB framed in orange in dashed line. The coordinates of the pixel at the top-left of this CB are y, x. The bin value of a nnFlag value appears in bold gray. Here, h=8, w=4, x=8, and y=0.

### Signaling of the neural network-based intra prediction in chrominance

For a given w×h chrominance CB whose top-left pixel is at position y,x in the current frame, if the luminance CB collocated with this chrominance CB is predicted by the neural network-based intra prediction mode, the DM becomes the neural network-based intra prediction mode if h,w∈T && x≥nl && y≥na. Otherwise, DM is set to PLANAR.

### Transformation of the context and the neural network prediction

It is possible that the neural network-based intra prediction mode must predict a given w×h block as h,w∈T but the neural network-based intra prediction mode does not contain fh, w . , θh,w. This happens for instance when h=8 and w=4. In this case, the context of the current block can be down-sampled vertically by a factor δ and/or down-sampled horizontally by a factor γ and/or transposed before the step called “preprocessing” in Figure 1. Then, the prediction of the current block can be transposed and/or up-sampled by the factor δ and/or up-sampled horizontally by the factor γ after the step called “postprocessing” in Figure 1. The transposition of the context of the current block and the prediction, δ, and γ are chosen so that a neural network belonging to the neural network-based intra prediction mode is used for prediction, see Table 9.

| height and width of the block to be predicted h, w | γ | δ | transposition | neural network used for prediction |
| --- | --- | --- | --- | --- |
| 4, 4 | 1 | 1 | no | f4, 4 . , θ4,4 |
| 4, 8 | 1 | 1 | no | f4, 8 . , θ4,8 |
| 8, 4 | 1 | 1 | yes | f4, 8 . , θ4,8 |
| 4, 16 | 1 | 1 | no | f4, 16 . , θ4,16 |
| 16, 4 | 1 | 1 | yes | f4, 16 . , θ4,16 |
| 4, 32 | 1 | 1 | no | f4, 32 . , θ4,32 |
| 32, 4 | 1 | 1 | yes | f4, 32 . , θ4,32 |
| 8, 8 | 1 | 1 | no | f8, 8 . , θ8,8 |
| 8, 16 | 1 | 1 | no | f8, 16 . , θ8,16 |
| 16, 8 | 1 | 1 | yes | f8, 16 . , θ8,16 |
| 16, 16 | 1 | 1 | no | f16, 16 . , θ16,16 |
| 16, 32 | 2 | 1 | no | f16, 16 . , θ16,16 |
| 32, 16 | 1 | 2 | no | f16, 16 . , θ16,16 |
| 32, 32 | 1 | 1 | no | f32, 32 . , θ32,32 |
| 64, 64 | 2 | 2 | no | f32, 32 . , θ32,32 |

Table 9: decision of transposing the context of the current w×h block to be predicted and the prediction of this block, the value of γ, and the value of δ, and the neural network belonging to the neural network-based intra prediction mode used for prediction for each h, w∈T.

## Signaling of LFNST in case the neural network-based mode predicts the LFNST to be applied

In this section, the prediction of the LFNST to be applied, described in Section 2.2.2, is used. If the neural network-based intra prediction mode is selected to predict the current w×h block and lfnstIdx∈1,2, the predictive encoding of the grpIdxlfnstIdx selected by the encoder with respect to the grpPredIdxlfnstIdx provided by the neural network-based intra prediction mode is given by Figure 5.

Figure 5: predictive encoding of the grpIdxlfnstIdx selected by the encoder with respect to the grpPredIdxlfnstIdx provided by the neural network-based intra prediction mode when the neural network-based intra prediction mode is selected to predict the current w×h block and lfnstIdx∈1,2.

On the decoder side, if the neural network-based intra prediction mode is selected to predict the current w×h block and lfnstIdx∈1,2, the predictive decoding of grpIdxlfnstIdx with respect to the grpPredIdxlfnstIdx given by the neural network-based intra prediction mode is depicted in Figure 6.

Figure 6: predictive decoding of grpIdxlfnstIdx with respect to the grpPredIdxlfnstIdx provided by the neural network-based intra prediction mode when the neural network-based intra prediction mode is selected to predict the current w×h block and lfnstIdx∈1,2. The predictive code C is defined in Figure 5.

# Reporting template

| Network information in the training stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment | CPU: Intel Xeon E5-2680v4 14-cores@2.40GHz GPU: Tesla M60 Maxwell (8 GB) |
|  | SW environment | CentOS 7.6.1810 |
|  | framework | Tensorflow 1.9.0 |
|  | number of epochs | 15 |
|  | batch size | 210000 batches of 100 examples |
|  | training time | 8 hours |
|  | training data information | The set of images used to create the training sets combines the first 106 RGB images in the ILSVRC2012 training set, the RGB images in DIV2K, and the RGB images in the CLIC2020 training set for low-rate image compression. Then, the RGB images are converted into YCbCr. |
|  | configurations for generating compressed training data | The configuration for all-intra in VTM-8.0 is used to generate the compressed training data. Moreover, QPs in 22, 27, 32, 37 are used. |
| Optional | patch size | fh,w . , θh,w is trained on pairs of a w×h block and its context. |
|  | preprocessing | The preprocessing of a training block and its context is directly derived from the step called “preprocessing” and the step called “postprocessing” in Figure 1. |

Table 10: neural network information in the training stage.

| Network information in inference stage |  |  |  |
| --- | --- | --- | --- |
| Mandatory | HW environment | CPU: E5-2697Av4 @ 2.6GHz using a single core |
|  | SW environment | CentOS7 |
|  | framework | Tensorflow 1.9.0 native, CPU single core, inference in floats |
|  | total conv. layers | See Figure 3, Table 2, Table 3, Table 5, Table 6 |
|  | total fully-connected layers | See Table 1, Table 4, Table 7 |
|  | total parameter number | θ4,4 contains 1574430 parameters. θ4,8 contains 1632046 parameters. θ4,16 contains 1747278 parameters. θ4,32 contains 1977742 parameters. θ8,8 contains 1920078 parameters. θ8,16 contains 2150542 parameters. θ16,16 contains6212821256282 parameters. θ32,32 contains 15639183904782 parameters. |
|  | parameter precision | 32 (F) |
|  | memory parameter (MB) | 50.3 MB in total |
|  | memory temp (MB) |  |
|  | MAC (Giga) | 1.6 Mmac for f4, 4 . , θ4,4 predicting a 4×4 block. 1.6 Mmac for f4, 8 . , θ4,8 predicting a 8×4 block. 1.7 Mmac for f4, 16 . , θ4,16 predicting a 16×4 block. 2.0 Mmac for f4, 32 . , θ4,32 predicting a 32×4 block. 1.9 Mmac for f8, 8 . , θ8,8 predicting a 8×8 block. 2.1 Mmac for f8, 16 . , θ8,16 predicting a 16×8 block. 4.3 Mmax for f16, 16 . , θ16,16 predicting a 16×16 block. 11.5 Mmax for f32, 32 . , θ32,32 predicting a 32×32 block. |

Table 11: neural network information in the inference stage.

|  | All intra over VTM-8.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | encoding time (CPU) | decoding time (CPU) |
| Class A1 | -4.89% | -4.16% | -4.61% | 366% | 3007% |
| Class A2 | -2.35% | -2.08% | -2.28% | 411% | 2955% |
| Class B | -2.99% | -2.59% | -2.50% | 429% | 3897% |
| Class C | -2.71% | -2.25% | -2.38% | 364% | 3916% |
| Class E | -4.30% | -4.14% | -3.57% | 400% | 3943% |
| Overall | -3.36% | -2.95% | -2.97% | 395% | 3575% |
| Class D | -3.02% | -2.63% | -2.71% | 348% | 4010% |
| Class F | -1.91% | -1.35% | -1.49% | 234% | 1844% |

|  | Random access over VTM-8.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | encoding time (CPU) | decoding time (CPU) |
| Class A1 | -2.06% | -1.00% | -1.87% | 150% | 761% |
| Class A2 | -1.04% | -0.42% | -0.55% | 150% | 577% |
| Class B | -1.42% | -1.16% | -1.17% | 160% | 593% |
| Class C | -1.61% | -1.24% | -1.44% | 173% | 1057% |
| Class E |  |  |  |  |  |
| Overall | -1.52% | -1.00% | -1.26% | 159% | 723% |
| Class D | -1.19% | -0.41% | -1.22% | 160% | 731% |
| Class F | -0.78% | -1.08% | -0.45% | 155% | 445% |

Table 12: BD-rate and coding time in the inference stage when the neural network-based intra prediction mode infers the LFNST to be applied as in Section 2.2.1.

|  | All Intra over VTM-8.0 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate (Y) |  | BD-rate (U) |  | BD-rate (V) |  | encoding time (CPU) |  |  | decoding time (CPU) |  |  |
| Class A1 | -5.14% |  | -4.01% | -4.74% |  |  | 404% |  |  |  | 3047% |  |
| Class A2 | -2.35% |  | -2.39% | -2.46% |  |  | 435% |  |  |  | 2942% |  |
| Class B | -3.11% |  | -2.74% | -2.72% |  |  | 492% |  |  |  | 3792% |  |
| Class C | -2.82% |  | -2.43% | -2.55% |  |  | 441% |  |  |  | 5153% |  |
| Class E | -4.48% |  | -4.01% | -3.29% |  |  | 494% |  |  |  | 4704% |  |
| Overall | -3.49% |  | -3.04% | -3.07% |  |  | 455% |  |  |  | 3889% |  |
| Class D | -3.16% |  | -2.73% | -2.80% |  |  | 426% |  |  |  | 4941% |  |
| Class F | -1.96% |  | -1.75% | -1.40% |  |  | 268% |  |  |  | 2046% |  |
|  | Random access over VTM-8.0 |  |  |  |  |  |  |  |  |  |  |
|  | BD-rate (Y) | BD-rate (U) |  |  | BD-rate (V) | encoding time (CPU) |  |  | decoding time (CPU) |  |  |
| Class A1 | -2.14% | -0.94% |  |  | -1.79% | 167% |  | 880% |  |  |  |
| Class A2 | -1.09% | -0.22% |  |  | -0.67% | 161% |  | 497% |  |  |  |
| Class B | -1.48% | -1.30% |  |  | -0.93% | 173% |  | 547% |  |  |  |
| Class C | -1.62% | -1.12% |  |  | -1.30% | 194% |  | 992% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.57% | -0.96% |  |  | -1.15% | 175% |  | 692% |  |  |  |
| Class D | -1.30% | -0.63% |  |  | -1.68% | 181% |  | 788% |  |  |  |
| Class F | -0.81% | -1.43% |  |  | -0.48% | 167% |  | 440% |  |  |  |

Table 13: BD-rate and coding time in the inference stage when the neural network-based intra prediction mode predicts the LFNST to be applied as in Section 2.2.2.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).