| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 October 2020 | Document: JVET-T0074-v1 |
| --- | --- |

| Title: | AHG10: LMCS encoder algorithm for YUV4:4:4 content |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangjun Pu, Taoran Lu, Peng Yin, Sean McCarthy, Walt Husak, Tao Chen 432 Lakeside Drive, Sunnyvale, CA, 94085 US | Tel: Email: | fpu@dolby.com tlu@dolby.com pyin@dolby.com |  |
| Source: | Dolby Laboratories, Inc. |  |  |  |

_____________________________

# Abstract

This contribution proposes an automatic non-normative encoder optimization algorithm for LMCS to improve coding performance for SDR YUV4:4:4 video content. In VTM10.0, the default LMCS encoder algorithm is optimized for YUV4:2:0 content. However, YUV4:2:0 and YUV4:4:4 signals have very different characteristics. Similarly, YUV4:4:4 camera (natural) content and screen content have different characteristics. The YUV4:4:4 LMCS encoder algorithm proposed in this contribution provides the following coding performance for SDR YUV4:4:4 CTC (reference: LMCS off; test: LMCS on) under VTM10.0 in terms of BDRate and processing time:

- For camera content (YUV4:4:4):

  - AI:  { -0.32%	0.69%	1.04% }, Enc: 104%, Dec: 101%

  - RA:  { -1.13%	2.98%	0.20% }, Enc: 104%, Dec: 100%

- For screen content (YUV4:4:4):

  - AI:  { -1.22%	2.89%	3.08% }, Enc: 104%, Dec: 101%

  - RA:  { -1.64%	-0.01%	0.16% }, Enc: 101%, Dec: 100%

# Introduction

At the 18th meeting of JVET, common test conditions (CTC) and software reference configurations were defined to be used in the development of VVC for non-4:2:0 colour format content [1]. Applicable test cases include 4:4:4, 4:2:2, 4:0:0 chroma formats; R’G’B’ and Y’CbCr colour spaces; sensor-generated content (former natural content); and computer-generated content (former screen content). (We refer to R’G’B’ and Y’CbCr as RGB and YUV, respectively, in this contribution)

In VTM10.0, the default LMCS encoder algorithm was originally designed for YUV4:2:0 camera content [2]. For YUV4:4:4 (camera content and screen content), additional aspects should be considered:

- Given the same content in YUV4:2:0 and YUV4:4:4 chroma formats, YUV4:4:4 contains four-times the number of chroma pixels than YUV4:2:0. Chroma variance of the YUV4:4:4 signal is much higher than chroma variance of the corresponding YUV4:2:0 signal. This consequently increases chroma bitrate after compression (see Appendix).

- The histogram of luma values for camera (natural) content tends to have a smooth distribution over a wide range whereas the histogram of luma values for screen content tends to have sharp concentrations for relatively few luma values (see Appendix).

# Proposed LMCS encoder algorithm for YUV4:4:4

For LMCS encoder algorithm, there are three major factors to be considered:

- total number of LMCS codewords to be allocated

- slice adaptation in RA case

- chroma residue scaling offset (CRSOffset) value

For all YUV444 content, the total number of LMCS code words allocated is set to 1022, which indicates that full range is used. As for slice adaptation in RA case, for camera content, LMCS is enabled only for inter slices. For screen content, LMCS is enabled only for temporal layer 0 and 1 slices. For CRSOffset, which influences luma-chroma balance, CRSOffset is set to 2 for both camera content and screen content for all intra and CRSOffset is set to 5 (camera content) and 6 (screen content) for random access.

The LMCS encoding parameters are uniform for all test sequences except BirdsInCage and Robot.

- BirdsInCage in camera content contains very high chroma variance compared with luma component. Chroma bitrate increases a lot after compression. In this case, LMCS is enabled for temporal layer 0 and 1 only for RA and set CRSOffset=7 to maintain balance between luma and chroma.

- For Robot in screen content, all pixels concentrate in dark area and the pixel variance for each luminance band are so close that it’s hard to adjust codewords among different bands (LMCS adapt codewords for each band based on pixel variance). In this case, LMCS is disabled both AI and RA.

# Simulation results

Simulation tests were conducted according to SDR YUV4:4:4 CTC on VTM10.0. Table 1 to Table 4 show LMCS tool on (reference: LMCS off, test: LMCS on) performance of YUV4:4:4 for both camera (natural) content and screen content with the proposed LMCS encoder algorithm.

Table 1 LMCS performance of YUV4:4:4 for camera content under AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Proposal LMCS on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | -0.44% | 2.39% | 1.49% | 104% | 105% |
| Kimono | -0.80% | 2.70% | -0.58% | 106% | 105% |
| EBULupoCandlelight | 0.00% | 0.00% | 0.00% | 100% | 99% |
| EBURainFruits | 0.00% | 0.00% | 0.00% | 101% | 99% |
| VenueVu | 0.00% | 0.00% | 0.00% | 100% | 99% |
| BirdsInCage | -0.40% | -2.52% | 3.54% | 106% | 104% |
| CrowdRun | -0.63% | 2.25% | 2.84% | 103% | 93% |
| Overall | -0.32% | 0.69% | 1.04% | 104% | 101% |

Table 2 LMCS performance of YUV4:4:4 for camera content under RA configuration

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Proposal LMCS on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | -1.25% | 3.49% | 4.02% | 105% | 99% |
| Kimono | -2.11% | 2.45% | 2.59% | 105% | 102% |
| EBULupoCandlelight | 0.00% | 0.00% | 0.00% | 100% | 101% |
| EBURainFruits | 0.00% | 0.00% | 0.00% | 100% | 99% |
| VenueVu | 0.00% | 0.00% | 0.00% | 100% | 104% |
| BirdsInCage | -3.25% | 10.55% | -9.41% | 104% | 97% |
| CrowdRun | -1.29% | 4.38% | 4.15% | 104% | 105% |
| Overall | -1.13% | 2.98% | 0.20% | 104% | 100% |

Table 3 LMCS performance of YUV4:4:4 for screen content under AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | YUV 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -1.65% | 2.06% | 1.83% | 101% | 102% |
| TGM 720p | -1.00% | 3.27% | 3.86% | 106% | 101% |
| Animation | -0.88% | 2.60% | 2.65% | 104% | 101% |
| Mixed content | -1.27% | 3.77% | 4.15% | 105% | 101% |
| Overall | -1.22% | 2.89% | 3.08% | 104% | 101% |

Table 4 LMCS performance of YUV4:4:4 for screen content under RA configuration

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | YUV 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -1.05% | 0.47% | 0.53% | 101% | 100% |
| TGM 720p | -1.45% | 0.43% | 0.23% | 102% | 101% |
| Animation | -1.45% | -1.19% | -0.86% | 100% | 100% |
| Mixed content | -2.87% | -0.07% | 0.58% | 100% | 101% |
| Overall | -1.64% | -0.01% | 0.16% | 101% | 100% |

# Conclusion

This proposal improves LMCS encoder algorithm for SDR YUV4:4:4 CTC content. The coding performance for YUV4:4:4 CTC test is consistent with that of YUV4:2:0. It is recommended to adopt the proposed automatic non-normative LMCS encoder algorithm into VTM for YUV4:4:4 content.

# Acknowledgements

We would like to thank Jie Chen from Alibaba for crosschecking the simulation results.

# References

- Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu, “JVET common test conditions and software reference configurations for non-4:2:0 colour formats”, JVET-R2013, Teleconference, 15–24 April 2020.

- F. Pu, S. McCarthy, T. Lu, P. Yin, W. Husak, T. Chen, “AHG10: LMCS encoder improvement”, JVET-O0432, Gothenburg, SE, 3–12 July 2019.

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 10 (VTM 10)”, JVET-S2002, Teleconference, 22 June – 1 July 2020.

- W.-J. Chien, J. Boyce, Y.-W. Chen, R. Chernyak, K. Choi, R. Hashimoto, Y.-W. Huang, H. Jang, R.-L. Liao, S. Liu, “JVET AHG report: Tool reporting procedure and testing (AHG13)”, JVET-S0013, Teleconference, 22 June – 1 July 2020.

- G. Sullivan, J.-R. Ohm, “Meeting Report of the 19th Meeting of the Joint Video Experts Team (JVET)”, JVET-S_Notes_d9, Teleconference, 22 June – 1 July 2020.

# Appendix

## Statistics of YUV4:4:4 and YUV4:2:0

Figure 1 and Figure 2 show two examples of bit allocations between luma and chroma components with LMCS disabled at QP22 and QP37. Chroma components comprise a much higher percentage in YUV4:4:4 than in YUV4:2:0 as expected. Therefore, we should to set LMCS encoder parameters for YUV4:4:4 differently from those for YUV4:2:0 in order to achieve good balance between luma and chroma components.

Figure 1  bits percentages of Traffic (QP22&37) with LMCS disabled

Figure 2 . bits percentages of Kimono (QP22&37) with LMCS disabled

## Camera (natural) content and screen content

Camera (natural) content and screen content have very different characteristics, as shown in Figure 3 and Figure 4, which illustrates some examples of luma histogram for camera content and screen content, respectively. It can be observed that camera (natural) content covers a wider and smoother luma range than does screen content. Thus, different settings are used for LMCS for these two categories.

Figure 3 luma histogram for sensor-generated content (natural content)

Figure 4 luma histogram for computer-generated content (screen content)

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).