    - Film grain characteristics SEI message semantics

This SEI message provides the decoder with a parameterized model for film grain synthesis.

NOTE 1 – For example, an encoder could use the film grain characteristics SEI message to characterize film grain that was present in the original source video material and was removed by pre-processing filtering techniques. Synthesis of simulated film grain on the decoded images for the display process is optional and does not need to exactly follow the specified semantics of the film grain characteristics SEI message. When synthesis of simulated film grain on the decoded images for the display process is performed, there is no requirement that the method by which the synthesis is performed be the same as the parameterized model for the film grain as provided in the film grain characteristics SEI message.

NOTE 2 – The display process is not specified in this Specification.

NOTE 3 – SMPTE RDD 5 specifies a film grain simulator based on the information provided in the film grain characteristics SEI message.

The film grain models specified in the film grain characteristics SEI message are expressed for application to decoded pictures that have 4:4:4 colour format with luma and chroma bit depths corresponding to the luma and chroma bit depths of the film grain model and use the same colour representation domain as the identified film grain model. When the colour format of the decoded video is not 4:4:4 or the decoded video uses a different luma or chroma bit depth from that of the film grain model or uses a different colour representation domain from that of the identified film grain model, an unspecified conversion process is expected to be applied to convert the decoded pictures to the form that is expressed for application of the film grain model.

NOTE 4 – Because the use of a specific method is not required for performing the film grain generation function used by the display process, a decoder could, if desired, down-convert the model information for chroma in order to simulate film grain for other chroma formats (4:2:0 or 4:2:2) rather than up-converting the decoded video (using a method not specified in this Specification) before performing film grain generation.

film_grain_characteristics_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous film grain characteristics SEI message in output order that applies to the current layer. film_grain_characteristics_cancel_flag equal to 0 indicates that film grain modelling information follows.

film_grain_model_id identifies the film grain simulation model as specified in Table D.4. The value of film_grain_model_id shall be in the range of 0 to 1, inclusive. The values of 2 and 3 for film_grain_model_id are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore film grain characteristic SEI messages with film_grain_model_id equal to 2 or 3.

Table D.4 – film_grain_model_id values

| Value | Description |
| --- | --- |
| 0 | Frequency filtering |
| 1 | Auto-regression |

separate_colour_description_present_flag equal to 1 indicates that a distinct combination of luma bit depth, chroma bit depth, video full range flag, colour primaries, transfer characteristics, and matrix coefficients for the film grain characteristics specified in the SEI message is present in the film grain characteristics SEI message syntax. separate_colour_description_present_flag equal to 0 indicates that the combination of luma bit depth, chroma bit depth, video full range flag, colour primaries, transfer characteristics, and matrix coefficients for the film grain characteristics specified in the SEI message are the same as indicated in VUI parameters for the CVS.

NOTE 5 – When separate_colour_description_present_flag is equal to 1, any of the luma bit depth, chroma bit depth, video full range flag, colour primaries, transfer characteristics, and matrix coefficients specified for the film grain characteristics specified in the SEI message could differ from that for the pictures in the CVS.

When VUI parameters are not present for the CVS, and equivalent information is not conveyed by external means, separate_colour_description_present_flag shall be equal to 1.

The decoded image Idecoded used in the equations in this clause is in the same colour representation domain as the simulated film grain signal. Therefore, when any of these parameters does differ from that for the pictures in CVS, the decoded image Idecoded used in the equations in this clause would be in a different colour reporesentation domain than that for the pictures in the CVS. For example, when the value of film_grain_bit_depth_luma_minus8 + 8 is greater than the bit depth of the luma component of the pictures in the CVS, the bit depth of Idecoded used in the equations in this clause is also greater than the bit depth of the luma component of the pictures in the CVS. In such a case, the decoded image Idecoded corresponding to an actual decoded picture would be generated by converting the actual decoded picture to be in the same colour representation domain as the the simulated film grain signal. The process for converting an actual decoded pictures to the 4:4:4 colour format with same colour representation domain as the the simulated film grain signal is not specified in this Specification.

film_grain_bit_depth_luma_minus8 plus 8 specifies the bit depth used for the luma component of the film grain characteristics specified in the SEI message. When film_grain_bit_depth_luma_minus8 is not present in the film grain characteristics SEI message, the value of film_grain_bit_depth_luma_minus8 is inferred to be equal to bit_depth_luma_minus8 − 8.

The value of filmGrainBitDepth[ 0 ] is derived as follows:

filmGrainBitDepth[ 0 ] = film_grain_bit_depth_luma_minus8 + 8	(D-3)

film_grain_bit_depth_chroma_minus8 plus 8 specifies the bit depth used for the Cb and Cr components of the film grain characteristics specified in the SEI message. When film_grain_bit_depth_chroma_minus8 is not present in the film grain characteristics SEI message, the value of film_grain_bit_depth_chroma_minus8 is inferred to be equal to bit_depth_chroma_minus8 − 8.

The value of filmGrainBitDepth[ c ] for c = 1 and 2 is derived as follows:

filmGrainBitDepth[ c ] = film_grain_bit_depth_chroma_minus8 + 8, with c = 1, 2	(D-4)

film_grain_full_range_flag has the same semantics as specified in clause E.3.1 for the video_full_range_flag syntax element, except as follows:

–	film_grain_full_range_flag specifies the video full range flag of the film grain characteristics specified in the SEI message, rather than the video full range flag used for the CVS.

–	When film_grain_full_range_flag is not present in the film grain characteristics SEI message, the value of film_grain_full_range_flag is inferred to be equal to video_full_range_flag.

film_grain_colour_primaries has the same semantics as specified in clause E.3.1 for the colour_primaries syntax element, except as follows:

–	film_grain_colour_primaries specifies the colour primaries of the film grain characteristics specified in the SEI message, rather than the colour primaries used for the CVS.

–	When film_grain_colour_primaries is not present in the film grain characteristics SEI message, the value of film_grain_colour_primaries is inferred to be equal to colour_primaries.

film_grain_transfer_characteristics has the same semantics as specified in clause E.3.1 for the transfer_characteristics syntax element, except as follows:

–	film_grain_transfer_characteristics specifies the transfer characteristics of the film grain characteristics specified in the SEI message, rather than the transfer characteristics used for the CVS.

–	When film_grain_transfer_characteristics is not present in the film grain characteristics SEI message, the value of film_grain_transfer_characteristics is inferred to be equal to transfer_characteristics.

film_grain_matrix_coeffs has the same semantics as specified in clause E.3.1 for the matrix_coeffs syntax element, except as follows:

–	film_grain_matrix_coeffs specifies the matrix coefficients of the film grain characteristics specified in the SEI message, rather than the matrix coefficients used for the CVS.

–	When film_grain_matrix_coeffs is not present in the film grain characteristics SEI message, the value of film_grain_matrix_coeffs is inferred to be equal to matrix_coeffs.

–	The values allowed for film_grain_matrix_coeffs are not constrained by the chroma_format_idc of the decoded pictures that is indicated by the value of chroma_format_idc for the semantics of the VUI parameters.

blending_mode_id identifies the blending mode used to blend the simulated film grain with the decoded images as specified in Table D.5. blending_mode_id shall be in the range of 0 to 1, inclusive. The values of 2 and 3 for blending_mode_id are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore film grain characteristic SEI messages with blending_mode_id equal to 2 or 3.

Table D.5 – blending_mode_id values

| Value | Description |
| --- | --- |
| 0 | Additive |
| 1 | Multiplicative |

Depending on the value of blending_mode_id, the blending mode is specified as follows:

–	If blending_mode_id is equal to 0, the blending mode is additive as specified by:

Igrain[ x, y, c ] = Clip3( 0, ( 1  <<  filmGrainBitDepth[ c ] ) − 1, Idecoded[ x, y, c ] + G[ x, y, c ] )	(D-5)

–	Otherwise (blending_mode_id is equal to 1), the blending mode is multiplicative as specified by:

Igrain[ x, y, c ] = Clip3( 0, ( 1  <<  filmGrainBitDepth[ c ] ) − 1, Idecoded[ x, y, c ] +	(D-6)
			Round( ( Idecoded[ x, y, c ] * G[ x, y, c ] ) ÷ ( ( 1  <<  bitDepth[ c ] ) − 1 ) ) )

where Idecoded[ x, y, c ] represents the sample value at coordinates x, y of the colour component c of the decoded image Idecoded, G[ x, y, c ] is the simulated film grain value at the same position and colour component, and filmGrainBitDepth[ c ] is the number of bits used for each sample in a fixed-length unsigned binary representation of the arrays Igrain[ x, y, c ], Idecoded[ x, y, c ], and G[ x, y, c ], where c = 0..2, x = 0..pic_width_in_luma_samples − 1, and y = 0..pic_height_in_luma_samples – 1

log2_scale_factor specifies a scale factor used in the film grain characterization equations.

comp_model_present_flag[ c ] equal to 0 indicates that film grain is not modelled on the c-th colour component, where c equal to 0 refers to the luma component, c equal to 1 refers to the Cb component, and c equal to 2 refers to the Cr component. comp_model_present_flag[ c ] equal to 1 indicates that syntax elements specifying modelling of film grain on colour component c are present in the SEI message.

When separate_colour_description_present_flag is equal to 0 and chroma_format_idc is equal to 0, the value of comp_model_present_flag[ 1 ] and comp_model_present_flag[ 2 ] shall be equal to 0.

num_intensity_intervals_minus1[ c ] plus 1 specifies the number of intensity intervals for which a specific set of model values has been estimated.

NOTE 6 – The intensity intervals could overlap in order to simulate multi-generational film grain.

num_model_values_minus1[ c ] plus 1 specifies the number of model values present for each intensity interval in which the film grain has been modelled. The value of num_model_values_minus1[ c ] shall be in the range of 0 to 5, inclusive.

intensity_interval_lower_bound[ c ][ i ] specifies the lower bound of the i-th intensity interval for which the set of model values applies.

intensity_interval_upper_bound[ c ][ i ] specifies the upper bound of the i-th intensity interval for which the set of model values applies.

The variable intensityIntervalIdx[ x, y, c, j ] represents the j-th index to the list of intensity intervals selected for the sample value Idecoded[ x, y, c ] for c = 0..2, x = 0..pic_width_in_luma_samples − 1, y = 0..pic_height_in_luma_samples − 1, and j = 0..numApplicableIntensityIntervals[ x, y, c ] − 1, where numApplicableIntensityIntervals[ x, y, c ] is derived below.

Depending on the value of film_grain_model_id, the selection of one or more intensity intervals for the sample value Idecoded[ x, y, c ] is specified as follows:

–	The variable numApplicableIntensityIntervals[ x, y, c ] is initially set equal to 0.

–	If film_grain_model_id is equal to 0, the following applies:

- The top-left sample location ( xB, yB ) of the current 8x8 block b that contains the sample value Idecoded[ x, y, c ] is derived as ( xB, yB ) = ( x / 8, y / 8 ).

- The average value bavg of the current 8x8 block is derived as follows:

sum8x8 = 0
for( i = 0; i  < 8; i++ )
	for( j = 0; j < 8; j++ )
	     sum8x8  +=  Idecoded[ xB * 8 + I, yB * 8 + j, c ]	(D-7)
bavg = Clip3( 0, 255, ( sum8x8 + ( 1  <<  ( filmGrainBitDepth[ c ] − 3 ) ) )  >>  ( filmGrainBitDepth[ c ] − 2 ) )

- The value of intensityIntervalIdx[ x, y, c, j ] is derived as follows:

for( i = 0, j = 0; i  <=  num_intensity_intervals_minus1[ c ]; i++ )
	if( bavg  >=  intensity_interval_lower_bound[ c ][ i ]  
			&&  bavg  <=  intensity_interval_upper_bound[ c ][ i ] ) {
		intensityIntervalIdx[ x, y, c, j ] = i		(D-8)
		j++
	}
numApplicableIntensityIntervals[ x, y, c ] = j

–	Otherwise (film_grain_model_id is equal to 1), the value of intensityIntervalIdx[ x, y, c, j ] is derived as follows:

I8[ x, y, c ] = ( filmGrainBitDepth[ c ]  = =  8 ) ? ( Idecoded[ x, y, c ] :
		Clip3( 0, 255, ( Idecoded[ x, y, c ] + ( 1  <<  ( filmGrainBitDepth[ c ] − 9 ) ) )  >>  ( filmGrainBitDepth[ c ] − 8 ) )
for( i = 0, j = 0; i  <=  num_intensity_intervals_minus1[ c ]; i++ )
	if( I8[ x, y, c ]  >=  intensity_interval_lower_bound[ c ][ i ]  &&  
			I8[ x, y, c ]  <=  intensity_interval_upper_bound[ c ][ i ] ) {	(D-9)
		intensityIntervalIdx[ x, y, c, j ] = i
		j++
	}
numApplicableIntensityIntervals[ x, y, c ] = j

Samples that do not fall into any of the defined intervals (i.e., those samples for which the value of numApplicableIntensityIntervals[ x, y, c ] is equal to 0) are not modified by the grain generation function. Samples that fall into more than one interval (i.e., those samples for which the value of numApplicableIntensityIntervals[ x, y, c ] is greater than 1) will originate multi-generation grain. Multi-generation grain results from adding the grain computed independently for each ot the applicable intensity intervals.

In the equations in the remainder of this clause, the variable sj in each instance of the list comp_model_value[ c ][ sj ] is the value of intensityIntervalIdx[ x, y, c, j ] derived for the sample value Idecoded[ x, y, c ].

comp_model_value[ c ][ i ][ j ] specifies the j-th model value present for the colour component c and the i-th intensity interval. The set of model values has different meaning depending on the value of film_grain_model_id.

The value of comp_model_value[ c ][ i ][ j ] is constrained as follows, and could be additionally constrained as specified elsewhere in this clause:

–	If film_grain_model_id is equal to 0, comp_model_value[ c ][ i ][ j ] shall be in the range of 0 to 2filmGrainBitDepth[ c ] − 1, inclusive.

–	Otherwise (film_grain_model_id is equal to 1), comp_model_value[ c ][ i ][ j ] shall be in the range of −2( filmGrainBitDepth[ c ] − 1 ) to 2( filmGrainBitDepth[ c ] − 1 ) − 1, inclusive.

Depending on the value of film_grain_model_id, the synthesis of the film grain is modelled as follows:

–	If film_grain_model_id is equal to 0, a frequency filtering model enables simulating the original film grain for c = 0..2, x = 0..pic_width_in_luma_samples − 1 and y = 0..pic_height_in_luma_samples − 1 as specified by:

G[ x, y, c ] = ( comp_model_value[ c ][ sj ][ 0 ] * Q[ c ][ x, y ] + comp_model_value[ c ][ sj ][ 5 ] *
			G[ x, y, c − 1 ] )  >>  log2_scale_factor	(D-10)

where Q[ c ] is a two-dimensional random process generated by filtering 16x16 blocks gaussRv with random-value elements gaussRvij generated with a normalized Gaussian distribution (independent and identically distributed Gaussian random variable samples with zero mean and unity variance) and where the value of an element G[ x, y, c − 1 ] used in the right-hand side of the equation is inferred to be equal to 0 when c − 1 is less than 0.

NOTE 7 – A normalized Gaussian random variable can be generated from two independent, uniformly distributed random values over the interval from 0 to 1 (and not equal to 0), denoted as uRv0 and uRv1, using the Box-Muller transformation specified by:

gaussRvi,j = Sqrt( −2 * Ln( uRv0 ) ) * Cos( 2 * π * uRv1 )	(D-11)

where  is Archimedes' constant 3.141 592 653 589 793....

The band-pass filtering of blocks gaussRv can be performed in the discrete cosine transform (DCT) domain as follows:

for( y = 0; y < 16; y++ )
	for( x = 0; x < 16; x++ )
		if( ( x < comp_model_value[ c ][ sj ][ 3 ]  &&  y < comp_model_value[ c ][ sj ][ 4 ] )  | |	(D-12)
				x > comp_model_value[ c ][ sj ][ 1 ]  | |  y > comp_model_value[ c ][ sj ][ 2 ] )
			gaussRv[ x, y ] = 0
filteredRv = IDCT16x16( gaussRv )

where IDCT16x16( z ) refers to a unitary inverse discrete cosine transformation (IDCT) operating on a 16x16 matrix argument z as specified by:

IDCT16x16( z ) = r * z * rT		(D-13)

where the superscript T indicates a matrix transposition and r is the 16x16 matrix with elements rij specified by:

ri,j = (  i  = =  0  ? 1  : Sqrt( 2 )4* Cos i *  2 * j + 1   * π32 	(D-14)

where  is Archimedes' constant 3.141 592 653 589 793....

Q[ c ] is formed by the frequency-filtered blocks filteredRv.

NOTE 8 – Coded model values are based on blocks of size 16x16, but a decoder implementation could use other block sizes. For example, decoders implementing the IDCT on 8x8 blocks could down-convert by a factor of two the set of coded model values comp_model_value[ c ][ sj ][ i ] for i equal to 1..4.

NOTE 9 – To reduce the degree of visible blocks can result from mosaicking the frequency-filtered blocks filteredRv, decoders could apply a low-pass filter to the boundaries between frequency-filtered blocks.

–	Otherwise (film_grain_model_id is equal to 1), an auto-regression model enables simulating the original film grain for c = 0..2, x = 0..pic_width_in_luma_samples – 1, and y = 0..pic_height_in_luma_samples − 1 as specified by:

G[ x, y, c ] = ( comp_model_value[ c ][ sj ][ 0 ] * n[ x, y, c ] +
comp_model_value[ c ][ sj ][ 1 ] * ( G[ x − 1, y, c ] + ( ( comp_model_value[ c ][ sj ][ 4 ] * G[ x, y − 1, c ] )  >>
		log2_scale_factor ) ) +
comp_model_value[ c ][ sj ][ 3 ] * ( ( ( comp_model_value[ c ][ sj ][ 4 ] * G[ x − 1, y − 1, c ] )  >>
		log2_scale_factor ) + G[ x + 1, y − 1, c ] ) +
comp_model_value[ c ][ sj ][ 5 ] * ( G[ x − 2, y, c ] +
		( ( comp_model_value[ c ][ sj ][ 4 ] * comp_model_value[ c ][ sj ][ 4 ] * G[ x, y − 2, c ] )  >>
			( 2 * log2_scale_factor ) ) ) +
	comp_model_value[ c ][ sj ][ 2 ] * G[ x, y, c − 1 ] )  >>  log2_scale_factor	(D-15)

where n[ x, y, c ] is a random value with normalized Gaussian distribution (independent and identically distributed Gaussian random variable samples with zero mean and unity variance for each value of x, y and c) and where the value of an element G[ x, y, c ] used in the right-hand side of the equation is inferred to be equal to 0 when any of the following conditions are true:

–	x is less than 0,

–	y is less than 0,

–	c is less than 0.

comp_model_value[ c ][ i ][ 0 ] provides the first model value for the model as specified by film_grain_model_id. comp_model_value[ c ][ i ][ 0 ] corresponds to the standard deviation of the Gaussian noise term in the generation functions specified in Equations D-10 through D-15.

comp_model_value[ c ][ i ][ 1 ] provides the second model value for the model as specified by film_grain_model_id. When film_grain_model_id is equal to 0, comp_model_value[ c ][ i ][ 1 ] shall be greater than or equal to 0 and less than 16.

When not present in the film grain characteristics SEI message, comp_model_value[ c ][ i ][ 1 ] is inferred as follows:

–	If film_grain_model_id is equal to 0, comp_model_value[ c ][ i ][ 1 ] is inferred to be equal to 8.

–	Otherwise (film_grain_model_id is equal to 1), comp_model_value[ c ][ i ][ 1 ] is inferred to be equal to 0.

comp_model_value[ c ][ i ][ 1 ] is interpreted as follows:

–	If film_grain_model_id is equal to 0, comp_model_value[ c ][ i ][ 1 ] indicates the horizontal high cut frequency to be used to filter the DCT of a block of 16x16 random values.

–	Otherwise (film_grain_model_id is equal to 1), comp_model_value[ c ][ i ][ 1 ] indicates the first order spatial correlation for neighbouring samples ( x − 1, y ) and ( x, y − 1 ).

comp_model_value[ c ][ i ][ 2 ] provides the third model value for the model as specified by film_grain_model_id. When film_grain_model_id is equal to 0, comp_model_value[ c ][ i ][ 2 ] shall be greater than or equal to 0 and less than 16.

When not present in the film grain characteristics SEI message, comp_model_value[ c ][ i ][ 2 ] is inferred as follows:

–	If film_grain_model_id is equal to 0, comp_model_value[ c ][ i ][ 2 ] is inferred to be equal to comp_model_value[ c ][ i ][ 1 ]

–	Otherwise (film_grain_model_id is equal to 1), comp_model_value[ c ][ i ][ 2 ] is inferred to be equal to 0.

comp_model_value[ c ][ i ][ 2 ] is interpreted as follows:

–	If film_grain_model_id is equal to 0, comp_model_value[ c ][ i ][ 2 ] indicates the vertical high cut frequency to be used to filter the DCT of a block of 16x16 random values.

–	Otherwise (film_grain_model_id is equal to 1), comp_model_value[ c ][ i ][ 2 ] indicates the colour correlation between consecutive colour components.

comp_model_value[ c ][ i ][ 3 ] provides the fourth model value for the model as specified by film_grain_model_id. When film_grain_model_id is equal to 0, comp_model_value[ c ][ i ][ 3 ] shall be greater than or equal to 0 and less than or equal to comp_model_value[ c ][ i ][ 1 ].

When not present in the film grain characteristics SEI message, comp_model_value[ c ][ i ][ 3 ] is inferred to be equal to 0.

comp_model_value[ c ][ i ][ 3 ] is interpreted as follows:

–	If film_grain_model_id is equal to 0, comp_model_value[ c ][ i ][ 3 ] indicates the horizontal low cut frequency to be used to filter the DCT of a block of 16x16 random values.

–	Otherwise (film_grain_model_id is equal to 1), comp_model_value[ c ][ i ][ 3 ] indicates the first order spatial correlation for neighbouring samples ( x − 1, y − 1 ) and ( x + 1, y − 1 ).

comp_model_value[ c ][ i ][ 4 ] provides the fifth model value for the model as specified by film_grain_model_id. When film_grain_model_id is equal to 0, comp_model_value[ c ][ i ][ 4 ] shall be greater than or equal to 0 and less than or equal to comp_model_value[ c ][ i ][ 2 ].

When not present in the film grain characteristics SEI message, comp_model_value[ c ][ i ][ 4 ] is inferred to be equal to film_grain_model_id.

comp_model_value[ c ][ i ][ 4 ] is interpreted as follows:

–	If film_grain_model_id is equal to 0, comp_model_value[ c ][ i ][ 4 ] indicates the vertical low cut frequency to be used to filter the DCT of a block of 16x16 random values.

–	Otherwise (film_grain_model_id is equal to 1), comp_model_value[ c ][ i ][ 4 ] indicates the aspect ratio of the modelled grain.

comp_model_value[ c ][ i ][ 5 ] provides the sixth model value for the model as specified by film_grain_model_id.

When not present in the film grain characteristics SEI message, comp_model_value[ c ][ i ][ 5 ] is inferred to be equal to 0.

comp_model_value[ c ][ i ][ 5 ] is interpreted as follows:

–	If film_grain_model_id is equal to 0, comp_model_value[ c ][ i ][ 5 ] indicates the colour correlation between consecutive colour components.

–	Otherwise (film_grain_model_id is equal to 1), comp_model_value[ c ][ i ][ 5 ] indicates the second order spatial correlation for neighbouring samples ( x, y − 2 ) and ( x − 2, y ).

film_grain_characteristics_persistence_flag specifies the persistence of the film grain characteristics SEI message for the current layer.

film_grain_characteristics_persistence_flag equal to 0 specifies that the film grain characteristics SEI message applies to the current decoded picture only.

Let picA be the current picture. film_grain_characteristics_persistence_flag equal to 1 specifies that the film grain characteristics SEI message persists for the current layer in output order until any of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing a film grain characteristics SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

____________________