| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0076-v2 |
| --- | --- |

| Title: | AHG9: ALF control SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Boon Teo Hai Wei Sun Chong Soon Lim | Tel: Email: | +65-6550-5417 hanboon.teo@sg.panasonic.com haiwei.sun@sg.panasonic.com chongsoon.lim@sg.panasoni.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This contribution is a further follow-up proposal on JVET-R0190, JVET-Q0319 and JVET-P0179. In the case when ALF module is disabled through bitstream header, post-filter hint based on ALF control SEI messages are decoded. The proposed SEI message is used to enable the ALF module to filter the reconstructed image for output using the accompanied ALF filter flags and filter coefficients. This contribution provides evidences about possible application cases and its benefit. The source code for encoder and decoder implementations are available.

# Introduction

## ALF Control SEI

In the VTM10, an Adaptive Loop Filter (ALF) with block-based filter adaption is applied when ALF module is enabled in the bitstream (Figure 1).

Figure 1 – VTM10 decoder block diagram

When ALF module is disabled in the bitstream header, the decoder block diagram becomes Figure 2.

Figure 2 – VTM10 decoder block diagram when ALF is disabled

The adaptive loop filter control SEI (ALF control SEI) was proposed in JVET-Q0319 [1] and JVET-P0179 [2]. The SEI is used to enable the ALF module to filter the reconstructed image for output (Figure 3).

Figure 3 – VTM10 decoder block diagram when ALF is disabled with proposed SEI

## Software implementation

For effective use of adaptive loop filter control SEI, one implementation method is suggested. In VTM10, maxDpbPicBuf size is 8. Figure 4 shows an example of VTM10 DPB buffer implementation where each buffer space consists of a flag to indicate “needed/not needed for output” and a flag to indicate “used/unused for short/long-term reference”.

Figure 4 – VTM10 DPB buffer implementation

Figure 5 shows an example of the proposed DPB buffer implementation. Each buffer space consists of:

- A set of flag and memory space to indicate “used/unused for short/long-term reference” and to store reconstructed image (before ALF), and

- A set of flag and memory space to indicate “needed/not needed for output” and to store images for output/display (e.g. ALF filtered reconstructed image).

The buffer space which holds the reconstructed image (before ALF) will be referred to in the decoding loop while the buffer space which holds the ALF filtered reconstructed image will be used for outputting to the display.

Figure 5 – Proposed DPB buffer implementation

# Use case applications

ALF is designed in VTM to improve PSNR quality. Its filter coefficients are calculated in the encoder and the filter sets with the lowest rate distortion costs are selected and signalled in APS. In our proposal, we are exploiting the classification feature of ALF for image control and not only for quality improvement. The post-filter hint SEI in HEVC has the similar concept. The categorizing (classification) is an important feature of ALF module which enables finer image control.

Table 1 shows the advantages of using the proposal with VVC. In the pre-filter approach, bits will increase when certain type of filters (example sharpening filters) are used. In the post-filter hint SEI (in HEVC) approach, only one type of filter can be applied to the whole frame at a time. Our proposal eliminates the bad effect caused by the pre-filter approach and at the same time shows better flexibility then the post-filter hint SEI approach with the capability of utilizing up to 25 sets of luma filters. On top of these benefits, our proposal doesn’t need additional hardware/software filter modules to be added to the codec as ALF module is already part of VVC.

Table 1 Advantages of proposed comparing to different methods of apply filters

| Comparisons | Pre-filter | Post-filter hint SEI | Proposed |
| --- | --- | --- | --- |
| Additional modules needed | Separate module | Separate module | Built-in VVC |
| Customization level | Determined by content provider, but typically: Picture level on/off No classification 1 set of filters | Picture level on/off | CTU level on/off |
|  |  | No classification | 4x4 level luma block classification |
|  |  | 1 set of filters for luma, 1 set of filters for Cb, 1 set of filters for Cr. | Up to 25 sets of luma filters, 1 set of filters for Cb, 1 set of filters for Cr. |
| E.g. Effect of using of sharpening filters | Sharpening original source | At same bitrate, smaller QP used compared to pre-filter. | At same bitrate, smaller QP used compared to pre-filter. |

## Application flowchart

A typical use case of the ALF control SEI is in post-production. In the case of User-Generated Content (UGC), the user captures video at the instance. Then by reviewing the video together with each classification, the user can design the type of video effects he/she desires. User can easily test each video effect by just appending the ALF control SEI to the bitstream. After that the user can share the appended bitstream to others (for example, upload to social media platforms) to show the video effect.

Figure 6 Application flow

## Use case examples

RaceHorses_832x480_30.yuv original

QP22 with ALF on: bitrate 4186.3bps, Y-PSNR 39.56dB

QP32 with ALF on: bitrate 787.5bps, Y-PSNR 33.79dB

QP32 with ALF off: bitrate 783.0bps, Y-PSNR 33.67dB

QP32 with ALF off plus ALF control SEI (2 sharpening filters): 2 types of sharpening filters are used to avoid image overexposing due to strong filters. Visually, video looks sharper than before.

QP32 with ALF off plus ALF control SEI (1 sharpening filter, 2 types of blur filters): Bokeh effect is achieved by the strategic use of sharpening and blur filters.

With the addition of ALF control SEI messages, it is possible to customize different types of video effects on the same bitstream.

# Proposal Syntax

## VSEI specification changes

8.x		Adaptive loop filter control SEI message

8.x.1	Adaptive loop filter control SEI message syntax

| adaptive_loop_filter_control( payloadSize ) { | Descriptor |
| --- | --- |
| alf_luma_filter_enable_flag | u(1) |
| alf_cb_filter_enable_flag | u(1) |
| alf_cr_filter_enable_flag | u(1) |
| alf_cc_cb_filter_enable_flag | u(1) |
| alf_cc_cr_filter_enable_flag | u(1) |
| alf_data( ) |  |
| } |  |

8.x.2	Adaptive loop filter control SEI message semantics

The adaptive loop filter control SEI message provides information to use the adaptive loop filter to modify the pictures for output. When alfEnabledFlag is equal to 1, it is a requirement of bitstream conformance that there shall be no adaptive loop filter control SEI messages present in the CLVS.

alf_luma_filter_enable_flag equal to 1 specifies that adaptive loop filter is applied to the luminance component. alf_luma_filter_enable_flag equal to 0 specifies that adaptive loop filter is not applied to the luminance component. When alf_luma_filter_enable_flag is not present, it is inferred to be equal to 0. If alf_luma_filter_enable_flag equal to 1, alfCtbYFlag for all ctb in the luminance component shall be equal to 1.

alf_cb_filter_enable_flag equal to 1 specifies that adaptive loop filter is applied to the Cb color component. alf_cb_filter_enable_flag equal to 0 specifies that adaptive loop filter is not applied to the Cb color component. When alf_cb_filter_enable_flag is not present, it is inferred to be equal to 0. If alf_cb_filter_enable_flag equal to 1, alfCtbCbFlag for all ctb in the Cb color component shall be equal to 1.

alf_cr_filter_enable_flag equal to 1 specifies that adaptive loop filter is applied to the Cr color component. alf_cr_filter_enable_flag equal to 0 specifies that adaptive loop filter is not applied to the Cr color component. When alf_cr_filter_enable_flag is not present, it is inferred to be equal to 0. If alf_cr_filter_enable_flag equal to 1, alfCtbCrFlag for all ctb in the Cr color component shall be equal to 1.

alf_cc_cb_filter_enable_flag equal to 1 specifies that cross-component filter is applied to the Cb color component. alf_cc_cb_filter_enable_flag equal to 0 specifies that cross-component filter is not applied to the Cb color component. When alf_cc_cb_filter_enable_flag is not present, it is inferred to be equal to 0.

alf_cc_cr_filter_enable_flag equal to 1 specifies that cross-component filter is applied to the Cr color component. alf_cc_cr_filter_enable_flag equal to 0 specifies that cross-component filter is not applied to the Cr color component. When alf_cc_cr_filter_enable_flag is not present, it is inferred to be equal to 0.

When ChromaFormatIdc is equal to 0, alf_cb_filter_enable_flag, alf_cr_filter_enable_flag, alf_cc_cb_filter_enable_flag and alf_cc_cr_filter_enable_flag shall be equal to 0.

At least one of the values of alf_luma_filter_enable_flag, alf_cb_filter_enable_flag, alf_cr_filter_enable_flag, alf_cc_cb_filter_enable_flag, and alf_cc_cr_filter_enable_flag shall be equal to 1.

alf_data() syntax is specified in Rec. ITU-T H.266 | ISO/IEC 23090-3.

## VVC specification changes

D.2.1	General SEI payload syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| else if( payloadType = = 1 ) |  |
| pic_timing( payloadSize ) |  |
| else if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| else if( payloadType = = 4 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| user_data_registered_itu_t_t35( payloadSize ) |  |
| else if( payloadType = = 5 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| user_data_unregistered( payloadSize ) |  |
| else if( payloadType = = 19 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| film_grain_characteristics( payloadSize ) |  |
| else if( payloadType = = 45 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| frame_packing_arrangement( payloadSize ) |  |
| else if( payloadType = = 129 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| parameter_sets_inclusion_indication( payloadSize ) |  |
| else if( payloadType = = 130 ) |  |
| decoding_unit_info( payloadSize ) |  |
| else if( payloadType = = 133 ) |  |
| scalable_nesting( payloadSize ) |  |
| else if( payloadType = = 137 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| mastering_display_colour_volume( payloadSize ) |  |
| else if( payloadType = = 144 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| content_light_level_info( payloadSize ) |  |
| else if( payloadType = = 145 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| dependent_rap_indication( payloadSize ) |  |
| else if( payloadType = = 147 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| alternative_transfer_characteristics( payloadSize ) |  |
| else if( payloadType = = 148 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| ambient_viewing_environment( payloadSize ) |  |
| else if( payloadType = = 149 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| content_colour_volume( payloadSize ) |  |
| else if( payloadType = = 150 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| equirectangular_projection( payloadSize ) |  |
| else if( payloadType = = 153 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| generalized_cubemap_projection( payloadSize ) |  |
| else if( payloadType = = 154 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| sphere_rotation( payloadSize ) |  |
| else if( payloadType = = 155 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| regionwise_packing( payloadSize ) |  |
| else if( payloadType = = 156 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| omni_viewport( payloadSize ) |  |
| else if( payloadType = = 168 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| frame_field_info( payloadSize ) |  |
| else if( payloadType = = 195 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| adaptive_loop_filter_control( payloadSize ) |  |
| else if( payloadType = = 203 ) |  |
| subpic_level_info( payloadSize ) |  |
| else if( payloadType = = 204 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| sample_aspect_ratio_info( payloadSize ) |  |
| else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| reserved_message( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |
| if( payloadType = = 3 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| filler_payload( payloadSize ) |  |
| if( payloadType = = 132 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| decoded_picture_hash( payloadSize ) |  |
| else if( payloadType = = 133 ) |  |
| scalable_nesting( payloadSize ) |  |
| else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| reserved_message( payloadSize ) |  |
| if( more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| sei_reserved_payload_extension_data | u(v) |
| sei_payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| sei_payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

D.9.7	Use of the adaptive loop filter control SEI message

For purposes of interpretation of the adaptive loop filter control SEI message, the following variables are specified:

–	ChromaFormatIdc is set equal to sps_chroma_format_idc.

–	alfEnabledFlag is set equal to sps_alf_enabled_flag.

–	alfCtbYFlag is set equal to alf_ctb_flag[0].

–	alfCtbCbFlag is set equal to alf_ctb_flag[1].

–	alfCtbCrFlag is set equal to alf_ctb_flag[2].

When an adaptive loop filter control SEI message is present, the following constraint apply:

–	The sps_alf_enabled_flag shall be equal to 0.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).