| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting: by teleconference, 7 Oct – 16 Oct 2020 | Document: JVET-T0078 |
| --- | --- |

| Title: | GDR Software |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seungwook Hong Limin Wang Krit Panusopone | Email: | seungwook.hong@nokia.com limin.2.wang@nokia.com krit.panusopone@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution includes the source code to enable GDR functionality for VVC and proposes adding the software in the VTM source code package. The software offers the following setups:

- Encoding input video sequences into GDR bitstreams using the VTM encoder with the following features:

  - Flexible GDR period configured through parameter set by user,

  - Similar number of bits per picture, implying the delay is as low as one frame interval,

  - Progressive intra refresh over a GDR period with even distribution of the forced intra areas over pictures within the GDR period using virtual boundary syntax in picture header,

  - Necessary (encoding) constraints on coding tools to prevent the leaks and

  - Exact match at the recovery point (or leak-free).

- Decoding the leak-free GDR bitstream using the VTM decoder.

# Introduction

VVC spec supports gradual decoding refresh (GDR). However, VTM reference software does not fully support GDR feature. It is desirable to make the source code of the GDR software available and maintained as a part of the VTM package. Our motivation and asserted benefits for the GDR setup include the followings:

- It is envisioned that there could be VVC decoder implementations that provide support for GDR. An intent of the GDR software is to allow creation of GDR bitstream needed to verify that any decoders can be used to decode a VVC bitstream with GDR.

- It is assumed that encoding of GDR can be achieved with the tested encoding setup. With the given approach, the encoding approach used in the experiments could be applied e.g. ultra low delay (one frame delay) setup.

- One of the goals of the GDR design in the VVC standard is to enable straightforward low delay decoding. The GDR software intends to serve as evidence that this design goal has been successfully met.

- The GDR software can be regarded as an independent implementation that generates bitstreams of GDR. However, it is remarked that GDR software shares most VTM source code modules for low-level coding processes, high-level syntax parsing and generation.

- The system setup can also be used to verify that the VTM decoder is capable of decoding GDR bitstreams without leak at recovery point.

# Software

This contribution includes the source code for the GDR software and proposes adding the software in the VTM source code package. In this GDR software,

- GDR period can be flexible. For example, a GDR period can cover all the pictures of a full second.

- Vertical progressive intra refresh over a GDR period is implemented. The width of vertical intra refresh strip per picture can be as small as 8 pixels. For GDR period equal to one second, the width of vertical intra refresh strip is calculated based on the frame rate and the picture’s width.

- The boundary of clean area and dirty area is specified by virtual boundary syntax in picture header.

- CUs in clean area will not use any coding information in dirty areas of the current picture and the reference pictures to prevent leaks and achieve exact match at the recovery point.

- For CUs in clean area, coding modes involving any coding information in dirty are considered as invalid.  Mode selection for CUs in clean area is performed based upon both the coding costs and the validities of possible coding modes.

- Decoder also provides an optional functionality for leak testing.

The GDR software can be run with low-delay B configuration in current common test condition (CTC) (i.e., encoder_lowdelay_vtm.cfg) with a few necessary changes. A sample confg file for GDR is provided, where

- The first picture is coded as an IDR picture,

- DecodingRefreshType is 3,

- GOPSize is 1.

- Each picture refers up to 4 immediately preceding pictures in decoding order,

- IntraQPOffset and QPoffset are set equal to 0.

- SEIDecodedPictureHash is set to 1.

- Disable in-loop filters across virtual boundaries,

- GDR refresh period is set to frame rate.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).