| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0079-v3 |
| --- | --- |

| Title: | AHG11: Neural Network-based In-Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongtao Wang Marta Karczewicz Jianle Chen Anand Meher Kotra | Email: | hangtaow@qti.qualcomm.commartak@qti.qualcomm.com cjianle@qti.qualcomm.com akotra@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a Neural Network-based In-loop filter (NN filter). The proposed NN filter adds an additional in-loop filter stage and is positionally placed after the Adaptive loop filter (ALF).

It is observed that averagely 4.11%, 13.83% and 13.28% BD rate saving for RA, and 3.63%, 9.47% and 9.98% BD rate saving for AI, for Y, Cb and Cr components respectively, are achieved.

# Introduction

VVC[1] consists of mainly three cascaded loop filters namely Deblocking Filter, Sample Adaptive Offset (SAO) and Adaptive Loop Filter (ALF). Deblocking filter is mainly used to attenuate blocking artifacts. SAO filter mainly filters ringing artifacts. ALF tries to minimize the mean square error between original samples and reconstructed samples using Wiener-based adaptive filter coefficients.

Convolutional Neural Networks (CNN)-based methods recently showed lot of potential in artifact removal and objective quality improvement. During the course of the VVC standardization they were briefly investigated in few core experiment cycles [2][3]. However due to the high computational complexity of the investigated methods, they were not further considered for adoption in version 1 of VVC.

In this contribution, a Neural Network based In-Loop Filter is proposed as an additional In-loop filter stage and is positionally placed after the ALF filter. Therefore, the output samples of ALF are given as input samples to the NN filter.

Simulation results show that the proposed filter can still bring considerable objective gain on top of VTM-10.

# Proposal

## Proposed NN Filter

The proposed NN filter is illustrated in Figure 1. It contains 14 layers, including 12 hidden layers with 96 channels in each layer. Each hidden layer consists of a 3x3 convolutional layer and a Leaky RELU. There are about 1.0 million model parameters in total.

Figure 1. Proposed network architecture

The inputs of the NN filtering process are a 128x128 size luma block and 2 64x64 chroma blocks. The luma samples are interleaved into four 64x64 size blocks before used as inputs of the filtering process as proposed in [5].

The detailed information of the proposed network in the inference stage is listed in Table 1.

Table 1 Network information in the inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | XEON_E5_2690v4 256 GB Memory No GPU |
|  | SW environment: | SUSE Linux Enterprise Server 11 |
|  | Framework: | Tensorflow 2.2.0 |
|  | Total Conv. Layers | 14 |
|  | Total FC Layers | 0 |
|  | Total Parameter Number | 1,006,950 |
|  | Parameter Precision | 32(F) |
|  | Memory Parameter (MB) | 3.84 MB |
|  | Memory Temp (MB) | 1.5 (input size 128x128) |
|  | MAC (Giga) | 3.8 (input size 128x128) |
| Optional | Preprocessing: | Convert 128x128 YUV420 signal to 6 64x64 blocks. Normalize to 0~1 |
|  | Other information: |  |

## Training process

### Training based on the DIV2K dataset

The DIV2K dataset [6] was used in our training. The raw images are compressed by VTM-10.0 with AI configuration and the reconstructed frames are obtained as the training dataset. The QPs used for training process are 20, 25, 30, 35, 40 and 45. During the training, the reconstructed pictures were split into 128x128 luma and 64x64 chorma blocks. The batch size is set to 64.

Each model was trained with 400 epochs. Additional information of the network training process can be found in Table 2.

Table 2 Network information in the training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | CPU: Intel(R) Xeon(R) Gold 6138 CPU @ 2.00GHz, 754GB Memory GPU: NVIDIA Tesla V100-SXM2-32GB |
|  | SW environment: | Ubuntu 18.04.4 LTS CUDA Version:10.1, Driver Version: 418.87.00 |
|  | Framework: | TensorFlow 2.2.0 |
|  | Epoch: | 400 |
|  | Batch size: | 1.9K * 64 |
|  | Training time: | 32h |
|  | Training data information: | DIV2K |
|  | Configurations for generating compressed training data (if different to VTM CTC): | QP values: 20, 25, 30, 35, 40, 45 |
| Optional |  |  |
|  | Patch size | 128x128 |
|  | Learning rate: | 1e-3 |
|  | Optimizer: | Adam |
|  | Loss function: | L2 |
|  | Preprocessing: | Convert 128x128 YUV420 signal to 6 64x64 blocks. Normalize to 0~1 |
|  | Other information: |  |

### Model refinement for inter frames

Additional models trained using the BVI-DVC dataset[7] are also tested in this contribution. To create the training set, the first 17 frames of each training sequence were compressed by the encoder with models trained as described in section 2.2.1 used as an additional in-loop filter. For each of the coded frame with temporal level id equals to 4 or 5 (the 2 highest temporal levels), 10 CTUs were selected randomly as training examples. The models trained on DIV2K dataset were used as start points and 100 epochs were trained. Other training configurations are set to be same with those described in section 2.2.1.

## Implementation in VTM-10.0

The proposed NN filter is incorporated into VTM-10.0 as in-loop filter after ALF, with CTU level switching (on/off) flags.

At the encoder side, RD selection was performed on a set of candidate models and the index of the select model is signaled in the picture header. Simulation results of 3 different methods of constructing the candidate set are reported in this contribution:

- RD selection based on the 2 closest QP models: The RD selection process is applied to the two DIV2K models trained based on the two closest QPs to the current input QP, and a 1-bit flag is signaled in the picture header to indicate which NN model is applied to the current picture.

- RD selection based on all the models: All the 6 models (QP=20,25,30,35,40,45) trained on the DIV2K dataset are always used as the candidate set. A 3-bit index value is signaled in the picture header to indicate which NN model is applied to the current picture.

- RD selection with models refined for inter frames included: In addition to the DIV2K models, the additional models trained based on the BVI-DVC data set are used as additional candidates for each frame. An index value is signaled in the picture header.

### Scaling of NN filter residues

When the proposed NN filter is being applied to reconstructed pictures, a scaling factor is signaled for each color component in slice header. The output of the NN filter (residuals) are scaled by the scaling factors before being added to input samples.

# Experimental report

## Simulation results of RD based on the 2 closest QP models

The common test conditions defined in [4] are used to evaluate the proposed NN filter. The results of using the 2 closest QP models as the candidate set are summarized in Table 1. It is observed that averagely 3.20%, 12.25% and 11.39% BD rate saving for RA and 3.63%, 9.47% and 9.98% BD rate saving for AI, for Y, Cb and Cr components respectively, are achieved.

The average encoding time and decoding time are 118% and 13967% for RA, and 119% and 7853% for AI.

Table 3 Simulation results of using proposed NN filter as an in-loop filter

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.12% | -9.00% | -10.64% | 119% | 13839% |
| Class A2 | -3.63% | -14.16% | -12.29% | 117% | 12832% |
| Class B | -3.25% | -13.33% | -11.51% | 120% | 14933% |
| Class C | -2.85% | -11.89% | -11.14% | 114% | 13786% |
| Class E |  |  |  |  |  |
| Overall | -3.20% | -12.25% | -11.39% | 118% | 13967% |
| Class D | -3.13% | -12.25% | -12.49% | 116% | 14492% |
| Class F | -1.41% | -8.23% | -7.34% | 134% | 19191% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -2.49% | -13.52% | -12.46% | 122% | 17388% |
| Class C | -2.42% | -14.12% | -12.65% | 113% | 14743% |
| Class E | -2.83% | -8.82% | -6.97% | 148% | 26084% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -2.73% | -15.26% | -14.30% | 114% | 14159% |
| Class F | -1.43% | -8.42% | -7.51% | 135% | 24146% |
|  | All Intra Main10 |  |  |  |  |
|  | Over VTM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.16% | -6.53% | -8.92% | 127% | 10639% |
| Class A2 | -2.90% | -9.27% | -8.92% | 121% | 7572% |
| Class B | -3.22% | -10.10% | -10.33% | 118% | 7807% |
| Class C | -4.02% | -12.00% | -12.46% | 109% | 5908% |
| Class E | -5.01% | -8.20% | -8.24% | 121% | 8871% |
| Overall | -3.63% | -9.47% | -9.98% | 119% | 7853% |
| Class D | -4.32% | -11.64% | -13.77% | 109% | 6326% |
| Class F | -2.46% | -9.57% | -8.56% | 109% | 7151% |

The simulation results of using the proposed filter as a post filter are summarized in Table 2.

Table 4 Simulation results of using proposed NN filter as a post filter

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.86% | -9.58% | -11.05% | 119% | 13846% |
| Class A2 | -2.98% | -13.47% | -12.05% | 118% | 12830% |
| Class B | -2.92% | -13.32% | -12.48% | 120% | 14915% |
| Class C | -2.96% | -13.04% | -12.75% | 114% | 13769% |
| Class E |  |  |  |  |  |
| Overall | -2.93% | -12.53% | -12.18% | 118% | 13958% |
| Class D | -3.48% | -12.47% | -13.75% | 117% | 14453% |
| Class F | -1.39% | -8.61% | -7.96% | 134% | 19198% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -2.59% | -18.10% | -16.56% | 122% | 17329% |
| Class C | -2.91% | -19.10% | -18.13% | 114% | 14735% |
| Class E | -3.30% | -10.11% | -10.97% | 148% | 26063% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -3.48% | -20.05% | -20.62% | 115% | 14166% |
| Class F | -1.41% | -12.61% | -11.65% | 136% | 24182% |
|  | All Intra Main10 |  |  |  |  |
|  | Over VTM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.17% | -6.51% | -8.57% | 127% | 10677% |
| Class A2 | -2.92% | -8.93% | -8.64% | 121% | 7564% |
| Class B | -3.24% | -9.84% | -10.22% | 117% | 7805% |
| Class C | -4.03% | -11.88% | -12.42% | 109% | 5905% |
| Class E | -5.01% | -8.20% | -8.21% | 121% | 8947% |
| Overall | -3.65% | -9.31% | -9.83% | 118% | 7866% |
| Class D | -4.34% | -11.60% | -13.74% | 108% | 6324% |
| Class F | -2.46% | -8.86% | -8.14% | 108% | 7204% |

## Simulation results of RD based on all models

In the section, the simulation results of using all the 6 models (QP=20,25,30,35,40,45) as the fixed candidate set are provided. It is observed that averagely 3.34%, 13.38% and 12.57% BD rate saving for RA and 3.63%, 9.47% and 9.98% BD rate saving for AI, for Y, Cb and Cr components respectively, are achieved.

The average encoding time and decoding time are 150% and 13885% for RA, and 154% and 7813% for AI.

Table 5 Simulation results of all-models RD selection

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.47% | -9.48% | -10.53% | 154% | 13772% |
| Class A2 | -3.70% | -16.15% | -14.42% | 150% | 12803% |
| Class B | -3.35% | -14.22% | -12.64% | 155% | 14919% |
| Class C | -2.98% | -13.17% | -12.62% | 140% | 13572% |
| Class E |  |  |  |  |  |
| Overall | -3.34% | -13.38% | -12.57% | 150% | 13885% |
| Class D | -3.29% | -13.40% | -13.83% | 145% | 14458% |
| Class F | -1.43% | -9.87% | -9.13% | 194% | 19180% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -2.56% | -16.49% | -14.98% | 139% | 14573% |
| Class E | -2.86% | -10.21% | -10.40% | 238% | 26117% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -2.96% | -17.69% | -16.62% | 143% | 14062% |
| Class F | -1.62% | -12.37% | -11.19% | 197% | 23952% |
|  | All Intra Main10 |  |  |  |  |
|  | Over VTM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.18% | -6.48% | -8.86% | 191% | 10561% |
| Class A2 | -2.89% | -9.28% | -8.94% | 160% | 7520% |
| Class B | -3.22% | -10.10% | -10.34% | 150% | 7838% |
| Class C | -4.02% | -12.00% | -12.47% | 127% | 5811% |
| Class E | -5.01% | -8.20% | -8.25% | 161% | 8860% |
| Overall | -3.63% | -9.47% | -9.98% | 154% | 7813% |
| Class D | -4.32% | -11.64% | -13.77% | 125% | 6262% |
| Class F | -2.46% | -9.67% | -8.67% | 124% | 7117% |

## Simulation results with models refined for inter frames included

In this section, the simulation results of using models refined for inter frames as additional candidates are reported. Two cases were tested, 1) use the 2 DIV2K models and the 2 refined models with the QPs that are closest to the sequence QP as candidate set. 2) Use the 6 DIV2K models and 6 refined models as candidate set.

- In the case of 2 + 2 models RD, It is observed that averagely 3.93%, 13.28% and 12.87% BD rate saving for RA and 3.63%, 9.47% and 9.99% BD rate saving for AI, for Y, Cb and Cr components respectively, are achieved. The average encoding and decoding running time are 135% and 13964% for RA, and 136% and 7834% for AI.

- In the case of 6 + 6 models RD, It is observed that averagely 4.11%, 13.83% and 13.28% BD rate saving for RA and 3.63%, 9.47% and 9.98% BD rate saving for AI, for Y, Cb and Cr components respectively, are achieved. The average encoding and decoding running time are 196% and 14058% for RA, and 204% and 7877% for AI.

Table 6 Simultaion results of using 2 DIV2K models + 2 BVI-DVC refined models

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.53% | -9.41% | -11.44% | 138% | 13880% |
| Class A2 | -4.21% | -15.13% | -13.49% | 135% | 12801% |
| Class B | -3.88% | -14.26% | -13.08% | 139% | 14983% |
| Class C | -4.09% | -13.56% | -13.21% | 128% | 13711% |
| Class E |  |  |  |  |  |
| Overall | -3.93% | -13.28% | -12.87% | 135% | 13964% |
| Class D | -5.05% | -14.64% | -15.36% | 132% | 14524% |
| Class F | -1.65% | -8.65% | -7.88% | 165% | 19218% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -3.91% | -17.00% | -16.78% | 127% | 14589% |
| Class E | -3.55% | -9.79% | -8.82% | 191% | 25849% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -4.86% | -18.78% | -18.47% | 129% | 14197% |
| Class F | -1.82% | -10.58% | -9.78% | 166% | 23844% |
|  | All Intra Main10 |  |  |  |  |
|  | Over VTM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.16% | -6.52% | -8.94% | 160% | 10573% |
| Class A2 | -2.90% | -9.26% | -8.92% | 140% | 7506% |
| Class B | -3.22% | -10.10% | -10.33% | 134% | 7813% |
| Class C | -4.02% | -12.00% | -12.46% | 119% | 5873% |
| Class E | -5.01% | -8.20% | -8.24% | 141% | 8935% |
| Overall | -3.63% | -9.47% | -9.99% | 136% | 7834% |
| Class D | -4.32% | -11.63% | -13.77% | 117% | 6222% |
| Class F | -2.46% | -9.57% | -8.56% | 116% | 7159% |

Table 7 Simultaion results of using 6 DIV2K models + 6 BVI-DVC refined models

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.85% | -9.38% | -10.75% | 203% | 13808% |
| Class A2 | -4.26% | -15.93% | -14.63% | 195% | 12877% |
| Class B | -3.99% | -14.73% | -13.44% | 207% | 14986% |
| Class C | -4.33% | -14.48% | -13.96% | 178% | 14049% |
| Class E |  |  |  |  |  |
| Overall | -4.11% | -13.83% | -13.28% | 196% | 14058% |
| Class D | -5.40% | -15.20% | -16.30% | 188% | 15153% |
| Class F | -1.68% | -9.88% | -9.29% | 280% | 19437% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -4.20% | -19.19% | -17.92% | 175% | 14636% |
| Class E | -3.66% | -11.01% | -9.43% | 362% | 26010% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -5.31% | -20.55% | -19.82% | 184% | 14396% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | All Intra Main10 |  |  |  |  |
|  | Over VTM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.18% | -6.48% | -8.86% | 278% | 10550% |
| Class A2 | -2.89% | -9.28% | -8.94% | 211% | 7516% |
| Class B | -3.22% | -10.10% | -10.34% | 196% | 7812% |
| Class C | -4.02% | -12.00% | -12.47% | 155% | 5995% |
| Class E | -5.01% | -8.20% | -8.25% | 220% | 8991% |
| Overall | -3.63% | -9.47% | -9.98% | 204% | 7877% |
| Class D | -4.32% | -11.64% | -13.77% | 151% | 6450% |
| Class F | -2.45% | -9.67% | -8.67% | 146% | 7194% |

# References

- Versatile Video Coding (Draft 10), JVET-S2001, B. Bross, J. Chen, S. Liu, Y.-K. Wang

- CE10: Summary Report on Neural Network based Filter for Video Coding, JVET-O0030, Y.Li, S. Liu, K. Kawamura

- CE13: Summary Report on Neural Network based Filter for Video Coding, JVET-N0033, Y.Li, S. Liu, K. Kawamura

- Methodology and reporting template for neural network coding tool testing, JVET-T0041, S. Liu, A. Segall, E. Alshina, J. Boyce, M. Wien, D. Grois

- AHG9: Convolutional neural network loop filter, JVET-M0159, Y.-L. Hsiao, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)

- https://data.vision.ee.ethz.ch/cvl/DIV2K/ R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool

- BVI-DVC: A Training Database for Deep Video Compression (https://data.bris.ac.uk/data/dataset/3hj4t64fkbrgn2ghwp9en4vhtn ), Di Ma, Fan Zhang, David R. Bull

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).