| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0080 |
| --- | --- |

| Title: | VSEI: Multilayered OLS decoded picture assembling SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Emmanuel Thomas | Email: | emmanuel.thomas@tno.nl |
| Source: | TNO |  |  |

_____________________________

# Abstract

This contribution presents the motivation for the combination of the decoded pictures from different layers based on PoC value as opposed to time-based combination for composition use cases.

The proposal is based on the section 2 of JVET-S2017 “Technologies under consideration for VSEI” and can be summarized as follows:

- Each layer contains the information determining its position in the assembled output picture.

- Each layer holds the information of its neighbouring layer.

- The position of each layer in the assembled picture is thus calculated based on the signalled information and the target OLS.

- Decoded pictures with the same PoC are assembled into the assembled picture.

- The existing cropping operation is applied to each decoded picture before assembled into the assembled picture.

# Introduction

The support for combining the decoded pictures has been motivated through several contribution over the past meeting with JVET-R0307 [1], JVET-R0308 [2], JVET-S0107 [3] and JVET-S0108 [4]. These contributions aim to enable use cases such as multi-party conferencing, video game streaming, video-based point cloud compression, wherein the application could benefit from the combinations of the decoded pictures of different layers into a single picture.

The contribution first describes a typical video pipeline where a VVC decoder could be used. After outputting the decoded pictures, a rendering step is performed which creates the composition unit passed on to the compositor where the composition unit and compositor are taken from MPEG-4 part 1 design. The aim of the proposal to enable PoC-based assembling of decoded pictures before the renderer step.

The proposal is based on the section 2 of JVET-S2017 “Technologies under consideration for VSEI” and can be summarized:

- Each layer contains the information determining its position in the assembled output picture.

- Each layer holds the information of its neighbouring layer.

- The position of each layer in the assembled picture is thus calculated based on the signalled information and the target OLS.

- Decoded pictures with the same PoC are assembled into the assembled picture.

- The existing cropping operation is applied to each decoded picture before assembled into the assembled picture.

The software implementation in a VTM version prior to VTM 10.0 was provided JVET-S0214 [5] for which the source code is available here: https://vcgit.hhi.fraunhofer.de/edrthomas/VVCSoftware_VTM_multi-layer .

# Motivation: POC-based decoded picture assembling

Figure 1 presents a typical video pipeline where a VVC decoder could be used. After outputting the decoded pictures, a rendering step is performed which creates the composition unit passed on to the compositor. The terms and definition of a composition unit and compositor are taken form MPEG-4 part 1.

The composition is conventionally a timing-based while the renderer synchronises the incoming decoded pictures based on the PoC order. Indeed, the renderer typically handles texture buffers and it is unaware of timing information. This step is crucial since the slightest asynchrony between the decoded pictures used by the renderer may cause visual artefacts in the generated composition unit. This issue was reported in the V-PCC group at MPEG when the geometry component and the texture component are fed to the synthesiser with a delta in the picture order count between both components. Same applies for other applications where picture accuracy is essential.

Figure 1 - PoC-based and time-based synchronization in video pipeline

Since the interface to the renderer process is a texture buffer, it is beneficial to feed a single texture buffer to the renderer containing the combination of PoC-aligned decoded pictures, hence ensuring the accurate synchronisation during the execution of rendering process.

Figure 2 illustrates the PoC-based layer combination as targeted by the proposal.

Figure 2 - PoC-based layer combination

It is worth to note that both PoC-based and time-based synchronisation are useful and are thus complimentary. In addition, the PoC-based operations are not of the same nature as what is called composition. Therefore, the proposal does not aim at enabling specific needs for composition such as overlay, scaling, gaps, etc…

# Proposal

The following text is taken from JVET-S2017 “Technologies under consideration for VSEI”.

## Independent layer info SEI message syntax

| independent_layer_info( payloadSize ) { | Descriptor |
| --- | --- |
| boundary_identifier_north_present_flag | u(1) |
| if( boundary_identifier_north_present_flag ) |  |
| boundary_identifier_north | u(16) |
| boundary_identifier_east_present_flag | u(1) |
| if( boundary_identifier_east_present_flag ) |  |
| boundary_identifier_east | u(16) |
| boundary_identifier_south_present_flag | u(1) |
| if( boundary_identifier_south_present_flag ) |  |
| boundary_identifier_south | u(16) |
| boundary_identifier_west_present_flag | u(1) |
| if( boundary_identifier_west_present_flag ) |  |
| boundary_identifier_west | u(16) |
| } |  |

## Independent layer info SEI message semantics

boundary_identifier_north_present_flag, boundary_identifier_east_present_flag, boundary_identifier_south_present_flag and boundary_identifier_west_present_flag equal to 1 specify that the SEI message contains a boundary identifier, respectively, for the north, east, south and west boundary.

boundary_identifier_north, boundary_identifier_east, boundary_identifier_south and boundary_identifier_west specifiy the boundary identifier, respecitvely, at the north, east, south and west boundary of the decoded picture of the associated layer; the associated layer being the layer whose nuh_layer_id is equal to the nuh_layer_id of the SEI message. If not present, the boundary identifiers respecitvely at the north, east, south and west boundary of the decoded picture of the associated layer are not defined.

Figure X - Representation of where the bououndaries are on the layers

For two layers, the i-th and j-th layers, when the pair of the boundary_identifier_north value of the i-th layer and the boundary_identifier_south value of the j-th layer are equal then the decoded picture of the i-th layer and the decoded picture of the j-th layer are adjacent in the composed output picture and they share a common boundary at the boundary north/south. For i-th and j-th layers, when the pair of the boundary_identifier_east value of the i-th layer and the boundary_identifier_west value of the j-th layer are equal then the decoded picture of the i-th layer and the decoded picture of the j-th layer are adjacent in the composed output picture and they share a common boundary at the boundary east/west.

Two decoded pictures adjacent by the north/south boundary are aligned on their west boundary in the final output picture. Two decoded pictures adjacent by the east/west boundary are aligned on their north boundary in the final output picture.

All the Independent layer info SEI message present in layers of an OLS shall collectively describe a 4-connected graph and each layer of the OLS shall be connected to the graph.

### D.x.2 Process for generating the aggregated output picture

### (The intent of the following section is to explain the process for aggregating the output picture, but it can be removed as it is informative text)

The process for generating the final output picture is informative. The following section provides the expected operations performed for generating the final output picture based on the decoded pictures of each layer from a selected OLS:

- For each Access Unit:

  - If VPS present, parse VPS and store the list of layers in the bitstream.

  - For each present PPS, determine the size in luma samples of the corresponding layer.

  - For each present Independent layer info SEI, parse the payload and store the boundary identifiers for the corresponding layer.

  - If any of VPS, PPS or Independent layer info SEI is present in the current Access Unit, execute calculate_layer_layout_in_final_output_picture(), that returns the horizontal, XPos, and vertical, YPos, position of the top-left corner of each cropped decoded picture per layer in the final output picture.

  - Initialize a picture buffer of size FinalWidth of width and FinalHeight of height for the final output picture.

  - For each Picture Unit:

    - Decode the coded picture.

  - If pictures are ready for output:

    - For each layer in selected OLS.

      - Apply conformance window cropping on the decoded picture of the current layer.

      - Retrieve XPos and YPos, the positions of the current layer in the final output picture in luma sample.

      - Copy cropped decoded picture in final output picture buffer at position (XPos, YPos) corresponding to the top-left corner of the cropped decoded picture.

### D.x.2.1 Process for calculating the layer layout in the final output picture

### (The intent of the following section is to explain the process for determining the layer layout, but it can be removed as it is informative text)

Input to this process are:

- the list of layer identifiers included in the OLS of index TargetOlsIdx, LayerIdList

- the list of cropped picture size of each layer, LayerPicSizeList

- the boundary identifiers for each layer.

Outputs of this process are:

- the horizontal and vertical position in luma sample of the cropped decoded picture in each layer in the final output picture, [{layer_id, XPos, YPos}][NbLayer]

- the size in luma samples of the final output picture, {FinalWidth, FinalHeight}

NOTE: There may be several ways to implement the function that returns the correct output given the input above. The following is an exemplary algorithm.

The steps of the process are:

- Select a layer as starting point, whose layer identifier is layer0. The top-left corner of this layer will be the origin for calculating the coordinates top-left corner of each layer.

- Set curLayerId to layer0.

- Until all layers are traversed, do:

  - Set XPos and Ypos for the current layer using LayerPicSizeList[curLayerId].

NOTE: For layer0, XPos are Ypos set to ‘0’. Otherwise, the rule defined in the semantic defines hwo to derive XPos and YPos. That is, for two vertical neighbors (North or South), the top-left corner of both layer have the same horizontal coordinate. Similarly, for two horizonatal neighbors (East or West), the top-left corner of both layer have the same vertical coordinate. The second component of the top-left corner position, vertical or horizontal, is determined by using the width or height of the cropped decoded picture of the layers.

  - Find a neaighbor of the current layer, and set curLayerId to this layer id.

- At this point XPos and YPos are relative to the top-left corner layer0. Apply a change of origin such that all the XPos and YPos values are positive or null.

- Set FinalWidth to the maximum value calculated by summing each row of grid of horizontal neighbor layers (layers having the same YPos)

- Set FinalHeight to the maximum value calculated by summing each height of vertical neighbor layers (layers having the same XPos)

# References

- JVET-R0307, “AHG8/AHG9: Positioning information SEI message of output independent layers”, Emmanuel Thomas (TNO)

- JVET-R0308, “AHG8: Implementation of multi-layer decoding and output independent layer composition in VTM”, Emmanuel Thomas, Karim El Assal, Alexandre Gabriel (TNO)

- J. Boyce, “AHG8/AHG12: Recommended multi-layer composite picture SEI messages”, JVET-S0107, the 19th JVET meeting, by teleconference, 22 June – 1 July 2020.

- JVET-S0108, “AHG8/AHG9: Refinement of proposed positioning information SEI message of output independent layers”, Emmanuel Thomas (TNO)

- JVET-S0214, “AHG8/AHG9: Updates on the implementation of multi-layer decoding and output independent layer arrangement in VTM”, Emmanuel Thomas (TNO)

# Patent rights declaration(s)

TNO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).