| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0081 |
| --- | --- |

| Title: | AHG3: On StreamMergeApp design |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Emmanuel Thomas | Email: | emmanuel.thomas@tno.nl |
| Source: | TNO |  |  |

_____________________________

# Abstract

The StreamMergeAp allows to merge single-layer bitstreams into a multi-layer bitstream. This contribution discusses issues encountered when using the StreamMergeApp together with bitstreams generated with VTM 10.0.

Possible directions are proposed to solve these issues.

# Introduction

This contribution discusses issues encountered when using the StreamMergeApp together with bitstreams generated with VTM 10.0.

The StreamMergeApp tool allows the merging of several single-layer input bitstream into a single multi-layer bitstream. Each of the input bitstream constitutes an independent layer in the output multi-layer bitstream. Figure 1 depicts this merging process.

Figure 1- Purpose of StreamMergeApp

In the current version of the tool, the merging process operates as follows:

- A new VPS is created for the output bitstream

- One NALU is read in each input bitstream in a round-robin fashion and written to the output bitstream, hence interleaving the NALUs.

- When a VPS NAL is encountered in the input bitstream, the new VPS is written to the output bitstream with the layer ID corresponding to the current layer. The new VPS is thus duplicated in all the layers of the output bitstream.

# Problem

## #1: Absence of VPS in single layer bitstream

At the time the StreamMergeApp was first developed, the VPS NAL was always written even for a single layer bitstream. This way, the StreamMergeApp could use the detection of a VPS NAL as a trigger to write the new VPS in the output bitstream.

However, it has been observed that the writing of the VPS in a single layer bitstream is conditioned to the vps identifier being greater than 0 in VTM 10.0.

int EncGOP::xWriteParameterSets(AccessUnit &accessUnit, Slice *slice, const bool bSeqFirst, const int layerIdx) { […] if (slice->getSPS()->getVPSId() != 0) { actualTotalBits += xWriteVPS(accessUnit, m_pcEncLib->getVPS()); } […] }

The encoder sets the VPS identifier to 1 when the number of max layers is greater than 1:

void EncApp::xInitLibCfg() { […] if (vps.getMaxLayers() > 1) { vps.setVPSId(1); //JVET_P0205 vps_video_parameter_set_id shall be greater than 0 for multi-layer coding } else { vps.setVPSId(0); vps.setEachLayerIsAnOlsFlag(1); } [..] }

As a result, the encoding of single layer bitstream does not cause the presence of the VPS NAL in the output bitstream which breaks the design of the StreamMergeApp.

Figure 2 - Missing VPS in input bitstreams causes the absence of the VPS in the output bitstream

## #2: Outdated initialisation of the new VPS

The StreamMergeApp must create a VPS for the output bitstream which is the similar task of the VTM encoder when creating a multi-layer bitstream. As a result, the VTM encoder executes functions to 1) initialise VPS parameters according to the encoding configuration (e.g. dependency between layers or not) and 2) derive remaining VPS parameters based on the conditions set in the previous step.

It is our understanding that those two functions are:

void EncApp::xInitLibCfg() void EncLib::xInitVPS( const SPS& sps )

xInitLibCfg() sets VPS parameters based on the input config file while xInitVPS() derives the remaining VPS parameter based on the parameters set in xInitLibCfg() and SPS parameters.

Currently, the StreamMergeApp simply creates a VPS object and relies on the default values set in the object constructor and is thus outdated. As a result, it would be advantageous to at least reuse the logic implemented in the encoding library and possibly some part of the encoding app logic although the StreamMergeApp does not parse a config file as the encoding app does.

# Proposal

## On #1: Absence of VPS in single layer bitstream

Possible options are:

- VTM encoder to write the VPS for bitstreams with one layer

- StreamMergeApp to determine when to write the VPS differently

## On #2: Outdated initialisation of the new VPS

Possible options are:

- Transfer initialization logic of VPS to a generic place

- Duplicate initialization logic of VPS in StreamMergeApp

# Patent rights declaration(s)

TNO does not have any current or pending patent rights relating to the technology described in this contribution.