| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0082-v3 |
| --- | --- |

| Title: | New HLG sequences for high bit-depth video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo Masaru Ikeda Adrian Browne Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-70-7598-7407 Kenji.Kondo@sony.com Masaru.Ikeda@sony.com Adrian.Browne@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes new HLG (Hybrid Log-Gamma) test sequences for high bit-depth video coding. This provides information on the test sequences, which are resolution, frame rate, chroma format, bit-depth, frame count and the coding results of VTM-10.0 compared to HM-16.22-RExt.

Introduction

We, Sony, proposed five HLG sequences, YUV4:2:0/10-bit, for HDR CTC in the VVC Ver.1 [1][2], and two of them were adopted at that time. The VVC Ver.1 was finalized in the last meeting, and a new AHG is established where high bit-depth, high bit rate, and high frame rate coding are studied. One of the requirements for the AHG is to define test sequences. So, we would like to propose the same content as previously proposed, but at 12-bit. These sequences are captured by the 4K camera below in Table 1.

Table 1: Specifications of 4K camera

| Spec. |  |
| --- | --- |
| Model/Manufacturer | CineAlta F65RS / Sony |
| Pickup device | Super 35mm equivalent Single-chip CMOS |
| Frame rate | 59.94fps, progressive |
| Bit depth | 16bit |
| Signal format | RGB RAW |

Test sequence

The developed data, YUV444/12-bit, were converted to YUV422/12-bit by HDR-tools using the attached configuration file. The chroma position was set to 2 in the configuration file. The thumbnails and brief descriptions are provided below in Table 2. We can also provide these sequences for SDR verification test if necessary.

Table 2: Proposed HLG test sequences

| Thumbnail | File name | Brief description |
| --- | --- | --- |
|  | DayStreet_3840x2160_60p_12bit_444_hlg DayStreet_3840x2160_60p_12bit_422_hlg | Reflection on the glass of the cars and heat haze (fixed) |
|  | PeopleInShoppingCenter_3840x2160_60p_12bit_444_hlg PeopleInShoppingCenter_3840x2160_60p_12bit_422_hlg | People walking in shopping center (fixed) |
|  | NightStreet_3840x2160_60p_12bit_444_hlg NightStreet_3840x2160_60p_12bit_422_hlg | Reflection on the glass and the body of the cars (fixed) |
|  | SeaSurface_3840x2160_60p_12bit_444_hlg SeaSurface_3840x2160_60p_12bit_422_hlg | Birds flying around on the sea (fixed) |
|  | StainedGlass_3840x2160_60p_12bit_444_hlg StainedGlass_3840x2160_60p_12bit_422_hlg | Stained glass (tilting) |

The frame count of all sequences is 600.

MD5SUMs of test sequence files

Regarding to proposed test sequences, MD5SUMs of files are computed as shown below.

22d84b5024ff0477a729d5a16b29bf61  DayStreet_3840x2160_60p_12bit_444_hlg.yuv

136cbeb9cea1fd6969366b5979b89eb8  DayStreet_3840x2160_60p_12bit_422_hlg.yuv

dcdc82fcbf693c0bb0dcf16abad64c75  PeopleInShoppingCenter_3840x2160_60p_12bit_444_hlg.yuv

8fc91ff312f3a729bf6c5e149bbaaf5e  PeopleInShoppingCenter_3840x2160_60p_12bit_422_hlg.yuv

da10d72731df64910269f7a193021cae  NightStreet_3840x2160_60p_12bit_444_hlg.yuv

a9773f3df122061379888a5268d0d50e  NightStreet_3840x2160_60p_12bit_422_hlg.yuv

f40ecd853a073682964c05e481a8b554  SeaSurface_3840x2160_60p_12bit_444_hlg.yuv

ab2980932ebe6a254b5407efed123896  SeaSurface_3840x2160_60p_12bit_422_hlg.yuv

cc6f6f8c1d5c7143210e602c3d9b7471  StainedGlass_3840x2160_60p_12bit_444_hlg.yuv

5cf45f6880652eb9b9ddac84f3d20e66  StainedGlass_3840x2160_60p_12bit_422_hlg.yuv

Coding result

The test was conducted in the case of common test condition, low QP, and lossless which are aligned with AHG12 preliminary common test conditions.[3] The coding results of VTM-10.0 can be shown compared to HM-16.22 RExt in Table 3. For encoding time, it is not reliable because several machine platforms are used. For decoding time, same type machine platform is used to measure it.

Table 3 Test results for AI, LDB and RA (QP range: -13, -8, -3, 2, 7 and 12)

|  |  |  |  |  | AI-12 Low-QP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Over HM-16.22-Rext |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |  |
| HLG444 | -7.33% |  | -9.46% |  | -10.45% |  | 5649% |  | 190% |  |
| HLG422 | -7.51% |  | -12.66% |  | -12.77% |  | 4865% |  | 172% |  |
| Overall | -7.42% |  | -11.06% |  | -11.61% |  | 5243% |  | 181% |  |
|  |  |  |  |  | LDB-12 Low-QP |  |  |  |  |  |
|  |  |  |  |  | Over HM-16.22-Rext |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |  |
| HLG444 | -5.02% |  | -7.68% |  | -8.06% |  | 1084% |  | 181% |  |
| HLG422 | -6.74% |  | -13.28% |  | -12.79% |  | 1499% |  | 171% |  |
| Overall | -5.88% |  | -10.48% |  | -10.42% |  | 1275% |  | 176% |  |
|  |  |  |  |  | RA-12 Low-QP |  |  |  |  |  |
|  |  |  |  |  | Over HM-16.22-Rext |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |  |
| HLG444 | -5.96% |  | -9.61% |  | -9.98% |  | 1683% |  | 180% |  |
| HLG422 | -7.45% |  | -14.20% |  | -13.56% |  | 2138% |  | 178% |  |
| Overall | -6.70% |  | -11.91% |  | -11.77% |  | 1683% |  | 180% |  |

Copyright information

The sequences provided in this contribution are owned by Sony Corporation. The copyright and disclaimer are attached with the test sequences. The test sequences shall be copied together with the copyright and disclaimer which is not subject to be encoded.

The sequence has been generated by Sony Corporation, and all intellectual property rights remain with Sony Corporation. The material shall only be used for ISO/IEC JTC1 SC29/WG11 | ITU-T SG16 WP3 Q.6 (MPEG/VCEG). The sequences should be processed in the original 4K resolution.

This copyright and permission notice shall be duplicated together whenever the data are copied or distributed. The material cannot be charged for any copy or distribution. Sony Corporation makes no warranty with respect to the material and expressly disclaims any warranty regarding any use. If you do not agree to the above polity, no permission is granted for any use of the data. We will regard that you have agreed to the policy of the copyright and disclaimer if you use the data.

The following uses are NOT allowed for the proposed sequences:

    - Do not publish snapshots in product brochures.

    - Do not use video for marketing purposes

    - Do not redistribute video with a commercial product.

    - Do not use in television shows, commercials, or movies.

Patent rights declaration(s)

Sony Corporation does not have any current or pending patent rights relating to the technology described in this contribution.

References

[1] T. Tsukuba, M. Ikeda, T. Suzuki, “Test Sequences of 4k Hybrid Log-Gamma”, JVET-G0063, JVET, 7th Meeting: Torino, IT, 13–21 July 2017

[2] T. Tsukuba, M. Ikeda, T. Suzuki, “AHG7: Test Sequences of 4K Hybrid Log-Gamma”, JVET-H0038, JVET, 8th Meeting: Macao, CN, 18–24 Oct. 2017

[3] T. Ikai, A. Browne, X. Xiu, “AHG12: Preliminary common test conditions for high bitdepth video coding”, JVET-T0047, JVET, 20th Meeting: by teleconference, 7–16 Oct. 2020