| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0085 |
| --- | --- |

| Title: | AHG12: Rice parameter derivation for high bit-depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomonori Hashimoto Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-50-5213-6337 tomonori.hashimoto@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to improve the derivation of rice parameter for regular residual coding (RRC) for high bit-depth coding. VVC derives rice parameter in RRC with look-up table and its range is restricted from 0 to 3. It is asserted that this restriction causes worse performance of VTM in high bit-depth coding compared to HEVC Format Range extension profile where rice parameter derivation can be extended when persistent_rice_adaptation_enabled_flag is equal to 1. To resolve this problem, this contribution introduces a rice parameter derivation method using a predicted remainder value.

It is reported 1.52 % and 25.02 % in bdrateY in AI-12 qp=-13, -8, -3, 2, 7, and AI-16 -33, -28, -23, 18, 13, and 0.24 % and 16.95 % in bdrateY in LDB-12 qp=-13, -8, -3, 2, 7, and LDB-16 -33, -28, -23, 18, 13 respectively.

Introduction

In the current VVC design [1], VTM derives rice parameter for RRC with look-up table with coefficient value referring to the neighboring transform coefficients within a template. First, locSumAbs (the sum of absolute coefficient values of available neighboring five blocks in template) is calculated (Figure 1).

Figure 1. Template area referred by current block

locSumAbs is subtracted considering baselevel and cliped as followings:

locSumAbs = Clip3( 0, 31, locSumAbs - baseLevel * 5 )

Using this locSumAbs, rice parameter is derived with look-up table (Table 1) .

Table 1. Look-up table for Rice Parameter based on locSumAbs in current spec

| locSumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |
| locSumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| cRiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |

In this design, rice parameter range is restricted from 0 to 3. This restriction causes worse performance of VVC in high bit-depth coding compared to HEVC Format range extension profile where rice parameter derivation can be extended when persistent_rice_adaptation_enabled_flag is equal to 1

As a resolution for this problem, this contribution introduces the method for derive the rice parameter with a predicted remainder. This method removes the restriction of rice parameter in current design.

Proposed methods

Proposed method derives as two steps.

The first step is remainder prediction. Instead of look-up table, the proposed method predicts a remainder value by a simple linear formula calculation. This change solves the range problem of the look-up table and provides adaptability. As the second step, rice the parameter cRiceParam is derived from a formula with predRem as follows:

- predRem = ( locSumAbs0 * a + b ) >> 4

- cRiceParam = predRem * 5 < 16 ? 0 : floorLog2((predRem * 5) >> 4) + 1

Where the variable a and b are derived depending on syntax. i.e. abs_remainder or dec_abs_level as following pseudo code:

if (abs_remainder) {
  a = 3, b = 0
}
else { // dec_abs_level
  a = 7, b = 16
}

locSumAbs0 is the sum of absolute coefficient values of neighboring blocks (before subtraction considering baselevel) as below.

locSumAbs = 0
if( xC < ( 1  <<  log2TbWidth ) − 1 ) {
	locSumAbs  +=  AbsLevel[ xC + 1 ][ yC ]
	if( xC < ( 1  <<  log2TbWidth ) − 2 )
		locSumAbs  +=  AbsLevel[ xC + 2 ][ yC ]
	if( yC < ( 1  <<  log2TbHeight ) − 1 )
		locSumAbs  +=  AbsLevel[ xC + 1 ][ yC + 1 ]	(1517)
}
if( yC < ( 1  <<  log2TbHeight ) − 1 ) {
	locSumAbs  +=  AbsLevel[ xC ][ yC + 1 ]
	if( yC < ( 1  <<  log2TbHeight ) − 2 )
		locSumAbs  +=  AbsLevel[ xC ][ yC + 2 ]
} 
locSumAbs0 = locSumAbs // use the value before subtraction
locSumAbs = Clip3( 0, 31, locSumAbs - baseLevel * 5 )

Test condition

The test sequences and condition are following AHG12 CTC [3]. The anchors and tests use the latest VTM-10.0 with

#define JVET_R0351_HIGH_BIT_DEPTH_ENABLED                 1

Anchor:

Anchor0: VTM-10.0 with ExtendedPrecision = 0

Anchor1: VTM-10.0 with ExtendedPrecision = 1

Anchor2: HM-16.22 (based on JVET-T0047 cfg)

Tests:

Test: VTM-10.0 with ExtendedPrecision = 1

Simulation results

The higher bit rate, the greater the gain of the proposed method against current VTM. In terms of the comparison with HM, proposed method exceeds when QP is -23 or more, but HM exceeds after that.

Following tables show detail results.

VVC (extended_precistion_flag = 1)

Test vs Anchor1

|  |  |  | AI-12 |  |  |  |  |  |  | AI-16 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0 exp1 |  |  |  |  |  |  | Over VTM-10.0 exp1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |  | Y | U | V | EncT | DecT |
| SVT | -3.32% | -3.13% | -3.14% | 108% | 80% |  | SVT | -25.02% | -23.92% | -23.97% | 120% | 67% |
| PQ444 | -0.57% | -0.61% | -0.64% | 105% | 91% |  |  |  |  |  |  |  |
| PQ422 | -0.66% | -0.74% | -0.76% | 105% | 91% |  |  |  |  |  |  |  |
| Overall | -1.52% | -1.49% | -1.51% | 106% | 87% |  | Overall | -25.02% | -23.92% | -23.97% | 120% | 67% |
|  |  |  | LDB-12 |  |  |  |  |  |  | LDB-16 |  |  |
|  | Over VTM-10.0 exp1 |  |  |  |  |  |  | Over VTM-10.0 exp1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |  | Y | U | V | EncT | DecT |
| SVT | -0.61% | -0.64% | -0.64% | 107% | 87% |  | SVT | -16.95% | -15.49% | -15.81% | 130% | 74% |
| PQ444 | -0.06% | -0.07% | -0.07% | 101% | 89% |  |  |  |  |  |  |  |
| PQ422 | -0.07% | -0.10% | -0.10% | 101% | 89% |  |  |  |  |  |  |  |
| Overall | -0.24% | -0.27% | -0.27% | 103% | 88% |  | Overall | -16.95% | -15.49% | -15.81% | 130% | 74% |
|  |  |  | RA-12 |  |  |  |  |  |  | RA-16 |  |  |
|  | Over VTM-10.0 exp1 |  |  |  |  |  |  | Over VTM-10.0 exp1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |  | Y | U | V | EncT | DecT |
| SVT | -0.65% | -0.68% | -0.68% | 107% | 85% |  | SVT | -15.57% | -15.34% | -15.48% | 124% | 75% |
| PQ444 | -0.13% | -0.13% | -0.15% | 81% | 94% |  |  |  |  |  |  |  |
| PQ422 | -0.14% | -0.17% | -0.16% | 81% | 94% |  |  |  |  |  |  |  |
| Overall | -0.31% | -0.33% | -0.33% | 98% | 93% |  | Overall | -15.57% | -15.34% | -15.48% | 124% | 75% |

Additional results (extended_precision_flag = 1 in test, 0 in anchor)

Test vs Anchor0

|  |  |  | AI-12 |  |  |  |  |  |  | AI-16 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-10 |  |  |  |  |  |  | Over VTM-10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |  | Y | U | V | EncT | DecT |
| SVT | -4.13% | -4.22% | -4.23% | 113% | 71% |  | SVT | -22.51% | -28.66% | -27.05% | 120% | 55% |
| PQ444 | -1.94% | -2.51% | -2.68% | 106% | 92% |  |  |  |  |  |  |  |
| PQ422 | -1.81% | -2.09% | -2.21% | 106% | 92% |  |  |  |  |  |  |  |
| Overall | -2.63% | -2.94% | -3.04% | 108% | 84% |  | Overall | -22.51% | -28.66% | -27.05% | 120% | 55% |
|  |  |  | LDB-12 |  |  |  |  |  |  | LDB-16 |  |  |
|  | Over VTM-10 |  |  |  |  |  |  | Over VTM-10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |  | Y | U | V | EncT | DecT |
| SVT | -1.45% | -1.40% | -1.41% | 120% | 80% |  | SVT | -20.67% | -20.21% | -20.29% | 200% | 62% |
| PQ444 | -1.48% | -2.38% | -2.39% | 103% | 94% |  |  |  |  |  |  |  |
| PQ422 | -1.29% | -1.91% | -1.81% | 103% | 94% |  |  |  |  |  |  |  |
| Overall | -1.40% | -1.89% | -1.87% | 84% | 71% |  | Overall | -20.67% | -20.21% | -20.29% | 200% | 62% |
|  |  |  | RA-12 |  |  |  |  |  |  | RA-16 |  |  |
|  | Over VTM-10 |  |  |  |  |  |  | Over VTM-10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |  | Y | U | V | EncT | DecT |
| SVT | -1.43% | -1.34% | -1.34% | 116% | 81% |  | SVT | -19.16% | -18.75% | -18.81% | 183% | 63% |
| PQ444 | -1.56% | -2.23% | -2.33% | 102% | 89% |  |  |  |  |  |  |  |
| PQ422 | -1.29% | -1.83% | -1.80% | 102% | 89% |  |  |  |  |  |  |  |
| Overall | -1.43% | -1.80% | -1.83% | 109% | 89% |  | Overall | -19.16% | -18.75% | -18.81% | 183% | 63% |

Additional results (extended_precision_flag = 1 in test, HM in anchor)

Test vs Anchor2

|  |  |  | AI-12 |  |  |  |  |  |  | AI-16 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM-16.22 |  |  |  |  |  |  | Over HM-16.22 |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |  | Y | U | V | EncT | DecT |
| SVT | -3.34% | -2.94% | -3.03% | 8079% | 77% |  | SVT | 0.07% | 0.35% | 0.33% | 12031% | 105% |
| PQ444 | -5.32% | -6.73% | -7.44% | 4027% | 167% |  |  |  |  |  |  |  |
| PQ422 | -10.16% | -14.87% | -15.54% | 4027% | 167% |  |  |  |  |  |  |  |
| Overall | -6.28% | -8.18% | -8.67% | 5057% | 131% |  | Overall | 0.07% | 0.35% | 0.33% | 12031% | 105% |
|  |  |  | LDB-12 |  |  |  |  |  |  | LDB-16 |  |  |
|  | Over HM-16.22 |  |  |  |  |  |  | Over HM-16.22 |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |  | Y | U | V | EncT | DecT |
| SVT | -4.20% | -1.82% | -1.83% | 1386% | 143% |  | SVT | -0.93% | 0.50% | 0.49% | 1744% | 136% |
| PQ444 | -7.24% | -8.64% | -8.29% | 1015% | 184% |  |  |  |  |  |  |  |
| PQ422 | -10.92% | -16.53% | -17.36% | 1015% | 184% |  |  |  |  |  |  |  |
| Overall | -7.45% | -9.00% | -9.16% | 1224% | 168% |  | Overall | -0.93% | 0.50% | 0.49% | 1744% | 136% |
|  |  |  | RA-12 |  |  |  |  |  |  | RA-16 |  |  |
|  | Over HM-16.22 |  |  |  |  |  |  | Over HM-16.22 |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |  | Y | U | V | EncT | DecT |
| SVT | -4.34% | -2.09% | -2.10% | 2064% | 137% |  | SVT | -1.27% | 0.49% | 0.44% | 2530% | 134% |
| PQ444 | -7.98% | -9.55% | -9.35% | 1342% | 186% |  |  |  |  |  |  |  |
| PQ422 | -11.69% | -17.30% | -18.47% | 1342% | 186% |  |  |  |  |  |  |  |
| Overall | -8.00% | -9.65% | -9.98% | 1746% | 175% |  | Overall | -1.27% | 0.49% | 0.44% | 2530% | 134% |

Conclusion

This contribution proposes the method to derive the rice parameter with a predicted remainder value for high bit-depth coding. This method removes the restriction of rice parameter range in current design and shows coding efficiency against current VTM. It is recommended to conduct core experiment for this method.

References

[1] B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 10),” JVET-S2001, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 19th Meeting: by teleconference, 22 June – 1 July 2020.

[2] A. Browne, S. Keating, K. Sharman, “On operation beyond 10-bit”, JVET-S0243

[3] T. Ikai, A. Browne, X. Xiu, “AHG12: Preliminary common test conditions for high bitdepth video coding”, JVET-T0047

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).