| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0086-v2 |
| --- | --- |

| Title: | AHG12: SIMD implementation of BDOF for high bit-depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Eiich Sasaki Tomohiro Ikai | Tel: Email: | +81-50-5213-6334 chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

Since on the current configuration for high bit-depth coding, the SIMD code is not applied, the encoding times are very huge. In the current BDOF processing, the internal precision exceeds 16-bit when the bit depth is greater than 12. In this contribution, by limiting the values of the two variables, the internal precision of BDOF is within 16-bit and efficient SIMD codes can be implemented. As a result of the experiment, the coding efficiency was not changed, and the encoding and decoding times have been improved. The specification is only influenced by this change in the case of the bit-depth greater than 12.

Problem Statement

On the current configuration for high bit-depth coding, the encoding times are very huge. One of the reasons is that SIMD codes are not applied to the high bit-depth coding configuration. On the current specification of BDOF, the internal precision exceeds 16-bit when the bit depth is greater than 12-bit [1].  If the internal precision is within 16-bit, efficient SIMD codes can be implemented.

Proposal

The working draft [2] should be changed by the following. The values of the two variables should be limited because the internal precision keeps within 16-bit.

- 8.5.6.5	Bi-directional optical flow prediction process

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current coding block,

- two (nCbW + 2)x(nCbH + 2) luma prediction sample arrays predSamplesL0 and predSamplesL1,

- the prediction list utilization flags predFlagL0 and predFlagL1,

- the reference indices refIdxL0 and refIdxL1,

Output of this process is the (nCbW)x(nCbH) array pbSamples of luma prediction sample values.

The variables shift1, shift2, shift3, shift4, offset4, and mvRefineThres are derived as follows:

- The variable shift1 is set to be equal to 6.

- The variable shift2 is set to be equal to 4Max( 4, bitDepth − 8 ).

- The variable shift3 is set to be equal to 1Max( 1, bitDepth − 11 ).

- The variable shift4 is set equal to Max( 3, 15 − BitDepth ) and the variable offset4 is set equal to 1  <<  ( shift4 − 1 ).

- The variable mvRefineThres is set equal to 1  <<  4.

For xIdx = 0..( nCbW  >>  2 ) − 1 and yIdx = 0..( nCbH  >>  2 ) − 1, the following applies:

- The variable xSb is set equal to ( xIdx  <<  2) + 1 and ySb is set equal to ( yIdx  <<  2 ) + 1.

- The prediction sample values of the current subblock are derived as follows:

  - For x =xSb − 1..xSb + 4, y = ySb − 1..ySb + 4, the following ordered steps apply:

- The locations ( hx, vy ) for each of the corresponding sample locations ( x, y ) inside the prediction sample arrays are derived as follows:

hx = Clip3( 1, nCbW, x )		(991)

vy = Clip3( 1, nCbH, y )		(992)

- The variables gradientHL0[ x ][ y ], gradientVL0[ x ][ y ], gradientHL1[ x ][ y ] and gradientVL1[ x ][ y ] are derived as follows:

gradientHL0[ x ][ y ]  =  ( predSamplesL0[ hx + 1 ][vy]  >>  shift1 ) −	(993)
	( predSampleL0[ hx − 1 ][ vy ] )  >>  shift1 )

gradientVL0[ x ][ y ]  =  ( predSampleL0[ hx ][ vy + 1 ]  >>  shift1 ) −	(994)
	( predSampleL0[ hx ][vy − 1 ] )  >>  shift1 )

gradientHL1[ x ][ y ]  =  ( predSamplesL1[ hx + 1 ][vy]  >>  shift1 ) −	(995)
	( predSampleL1[ hx − 1 ][ vy ] )  >>  shift1 )

gradientVL1[ x ][ y ]  =   ( predSampleL1[ hx ][ vy + 1 ]  >>  shift1 ) −	(996)
	( predSampleL1[ hx ][vy − 1 ] )  >>  shift1 )

- The variables diff[ x ][ y ], tempH[ x ][ y ] and tempV[ x ][ y ] are derived as follows:

diff[ x ][ y ] = (predSamplesL0[ hx ][ vy ]  >>  shift2 ) − ( predSamplesL1[ hx ][ vy ]  >>  shift2 )	(997)

tempH[ x ][ y ] = (gradientHL0[ x ][ y ] + gradientHL1[ x ][ y ] )  >>  shift3	(998)

tempV[ x ][ y ] = (gradientVL0[ x ][ y ] + gradientVL1[ x ][ y ] )  >>  shift3	(999)

  - The variables sGx2, sGy2, sGxGy, sGxdI and sGydI are derived as follows:

sGx2 = ij Abs( tempH[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(1000)

sGy2 = ij Abs( tempV[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(1001)

sGxGy = ij( Sign( tempV[ xSb + i ][ ySb + j ] ) * tempH[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(1002)

sGxdI = ij( −Sign( tempH[ xSb + i ][ ySb + j ] ) * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(1003)

sGydI = ij( −Sign( tempV[ xSb + i ][ ySb + j ] ) * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(1004)

  - The horizontal and vertical motion offset of the current subblock are derived as:

vx = sGx2 > 0 ? Clip3( −mvRefineThres + 1, mvRefineThres − 1,	(1005)
	( sGxdI  <<  2 )  >>  Floor( Log2( sGx2 ) ) ) : 0

vy = sGy2 > 0 ? Clip3( −mvRefineThres + 1, mvRefineThres − 1, ( ( sGydI  <<  2 ) −	(1006)
	( ( vx * sGxGy )  >>  1 ) )  >>  Floor( Log2( sGy2 ) ) ) : 0

  - For x =xSb − 1..xSb + 2, y = ySb − 1..ySb + 2, the prediction sample values of the current sub-block are derived as follows:

bdofOffset = vx * ( gradientHL0[ x + 1 ][ y + 1 ] − gradientHL1[ x + 1 ][ y + 1 ] )	(1007)
	+ vy * (gradientVL0[ x + 1 ][ y + 1 ] − gradientVL1[ x + 1 ][ y + 1 ] )

pbSamples[ x ][ y ] = Clip3( 0, ( 2BitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ] + offset4 +	(1008)
	predSamplesL1[ x + 1 ][ y + 1 ] + bdofOffset )  >>  shift4 )

In the current specification, when the bit depth is greater than 12-bit, the precision of the variables diff, tempH and tempV is increased as shown in Table 1. In the proposed specification change, when the bit depth is greater than 12-bit, the values of shift2 and shift3 are increased; however, the precision of the variables diff, tempH and tempV is constant as shown in Table 2.

Table 1  Internal bit precision of BDOF (Current specification)

|  |  | bitDepth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| Value | shift1 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
|  | shift2 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
|  | shift3 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | shift4 | 7 | 6 | 5 | 4 | 3 | 3 | 3 | 3 | 3 |
|  | mvRefineThres | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| Precision (dynamic range) | predSamplesL0 predSamplesL1 | 14 (16) | 14 (16) | 14 (16) | 14 (16) | 14 (16) | 15 (17) | 16 (18) | 17 (19) | 18 (20) |
|  | gradientHL0 gradientVL0 gradientHL1 gradientVL1 | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 10 (12) | 11 (13) | 12 (14) | 13 (15) |
|  | diff | 11 (13) | 11 (13) | 11 (13) | 11 (13) | 11 (13) | 12 (14) | 13 (15) | 14 (16) | 15 (17) |
|  | tempH tempV | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 10 (12) | 11 (13) | 12 (14) | 13 (15) |
|  | bdofOffset | 15 (17) | 15 (17) | 15 (17) | 15 (17) | 15 (17) | 16 (18) | 17 (19) | 18 (20) | 19 (21) |
|  | pbSamples | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |

Table 2  Internal bit precision of BDOF (Proposal)

|  |  | bitDepth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| Value | shift1 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
|  | shift2 | 4 | 4 | 4 | 4 | 4 | 5 | 6 | 7 | 8 |
|  | shift3 | 1 | 1 | 1 | 1 | 1 | 2 | 3 | 4 | 5 |
|  | shift4 | 7 | 6 | 5 | 4 | 3 | 3 | 3 | 3 | 3 |
|  | mvRefineThres | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| Precision (dynamic range) | predSamplesL0 predSamplesL1 | 14 (16) | 14 (16) | 14 (16) | 14 (16) | 14 (16) | 15 (17) | 16 (18) | 17 (19) | 18 (20) |
|  | gradientHL0 gradientVL0 gradientHL1 gradientVL1 | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 10 (12) | 11 (13) | 12 (14) | 13 (15) |
|  | diff | 11 (13) | 11 (13) | 11 (13) | 11 (13) | 11 (13) | 11 (13) | 11 (13) | 11 (13) | 11 (13) |
|  | tempH tempV | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 9 (11) |
|  | bdofOffset | 15 (17) | 15 (17) | 15 (17) | 15 (17) | 15 (17) | 16 (18) | 17 (19) | 18 (20) | 19 (21) |
|  | pbSamples | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |

Experimental results

The common test conditions for high bit-depth video coding is based on JVET-T0047[3]. Since this proposal is only applied when the configuration is RA and the bit-depth is greater than 12-bit, the result of RA-16 is shown in Table 3.

Table 3  Experimental results of RA-16

|  | RA-16 (QP=-33,-28,-23,-18,-13,-8) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0 lmcs off and bdpcm on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| SVT | 0.00% | -0.01% | 0.00% | 100% | 101% |

Since the value of QP is very small, the overall encoding and decoding times are too long to evaluate the BDOF processing time correctly and the loss is -0.02% when BDOF is off. BDOF is not used on this condition.

Table 4 shows the result of the QP range from 22 to 37. The loss is -0.02% and the encoding and decoding time become faster. The loss is 0.66% when BDOF is off.

Table 4  Experimental results of RA-16 (QP=22,27,32,37)

|  | RA-16 (QP=22,27,32,37) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0 lmcs off and bdpcm on |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| SVT | -0.02% | 0.01% | -0.01% | 98% | 96% |

Additionally, several results are shown based on the previous CTC [4]. Table 4 shows the results of our proposal with InternalBitDepth equal to 14. The anchor is the result of CTC with InternalBitDepth equal to 14. The loss is -0.02% and the encoding and decoding time become faster. The loss is 0.77% when BDOF is off.

Table 5  Experimental results of IBDI14

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0_IBDI14 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.02% | 0.09% | 97% | 96% |
| Class A2 | -0.02% | 0.03% | 0.00% | 97% | 95% |
| Class B | -0.01% | 0.01% | 0.04% | 97% | 94% |
| Class C | -0.02% | 0.06% | -0.07% | 97% | 94% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.03% | 0.01% | 97% | 95% |
| Class D | -0.02% | 0.01% | 0.08% | 97% | 92% |
| Class F | -0.02% | -0.03% | -0.03% | 97% | 99% |

The reason for testing IBDI14 is that the result of VTM-10 for 16-bit coding may have some problems as Table 5. Usually, as the internal bit depth is increased, the coding efficiency is a little improved.

Table 6  Experimental results of internalBitDepth from 12 to 16

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0 (10-bit) |  |  |  |  |  |
|  | Y | U |  | V | EncT | DecT |
| InternalBitDepth 12 | -0.51% |  | 0.61% | 0.58% | 131% | 104% |
| InternalBitDepth 14 | -0.57% |  | 1.06% | 1.02% | 177% | 189% |
| InternalBitDepth 16 | 0.91% |  | 2.35% | 1.86% | 180% | 185% |

Reference

[1]	T. Chujoh, E. Sasaki and T. Ikai, “Non-CE9: A bug fix of BDOF specification”, Joint Video Experts Team Document, JVET-O0207, Gothenburg, July 2020.

[2]	B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 10)”, Joint Video Experts Team Document, JVET-S2001, teleconference, July 2020.

[3] T. Ikai, A. Browne and X. Xiu, “AHG12: Preliminary common test conditions for high bitdepth video coding”, JVET-T0047, Oct. (2020)

[4]	F. Bossen, J. Boyce, K. Sühring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, Joint Video Experts Team Document, JVET-N1010, Geneva, March 2019.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).