| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0087 |
| --- | --- |

| Title: | AHG12: VVC tool evaluation in high bit rate coding |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Tianyang Zhou Tomonori Hashimoto 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution checks VVC coding tools in high bitrate range. It is reported that MTS, CCLM, DEPQUANT shows more than 0.5 % gain, i.e. 0.80 %, 1.51 %, and 1.41 % gain in average bdrateY in AI-12 (QP=-13, -8, -3, 2, 7, 12) respectively. It is also reported LFNST shows 0.47% loss in terms of coding efficiency.

It is suggested to define common test conditions for high bit-depth coding or recommends experts to study coding tools to confirm if it is performed appropriately or it can be improved by adjustments.

Introduction

VVC tools are designed in 420, 422, 444 and 8 to 12bits. However, it has been not well known how much effective in low QPs for appropriate high bit depth.

Experiment

Table1 shows bd-rate changes of tools compared to all-tools on anchor. plus means gain,

It is noted that EncT and DecT are not accurate. We will fix this issue in update.

Table 1 Result on AI-12 (QP=-13, -8, -3, 2, 7, 12)

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| ALF | 0.20% | 0.56% | 0.99% | 101% | 90% |
| SAO | 0.00% | -0.01% | -0.03% | 102% | 97% |
| MTS | 0.80% | 0.48% | 0.47% | 83% | 97% |
| LFNST | -0.47% | 0.08% | 0.12% | 92% | 100% |
| MRL | 0.14% | 0.06% | 0.06% | 101% | 97% |
| ISP | 0.04% | 0.04% | 0.04% | 95% | 96% |
| CCLM | 1.51% | 2.76% | 2.88% | 93% | 98% |
| JCCR | 0.20% | 0.06% | 0.08% | 89% | 100% |
| DEPQUQNT | 1.41% | 3.29% | 3.34% | 82% | 105% |

References

[1]	T. Ikai, A. Browne, X. Xiu, “AHG12: Preliminary common test conditions for high bitdepth video coding”, JVET-T0047, teleconference, October 2020.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).