| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0087 |
| --- | --- |

| Title: | AHG12: VVC tool evaluation in high bit rate coding |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Tianyang Zhou Tomonori Hashimoto 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution checks VVC coding tools in high bitrate range. It is reported that MTS, CCLM, DEPQUANT shows more than 0.5 % gain, i.e. 0.80 %, 1.51 %, and 1.41 % gain in average bdrateY in AI-12 (QP=-13, -8, -3, 2, 7, 12) respectively. It is also reported LFNST shows 0.47% loss in terms of coding efficiency.

It is suggested to define common test conditions for high bit-depth coding based on experiments. And it is also recommended experts to study coding tools to confirm if it is performed appropriately or it can be improved by adjustments.

v2 added encoder configuration test, which include CTU size and max MTT size configuration. It is proposed that a configuration with maxmtt size 16 shows better balance between encoding time and gain where the maximum loss with 0.19% loss and EncT is 61% in LDB-16, which changes the maximum encoding time from 3.6 days to 1.7 days.

Introduction

VVC tools are designed in 420, 422, 444 and 8 to 12bits. However, it has been not well known how much effective in low QPs for appropriate high bit depth.

Experiment

Table1 shows bd-rate changes of tools compared to all-tools on anchor. plus means gain,

It is noted that EncT and DecT are not accurate. We will fix this issue in update.

Table 1 Result on AI-12 (QP=-13, -8, -3, 2, 7, 12) and AI-16 (QP=-33, -28, -23, -18, -13, -8)

|  | AI-12 |  |  |  |  |  |  |  |  |  | AI-16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U |  | V |  | EncT |  | DecT |  | Y |  | U |  |  | V |  | EncT |  | DecT |
| MRL | 0.14% | 0.06% |  | 0.06% |  | 100% |  | 96% |  | 0.03% |  | 0.02% |  | 0.02% | 101% |  | 102% |  |
| ISP | 0.04% | 0.04% |  | 0.04% |  | 95% |  | 96% |  | 0.00% |  | 0.04% |  | 0.04% | 98% |  | 100% |  |
| CCLM | 1.51% | 2.76% |  | 2.88% |  | 93% |  | 98% |  | -0.45% |  | 2.21% |  | 2.26% | 92% |  | 102% |  |
| MIP | 0.30% | 0.13% |  | 0.13% |  | 90% |  | 96% |  | 0.57% |  | 0.29% |  | 0.29% | 85% |  | 98% |  |
| MTS | 0.80% | 0.48% |  | 0.47% |  | 81% |  | 95% |  | 0.50% |  | 0.34% |  | 0.34% | 67% |  | 97% |  |
| LFNST | -0.47% | 0.08% |  | 0.12% |  | 89% |  | 96% |  | -0.49% |  | -0.17% |  | -0.17% | 68% |  | 104% |  |
| JCCR | 0.20% | 0.06% |  | 0.08% |  | 87% |  | 98% |  | 0.00% |  | 0.00% |  | 0.00% | 78% |  | 100% |  |
| DEPQUQNT | 1.41% | 3.29% |  | 3.34% |  | 81% |  | 104% |  | 2.16% |  | 2.49% |  | 2.55% | 90% |  | 101% |  |
| SAO | 0.00% | -0.01% |  | -0.03% |  | 101% |  | 96% |  | 0.00% |  | 0.00% |  | 0.00% | 99% |  | 100% |  |
| ALF | 0.20% | 0.56% |  | 0.99% |  | 100% |  | 89% |  | 0.00% |  | 0.00% |  | 0.00% | 99% |  | 100% |  |
| ctu32 | 0.11% | 0.01% |  | -0.04% |  | 97% |  | 102% |  | 0.08% |  | 0.12% |  | 0.12% | 61% |  | 102% |  |
| maxmtt8 | 0.60% | 0.49% |  | 0.48% |  | 36% |  | 98% |  | 0.40% |  | 0.66% |  | 0.65% | 16% |  | 103% |  |
| maxmtt16 | 0.05% | 0.08% |  | 0.07% |  | 75% |  | 97% |  | 0.08% |  | 0.12% |  | 0.12% | 61% |  | 102% |  |

Fig 2. Gain per tools in AI-12 and AI-16

EncT vs Gain in tools

Fig 1. and Table 2 show average of AI-12 and AI-16 results. As can be seen, depquant show relatively good gain while JCCR, ISP shows less gain in terms of EncT – gain balance. Nevertheless MTS and MIP shows relatively good gain, encoding time increase can be seen not be small.

Fig 3. EncT vs Gain in AI

Table 2. EncT vs Gain in AI

|  | EncT | Gain |
| --- | --- | --- |
| MRL | 101% | 0.08% |
| ISP | 96% | 0.02% |
| CCLM | 93% | 0.53% |
| MIP | 87% | 0.43% |
| MTS | 74% | 0.65% |
| LFNST | 78% | -0.48% |
| JCCR | 82% | 0.10% |
| DEPQUQNT | 85% | 1.78% |
| SAO | 100% | 0.00% |
| ALF | 100% | 0.10% |

Encoder configuration with respect to CTU size and MTT

We compared the following three configuration in CTU size and MTT.

Table 3.  Encoder configuration

|  | CTC | CTU32 | maxmtt8 | maxmtt16 |
| --- | --- | --- | --- | --- |
| CTUSize | 128 | 32 | 128 | 128 |
| MaxBTlumaISlice | 32 | 32 | 8 | 16 |
| MaxBTChromaISlice | 64 | 32 | - | - |
| MaxBTNonISlice | 128 | 32 | 8 | 16 |
| MaxTTLumaISlice | 32 | 32 | 8 | 16 |
| MaxBTChromaISlice | 32 | 32 | - | - |
| MaxTTNonISlice | 64 | 32 | 8 | 16 |
| Log2MaxTbSize | 6 | 5 | - | - |

or

[ctu32]

--CTUSize=32 --MaxBTLumaISlice=32 --MaxBTChromaISlice=32 --MaxBTNonISlice=32 --MaxTTLumaISlice=32 --MaxTTChromaISlice=32 --MaxTTNonISlice=32 --Log2MaxTbSize=5

[maxmtt8]

--MaxBTLumaISlice=8 --MaxBTNonISlice=8 --MaxTTLumaISlice=8 --MaxTTNonISlice=8

[maxmtt16]

--MaxBTLumaISlice=16 --MaxBTNonISlice=16 --MaxTTLumaISlice=16 --MaxTTNonISlice=16

Results

Experimental results shows that CTU32 speed-up achieves up-to 23% in LDB-16 while max MTT size restriction achieves up to 67% and 56% in LDB-16 which is very significant. In terms of coding efficiency, loss on LDB-12 is relatively large, 0.32 %, 0.53 % and 0.19% in CTU32, maxmtt8 and maxmtt16 respectively.

Table 3.  Summary results on encoder configuration

|  | AI-12 | AI-16 | LDB-12 | LDB-16 | AI-12 | AI-16 | LDB-12 | LDB-16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CTU32 | 0.11% | 0.00% | 0.32% | 0.14% | 97% | 98% | 85% | 77% |
| maxmtt8 | 0.60% | 0.40% | 0.53% | 0.18% | 36% | 16% | 42% | 37% |
| maxmtt16 | 0.05% | 0.08% | 0.19% | 0.08% | 75% | 61% | 61% | 54% |

Table 4  Maximum encoding time on encoder configuration

|  |  | AI-12 | LDB-12 | RA-12 | AI-16 | LDB-16 | RA-16 | All [hours] | All [days] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T0047 | Max | 20 | 67 | 231 | 21 | 86 | 260 | 86 | 3.6 |
| CTU32 | Max | 20 | 58 | 0 | 21 | 58 | 0 | 58 | 2.4 |
| maxmtt8 | Max | 5 | 27 | 0 | 3 | 29 | 0 | 29 | 1.2 |
| maxmtt16 | Max | 12 | 41 | 0 | 12 | 38 | 0 | 41 | 1.7 |

Fig 4. EncT vs Gain in AI

Proposal

In summary, maxmtt16 is suggested as common test condition for AHG12 high bit depth coding as

- Loss: 0.05%	, 0.08%, 0.19% and 0.08% in AI-12, AI-16, LDB-12, LDB-16

- EncT: 75%, 61%, 61% and 64% in AI-12, AI-16, LDB-12, LDB-16

- Max encoding time reduced 3.6 day to 1.7 day in LDB-16

References

[1]	T. Ikai, A. Browne, X. Xiu, “AHG12: Preliminary common test conditions for high bitdepth video coding”, JVET-T0047, teleconference, October 2020.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).