| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0089 |
| --- | --- |

| Title: | AHG12: Slice based Rice parameter selection for transform skip residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu, Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Che-Wei Kuo, Xianglin Wang | Email: | jhuhong-jheng@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In residual coding of a transform skip block, for each sample, the remaining absolute level “abs_remainder” is binarized using the fixed Rice parameter equal to 1. However, significant performance degradation is reported in VVC high bit depth configuration. It is observed that using different Rice parameters for different conditions (e.g. different sequences, different QPs, different bit depths) improves the coding efficiency especially for high bit depth conditions. In this proposal, it is proposed to explicitly signal the Rice parameter for each slice to indicate the Rice parameter for the binary codewords of abs_remainder. The results are summarized as below.

The results of slice-level Rice parameter compared to VTM10.0 on SVT sequences:

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | G | B | R | EncT | DecT |
| 12-bit | -1.1% | -1.2% | -1.2% | 88% | 97% |
| 14-bit | -7.4% | -8.9% | -8.8% | 90% | 98% |
| 16-bit | -27.3% | -27.2% | -27.4% | 88% | 99% |

# Introduction

Above 12-bit an issue with the Golomb-Rice coding is reported as the cause of a significant reduction in performance from VVC. In residual coding of a transform skip block, the remainder of the absolute levels are binarized using fixed Rice parameter “1”. But it is observed that different coding bit-depths use different Rice parameters obtains higher coding performance. In this proposal, a slice based Rice parameter is proposed for the transform skip residual coding for higher coding efficiency especially for high bit depth conditions.

# Proposal

In the proposed method, one control flag is signaled in sequence parameter set to indicate the signaling of Rice parameter for the transform skip blocks is enabled or disabled. When the control flag is signaled as enabled, one syntax element is further signaled for each transform skip slice to indicate the Rice parameter of that slice. When the control flag is signaled as disabled (e.g. set equal to “0”), no further syntax element is signaled at lower level to indicate the Rice parameter for the transform skip slice and a default Rice parameter (e.g. 1) is used for all the transform skip slice.

In the proposed method, the Rice parameter is coded by ue(v), unsigned integer 0-th order Exp-Golomb-coded syntax element with the left bit first. The used Rice parameters for each slice and the corresponding bit depth are given in Table 1.

Table 1 Proposed Rice parameters for high bit depths

| Rice parameter | Bit depth |
| --- | --- |
| 4 | 12 |
| 6 | 14 |
| 8 | 16 |

# Simulation Results

The proposed method is implemented based on the VTM-10.0 reference software [1] and tested using SVT 16-bit RGB 4:4:4 sequences defined in [2], High tier (HT) and Medium tier (MT) QP range and 50 frames. In the testing, Intra block copy (IBC), BDPCM, Hash-based motion search, palette mode (PLT), and Adaptive colour transform (ACT) were enabled in the RGB 4:4:4 test condition defined in [3] with RExt__HIGH_BIT_DEPTH_SUPPORT = 1.

Table 2: Results of Slice based Rice Parameters for high bit depth

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | G | B | R | EncT | DecT |
| 12-bit | -1.1% | -1.2% | -1.2% | 88% | 97% |
| 14-bit | -7.4% | -8.9% | -8.8% | 90% | 98% |
| 16-bit | -27.3% | -27.2% | -27.4% | 88% | 99% |

# Working Draft Text

The changes to the VVC Draft [4] are highlighted below.

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_sign_data_hiding_enabled_flag | u(1) |
| sps_ts_residual_coding_rice_present_in_sh_flag | u(1) |
| sps_virtual_boundaries_enabled_flag | u(1) |
| … |  |
| } |  |

sps_ts_residual_coding_rice_present_in_sh_flag equal to 1 specifies that sh_ts_residual_coding_rice_idx could be present in SH syntax structures referring to the SPS. sps_ts_residual_coding_rice_present_in_sh_flag equal to 0 specifies that sh_ts_residual_coding_rice_idx is not present in SH syntax structures referring to the SPS. When sps_ts_residual_coding_rice_present_in_sh_flag is not present, the value of sps_ts_residual_coding_rice_present_in_sh_flag is inferred to be equal to 0.

### Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_transform_skip_enabled_flag && !sh_dep_quant_used_flag && !sh_sign_data_hiding_used_flag ) |  |
| sh_ts_residual_coding_disabled_flag | u(1) |
| if((!sh_ts_residual_coding_disabled_flag) && sps_ts_residual_coding_rice_enabled _flag ) |  |
| sh_ts_residual_coding_rice_idx | ue(v) |
| … |  |
| } |  |

sh_ts_residual_coding_rice_idx specifies the rice parameter used for the residual_ts_coding( ) syntax structure.

# Conclusions

- A slice based Rice parameter for transform skip is proposed in this contribution, which has no negligible complexity to the decoder but provides significant BD-rate saving for high bit depth conditions. It is recommended the proposed method be adopted into VVC version 2.

# References

- VTM-10.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-10.0.

- C. Rosewarne, K. Sharman, D. Flynn, “ Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-P1006, 16th meeting, San José, US, 9–17 Jan. 2014

- Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu, “JVET common test conditions and software reference configurations for non-4:2:0 colour formats”, JVET-R2013, by teleconference, 15–24 April 2020

- B. Bross, J. Chen, S. Liu, and Y.-K. Wang, “Versatile Video Coding (Draft 10)”, JVET-S2001, by teleconference, 22 June – 1 July 2020.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).