| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0090 |
| --- | --- |

| Title: | AHG9: Digital signature SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Ye-Kui Wang, Li Zhang, Kai Zhang 8910 University Center Ln, San Diego, CA92122, USA | Tel: Email: | +1-619-436-7216 xujizheng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This document proposes a digital signature SEI message. The purpose of this SEI message is to provide a mechanism to protect a segment/whole video from tampering.

# Introduction

This document introduces digital signature SEI into video. It is a straightforward extension of current widely used file verification to a segment (including whole) video. Compared with JVET-S0051, this document tries to address the concerns raised by the experts and complete the design.

# Digital signature

Digital signature is widely used in file verification. This document is to extend the solution of file verification to a segment of video signals via a digital signature SEI message.

Figure 1. Digital signature process (from [2]).

Figure 1 shows the general structure of digital signature. To apply it to video segment, a spatiotemporal region needs to be specified and a secure hash of the region is needed to be sent via a secure way.

# Changes to the text

## Digital signature SEI message syntax

| digital_signature( payloadSize ) { | Descriptor |
| --- | --- |
| ds_cancel_flag | u(1) |
| if ( !ds_cancel_flag ) { |  |
| ds_persistent_flag | u(1) |
| ds_reserved_zero_bit | u(1) |
| ds_type | u(3) |
| ds_secure_hash_type | u(8) |
| ds_single_component_flag | u(1) |
| ds_region_params_present_flag | u(1) |
| if( ds_region_params_present_flag ) { |  |
| ds_region_start_left | u(16) |
| ds_region_start_top | u(16) |
| ds_region_width | u(16) |
| ds_region_height | u(16) |
| } |  |
| ds_fingerprint_present_flag | u(1) |
| if( ds_ fingerprint_present_flag ) { |  |
| ds_fingerprint | u(160) |
| } |  |
| ds_digital_signature | u(v) |
| } |  |
| } |  |

## Digital signature SEI message semantics

This message provides a secure hash for a rectangular region of the associated decoded pictures.

Use of this SEI message requires the definition of the following variables:

–	A picture width and picture height in units of luma samples, denoted herein by PicWidthInLumaSamples and PicHeightInLumaSamples, respectively.

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in clause 7.3 of JVET-S2007-v7.

–	A bit depth for the samples of the luma component, denoted herein by BitDepthY, and when ChromaFormatIdc is not equal to 0, a bit depth for the samples of the two associated chroma components, denoted herein by BitDepthC.

–	For each colour component cIdx, an array ComponentSamples[ cIdx ][ i ] in raster scan order of decoded sample values in two's complement representation.

ds_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous digital signature SEI message in decoding order. ds_cancel_flag equal to 0 indicates that digital signature information follows.

ds_persistence_flag specifies the persistence of the digital signature SEI message for the current layer.

ds_persistence_flag equal to 0 specifies that the digital signature SEI message applies to the current decoded picture only.

ds_persistence_flag equal to 1 specifies that the digital signature SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a digital signature SEI message that follows the current picture in decoding order.

Let the the number of decoded pictures associated with a digital signature SEI message with ds_cancel_flag equal to 0, be numAssociatedPics.

Prior to computing the hash, the decoded picture data are arranged into one or three strings of bytes called regionData[ cIdx ] of lengths dataLen as follows:

dataLen = 0
for( i = 0; i < numAssociatedPics; i++) {
	for( cIdx = 0; cIdx < ds_single_component_flag ? 1 : 3; cIdx++ ) {
		if( cIdx  = =  0 ) {
			startX[ cIdx ] = ds_region_start_left * SubWidthC
			startY[ cIdx ] = ds_region_start_top * SubHeightC
			regionWidth = ds_region_width * SubWidthC
			regionHeight = ds_region_height * SubHeightC
			compDepth[ cIdx ] = BitDepthY
		} else {
			startX[ cIdx ] = ds_region_start_left
			startX[ cIdx ] = ds_region_start_top 
			regionWidth = ds_region_width 
			regionHeight = ds_region_height 
			compDepth[ cIdx ] = BitDepthC
		}
		for( i = 0; i < regionHeight; i++ ) {
			for( j = 0; j < regionWidth; j++ ) {
				regionData[ cIdx ][ dataLen++ ] = ComponentSamples[ cIdx ][ i + startY ][ j + startX ] & 	0xFF
				if( compDepth[ cIdx ] > 8 )
					    regionData[ cIdx ][ dataLen++ ] = ComponentSamples[ cIdx ][ i ]  >>  8
			}
		}
	}
}

ds_reserved_zero_bit shall be equal to 0. The value 1 for ds_reserved_zero_bit is reserved for future use by ITU-T | ISO/IEC.

ds_type indicates the method used to calculate the digital signature as specified in the following table. Values of ds_type that are not listed in in the table are reserved for future use by ITU-T | ISO/IEC and shall not be present in payload data conforming to this version of this Specification. Decoders shall ignore digital signature SEI messages that contain reserved values of ds_type.

| digital_signature_type | Method |
| --- | --- |
| 0 | DSA |
| 1 | RSA |
| 2 | ECDSA |
| 3~7 | reserved |

ds_secure_hash_type indicates the method used to calculate the hash value as specified in the following table. Values of ds_secure_hash_type that are not listed in in the table are reserved for future use by ITU-T | ISO/IEC and shall not be present in payload data conforming to this version of this Specification. Decoders shall ignore secure picture hash SEI messages that contain reserved values of ds_secure_hash_type.

| Secure_hash_type | Method |
| --- | --- |
| 0 | SHA-1 |
| 1 | SHA-224 |
| 2 | SHA-256 |
| 3 | SHA-384 |
| 4 | SHA-512 |
| 5 | SHA-512/224 |
| 6 | SHA-512/256 |
| 7~255 | reserved |

ds_single_component_flag equal to 1 specifies that the pictures associated with the digital signaure SEI message contains a single colour component. ds_single_component_flag equal to 0 specifies that the pictures associated with the digital signaure SEI message contains three colour components. The value of ds_single_component_flag shall be equal to ( ChromaFormatIdc  = =  0 ).

ds_region_params_present_flag equal to 1 specifies that ds_region_start_left, ds_region_start_top, ds_region_width and ds_region_height are present. ds_region_params_present_flag equal to 0 specifies that ds_region_start_left, ds_region_start_top, ds_region_width and ds_region_height are not present.

If the SEI message is contained in a scalable nesting SEI message with sn_subpic_flag equal to 1, the variable subpicFlag is set equal to 1. Otherwise (the SEI message is not contained in a scalable nesting SEI message with sn_subpic_flag equal to 1), subpicFlag is equal to 0.

ds_region_start_left and ds_region_start_top specify the left and right offsets of the upper left corner of the region relative to the upper left corner of the decoded picture (when subpicFlag is equal to 0) or the decoded subpicture (when subpicFlag is equal to 1), in units of chroma samples for non-4:0:0 colour formats and in units of luma sample for 4:0:0 colour format. When ds_region_params_present_flag is equal to 0, the values of ds_region_start_left and ds_region_start_top are both inferred to be equal to 0.

ds_region_width and ds_region_height specifies the width and height, respectively, of the region in units of chroma sample. When ds_region_params_present_flag is equal to 0, the values of ds_region_width and ds_region_height are inferred to be the width and height, respectively, of the decoded picture (when subpicFlag is equal to 0) or the decoded subpicture (when subpicFlag is equal to 1), in units of chroma samples for non-4:0:0 colour formats and in units of luma sample for 4:0:0 colour format.

It is required that when present, both ds_region_width and ds_region_height shall be larger or equal to 1.

The value of ( ds_region_start_left + ds_region_width ) shall be less than or equal to the width of the decoded picture (when subpicFlag is equal to 0) or the decoded subpicture (when subpicFlag is equal to 1) in units of chroma samples for non-4:0:0 colour formats and in units of luma samples for 4:0:0 colour format.

The value of ( ds_region_start_top + ds_region_height ) shall be less than or equal to the height of the decoded picture (when subpicFlag is equal to 0) or the decoded subpicture (when subpicFlag is equal to 0) in units of chroma samples for non-4:0:0 colour formats and in units of luma samples for 4:0:0 colour format.

digital_signature specifiies information including multiple bytes for verification the integraty of the protected region.

For DSA(2048, 256), the digital signature contains two integers, r and s, both of them are of 256 bits. The syntax element is 64 bytes, having a value dsVal defined as follows:

dsVal = 0
	for( i = 248; i  >=  0; i  −=  8 )
		dsVal  +=  r  >>  i & 0xFF
	dsVal = dsVal  <<  256
	for( i = 248; i  >=  0; i  −=  8 )
		dsVal  +=  s  >>  i & 0xFF

# References

[1] J. Xu etc, “AHG9: Digital signature SEI,” JVET-S0051

[2] FIPS PUB 186-4, “Digital Signature Standard,” http://dx.doi.org/10.6028/NIST.FIPS.186-4

[3] FIPS PUB 180-4, “Secure Hash Standard,” http://dx.doi.org/10.6028/NIST.FIPS.180-4

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).