| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0091 |
| --- | --- |

| Title: | AHG12: Fix on encoder overflow issue of the LMCS at high bit-depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Hong-Jheng Jhu, Yi-Wen Chen, Tsung-Chuan Ma, Xianglin Wang | Email: | {xiaoyuxiu, jhuhong-jheng, yiwenchen, tsung-chuanma, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

In this contribution, one encoder overflow problem was identified in current VTM-10 reference software for the calculation of the LMCS parameters of SDR content when internal bit-depth is 16-bit. Encoder fixes are provided in the contribution to resolve such overflow issue. Simulation results reportedly show compared to the VTM-10.0 16-bit anchors, the average {Y, U, V} BD-rate performance of the proposed fixes are {-0.88%, 0.40%, 0.12%}, {-1.31%, 1.61%, 1.61%} and {-0.55%, 2.3%, 2.51%} for AI, RA and LDB configurations, respectively.

- Proposal

In VTM-10.0 [1], when the LMCS is enabled, one encoder algorithm based on local sample variances is applied to calculate the piece-wise linear luma mapping function of SDR video contents. The method intends to map the segments with smaller variances to a larger dynamic range (i.e., assigning more code-words), and the segments with larger variances to a smaller dynamic range (i.e., assigning fewer code-words), after the luma mapping is applied. In the current LMCS encoder implementation, the local variance of each luma samples is calculated based on a window of neighboring samples centered at the current position and the window size is adaptively adjusted according to the resolution of the picture. To calculate the variance of each luma sample, both the sample values and the squared sample values need to be accumulated over all the luma samples within the local window. In the VTM-10.0, because the macro for high bit-depth coding (i.e., RExt__HIGH_BIT_DEPTH_SUPPORT) is enabled by default, the data type used for storing the intermediate variables during the sample variance calculation are signed 32-bit integer. However, such precision may not be sufficient when the internal bit-depth is 16-bit where the squared sample values used by the LMCS encoder algorithm may go beyond 32-bit. As indicated by the experimental results, such 32-bit overflow problem may lead to abnormal luma mapping functions derived by the current VTM-10.0 encoder.

To fix such issue, encoder bug fixes are provided in this contribution where the data type used for a number of intermediate parameters used for the LMCS parameter derivation for SDR content are increased from signed 32-bit to signed 64-bit to allow enough precision to match the actual dynamic of those parameters. The implementation of the proposed encoder fixes is based on the VTM-10.0 and one merge request will be submitted if requested.

- Simulation results

The proposed encoder fixes are implemented on top of the VTM-10.0 reference software and tested by following the current CTC [2] except that the internal bit-depth, i.e., InterBitDepth, is set to 16 for both the anchor and the test.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-9.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.72% | -3.17% | -5.11% | 106% | 100% |
| Class A2 | -2.82% | 0.12% | 0.47% | 102% | 100% |
| Class B | -0.48% | -0.89% | -0.85% | 103% | 98% |
| Class C | -0.07% | 2.44% | 2.65% | 103% | 104% |
| Class E | 0.12% | 3.67% | 3.23% | 102% | 104% |
| Overall | -0.88% | 0.40% | 0.12% | 103% | 101% |
| Class D | 0.16% | 1.77% | 2.03% | 104% | 105% |
| Class F | -0.56% | 1.53% | 1.56% | 105% | 102% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-9.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.98% | 1.03% | 0.26% | 102% | 100% |
| Class A2 | -2.00% | 4.52% | 4.53% | 105% | 100% |
| Class B | -1.14% | 0.30% | 0.34% | 106% | 103% |
| Class C | -1.23% | 1.49% | 2.03% | 100% | 104% |
| Class E |  |  |  |  |  |
| Overall | -1.31% | 1.61% | 1.61% | 103% | 102% |
| Class D | -0.86% | 0.69% | 0.99% | 103% | 99% |
| Class F | -1.09% | 0.41% | 0.93% | 104% | 100% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-9.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.81% | 1.43% | 1.19% | 122% | 110% |
| Class C | -0.72% | 2.83% | 3.28% | 106% | 107% |
| Class E | -0.06% | 2.46% | 2.80% | 106% | 106% |
| Overall | -0.55% | 2.30% | 2.51% | 106% | 105% |
| Class D | -1.10% | 2.06% | 3.37% | 112% | 109% |
| Class F | -0.37% | 1.38% | 1.35% | 108% | 105% |

It should be mentioned that in the above table, four sequences “MarketPlace”, “RitualDance”, “RaceHorses” and “BasketballDrillText” are excluded when calculating the average BD-rate numbers for the LDB configuration, due to its abnormal bitrate and PSNR behavior of luma component in the VTM-10.0 anchor which is caused by the incorrect luma mapping function due to the 32-bit overflow problem existing in the current LMCS parameter derivation. As shown in the attached spreadsheet, after the proposed encoder fixes are applied, the performance of the four problematic sequences become normal.

- Reference

- VTM-9.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-9.0.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, Switzerland.

- Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).