| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0092-v1 |
| --- | --- |

| Title: | AHG9/AHG11: Neural network based super resolution SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Eiich Sasaki Tomohiro Ikai | Tel: Email: | +81-50-5213-6334 chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

In this contribution, a framework for introducing a super-resolution post-filter using CNN is proposed. In VVC, RPR (Reference Picture Re-sampling) has been introduced. By using VTM-10.0, as a result of some experiments to change the resolution of the whole sequence, in several 4K sequences, there were some coding gains. In this experiment, the up-sampling filter is used the interpolation filter of VVC. If some super-resolution post-filters using CNN are introduced, the visual quality will be more improved. For example, MPEG NNR which is discussed in SC29/WG4 can describe neural networks efficiently. In this contribution, a simple SEI which can encode and decode a neural network for super-resolution is proposed. It is recommended that AHG11 should consider a method of using RPR.

Problem Statement

It has been known that there are some cases that the coding efficiency of down-sampled coding is better than that of original resolution coding when high-resolution sequences are encoded in the condition of low-bit rate. Since recently super-resolution technologies using CNN have been significantly improved [1], its possibility should be utilized.

Some experiments to change the resolution of the whole sequence have been conducted by RPR coding on VTM-10.0. As in Tables 1 and 2 (see attached Excel sheets), several 4K sequences get good coding gains. In this experiment, the cases of ScalingRatioHor and ScalingRatioVer equal to 1.5 and 2.0 are tested. The QP range of anchor is 32, 37, 42, 47 and that of RPR coding is 27, 32, 37, 42. PSNR between the original sequence and the decoded up-sampled (--UpscaledOutput = 2) sequence is calculated. In order to valance between luma and chroma gains, the chroma QP offset of RPR coding were changed.

Table 1  Experimental results of ScalingRatioHor and ScalingRatioVer equal to 1.5

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0(QP=32,37,42,47) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -5.59% | -5.02% | -8.49% | 177% | 101% |
| Class A2 | -1.44% | 11.53% | 2.40% | 160% | 99% |

Table 2  Experimental results of ScalingRatioHor and ScalingRatioVer equal to 2.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0 (QP32,37,42,47) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -5.32% | -12.38% | -11.79% | 107% | 57% |

Proposal

The following is a simple SEI message is proposed.

| output_picture_upsampling_parameter_sei_message( payloadSize ) { | Descriptor |
| --- | --- |
| upsampling_parameter_idc | u(2) |
| upsampling_parameter_format_idc | u(6) |
| for (i = 1; i < payloadSize;_i++) { |  |
| upsampling_parameter_payload_byte | b(8) |
| } |  |
| } |  |

upsampling_parameter_idc specifies the value of the index of neural network parameters.

upsampling_parameter_format_idc specifies the format of the neural network parameter. For example, MPEG NNR [3] is a good format for compressed neural network parameters. Otherwise, ONNX (Open Neural Network eXchange) and NNEF (Neural Network Exchange Format) will be one of the candidates for parameter format.

upsampling_parameter_payload_byte is a byte of the payload type of the SEI message. According to the format which is specified by upsampling_paramter_format_idc, the bytes are decoded, and the neural network parameters are received.

Reference

[1]	W. Yang, X. Zhang, Y. Tian, W. Wang, J-H Xue and Q. Liao, “Deep Learning for Single Image Super-Resolution: A Brief Review”, IEEE Trans. Multimedia, vol.21, no.12, pp.3106-3121, Dec. 2019.

[2]	S. Liu, A. Segall, E. Alshina, J. Boyce, M. Wien and D. Grois, “Methodology and reporting template for neural network coding tool testing”, Joint Video Experts Team Document, JVET-T0041, teleconference, Oct. 2020.

[3] “Text of ISO/IEC CD 15938-17 Compression of Neural Networks for Multimedia Content Description and Analysis”, ISO/IEC JTC 1/SC29/WG11 Documents, N19499, Online, July 2020.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).