| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0094_r1 |
| --- | --- |

| Title: | AHG11: In-loop filtering based on neural network |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tsung-Chuan Ma, Wei Chen, Xiaoyu Xiu, Yi-Wen Chen, Hong-Jheng Jhu, Che-Wei Kuo, Xianglin Wang | Email: | {tsung-chuanma, chenwei06, xiaoyuxiu, yiwenchen, jhuhong-jheng, cheweikuo, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

A deep-learning based in-loop filter is proposed in this contribution. The proposed in-loop filter is located between the de-blocking and SAO stages. The implementation based on VTM9.0, the average {Y, U, V} BD-rate performance of the proposed deep-learning based filter is {-4.99%, -16.39%, -17.34%} and {-3.92%, -18.09%, -16.93%} for AI and RA configurations, respectively.

- Proposal

In this contribution, a neutral network based in-loop filter is proposed and placed between the de-blocking and SAO process in order to improve the BD-rate over VTM9.0 [1].

As shown in Fig. 1 (a), the architecture of proposed in-loop filter consists of the residual block (RB) [2] and the 2-dimensional up-sampling convolution. The chroma samples are up-sampled from 64x64 to 128x128 and then concatenated with luma sample to form a 3x128x128 input features. In Fig. 1 (b), the RB contains two 3x3 Convolution filter with K input/output features. In this contribution, N and K are set equal to 20 and 64, respectively.

Figure 1 (a) the proposed deep-learning in-loop filter (b) the detail of residual block

- Training Process

In this proposal, the DIV2K dataset [3] was used in the training process. The original RGB images are encoded and decoded by VTM9.0 with AI configuration after converted into YUV420 format. The QPs for training are 22, 27, 32, and 37. The reconstructed images are then split into 128x128 luma and 64x64 chroma blocks. The batch size is set equal to 8 in our training process. The epoch is 20 for the training process.

- Inference in VTM9.0

The proposed neutral network based in-loop filter is implemented in VTM9.0 with CTU and frame level flags. If the frame level flag is turn-off, all CTU in current frame are not applied with proposed filter. If the frame level flag is turn-on, the CTU level flag is signaled to indicate whether the proposed filter is applied.

- Simulation Results

The proposed in-loop filter is implemented on top of the VTM-9.0 [1] reference software.

The CTC defined in [4] is used for simulation. However, due to limited resource, only the frames within first two intra periods are used for each sequence in our simulation. Additionally, both the anchor and test data are generated based on CPUs

Table 1 Simulation result of proposed neutral network based in-loop filter for AI configuration

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM9.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.61% | -10.86% | -13.64% | 1901% | 996565% |
| Class A2 | -4.14% | -15.29% | -16.62% | 850% | 743219% |
| Class B | -4.49% | -17.88% | -18.15% | 850% | 785819% |
| Class C | -5.76% | -19.18% | -20.71% | 478% | 563704% |
| Class E | -7.03% | -16.81% | -15.94% | 976% | 939230% |
| Overall | -4.99% | -16.39% | -17.34% | 875% | 775056% |
| Class D | -6.64% | -18.22% | -21.60% | 500% | 644479% |
| Class F | -3.29% | -15.75% | -15.50% | 430% | 700323% |

Table 2 Simulation result of proposed neutral network based in-loop filter for RA configuration

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM9.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.90% | -9.57% | -13.03% | 207% | 866536% |
| Class A2 | -4.48% | -18.40% | -18.85% | 191% | 506881% |
| Class B | -3.82% | -22.21% | -18.07% | 205% | 702260% |
| Class C | -3.65% | -19.09% | -16.99% | 171% | 1000062% |
| Class E |  |  |  |  |  |
| Overall | -3.92% | -18.09% | -16.93% | 193% | 754013% |
| Class D | -3.80% | -19.34% | -18.37% | 187% | 641745% |
| Class F | -1.91% | -11.57% | -11.72% | 278% | 596299% |

- Reference

- VTM-9.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-9.0.

- K. He, X. Zhang, S. Ren, and J. Sun. "Deep residual learning for image recognition." In Proceedings of the IEEE conference on computer vision and pattern recognition, pp. 770-778. 2016.

- https://data.vision.ee.ethz.ch/cvl/DIV2K/ R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool

- Methodology and reporting template for neural network coding tool testing, JVET-T0041, S. Liu, A. Segall, E. Alshina, J. Boyce, M. Wien, D. Grois

- Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).