| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0095-v2 |
| --- | --- |

| Title: | Performance of a VVC software decoder |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bin Zhu Shan Liu Xiaozhong Xu Xianguo Zhang Chenchen Gu Liang Wang Weiwei Feng | Tel: Email: | bzhu@tencent.com shanl@tencent.com xiaozhongxu@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

An independent VVC software decoder implemented by Tencent is demonstrated in this contribution. The performance of such a decoder is provided by decoding the CTC bitstreams generated by VTM-10 encoder. It is reported that for CTC RA configuration, an average of 48.5 fps can be achieved on an 8-core 8-thread (without overclocking) processor when decoding 4K bitstreams, a 21X speedup over the VTM decoder; an average of 182 fps can be achieved on an 8-core processor when decoding full HD (1080p) bitstreams; an average of 259 fps can be achieved on an 8-core processor when decoding full HD (1080p) SCC bitstreams.

Introduction

Upon the completion of VVC standard, the implementability of such a new codec format remains a challenge to its wide deployment in the industry. This is partly because its target applications include ultra high resolution and high bitrate video. The added new features on top of HEVC also contribute to the total complexity. As an early stage attempt, an independent VVC software decoder has been developed by Tencent, trying to demonstrate the capability of efficient decoding VVC bitstreams.

Below is a high-level summary of the highlights of the VVC software decoder implementation by Tencent:

- Picture level parallelism

All intra coded pictures, as well as inter coded picture that do not have dependency, can be decoded in parallel.

- CTU level parallelism

Typical WPP pattern can enable CTUs to be decoded in parallel.

- Module level parallelism

Pixel processor (MV derivation, prediction, De-quant, Inv-transform and Recon.) and filter processor (LMCS, DBK, SAO, ALF) can work in parallel. In addition, Decoder parser and pixel processor can work in parallel as long as they operate in different frames.

- Sub-CTU level parallelism

All inter coded CUs can be decoded in parallel, SMID optimization, etc.

Results

Table I below shows the results of decoding 4K bitstreams (generated with VTM version 10.0, RA configuration of common test conditions). The software was run on Windows Desktop i7-9700@3.0G platform. The average acceleration factor over VTM decoder is 21 (48.1/2.3) with an 8-core processor setting (hyperthreading not supported).

Table I: decoding speed comparison among VTM and Tencent O266 decoders, 4K CTC streams

| Sequence | QP | Frames | Bitrate [Mbps] | VTM [fps] | O266 Thread 1 [fps] | O266 Thread 2 [fps] | O266 Thread 4 [fps] | O266 Thread 8 [fps] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tango | 22 | 294 | 27.86 | 2.03 | 6.74 | 13.02 | 23.82 | 41.86 |
|  | 27 | 294 | 7.54 | 2.48 | 8.21 | 15.62 | 28.48 | 50.33 |
|  | 32 | 294 | 3.62 | 2.72 | 9.11 | 17.33 | 31.34 | 55.48 |
|  | 37 | 294 | 1.97 | 3.04 | 10.59 | 20.11 | 36.64 | 64.11 |
| FoodMarket | 22 | 300 | 16.98 | 2.11 | 7.02 | 13.43 | 24.85 | 44.47 |
|  | 27 | 300 | 7.58 | 2.40 | 7.92 | 15.16 | 27.98 | 48.62 |
|  | 32 | 300 | 3.91 | 2.66 | 8.96 | 17.07 | 31.41 | 56.37 |
|  | 37 | 300 | 2.06 | 2.95 | 10.21 | 19.5 | 35.92 | 63.58 |
| Campfire | 22 | 300 | 59.03 | 1.35 | 4.23 | 8.33 | 15.93 | 29.01 |
|  | 27 | 300 | 15.33 | 2.09 | 7.16 | 13.76 | 24.43 | 46.12 |
|  | 32 | 300 | 6.93 | 2.54 | 8.79 | 16.69 | 30.51 | 54.96 |
|  | 37 | 300 | 3.50 | 2.91 | 10.07 | 19.07 | 34.71 | 59.05 |
| CatRobot | 22 | 300 | 33.71 | 1.86 | 6.13 | 11.81 | 21.77 | 38.27 |
|  | 27 | 300 | 8.68 | 2.45 | 7.98 | 15.31 | 28.39 | 50.36 |
|  | 32 | 300 | 4.11 | 2.79 | 9.29 | 17.81 | 32.94 | 57.57 |
|  | 37 | 300 | 2.14 | 3.06 | 10.65 | 20.34 | 37.75 | 66.46 |
| DaylightRoad | 22 | 300 | 58.71 | 1.62 | 5.23 | 10.17 | 18.77 | 32.75 |
|  | 27 | 300 | 10.23 | 2.27 | 7.41 | 14.21 | 26.1 | 47.68 |
|  | 32 | 300 | 4.12 | 2.62 | 8.61 | 16.35 | 30.07 | 54.04 |
|  | 37 | 300 | 2.06 | 2.96 | 10.08 | 19.16 | 35.32 | 61.98 |
| ParkRunning | 22 | 300 | 108.17 | 1.17 | 3.57 | 6.99 | 13.53 | 24.75 |
|  | 27 | 300 | 41.64 | 1.46 | 4.72 | 9.17 | 17.35 | 31.82 |
|  | 32 | 300 | 18.47 | 1.76 | 5.83 | 11.19 | 20.96 | 38.49 |
|  | 37 | 300 | 8.01 | 2.22 | 7.27 | 13.86 | 25.43 | 45.64 |
| Average |  |  | 19.02 | 2.31 | 7.74 | 14.81 | 27.27 | 48.49 |

Table II and III below shows the results of decoding 2K bitstreams (generated with VTM version 10.0, RA configuration of common test conditions and SCC CE conditions). The software was run on Windows Desktop i7-9700@3.0G platform. It is shown that in call cases when 8 threads are used the decoding speed fulfills the real-time requirement which is typically set to 60fps. In fact, in most cases 4 threads can achieve real-time decoding while 8 threads can significantly exceed 60fps. At the similar bitrate, it was also observed that decoder can achieve higher performance(fps) while decoding SCC sequences compared with non-SCC ones.

Table II: decoding speed comparison among VTM and Tencent O266 decoders, 2K CTC streams

| Sequence | QP | Frames | Bitrate [Mbps] | VTM [fps] | O266 Thread 1 [fps] | O266 Thread 2 [fps] | O266 Thread 4 [fps] | O266 Thread 8 [fps] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MarketPlace | 22 | 600 | 14.39 | 6.74 | 21.02 | 40.06 | 73.13 | 129.63 |
|  | 27 | 600 | 5.12 | 8.54 | 26.84 | 50.78 | 91.12 | 162.95 |
|  | 32 | 600 | 2.26 | 10.12 | 32.66 | 61.53 | 108.68 | 192.03 |
|  | 37 | 600 | 1.00 | 11.6 | 38.92 | 73.51 | 127.79 | 219.38 |
| RitualDance | 22 | 600 | 9.22 | 7.13 | 23.12 | 43.74 | 79.37 | 143.36 |
|  | 27 | 600 | 4.57 | 8.78 | 28.79 | 54.21 | 97.84 | 166.05 |
|  | 32 | 600 | 2.43 | 10.35 | 34.23 | 64.29 | 114.96 | 201.47 |
|  | 37 | 600 | 1.27 | 11.84 | 40.17 | 75.27 | 132.6 | 233.2 |
| Cactus | 22 | 500 | 14.09 | 6.65 | 21.22 | 40.24 | 71.09 | 120.21 |
|  | 27 | 500 | 4.12 | 9.66 | 32.18 | 60.24 | 107.48 | 184.36 |
|  | 32 | 500 | 1.89 | 11.43 | 39.33 | 73.27 | 128.41 | 216.78 |
|  | 37 | 500 | 0.93 | 12.99 | 47.11 | 88.1 | 152.03 | 257.45 |
| BasketballDrive | 22 | 500 | 14.44 | 6.2 | 19.56 | 37.34 | 67.16 | 117.78 |
|  | 27 | 500 | 4.69 | 8.3 | 26.44 | 49.81 | 88.79 | 158.62 |
|  | 32 | 500 | 2.16 | 9.76 | 31.53 | 58.8 | 103.19 | 182.4 |
|  | 37 | 500 | 1.07 | 11.02 | 35.66 | 66.12 | 115.14 | 207.01 |
| BQTerrace | 22 | 600 | 34.07 | 4.65 | 14.38 | 27.91 | 52.19 | 93.8 |
|  | 27 | 600 | 5.47 | 9.75 | 31.75 | 59.04 | 101.57 | 174.95 |
|  | 32 | 600 | 1.69 | 12.16 | 39.79 | 73.26 | 126.24 | 223.15 |
|  | 37 | 600 | 0.75 | 13.46 | 45.14 | 83.49 | 143 | 254.02 |
| Average |  |  | 6.28 | 9.56 | 31.49 | 59.05 | 104.09 | 181.93 |

Table III: decoding speed comparison among VTM and Tencent O266 decoders, 2K SCC streams

| Sequence | QP | Frames | Bitrate [Mbps] | VTM [fps] | O266 Thread 1 [fps] | O266 Thread 2 [fps] | O266 Thread 4 [fps] | O266 Thread 8 [fps] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ArenaOfValor | 22 | 600 | 11.81 | 7.3 | 23.91 | 45.84 | 82.36 | 138.16 |
|  | 27 | 600 | 5.06 | 9.7 | 32.56 | 61.78 | 110.26 | 184.50 |
|  | 32 | 600 | 2.43 | 11.7 | 40.35 | 76.25 | 135.24 | 222.32 |
|  | 37 | 600 | 1.21 | 13.3 | 47.77 | 89.75 | 156.99 | 255.59 |
| FlyingGraphics | 22 | 300 | 24.06 | 5.1 | 16.53 | 32.14 | 59.17 | 105.20 |
|  | 27 | 300 | 12.32 | 6.1 | 20.67 | 39.85 | 72.06 | 125.59 |
|  | 32 | 300 | 7.15 | 7.1 | 24.21 | 46.43 | 82.74 | 144.37 |
|  | 37 | 300 | 4.04 | 8.7 | 28.88 | 54.97 | 96.60 | 166.74 |
| Desktop | 22 | 600 | 1.95 | 18.5 | 76.40 | 136.91 | 219.94 | 321.52 |
|  | 27 | 600 | 1.44 | 18.7 | 81.41 | 147.15 | 241.04 | 348.31 |
|  | 32 | 600 | 1.13 | 19.3 | 88.66 | 161.70 | 268.53 | 393.31 |
|  | 37 | 600 | 0.91 | 19.5 | 89.22 | 161.86 | 269.33 | 401.83 |
| Console | 22 | 600 | 3.64 | 14.5 | 54.25 | 97.35 | 159.72 | 264 |
|  | 27 | 600 | 2.70 | 15.5 | 58.73 | 105.64 | 171.39 | 276.23 |
|  | 32 | 600 | 2.05 | 16.1 | 63.07 | 112.75 | 184.02 | 293.37 |
|  | 37 | 600 | 1.54 | 17.2 | 69.32 | 124.26 | 201.62 | 313.01 |
| ChineseEditing | 22 | 600 | 4.03 | 17.2 | 69.45 | 126.33 | 196.31 | 273.68 |
|  | 27 | 600 | 2.99 | 17.4 | 74.16 | 135.57 | 214.83 | 302.41 |
|  | 32 | 600 | 2.20 | 17.6 | 75.90 | 139.09 | 223.26 | 315.82 |
|  | 37 | 600 | 1.57 | 18.0 | 78.06 | 142.40 | 231.68 | 336.42 |
| Average |  |  | 4.71 | 13.93 | 55.68 | 101.9 | 168.85 | 259.12 |

Player

A video player is implemented based on the presented VVC decoding capability and can display VVC bitstreams in real-time. This player is built based the open source solution of FFmpeg/VLC and comes with basic play/pause control.  It is complying with the GPL license and will be released to download soon at https://mmedia.tencent.com/vvc-player.

Conclusion

An example VVC software decoder is demonstrated in this contribution, with its performance reported when decoding the VVC CTC 4K/2K bitstreams in RA configurations. With multithreading, the decoder can decode 2K bitstreams in real-time with a large margin and achieve a speed of 48.5 fps when handling 4K bitstrteams.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).