| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0097 |
| --- | --- |

| Title: | Report on VVC compression performance verification testing in the SDR UHD Random Access Category |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Mathias Wien Vittorio Baroncini | Tel: Email: | wien@lfb.rwth-aachen.de baroncini@gmx.com |
| Source: | Verification Test Coordinators |  |  |

_____________________________

# Abstract

This document reports the first set of verification test results comparing VVC to its predecessor, HEVC. The compression performance capabilities of the two specifications are compared based on the HEVC reference software HM-16.22, and two VVC implementations, the VVC reference software VTM-9.0 and the open-source VVC implementation VVenC-0.1.0. For HEVC, the random-access configuration provided with the configuration file cfg/encoder_randomaccess_main10.cfg of HM-16.22 was used. For the VTM, the random-access configuration provided with the configuration file cfg/encoder_randomaccess_vtm_gop32.cfg of VTM-9.0 was employed. The results of a visual assessment of VVC compared to HEVC by naïve test subjects is reported. The assessment included five SDR UHD test sequences encoded in random-access configuration with a random-access interval of 1.07s. The measured MOS figures indicate a significant improvement of VVC over HEVC for both VVC implementations, VTM-9.0 and VVenC-0.1.0, resulting in an overall average bitrate saving of -43% and -49%, respectively. Based on the fact that the VVenC-0.1.0 software has been used at a medium configuration resulting in more than 100 times faster runtime than the VTM encoder, an overall average bitrate saving of about 50% of VVC over HEVC may be concluded.

# Introduction

A major design goal for the development of the VVC standard was to achieve a substantial improvement in compression capability relative to its predecessor, the HEVC standard. This document is the first verification test report in a series of reports addressing a variety of test categories and embracing some of the available versatile tools provided by the VVC standard. It reports the results of a verification test to confirm that this goal was achieved and to estimate the magnitude of that achievement.

A subjective evaluation was conducted at two test sites comparing the VVC Main 10 profile to the HEVC Main 10 profile for the SDR UHD test category with random-access configuration.

# Verification test logistics

The SDR UHD subjective test was carried out at the following test sites:

- GBTech, Rome, IT

- RWTH Aachen University, Aachen, DE

The tests were conducted using the DCR test method [9] with a 11-grade impairment scale. The verification test environment and the testing methodology is described in Anne A. The test set-up at the two test sites is shown in Table 1.

Table 1: Test site information and set up.

| Test Site | GBTech | RWTH Aachen University |
| --- | --- | --- |
| Display, size (resolution setting) | LG 65” CX6LA (3840x2160) | Sony PVM X550, 55” (3840×2160) |
| Viewing distance | 2 viewers at 1,5H | 1 viewer at 1.5H |
| Viewing angle | 60° (30° from screen center) | 90° (at screen center) |
| Total number of viewers | 16 (7 females, 9 males; age 18-24), all screened for visual acuity and normal colour vision. | 24 (5 females, 19 males, age 16-34), all screened for visual acuity and normal colour vision. |

# Verification test results

In the test, the HEVC bitstreams were encoded using the HEVC reference software HM16.22 [2]. For VVC, two encoder implementations were used. One set of bitstreams was encoded using the VTM-9.0 reference software [3], a second set of bitstreams was encoded using the open source VVC implementation VVenC-0.1.0 [4][5][6]. For all, the configuration enables random access to the bitstream every 1.07s, i.e. every 32 pictures for 30Hz test sequences and every 64 pictures for 60Hz test sequences.

For HEVC, the random-access configuration provided with the configuration file cfg/encoder_randomaccess_main10.cfg of HM-16.22 was used. In a pre-assessment, the verification test chairs compared the visual impact of the GOP32 and GOP16 configuration settings of VTM and decided for the GOP32 settings. Accordingly, for the VTM, the random-access configuration provided with the configuration file cfg/encoder_randomaccess_vtm_gop32.cfg of VTM-9.0 was employed. It is noted that the rate-distortion performance of VTM-10.0 has been asserted to be identical to VTM-9.0 subject to minor rate changes which are induced by modifications of high-level signaling in the final version of VVC. For the VVenc-0.1.0 software [4][5][6], the Medium configuration has been used. VVenC was operated with perceptual QP adaptation and without rate control. In this configuration, the VVenC-0.1.0 encoding speed is reportedly about 100 times faster than the VTM. For the purpose of the verification tests, the QPs were selected such that the VVenC rate was approximately the same or lower than the VTM bitrate. Details of the applied configuration and a comprehensive analysis of the VVenC performance are provided in input document JVET-T0103 [7].

The employed set of five test sequence candidates for the SDR category in UHD resolution as defined in the verification test plan document [1] is listed in Table 2.

Five rate points for each test sequence have been selected for the quality assessment of the test sequences. The rate points were chosen such that the VTM/HM pair for a rate point would represent approximately same quality while at the same time allow for approximate rate matching of the HM rate point with the next VTM rate point. Thereby both assessment of rate-savings at similar quality and assessment of quality improvement at similar rates are enabled. The selected QPs corresponding to these rate points are listed in Table 3.

Table 2: SDR UHD test sequences.

| No. | Test sequence | Resolution | fps | Frames | md5 |
| --- | --- | --- | --- | --- | --- |
| 01 | DrivingPOV3 | 3840x2160 | 60 | 0:599 | e81b65724c4235128b2749ccb3b0fb4a |
| 02 | Marathon2 | 3840x2160 | 30 | 0:299 | c065dfb87be3b2e2ab0ce35094fd4eb4 |
| 03 | MountainBay2 | 3840x2160 | 30 | 0:299 | f27b6b70244fb083baac546958fcf696 |
| 04 | NeptuneFountain3 | 3840x2160 | 60 | 0:599 | 88fd87ea57df4a36200946025e8618aa |
| 05 | TallBuildings2 | 3840x2160 | 30 | 0:299 | 9a0a3f261d004fa86754751c82fb8b47 |

Table 3: QP settings for HM, VTM, VVenC for the SDR UHD test sequences.

| No. | Test sequence | HM-16.22 QP | VTM-9.0 | VVenC-0.1.0 |
| --- | --- | --- | --- | --- |
| 01 | DrivingPOV3 | 30, 33, 36, 39, 43 | 30, 34, 39, 42, 46 | 29, 33, 38, 42, 46 |
| 02 | Marathon2 | 28, 32, 36, 39, 43 | 30, 34, 39, 42, 46 | 29, 33, 38, 41, 45 |
| 03 | MountainBay2 | 27, 30, 33, 36, 39 | 30, 33, 36, 39, 42 | 30, 33, 36, 39, 43 |
| 04 | NeptuneFountain3 | 30, 32, 35, 37, 39 | 33, 35, 37, 39, 42 | 32, 34, 36, 38, 43 |
| 05 | TallBuildings2 | 28, 31, 35, 39, 43 | 30, 34, 38, 42, 46 | 28, 32, 36, 40, 44 |

The test sequences are provided in ITU-R BT.709 colour space [12][13] and are available on the JVET ftp site (ftp://ftp.ient.rwth-aachen.de) in the folder /ahg/verificationtests/vtsequences using the general JVET login credentials (available from the JVET chairs). The bitstreams used in the verification tests are available in the folder /ahg/verificationtests/JVET-T_VT/sdr-uhd on the same ftp site.

The test sequences were evaluated using the 11-grade scale as specified in ITU-R BT.500-14, shown in Figure 1 below.

Figure 1: Meaning of the 11 grades numerical scale as specified in ITU-R BT.500-14 Table 2-4 [8].

## MOS plots

The results of the visual assessment in at the two test sites were evaluated by the verification test coordinators, and were found to be consistent in terms of quality range and quality progression over the rate points as voted by the test subjects. Therefore, the votes of both test sites were merged into a joint evaluation providing the results provided below. The measured MOS values of the reconstructed video on the 11-grade scale are plotted over the bitrate of the corresponding bitstream. The ±0.95% confidence intervals for the MOS values are indicated.

Figure 2: Collection of the MOS-over-rate plots for HM, VVC, and VVenC for the SDR UHD test sequences.

The MOS-over-rate plots provided in Figure 2 consistently indicate a significant subjective quality improvement of VVC over HEVC. For some sequences, a visual quality close to transparent (MOS higher than 8.5) is achieved already for the 4th rate point of VTM and/or VVenC. The plots further reveal that for some test sequences and some rate points, the intended quality matching of VTM and HM is not achieved in the subjective evaluation with naïve test subjects. This specifically holds for the DrivingPOV sequence, where the visual quality of the VVC encoders was consistently rated higher than the quality of the corresponding HM rate points.

# Analysis of the MOS over rate plots

The Bjøntegaard Delta rate (BD-rate) for both, the VTM-9.0 and VVenC, relative to HM-16.22 are reported in Table 4.  The numbers have been calculated based using the 5-point BD method provided in JVET-T0041 [16]. A further analysis of the test sequences compressed by HM, VTM, and VVenC including objective metrics is provided in JVET-T0103 [7].

The rate and MOS differences for all rate points are collected in Table 5. The rate saving is computed as the difference between the VVC rate point and the corresponding HEVC rate point relative to the HEVC bitrate. The MOS difference is reported as a number if the value is larger than the maximum of the VVC and the HEVC confidence intervals. Otherwise, “< CI” is indicated. The results are reported relative to the HM for both, the VTM and VVenC.

Table 4: Bjøntegaard Delta Rate relative to HM-16.22 based on bitrate and MOS.

| BD-Rate | VTM | VVenC |
| --- | --- | --- |
| DrivingPOV3 | -61% | -63% |
| Marathon2 | -37% | -42% |
| MountainBay2 | -37% | -39% |
| NeptuneFountain3 | -38% | -52% |
| TallBuildings2 | -41% | -51% |
| Overall | -43% | -49% |

Table 5: Rate savings and MOS deltas for the rate points

| VTM / HM |  | Rate Diff. | ΔMOS |  | VCenC / HM |  | Rate Diff. | ΔMOS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DrivingPOV3 | R1 | -51,0% | 0,5 |  | DrivingPOV3 | R1 | -51,2% | < CI |
| DrivingPOV3 | R2 | -49,3% | 1,0 |  | DrivingPOV3 | R2 | -50,6% | 1,3 |
| DrivingPOV3 | R3 | -48,0% | 0,7 |  | DrivingPOV3 | R3 | -48,3% | 0,8 |
| DrivingPOV3 | R4 | -30,1% | 1,7 |  | DrivingPOV3 | R4 | -29,5% | 2,3 |
| DrivingPOV3 | R5 | -16,4% | 0,9 |  | DrivingPOV3 | R5 | -15,3% | 1,4 |
| Marathon2 | R1 | -47,7% | < CI |  | Marathon2 | R1 | -48,7% | -0,5 |
| Marathon2 | R2 | -45,0% | < CI |  | Marathon2 | R2 | -46,4% | < CI |
| Marathon2 | R3 | -43,6% | < CI |  | Marathon2 | R3 | -48,3% | -0,8 |
| Marathon2 | R4 | -35,5% | < CI |  | Marathon2 | R4 | -38,7% | < CI |
| Marathon2 | R5 | -36,7% | < CI |  | Marathon2 | R5 | -37,4% | < CI |
| MountainBay2 | R1 | -48,8% | < CI |  | MountainBay2 | R1 | -50,9% | < CI |
| MountainBay2 | R2 | -47,4% | -0,6 |  | MountainBay2 | R2 | -50,3% | -0,7 |
| MountainBay2 | R3 | -47,1% | < CI |  | MountainBay2 | R3 | -50,7% | -0,8 |
| MountainBay2 | R4 | -46,3% | -0,8 |  | MountainBay2 | R4 | -50,4% | -0,5 |
| MountainBay2 | R5 | -45,7% | -0,5 |  | MountainBay2 | R5 | -48,8% | -0,5 |
| NeptuneFountain3 | R1 | -39,7% | < CI |  | NeptuneFountain3 | R1 | -39,8% | -0,5 |
| NeptuneFountain3 | R2 | -27,4% | 0,5 |  | NeptuneFountain3 | R2 | -36,7% | 1,3 |
| NeptuneFountain3 | R3 | -27,1% | 0,7 |  | NeptuneFountain3 | R3 | -34,1% | 0,9 |
| NeptuneFountain3 | R4 | -38,5% | < CI |  | NeptuneFountain3 | R4 | -43,3% | < CI |
| NeptuneFountain3 | R5 | -39,6% | < CI |  | NeptuneFountain3 | R5 | -41,6% | 0,4 |
| TallBuildings2 | R1 | -46,0% | -0,9 |  | TallBuildings2 | R1 | -46,6% | -1,4 |
| TallBuildings2 | R2 | -44,7% | < CI |  | TallBuildings2 | R2 | -46,9% | 0,8 |
| TallBuildings2 | R3 | -43,3% | < CI |  | TallBuildings2 | R3 | -44,8% | < CI |
| TallBuildings2 | R4 | -41,5% | < CI |  | TallBuildings2 | R4 | -43,0% | < CI |
| TallBuildings2 | R5 | -32,6% | < CI |  | TallBuildings2 | R5 | -33,6% | < CI |

The figures reported in Table 4 and Table 5 indicate significant compression performance improvements for VVC compared to its predecessor HEVC. It is observed that the VVC encoder implementation provided by VVenC provides more gain over HM than the VTM for the vast majority of rate points.

# Conclusions

The results of a visual assessment of VVC compared to HEVC by naïve test subjects is reported. The assessment included five SDR UHD test sequences encoded in random-access configuration with a random-access interval of 1.07s. The measured MOS figures indicate a significant improvement of VVC over HEVC for both VVC implementations, VTM-9.0 and VVenC-0.1.0, resulting in an overall average bitrate saving of -43% and -49%, respectively. Based on the fact that the VVenC-0.1.0 software has been used at a medium configuration resulting in more than 100 times faster runtime than the VTM encoder, an overall average bitrate saving of about 50% of VVC over HEVC may be concluded.

# References

- M. Wien, V. Baroncini, A. Segall, and Y. Ye, “VVC verification test plan (Draft 3),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 5, output document JVET-S2009, 19th JVET meeting by teleconference, June 2020.

- HEVC Test Model, https://vcgit.hhi.fraunhofer.de/jct-vc/HM/-/tree/HM-16.22, online, accessed Oct. 2020.

- VVC Test Model, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/VTM-9.0, online, accessed Oct. 2020.

- J. Brandenburg, A. Wieckowski, T. Hinz, A. Henkel, V. George, I. Zupancic, C. Stoffers, B. Bross, H. Schwarz, D. Marpe, “Towards Fast and Efficient VVC Encoding,” IEEE 22nd Workshop on Multimedia Signal Processing (MMSP 2020), Tampere, Finland, 2020.

- A. Wieckowski, J. Brandenburg, C. Bartnik, V. George, J. Güther, G. Hege, C. Helmrich, A. Henkel, T. Hinz, C. Lehmann, C. Stoffers, I. Zupancic, B. Bross, H. Schwarz, D. Marpe, “Open optimized VVC encoder (VVenC) and decoder (VVdeC) implementations,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 5, input document JVET-T0099, Oct. 2020.

- Fraunhofer HHI, “Fraunhofer Versatile Video Encoder (VVenC),” version 0.1.0 (initial release), https://github.com/fraunhoferhhi/vvenc/tags, online, accessed Oct. 2020.

- C. Helmrich, B. Bross, J. Pfaff, H. Schwarz, D. Marpe, T. Wiegand, “Information on and analysis of the VVC encoders in the SDR UHD verification test,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 5, input document JVET-T0103, Oct. 2020.

- Recommendation ITU-R BT.500-14 (2019), Methodologies for the subjective assessment of the quality of television images.

- Recommendation ITU-T P.910 (2008), Subjective video quality assessment methods for multimedia applications.

- Recommendation ITU-R BT.2100-2 (2018), Image parameter values for high dynamic range television for use in production and international programme exchange.

- SMPTE ST 2084, High Dynamic Range Electro-Optical Transfer Function of Mastering Reference Displays, 2014.

- SMPTE ST 2036-1, Ultra High Definition Television – Image Parameter Values for Program Production, 2014.

- ETSI TS 101 154, Digital Video Broadcasting (DVB); Specification for the use of Video and Audio Coding in Broadcast and Broadband Applications, 2019.

- K. Andersson, F. Bossen, J.-R. Ohm, A. Segall, R. Sjöberg, J. Ström, G. J. Sullivan, A. Tourapis, “Working practices using objective metrics for evaluation of video coding efficiency experiments (Draft 3),” Joint Video Experts Team, doc. JVET-S2016, teleconference, July 2020.

- G. Bjøntegaard, “Calculation of average PSNR differences between RD-curves,” in ITU-T SG 16 Q.6 document VCEG-M33, 13th VCEG meeting, Austin, Texas, USA, Apr. 2001.

- S. Liu, A. Segall, E. Alshina, J. Boyce, M. Wien, D. Grois, “Methodology and reporting template for neural network coding tool testing,” Joint Video Experts Team, doc. JVET-T0041, teleconference, Oct. 2020.

- J. Ström, K. Andersson, Rickard Sjöberg, A. Segall, F. Bossen, G. J. Sullivan, J.-R. Ohm, A. Tourapis, “Working practices using objective metrics for evaluation of video coding efficiency experiments (Draft 3),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 output document JVET-S2016, 19th JVET meeting by teleconference, June 2020.

# Acknowldegements

The verification test coordinators wish to thank the organizations who contributed to the VVC verification tests. [List to be provided in an output version of the verification test report]

# Annex A

The same evaluation method as for the HEVC verification tests is adopted for the VVC verification tests. The following description is based on JCTVC-Q1011 [A4] with minor adaptations.

## A.1 Test method

The test method adopted for this evaluation is DCR (Degradation Category Rating) [A1].

## A.1.1 Degradation Category Rating (DCR)

This test method is commonly adopted when the material to be evaluated shows a range of visual quality that well distributes across all quality scales. All the video material used for these tests consist of video clips of 10 seconds duration.

This method has been used under the schema of evaluation of the quality; for this reason, a quality rating scale made of 11 levels was adopted, ranging from "0" (lowest quality) to "10" (highest quality), see also Figure 1.

The structure of the Basic Test Cell (BTC) of DCR method is made by two consecutive presentations of the video clip under test; at first the original version of the video clip is displayed, immediately afterwards the coded version of the video clip is presented; then a message displays for 5 seconds asking the viewers to vote. The presentation of the video clips is preceded by a mid-grey screen displaying “Source” for the original and “Test” for the coded version of the sequence under test for one second.

Figure 3 - DCR BTC

## A.2 How to express the visual quality opinion with DCR

The viewers were asked to express their vote putting a mark on a scoring sheet.

The scoring sheet for a DCR test is made of a section for each BTC; each section has a box wherein which the viewer shall write the score ranging from 0 to 10. By writing a score of “10”, the subject will express an opinion of “best” quality, while by writing a score of “0” the subject will express an opinion of “worst” quality, as shown in Figure 1.

The vote has to be written when the message "Vote N" appears on the screen. The number "N" is a numerical progressive indication on the screen aiming to help the viewing subjects to use the appropriate box of the scoring sheet.

## A.4 Training and stabilization phase

The outcome of a test is highly dependent on a proper training of the test subjects.

For this purpose, each subject has to be trained by means of a short practice (training) session demonstrating the range of qualities to be expected in the test.

The stabilization phase uses the test material of a test session; three BTCs, containing one sample of best quality, one of the worst qualities and one of medium quality, are duplicated at the beginning of the test session. By this way, the test subjects have an immediate impression of the quality range they are expected to evaluate during that session.

The scores of the stabilization phase are discarded.

## A.5 The laboratory set-up

The laboratory for a subjective assessment were set up according to [A1], except for the selection of the display and the video play-out server. Play-out of the UHD video clips was done at the native resolution.

The PCs used to play the video sequence supported the display of 10 bit UHD at 30 and 60 frames per second, without any limitation, or without introducing any additional temporal or visual degradation. At GBTech, the connection between the PC and the display was provided by a 10 bit-capable HDMI connection. At RWTH Aachen University, the display was connected via quad-link SDI.

## A.5.2 Viewing environment

The viewing distance was 1.5H, where H is equal to the height of the active part of the screen, depending on the size of the active part of the screen and its native resolution.

The test laboratories were protected from external visual or audio pollution. Internal general light was low (just enough to allow the viewing subjects to fill out the scoring sheets) and a uniform light was placed behind the monitor, in a way no direct light hits the viewing subjects seated in front of the screen; the light behind the monitor must be dimmed to an intensity as specified in Table 4 of Recommendation ITU-T P.911 (“Typical viewing and listening conditions as used in audio-visual quality assessment”). No other light source was admitted, and in particular any light source directed to the screen or creating reflections.

## A.6 Overall test effort and subjects’ involvement

Each viewing session did not run for more than 20 minutes and the same viewing subject did not participate to the test run for more than six hours in total. Young people were hired as test subjects, selecting them for an age from 16 to 30, mostly students of scientific faculties. Viewing subjects were compensated for their participation to the testing activities.

## A.7 Statistical analysis and presentation of the results

The data collected from the score sheets, filled out by the viewing subjects, were stored in an Excel spread sheet. For each coding condition the Mean Opinion Score (MOS) and associated Confidence Interval (CI) values were computed in the spread-sheets.

The MOS and CI values are used to draw graphs. The graphs are drawn grouping the results for each video test sequence. No graph grouping results from different video sequences is considered.

From the “raw” data subject reliability should be calculated and the method used to assess subject reliability should be reported. Some criteria for subjective reliability are given in [A2] and [A3].

## A.8 References

- International Telecommunication Union Standardization Sector; Recommendation ITU-T P.910 “Subjective video quality assessment methods for multimedia applications”

- Pseudo Isochromatic Plates, engraved and printed by The Beck Engraving Co., Inc., Philadelphia and New York, United States.

- KIRK (R.E.): Experimental Design – Procedures for the Behavioural Sciences, 2nd Editions, Brooks/Cole Publishing Co., California, 1982.

- Tan, T. K.; Mrak, M.; Baroncini, V. & Ramzan, N, “Report on HEVC compression performance verification testing,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 2014.