| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 October 2020 | Document: JVET-T0098-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-T0074:AHG10: LMCS encoder algorithm for YUV4:4:4 content |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen | Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

This contribution reports crosscheck results of LMCS encoder algorithm for YUV 4:4:4 content which is proposed in JVET-T0074[1]. It is reported that the coding results are identical with those reported by the proponent.

- Introduction

JVET-T0074 proposes an automatic non-normative encoder optimization algorithm for LMCS to improve coding performance for SDR YUV4:4:4 video content. This contribution crosschecks the algorithm and the simulation were conducted for YUV444 contents. The simulation results are identical to what reported in JVET-T0074

Simulation results

Simulations were conducted under SDR YUV444 CTC. The test sequences including camera-captured contents and screen contents. The anchor is VTM10.0 without LMCS and the test is VTM10.0 with the proposed encoder algorithm.

Table 1.  LMCS with proposed algorithm vs. LMCS off in AI

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | YUV 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -1.65% | 2.06% | 1.83% | 105% | 103% |
| TGM 720p | -1.00% | 3.27% | 3.86% | 108% | 104% |
| Animation | -0.88% | 2.60% | 2.65% | 104% | 103% |
| Mixed content | -1.27% | 3.77% | 4.15% | 108% | 104% |
| Screen content Overall | -1.22% | 2.89% | 3.08% | 106% | 104% |
| camera capture | -0.32% | 0.69% | 1.04% | 108% | 103% |

Table 2.  LMCS with proposed algorithm vs. LMCS off in RA

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | YUV 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -1.05% | 0.47% | 0.53% | 101% | 99% |
| TGM 720p | -1.45% | 0.43% | 0.23% | 102% | 100% |
| Animation | -1.45% | -1.19% | -0.86% | 100% | 100% |
| Mixed content | -2.87% | -0.07% | 0.58% | 102% | 100% |
| Overall | -1.64% | -0.01% | 0.16% | 102% | 100% |
| Camera capture | -1.13% | 2.98% | 0.20% | 105% | 102% |

References

[1]F. Pu, T. Lu, P. Yin, S. McCarthy, W. Husak, T. Chen AHG10: LMCS encoder algorithm for YUV 4:4:4 content, JVET-T0074, teleconference, October, 2020.