| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0102 |
| --- | --- |

| Title: | Crosscheck of JVET-T0089:AHG12: Slice based Rice parameter selection for transform skip residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Adrian Browne | Email: | adrian.browne@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This document reports a cross check of contribution JVET-T0089 by Kwai which proposes changing the method for the selection of Rice parameters for transform skip residual coding (TSRC). The source code was studied and matched with the proposal. Experiments confirmed results for 12 and 16 bit AI.

# Cross check procedure

The source code for JVET-T0085 was supplied by Kwai and compiled using gcc-4.8.5. The experiment was run on a cluster of Intel i7-6700T CPUs under openSUSE 42.1 and the results were compared against an anchor of VTM 10.0 with high bit depth processing enabled. The tests were run using the test conditions defined in JVET-T0047 with extended precision disabled.

The source code modifications were compared to the proposal and found to match.

# Results

|  |  |  | AI-12 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-10.0 |  |  |
|  | Y | U | V | EncT | DecT |
| SVT | -1.06% | -1.07% | -1.00% | 91% | 101% |
| PQ444 | -0.10% | -0.08% | -0.08% | 100% | 101% |
| PQ422 | -0.11% | -0.07% | -0.08% | 100% | 101% |
| Overall | -0.42% | -0.41% | -0.39% | 97% | 100% |

|  |  |  | LDB-12 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-10.0 |  |  |
|  | Y | U | V | EncT | DecT |
| SVT | -0.36% | -0.39% | -0.34% | 100% | 100% |
| PQ444 | 0.00% | 0.02% | 0.02% | 99% | 100% |
| PQ422 | 0.01% | 0.04% | 0.02% | 99% | 100% |
| Overall | -0.12% | -0.11% | -0.10% | 99% | 100% |

|  |  |  | AI-16 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-10.0 |  |  |
|  | Y | U | V | EncT | DecT |
| SVT | -10.43% | -10.48% | -7.85% | 69% | 64% |
| Overall | -10.43% | -10.48% | -7.85% | 69% | 64% |

|  |  |  | LDB-16 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-10.0 |  |  |
|  | Y | U | V | EncT | DecT |
| SVT | -9.02% | -9.45% | -9.28% | 107% | 60% |
| Overall | -9.02% | -9.45% | -9.28% | 107% | 60% |

# Conclusions

The proposal and software changes in JVET-T0102 were found to be compatible.  No discrepancies have been found in the results for AI.