| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0107-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-T0093 (AHG12: On signalling for high-bit depth) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-50-5213-6334 chujoh.takeshi@sharp.co.jp |
| Source: | Sharp corporation |  |  |

_____________________________

- Abstract

In JVET-T0093, a new additional applicable condition of JCCR for high bit-depth coding is proposed. The cross-checker reports that the source code provided by KDDI who proposes JVET-T0093 has been carefully analyzed and has been found to be consistent with the description in JVET-T0093. The cross checker could reproduce the results reported in JVET-T0093. The difference from measured encoder and decoder run time ratios to those reported in JVET-T0093 has been acceptable.

Simulation Results

The test condition of anchor is based on JVET-T0047 [1] and the following parameters have been set.

InternalBitDepth : 12

LMCSEnable : 0

ExtendedPrecision : 0

TransformSkipLog2MaxSize : 2

ResidualRotation : 1

SingleSignificanceMapContext : 1

IntraReferenceSmoothing : 1

GolombRiceParameterAdaptation : 1

HighPrecisionPredictionWeighting : 1

For PQ444/422 sequences, yuv444.cfg and classH1.cfg are additionally included.

In the proposed method of JVET-T0093 [2], the following one parameter is added.

JointCbCrQpCheckEnabledFlag : 1

The summary of our cross-checking of JVET-T0093 is shown in Table 1. The results of Table 1 were consistent with that of JVET-T0093. The difference from measured encoder and decoder run time ratios to those reported in JVET-T0093 has been acceptable.

Table 1  Simulation results of AI condition where InternalBitDepth =12

|  | AI-12 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over reference |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| OldTownCross_1920x1080_50 | 0.00% | 0.00% | 0.00% | 79% | 100% |
| CrowdRun_1920x1080_50 | 0.00% | 0.00% | -0.01% | 80% | 100% |
| DucksTakeOff_1920x1080_50 | 0.00% | 0.00% | 0.00% | 80% | 101% |
| InToTree_1920x1080_50 | 0.00% | 0.00% | 0.00% | 80% | 101% |
| ParkJoy_1920x1080_50 | 0.00% | 0.00% | 0.00% | 78% | 100% |
| BalloonFestival_1920x1080p_24_12b_pq_709_ct2020_444 | 0.04% | 0.01% | -0.01% | 93% | 100% |
| FireEater2Clip4000r1_1920x1080p_25_12b_pq_709_ct2020_444 | 0.08% | -0.04% | 0.07% | 89% | 100% |
| Market3Clip4000r2_1920x1080p_50_12b_pq_709_ct2020_444 | 1.62% | 0.63% | 0.63% | 88% | 99% |
| EBU_04_Hurdles_1920x1080p_50_12b_pq_709_ct2020_444 | 0.05% | -0.07% | 0.06% | 86% | 100% |
| EBU_06_Starting_1920x1080p_50_12b_pq_709_ct2020_444 | 0.02% | -0.01% | 0.00% | 89% | 101% |
| BalloonFestival_1920x1080p_24_12b_pq_709_ct2020_422 | 0.04% | 0.01% | 0.00% | 94% | 101% |
| FireEater2Clip4000r1_1920x1080p_25_12b_pq_709_ct2020_422 | 0.04% | -0.01% | 0.03% | 92% | 100% |
| Market3Clip4000r2_1920x1080p_50_12b_pq_709_ct2020_422 | 1.09% | 0.40% | 0.41% | 92% | 101% |
| EBU_04_Hurdles_1920x1080p_50_12b_pq_709_ct2020_422 | 0.08% | -0.02% | 0.01% | 93% | 100% |
| EBU_06_Starting_1920x1080p_50_12b_pq_709_ct2020_422 | 0.03% | -0.01% | 0.03% | 93% | 100% |
| Overall | 0.20% | 0.06% | 0.08% | 87% | 100% |

Conclusion

In this contribution, the result of JVET-T0093 has been confirmed.

References

[1]	T. Ikai, A. Browne and X. Xiu, “AHG12: Preliminary common test conditions for high bitdepth video coding”, JVET-T0047, Oct. (2020)

[2]	Y. Kidani, K. Kawamura and K. Unno, “AHG12: On signalling for high-bit depth”, JVET-T0093, Oct. (2020)

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).