| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0111 |
| --- | --- |

| Title: | YCgCo-R: Observations and findings |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Derek Buitenhuis VideoLAN. Alexis Michael Tourapis Apple Inc. One Apple Park Way Cupertino, California, 95014 United States | Tel: Email: | derek@videolan.org atourapis@apple.com |
| Source: | VideoLAN, Apple Inc. |  |  |

_____________________________

# Abstract

This contribution discusses aspects of lossless coding for RGB and YCgCo image, and in particular still image, representations. It is identified that when using the currently specified YCgCo representation, as defined in ISO/IEC 23091-2 (CICP), in video/image specifications or encoder implementations that only support equal bit depth between the luma and chroma components, coding performance for lossless applications may be worse or even significantly worse than that of coding the same signal in an RGB representation. This is regardless of whether the coding specification can support a block level adaptive colour transform. An extension of the YCgCo representation intended for such applications is proposed in CICP, and in consequence to the AVC, HEVC, and VVC specifications among others, that alleviates this problem and can result in improved coding performance for most content.

# Introduction

Unlike previous video coding specifications, such as AVC and HEVC, which allowed for different bit depths between the luma and chroma components, the new VVC specification only allows representations that have the same bit depth. It was asserted during the development of the standard that this may impair the use of the YCgCo representation since the commonly accepted method of achieving a lossless transformation between the original RGB representation and YCgCo was with the extension of the chroma component by 1 bit compared to that of the luma component, and using a lifting scheme transformation method defined as follows:

Co = Round( R ) − Round( B ) + ( 1 << ( BitDepthC − 1 ) )

t = Round( B ) + ( ( Co − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )

Cg = Round( G ) − t + ( 1 << ( BitDepthC − 1 ) )

Y = t + ( ( Cg − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )

where R, G, and B are the red, green, and blue input colour component values, Co, Cg, and Y are the chroma orange-difference, chroma green-difference, and luma output colour component values, and BitDepthC corresponds to the output bit depth of the two chroma components. In this case, it is expected that the bit depth of the luma component would be equal to BitDepthC – 1.

Unfortunately, since VVC cannot support different luma and chroma bitdepths the actual transformation supported is as follows:

Y = Round( 0.5 * G + 0.25 * ( R + B ) )

Cg = Round( 0.5 * G − 0.25 * ( R + B ) ) + ( 1 << ( BitDepthC − 1 ) )

Co = Round( 0.5 * (R − B ) ) + ( 1 << ( BitDepthC − 1 ) )

In order for this transformation to be lossless BitDepthC, and essentially the bit depth of all colour components, needs to be higher of that of the original input R, G, and B signals. This, in effect, results in the luma component being MSB aligned in the resulting representation. Unfortunately, experiments have shown that this particular representation can be as efficient as expected in the context of such applications.

# Proposed improvement to the YCgCo-R representation

To avoid the above stated problem, we propose adding additional YCgCo based code points, i.e. new matrix coefficients entries, which would enforce an LSB alignment of the luma values in the luma component according to the YCgCo transformation process. In particular, we suggest introducing at least one more matrix coefficients code point in the CICP specification which would permit the same transformation as the current YCgCo matrix coefficients entry (value 8) specified for when BitDepthC != BitDepthY, i.e.:

Co = Round( R ) − Round( B ) + ( 1 << ( BitDepthC − 1 ) )

t = Round( B ) + ( ( Co − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )

Cg = Round( G ) − t + ( 1 << ( BitDepthC − 1 ) )

Y = t + ( ( Cg − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )

with however the following differences in the inverse process. In particular, in the first code point the inverse process would be recommended to be performed as follows:

maxValue = ( 1 << (BitDepthY – bitOffset) ) – 1

t = Y − ( ( Cg − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )

G = Clip3( 0, maxValue, t + ( Cg − ( 1 << ( BitDepthC − 1 ) ) ) )

B = Clip3( 0, maxValue, t − ( ( Co − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 ) )

R = Clip3(0, maxValue, B + ( Co − ( 1 << ( BitDepthC − 1 ) ) ) )

where the value of bitOffset is set equal to 2. The intent of this value is to allow for an "even" bit depth RGB representation, e.g. 8 bits, to be efficiently represented in a 10 bit input representation to the video encoder, or in general for applications where both the original RGB and intended input to the encoder representations are of odd or evn precision. For the second code point, the bitOffset is instead set to 1. This second code point is intended for applications where the RGB and input representations are of different bit depth parity, i.e. even vs odd representations. However, since odd bit-depth representations are relatively uncommon, this second code value may be omitted. Alternatively, it might be possible to reuse the "full range" flag indication to support both modes.

# Other observations

The VVC specification supports a block level adaptive residual colour transform that is based on the YCgCo transformation. So it was anticipated that the use of YCgCo for lossless coding would not result in any benefits compared to converting an entire RGB image to YCgCo and then compressing it using the new representations. However, and somewhat to our surprise, this was not the case. Careful investigation of the VTM reference software and the VVC specification identified the following:

- The adaptive colour transform is not supported on intra coded blocks when DualITree is used. This was by default enabled in the SW and in the current Common Test Conditions.

- Even when that mode is disabled the use of the adaptive colour transform on intra frames does not always guarantee a coding gain or that coding gain is small. It appears that either the implementation is inefficient or that since prediction is still taking place in the RGB domain and the transformation only applies to the residual, this transformation may be less efficient compared to doing so in the pixel domain. It should be highlighted that our experience has been different with the Screen Content Coding (SCC) extension of HEVC where the block level adaptive colour transform seemed to present some non negligible gains.

# Simulation results

We performed several encodings tests using the VTM and several publicly available image test sets. The performance of YCgCo using both the currently supported format and the proposed representation compared to RGB encoding is summarized in the four tables below.

|  | RGB |  |  | YCoCg-R |  |
| --- | --- | --- | --- | --- | --- |
|  | no DualItree + ACT 8 bit coded | DualItree 8-bit coded | no DualItree 8 bit coded | MSB aligned 10-bit coded | LSB aligned 10-bit coded |
| Mean | 100.00% | 100.98% | 101.52% | 123.27% | 97.20% |
| StdDev | 0.00% | 0.85% | 0.81% | 3.64% | 2.86% |
| Best | 100.00% | 99.63% | 100.04% | 111.51% | 87.58% |
| Worst | 100.00% | 103.69% | 104.18% | 131.11% | 103.19% |

Table 1: Lossless coding performance for subset1

|  | RGB |  |  | YCoCg-R |  |
| --- | --- | --- | --- | --- | --- |
|  | no DualItree + ACT 8 bit coded | DualItree 8-bit coded | no DualItree 8 bit coded | MSB aligned 10-bit coded | LSB aligned 10-bit coded |
| Mean | 100.00% | 100.94% | 101.49% | 123.26% | 97.19% |
| StdDev | 0.00% | 0.88% | 0.86% | 2.95% | 2.13% |
| Best | 100.00% | 99.08% | 99.93% | 117.01% | 91.07% |
| Worst | 100.00% | 103.17% | 103.70% | 131.53% | 101.11% |

Table 1: Lossless coding performance for subset2

|  | no DualItree + ACT 8 bit coded | DualItree 8-bit coded | no DualItree 8 bit coded | MSB aligned 10-bit coded | LSB aligned 10-bit coded |
| --- | --- | --- | --- | --- | --- |
| Mean | 100.00% | 101.12% | 101.40% | 119.75% | 95.69% |
| StdDev | 0.00% | 1.02% | 0.94% | 6.27% | 4.17% |
| Best | 100.00% | 95.18% | 96.22% | 90.85% | 74.00% |
| Worst | 100.00% | 108.66% | 105.58% | 153.45% | 115.25% |

Table 1: Lossless coding performance for subset3

|  | no DualItree + ACT 8 bit coded | DualItree 8-bit coded | no DualItree 8 bit coded | MSB aligned 10-bit coded | LSB aligned 10-bit coded |
| --- | --- | --- | --- | --- | --- |
| Mean | 100.00% | 101.23% | 101.53% | 119.82% | 95.67% |
| StdDev | 0.00% | 1.14% | 1.00% | 6.23% | 4.00% |
| Best | 100.00% | 92.66% | 91.67% | 84.26% | 67.66% |
| Worst | 100.00% | 111.75% | 105.72% | 146.40% | 114.84% |

Table 1: Lossless coding performance for subset4

It can be seen that, in general, coding gains for lossless applications of up to 25% can be achieved using this new, YCgCo-R based coding representation. The existing method is, unfortunately, quite inefficient, resulting in a loss of up to ~53% or on average ~20% increase in bitrate for all images that we have tested. Results per image as well as performance using the HEVC specification are also provided in the included Excel documents.

# Conclusion

It is proposed that one or more new matrix coefficients values are introduced in the CICP specification and consequently any video specification that utilize such code points.

# Patent rights declaration(s)

VideoLAN may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Apple Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).