| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0113 |
| --- | --- |

| Title: | Errata: 23091-4- Usage of video signal type code points |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET- Errata |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Name(s) Yasser Syed, Comcast Address line 2 Address line 3 | Tel: Email: | 303-246-8413 yasser_syed@comcast.com |
| Source: | Comcast |  |  |

_____________________________

# Abstract

The document lists the document errata or clarifications in the planned 3rd edition of ISO/IEC DTR 23091-4: 202x.

# ChromaTypeLocation Clarification

In table 5, the change is from “Interstitial” to “Vertically Interstitial” or else just put the value of ChromaLocType ( 3 or 11 occurences)

|  | System identifier | BT709_YCC | BT709_RGB | BT601_525 | BT601_625 |
| --- | --- | --- | --- | --- | --- |
| Colour properties | Colour primaries | BT.709 | BT.709 | BT.601 | BT.601 |
|  | Transfer characteristics | BT.709 | BT.709 | BT.709 | BT.709 |
|  | Colour representation | Y′CbCr | R′G′B′ | Y′CbCr | Y′CbCr |
| Other | Full/narrow range | Narrow | Narrow | Narrow | Narrow |
|  | 4:2:0 chroma sample location alignment | Vertically interstitial -or- 0 | N/A | Vertically interstitial -or- 0 | Vertically interstitial -or- 0 |
| CICP parameters Rec. ITU-T H.273 ¦ ISO/IEC 23091-2 | ColourPrimaries | 1 | 1 | 6 | 5 |
|  | TransferCharacteristics | 1 | 1 | 6 | 6 |
|  | MatrixCoefficients | 1 | 0 | 6 | 5 |
|  | VideoFullRangeFlag | 0 | 0 | 0 | 0 |
| SMPTE MXF parameters SMPTE ST 2067-21 | Colour primaries | urn:smpte:ul:06.0E.2B.34.04.01.01.06.04.01.01.01.03.03.00.00 |  | urn:smpte:ul:06.0E.2B.34.04.01.01.06.04.01.01.01.03.01.00.00 | urn:smpte:ul:06.0E.2B.34.04.01.01.06.04.01.01.01.03.02.00.00 |
|  | Transfer characteristic | urn:smpte:ul:06.0E.2B.34.04.01.01.01.04.01.01.01.01.02.00.00 |  |  |  |
|  | Coding equations | urn:smpte:ul:06.0E.2B.34.04.01.01.01.04.01.01.01.02.02.00.00 | N/R | urn:smpte:ul:06.0E.2B.34.04.01.01.01.04.01.01.01.02.01.00.00 |  |
|  | Full/narrow level range | Inferred (indicated in black reference level, white reference level, colour range) |  |  |  |
|  | 4:2:0 chroma sample location alignment | Inferred (ChromaLoc​Type = 0) | N/A | Inferred (ChromaLoc​Type = 0) | Inferred (ChromaLoc​Type = 0) |

ChromaLocType are listed as indicated in table 4

| ChromaLocType | 0 | Vertically interstitial, horizontally co-sited |
| --- | --- | --- |
|  | 1 | Vertically interstitial, horizontally interstitial |
|  | 2 | Vertically co-sited, horizontally co-sited |

References was the MXF values in the bottom row and Table 1

|  | Tag | Colour |  | Light |  | Container space properties |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Gamut | Primaries | Dynamic range | Transfer function | Colour represent ation | Integer code level scaling | 4:2:0 chroma sample location alignment (ChromaLocType) |
| HD or SD | BT601_525 | NCG | BT.601 | SDR | BT.709 | Y′CbCr | Narrow | Vertically interstitial (ChromaLocType = 0) |
|  | BT601_625 |  |  |  |  | Y′CbCr | Narrow | Vertically interstitial (ChromaLocType = 0) |
|  | BT709_YCC |  | BT.709 |  |  | Y′CbCr | Narrow | Vertically interstitial (ChromaLocType = 0) |
|  | BT709_RGB |  |  |  |  | R′G′B′ | Narrow | N/A |
|  | FR709_RGB |  |  |  |  | R′G′B′ | Full | N/A |
| UHD | BT2020_YCC_NCL | WCG | BT.2020 |  |  | Y′CbCr | Narrow | Co-sited (ChromaLocType = 2) |
|  | BT2020_RGB |  |  |  |  | R′G′B′ | Narrow | N/A |
|  | FR2020_RGB |  |  |  |  | R′G′B′ | Full | N/A |
|  | BT2100_PQ_YCC |  | BT.2100 | HDR | PQ | Y′CbCr | Narrow | Co-sited (ChromaLocType = 2) |
|  | BT2100_PQ_ICTCP |  |  |  |  | ICTCP | Narrow | Co-sited (ChromaLocType = 2) |
|  | BT2100_PQ_RGB |  |  |  |  | R′G′B′ | Narrow | N/A |
|  | BT2100_HLG_YCC |  |  |  | HLG | Y′CbCr | Narrow | Co-sited (ChromaLocType = 2) |
|  | BT2100_HLG_RGB |  |  |  |  | R′G′B′ | Narrow | N/A |

# Reference edit

Edit Error- Spaced out:

- SMPTE Registration Authority. https://registry.smpte-ra.org/view/published/elements_by_group_view.html

Should change to:

- SMPTE Registration Authority. https://registry.smpte-ra.org/view/published/elements_by_group_view.html

# Patent rights declaration(s)

Comcast Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).