| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0117 |
| --- | --- |

| Title: | AHG11: Methodology additional requirements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Franck Galpin Thierry Dumas Pavel Nikitin Fabrice Le Léannec Fabien Racapé Edouard François | Tel: Email: | Franck.Galpin@interdigital.com |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

Document JVET-T0041 describes the recommended JVET methodology and reporting template for neural network (NN) coding tool testing. This contribution suggests amendments to this document related to the reported complexity information template, in order to better evaluate the complexity of proposed NN-based video coding tools.

# Introduction

In [1], a methodology to evaluate neural network (NN) video coding tools in JVET is proposed. A template for reporting and analyzing the considered NN-based video coding tools is also proposed. Information in the template include network and complexity information for training and inference stages and the GPU runtime information (in the inference stage) are asked to be reported.

The limitations of the current template which are addressed in this contribution are the following.

- The number of CPU or GPU cores used is not detailed in the current reporting template.

- Depending on the network type (typically fully convolutional or not), the method for counting macs/pixel may differ.

- Details on architecture are sometimes missing and a huge gap in complexity estimation can be observed.

This contribution proposes to add some additional information to get a more accurate assessment of the complexity of the inference stage for the evaluated coding tools.

# Template changes

The proposed additions to the template from JVET-T0041, section 5, Table 2, are highlighted in the table below in yellow.

Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory |  |  |
|  | HW environment: | CPU: Intel Core i7-7820x CPU @ 3.60GHz x 16, 128GB GPU: GTX 1080ti x 4 x 12GB , 11 TFlops Peak |
|  | Framework: | TF v2.0, Pytorch v1.4, TensorRT, OpenVino, etc. |
|  | CPU/GPU | Eg: CPU |
|  | number of cores used for CPU | Eg: 1 (mono-thread process) |
|  | Total Number of parameters |  |
|  | Parameter Precision |  |
|  | Memory Parameter (MB) |  |
|  | Memory Temp (MB) |  |
|  | MAC (Giga) worst-case 1920x1080 image |  |
|  | Mac/pixel (worst case) |  |

In the above table, the following fields are refined as follows:

- HW: GPU: specify the peak TFlops when GPU is used for inference.

- CPU/GPU: which one is used

- Number of cores (for CPU mode):

  - CPU mode:

    - multithread disabled (1 core only is used)

    - multithread enabled: how many cores are used?

Motivations to specify this field are:

- Depending on the framework used and the way it is used, it might be difficult to enforce mono-core running.

- Tools are available to evaluate complexity at inference (for example https://github.com/Lyken17/pytorch-OpCounter for pytorch or native tf.profile for TF).

- MAC (Giga): Total number of multiply–accumulate operations at the inference stage, using a given (predefined) image size, typically 1920x1080 or 3840x2160 (to specify) in the worst-case.

- Mac/pixel: from the mac/frame, normalized results for all contributions.

# Network architecture

Number of layers is too restrictive to understand the network architecture. The network description is removed from the table. In order to better evaluate network complexity (for example number of stages, total complexity, data dependencies), an accurate description of the full network can be added in a separate file.

Specialized tools might be used to ease the visualization of network (e.g. tensorboard,  https://github.com/lutzroeder/netron ). Alternatively, a detailed description of the network with the parameters for each layer can be reported.

# References

[1] “Methodology and reporting template for neural network coding tool testing”, Shan Liu, Andrew Segall, Elena Alshina, Jill Boyce, Mathias Wien, Dan Grois. 20th Meeting: by teleconference, document JVET-T0041, 7 – 16 Oct. 2020.