| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0117-addon |
| --- | --- |

| Title: | Template sample |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Franck Galpin Thierry Dumas Pavel Nikitin Fabrice Le Léannec Fabien Racapé Jean Bégaint Edouard François | Tel: Email: | Franck.Galpin@interdigital.com |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

Template samples to report inference information and model architecture.

# Inference information

Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network information in inference stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment | CPU: E5-2697Av4 @ 2.6GHz |
|  | SW environment | CentOS7 |
|  | framework | Tensorflow 1.9.0 |
|  | CPU/GPU | CPU |
|  | number of cores used for CPU | 1 core, mono-thread |
|  | total parameter number (memory in bytes: x4) | 4×4: 1574430 4×8: 1632046 4×16: 1747278 4×32: 1977742 8×8: 1920078 8×16: 2150542 16×16: 1256282 32×32: 3904782 |
|  | parameter precision | 32 (F) |
|  | memory parameter (MB) | 61.7 MB in total |
|  | memory temp | 4×4.. 8×16: worst case 1200×4(F)=4.7kB 16×16: worst case (output layer 1): 20×4×32×4(F)=10kB 32×32: worst case (output layer 1): 40×8×32×4(F)=40kB |
|  | MAC per block size | 1.6 Mmac for 4×4 1.6 Mmac for 4×8 1.7 Mmac for 4×16 2.0 Mmac for 4×32 1.9 Mmac for 8×8 2.1 Mmac for 8×16 4.3 Mmac for 16×16 11.5 Mmac for 32×32 |
|  | MAC/pix (per block size) | 100k for 4×4 50k for 4×8 26k for 4×16 16k for 4×32 30k for 8×8 16k for 8×16 17k for 16×16 11k for 32×32 |

# Network structure description

## Text description example

As the model uses the Tensorflow framework, the summary of the model can be easily obtained (here for the 4×8 network):

Model: "intra_transfo_prediction" __________________________________________________________________________________________________ Layer (type) Output Shape Param # Connected to ================================================================================================== causal_border (InputLayer) [(None, 1, 112)] 0 __________________________________________________________________________________________________ dense_12 (Dense) (None, 1, 1200) 135600 causal_border[0][0] __________________________________________________________________________________________________ leaky_re_lu_6 (LeakyReLU) (None, 1, 1200) 0 dense_12[0][0] __________________________________________________________________________________________________ dense_13 (Dense) (None, 1, 1200) 1441200 leaky_re_lu_6[0][0] __________________________________________________________________________________________________ leaky_re_lu_7 (LeakyReLU) (None, 1, 1200) 0 dense_13[0][0] __________________________________________________________________________________________________ dense_14 (Dense) (None, 1, 32) 38432 leaky_re_lu_7[0][0] __________________________________________________________________________________________________ dense_15 (Dense) (None, 1, 14) 16814 leaky_re_lu_7[0][0] __________________________________________________________________________________________________ block_prediction (Reshape) (None, 4, 8) 0 dense_14[0][0] __________________________________________________________________________________________________ LFNST_group_idx (Reshape) (None, 1, 14) 0 dense_15[0][0] ================================================================================================== Total params: 1,632,046 Trainable params: 1,632,046 Non-trainable params: 0

The 16×16 block model has the following summary:

Model: "intra_transfo_prediction" __________________________________________________________________________________________________ Layer (type) Output Shape Param # Connected to ================================================================================================== top_border (InputLayer) [(None, 40, 8, 1)] 0 __________________________________________________________________________________________________ left_border (InputLayer) [(None, 8, 32, 1)] 0 __________________________________________________________________________________________________ conv2d_42 (Conv2D) (None, 20, 4, 32) 320 top_border[0][0] __________________________________________________________________________________________________ conv2d_46 (Conv2D) (None, 4, 16, 32) 320 left_border[0][0] __________________________________________________________________________________________________ leaky_re_lu_46 (LeakyReLU) (None, 20, 4, 32) 0 conv2d_42[0][0] __________________________________________________________________________________________________ leaky_re_lu_50 (LeakyReLU) (None, 4, 16, 32) 0 conv2d_46[0][0] __________________________________________________________________________________________________ conv2d_43 (Conv2D) (None, 10, 2, 64) 18496 leaky_re_lu_46[0][0] __________________________________________________________________________________________________ conv2d_47 (Conv2D) (None, 2, 8, 64) 18496 leaky_re_lu_50[0][0] __________________________________________________________________________________________________ leaky_re_lu_47 (LeakyReLU) (None, 10, 2, 64) 0 conv2d_43[0][0] __________________________________________________________________________________________________ leaky_re_lu_51 (LeakyReLU) (None, 2, 8, 64) 0 conv2d_47[0][0] __________________________________________________________________________________________________ conv2d_44 (Conv2D) (None, 5, 2, 128) 73856 leaky_re_lu_47[0][0] __________________________________________________________________________________________________ conv2d_48 (Conv2D) (None, 2, 4, 128) 73856 leaky_re_lu_51[0][0] __________________________________________________________________________________________________ leaky_re_lu_48 (LeakyReLU) (None, 5, 2, 128) 0 conv2d_44[0][0] __________________________________________________________________________________________________ leaky_re_lu_52 (LeakyReLU) (None, 2, 4, 128) 0 conv2d_48[0][0] __________________________________________________________________________________________________ conv2d_45 (Conv2D) (None, 3, 2, 128) 147584 leaky_re_lu_48[0][0] __________________________________________________________________________________________________ conv2d_49 (Conv2D) (None, 2, 2, 128) 147584 leaky_re_lu_52[0][0] __________________________________________________________________________________________________ leaky_re_lu_49 (LeakyReLU) (None, 3, 2, 128) 0 conv2d_45[0][0] __________________________________________________________________________________________________ leaky_re_lu_53 (LeakyReLU) (None, 2, 2, 128) 0 conv2d_49[0][0] __________________________________________________________________________________________________ reshape_5 (Reshape) (None, 1, 768) 0 leaky_re_lu_49[0][0] __________________________________________________________________________________________________ reshape_6 (Reshape) (None, 1, 512) 0 leaky_re_lu_53[0][0] __________________________________________________________________________________________________ concatenate_3 (Concatenate) (None, 1, 1280) 0 reshape_5[0][0] reshape_6[0][0] __________________________________________________________________________________________________ dense_6 (Dense) (None, 1, 500) 640500 concatenate_3[0][0] __________________________________________________________________________________________________ leaky_re_lu_54 (LeakyReLU) (None, 1, 500) 0 dense_6[0][0] __________________________________________________________________________________________________ dense_7 (Dense) (None, 1, 256) 128256 leaky_re_lu_54[0][0] __________________________________________________________________________________________________ dense_8 (Dense) (None, 1, 14) 7014 leaky_re_lu_54[0][0] __________________________________________________________________________________________________ block_prediction (Reshape) (None, 16, 16) 0 dense_7[0][0] __________________________________________________________________________________________________ LFNST_group_idx (Reshape) (None, 1, 14) 0 dense_8[0][0] ================================================================================================== Total params: 1,256,282 Trainable params: 1,256,282 Non-trainable params: 0

For PyTorch models, the pytorch-summary [3] package can be used to generate a TensorFlow/Keras-like text summary.

## Graphical description examples

An alternative visualization, using [2], provides a graphical view of the same network, e.g. for 4×8 blocks:

This graphical view can become difficult to analyze for very deep networks. An external file in SVG format can be used instead.

For 16×16 blocks, we obtain the following graph:

## Model file

Alternatively, standard file format (e.g. ONNX) can be used to describe the model (weights can be removed or zeroed out for compression purpose).

An example file for the 16×16 block model is provided for reference.

# References

[1] “Methodology and reporting template for neural network coding tool testing”, Shan Liu, Andrew Segall, Elena Alshina, Jill Boyce, Mathias Wien, Dan Grois. 20th Meeting: by teleconference, document JVET-T0041, 7 – 16 Oct. 2020.

[2] https://github.com/lutzroeder/netron

[3] https://github.com/sksq96/pytorch-summary