| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0120 |
| --- | --- |

| Title: | AHG4: Video conference sequence for HD verification testing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenneth Andersson Mats Folkesson | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The current plan for low-delay HD verification testing in JVET-S2009 [1] do not include any video conference sequences. This contribution proposes one video conference sequence for HD verification testing. PSNR and bitrates for low-delay B encodings with HM-16.22 and VTM-10.0 are provided in the contribution.

# Sequence

The sequence Beatriz_1920x1080_50fps_8bit_420 has 500 frames and was captured in 1080p 4:2:0 at 50 frames per second with a bit-depth of 8. The capture by the camera was originally compressed with H.264 high profile with an average bitrate of 25 Mbps. The camera brand is unknown.

The MD5 is fe74cd5046fa033b4f743f42b29e69cd.

Figure 1: Beatriz_1920x1080_50fps_8bit_420 (talking in shopping mall with some movements in background)

# RD performance for low-delay B configuration

The sequence has been encoded with low-delay B configuration with both VTM-10.0 [2] and HM-16.22 [3]. Bitrate and PSNR results are shown in Table 1.

Table 1: Bitrate and PSNR results

| QP | HM-16.22 |  |  |  | VTM-10.0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Rate [kbps] | PSNR Y [dB] | PSNR U [dB] | PSNR V [dB] | Rate [kbps] | PSNR Y [dB] | PSNR U [dB] | PSNR V [dB] |
| 48 | 70,1104 | 31,3872 | 39,0492 | 38,205 |  |  |  |  |
| 47 | 75,1576 | 31,8735 | 39,6367 | 38,8934 |  |  |  |  |
| 46 | 80,8408 | 32,3531 | 40,133 | 39,3902 | 59,7168 | 32,874 | 39,4087 | 38,5292 |
| 45 | 87,1648 | 32,8346 | 40,6636 | 40,0058 |  |  |  |  |
| 44 | 95,1736 | 33,2831 | 41,2034 | 40,5739 |  |  |  |  |
| 43 | 105,3064 | 33,7544 | 41,5694 | 41,0112 |  |  |  |  |
| 42 | 119,9168 | 34,2923 | 42,109 | 41,5704 | 97,4464 | 34,9246 | 41,218 | 40,5742 |
| 41 | 134,7848 | 34,7905 | 42,1193 | 41,625 |  |  |  |  |
| 40 | 154,5944 | 35,29 | 42,5516 | 42,1599 |  |  |  |  |
| 39 | 175,5016 | 35,7928 | 42,6105 | 42,2029 |  |  |  |  |
| 38 | 202,4512 | 36,2971 | 43,068 | 42,7262 | 164,3664 | 36,8352 | 42,3053 | 41,7972 |
| 37 | 230,264 | 36,774 | 43,0946 | 42,7797 |  |  |  |  |
| 36 | 267,3048 | 37,2379 | 43,5834 | 43,2961 |  |  |  |  |
| 35 | 306,8176 | 37,6996 | 43,6096 | 43,3522 |  |  |  |  |
| 34 | 356,556 | 38,1469 | 44,0322 | 43,8403 | 292,22 | 38,5992 | 43,5713 | 43,1485 |
| 33 | 412,4168 | 38,5957 | 44,0916 | 44,005 |  |  |  |  |
| 32 | 483,652 | 39,0264 | 44,5335 | 44,4329 |  |  |  |  |
| 31 | 569,9528 | 39,4474 | 44,8386 | 44,8412 |  |  |  |  |
| 30 | 705,5232 | 39,9427 | 45,1764 | 45,246 | 587,2744 | 40,2837 | 45,2218 | 45,0304 |
| 29 | 873,5064 | 40,3903 | 45,5503 | 45,7049 |  |  |  |  |
| 28 | 1039,331 | 40,7858 | 45,6012 | 45,7857 |  |  |  |  |
| 27 | 1261,977 | 41,1731 | 45,9312 | 46,2071 |  |  |  |  |
| 26 | 1640,531 | 41,6474 | 46,2849 | 46,6534 | 1400,65 | 41,9036 | 46,6319 | 46,7637 |
| 25 | 2110,079 | 42,0912 | 46,72 | 47,1486 |  |  |  |  |
| 24 | 2596,283 | 42,4793 | 46,9698 | 47,4498 |  |  |  |  |
| 23 | 3464,52 | 43,0035 | 47,4312 | 47,9278 |  |  |  |  |
| 22 | 4436,277 | 43,4531 | 47,8543 | 48,36 |  |  |  |  |

# Copyright

This test sequence and all intellectual property rights therein remain the property of Ericsson.

The following use is allowed for the test sequence:

Can be used by JVET for VVC verification testing.

# References

[1] M. Wien, V. Baroncini, A. Segall, Y. Ye, “VVC verification test plan (Draft 3)”, JVET-S2009, Teleconference, June 2020.

[2] VTM-10.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-10.0

[3] HM-16.22, available at https://vcgit.hhi.fraunhofer.de/jct-vc/HM/-/releases/HM-16.22

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).