| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0123 |
| --- | --- |

| Title: | DNNVC: A study of handling YUV420 input format for DNN-based video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ankitesh K. Singh Hilmi E. Egilmez Muhammed Coban Marta Karczewicz | Tel: Email: | ankites@qti.qualcomm.com hegilmez@qti.qualcomm.com mcoban@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

Most existing deep neural network (DNN) based video coding architectures are designed to operate in non-subsampled input formats such as RGB or YUV 4:4:4. However, video coding standards such as HEVC and VVC are designed to support  the YUV 4:2:0 colour format in their respective main profiles. This contribution document presents a study of three distinct DNN-based video coding architectures for handling YUV 4:2:0 colour format by comparing their performance against HEVC and VVC.

# Introduction

Recent studies in DNN based compression have shown that competitive coding gains can be achieved as compared to existing standards in RGB color format, where DNNs are trained and evaluated on RGB input frames. However, most practical codecs work with the subsampled YUV 4:2:0 input color format and there is very little or no work on DNN designs for YUV 4:2:0 in the literature. Although the same DNN architecture with different set of weights can be used to support non-subsampled colour formats such as RGB and YUV 4:4:4,  handling of YUV 4:2:0 in DNNs is non-trivial, and it is needed for a proper comparison against existing standards (e.g., HEVC [1] and VVC [2]) that are primarily focusing on the YUV 4:2:0 input.

This input document investigates three different front-end architectures to accommodate YUV 4:2:0 input format in DNNs designed for RGB input format. These front-end architectures are applicable to many DNN-based compression architectures including the ones in [3][4]. This document particularly integrates the front-end architectures to the mean-scale hyperprior based DNN scheme in [4] where both training and testing are performed on video datasets in YUV 4:2:0 format.

# Front-end architectures for YUV420 input

Three different methods for handling YUV 4:2:0 input  are integrated to the mean-scale hyperprior based DNN scheme (mbt2018) in [4].  In the first approach, luma and chroma channels go through separate convolution and GDN layers and combined before the second convolution layer as illustrated in Figure 1. This method introduces an additional GDN module compared to the original mbt2018 scheme. In the second method, luma and chroma channels are coded separately using separate NN codec based on mbt2018. This method doubles the number of parameters of the original network and disconnects luma and chroma channels as shown in Figure 2. In the third method, luma channel is subsampled by 2 in each dimension, resulting in 4 channels. The luma channels are stacked with 2 chroma channels (6-channel input) and processed by mbt2018 codec as shown in Figure 3. This method has additional parameters in the first convolutional layer due to increase input channel size.

There could be other viable ways of handling YUV 4:2:0 input such as converting to RGB before coding, or combining the luma and chroma channels before the GDN layers, etc. with different trade-offs, require further investigation.

Figure 1: Illustration of DNN in [4] with the joint front-end architecture that directly works with the 4:2:0 input (Y, U and V jointly).

Figure 2: Illustration of DNN in [4] that are trained and tested separately for luma (Y) and chroma channels (U and V) where K=5 for luma and K=3 for chroma channels.

Figure 3: Illustration of DNN in [4] with 6 input channels obtained by subsampling Y component into 4 partitions matching dimensions of U and Y.

# Training and Testing

The front-end architectures are integrated to [4] as discussed in Section 2, and the training and testing is performed on the CompressAI framework [5]. For training mean-squared error (MSE) is used as the distortion metric, and the trained network is tested and evaluated using PSNR. For testing the bitstream generation is done by using the ANS entropy coding (implemented in compressAI) on latent (hyper) variable coefficients, and the bitrates are obtained from the compressed bitstream files.

Throughout the experiments separate training and datasets are used. Specifically, training is performed on the Vimeo 90K triplet data set [8] where the RGB images are converted to YUV 4:2:0 using the method specified in JVET-T0041[10].

The model was trained for four rate points for each architecture. The training details are summarized in Table 1.

Table 1: Neural network training parameters.

| Dataset | Vimeo90K (converted to YUV420p) |
| --- | --- |
| Input Size | 256x256(Y),128x128(UV) |
| Iterations | 4M |
| Batch Size | 8 |
| Optimizer | Adam |
| Learning rate | 10-4 (main), 10-5(auxiliary) |
| Learning Rate decay schedule | 0.1x initial learning rate after 2M iteration (10-6) |

The entropy bottleneck channels are set to [192, 192, 320, 320] corresponding to lambda values of {0.005, 0.01, 0.025, 0.1} in compressAI framework. The associated rate points for these lambda parameters roughly correspond to bitrates for QP factors {42, 37, 32, 27} of HEVC. The reported PSNRs (integer) are according to the video’s input bit depth. Luma and chroma samples are treated with equal weights for distortion calculation.

The CTC sequences [7] and Kodak test set [6], converted to YUV 4:2:0, are used as the test set. Replicate padding of 64 and 32 is used for non multiple of 64 dimension sequences for luma and chroma channels, respectively during testing.

All-intra coding in CTC for VTM-10.0 and HM-16.20 are used as reference for evaluation of mbt2018 with front-end YUV420 architectures.

# Experimental Results

## RD-curves

The following figures compare the average coding performance of introduced architectures on mbt2018 [4] (i.e., joint, separate and six-channel front-end designs) by comparing against HM-16.20 and VTM-10.0. Specifically, Figure 1 shows the averate performance on CTC sequences [7] and Figure 2 shows on Kodak image dataset [6].

Figure 1: Rate-distortion performance of introduced architectures compared against VVC and HEVC on CTC in terms of PSNR and bits-per-pixel (BPP). PSNR values are shown for Y, U and V components separately at the top-left, top-right and bottom-left figures, respectively. The bottom-right figure shows the combined wPSNR values [11].

Figure 2: Rate-distortion performance of introduced architectures compared against VVC and HEVC on Kodak dataset in terms of PSNR and bits-per-pixel (BPP). PSNR values are shown for Y, U and V components separately at the top-left, top-right and bottom-left figures, respectively. The bottom-right figure shows the combined wPSNR values [11].

## BD-rates

Table 2: Performance of joint front-end architecture.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Over HM-16.20 |  |  |
|  | Y | U | V |
| Class A1 | -24.69% | 185.42% | -9.09% |
| Class A2 | -25.76% | 99.31% | 99.23% |
| Class B | -8.26% | 18.27% | 3.58% |
| Class C | -2.89% | 37.18% | 29.37% |
| Class E | -9.81% | 24.75% | -11.24% |
| Overall | -12.98% | 64.92% | 20.67% |
| Class D | 0.78% | 24.03% | 31.60% |
| Class F | 59.75% | 167.23% | 145.78% |

Table 3: Performance of separate front-end architecture.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Over HM-16.20 |  |  |
|  | Y | U | V |
| Class A1 | -22.20% | 249.54% | 21.41% |
| Class A2 | -24.31% | 108.14% | 110.13% |
| Class B | -9.05% | 16.20% | 6.66% |
| Class C | -3.38% | 39.97% | 34.78% |
| Class E | -10.81% | 8.09% | -15.91% |
| Overall | -12.82% | 74.34% | 28.85% |
| Class D | 0.85% | 39.57% | 45.37% |
| Class F | 79.50% | 130.71% | 129.49% |

Table 4: Performance front-end with 6-channel input.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Over HM-16.20 |  |  |
|  | Y | U | V |
| Class A1 | -22.07% | 194.39% | -7.90% |
| Class A2 | -24.82% | 95.22% | 86.54% |
| Class B | -7.64% | 19.24% | -0.76% |
| Class C | -1.73% | 34.03% | 22.07% |
| Class E | -9.29% | 28.11% | -13.04% |
| Overall | -11.87% | 65.86% | 15.63% |
| Class D | 1.58% | 19.41% | 24.82% |
| Class F | 60.23% | 67.69% | 66.42% |

As CTC [7] consist of classes of video sequences with diverse characteristics, training neural networks only on Vimeo90K dataset [8] may not be sufficient to generalize models well on CTC. This is also demonstrated in Tables 2, 3 and 4, where BD-rate values can be quite different for each class. For a better coding performance in general, larger training datasets capturing more diverse video characteristics might be required. The future work will include training the architectures in this document on larger datasets in order to generalize the average coding performance better across different classes of video sequences (including the ones in CTC).

# Conclusion

Three different front-end architectures for handling YUV420 input for codecs designed for processing of non-subsampled chroma formats, specifically RGB, have been presented. Performance evaluations of the  mbt2018  [4] codec with different YUV420 processing front-ends have been presented under draft DNNVC  test conditions. Further study of handling input YUV chroma formats in DNN-based video codecs is encouraged.

# References

- HEVC Test Model (HM) 16.20, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/ HM-16.20/.

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 10),” JVET-S2001, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 19th Meeting, 22 June–1 July 2020.

- J. Ballé, D. Minnen, S. Singh, S. J. Hwang, and N. Johnston, ‘Variational image compression with a scale hyperprior’, arXiv:1802.01436.

- D. Minnen, J. Ballé, and G. Toderici, ‘Joint Autoregressive and Hierarchical Priors for Learned Image Compression’, arXiv:1809.02736.

- J. Bégaint, F. Racapé, S. Feltman, A. Pushparaja, “CompressAI: A PyTorch library and evaluation platform for end-to-end compression research,” m54467, July 2020.

- Eastman Kodak, ‘Kodak Lossless True Color Image Suite (PhotoCD PCD0992)’. [Online]. Available: http://r0k.us/graphics/kodak.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting, Geneva, CH, 19–27 Mar. 2019.

- Vimeo 90K Triplet dataset, http://toflow.csail.mit.edu/index.html#triplet.

- Draft test conditions for DNNVC, w19583, ISO/IEC JTC 1/SC 29/WG 11,  July 2020.

- S. Liu, A. Segall, E. Alshina, J. Boyce, M. Wien, D. Grois, “Methodology and reporting template for neural network coding tool testing”, JVET-T0041, October 2020.

- A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video,” JVET-N1011, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting, Geneva, CH, 19–27 Mar. 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).