| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0124_v3 |
| --- | --- |

| Title: | BoG on common condition for high bit-rate, high bit-depth and high frame-rate coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai | Tel: Email: | ikai.tomohiro@sharp.co.jp |
| Source: | BoG |  |  |

_____________________________

- Abstract

This is about BoG on common condition for high bit-rate, high bit-depth and high frame-rate coding

Frist session took place from13 Oct, 7:20 UTC to 13 Oct, 9:30 UTC with around 30 participants.

sequences, bitdepth, colour format/sampling, QPs, tools and encoder cfgs has been recommended while evaluation criteria, spread sheet, high framerate was untouched.

Second session took place from14 Oct, 21:40 UTC to 14 Oct, 23:20 UTC with around 30 participants. evaluation criteria, spread sheet, high framerate has been confirmed and initial CE draft was quickly reviewed.

BoG Recommendation for high bit depth and high bit rate common test condition (Relative to JVET-T0047, Preliminary common test conditions for high bitdepth video coding test condition):

- Keep five SVT sequences, remove FireEater2Clip from PQ, adopt four new HLG sequences, Daystreet, PeopleInShoppingCenter, NightStreet, and StainedGlass

- Approve RGB444, YUV444 and YUV222 testing

- Approve 12bit and 16bit testing

- Approve QP ranges (QP=-13, -8, -3, 2, 7, 12) for 12bit and QP ranges (-33, -28, -23, -18, -13, -8) for 16 bit

- Approve lossless test as optional

- Enable LMCS but disable LFNST

- Restrict max MTT size (a.k.a maxmtt16) as proposed in JVET-T0087, for high bit depth low QP encoder configuration

- Set Extended precision flag equal to 0 for 12 bit testing and 1 for 16 bit testing

- Mandatory 12 bit RA with 97 frames. Use 50 frames for HD and 25 frames for UHD in LDB

- Choose PSNR for SDR and HLG content and both PSNR / wPSNR for PQ content as quality measure

- Test JVET-T0072, JVET-T0085, JVET-T0089 and JVET-T0105 in a CE and review CE description for rice parameter derivation

Recommendation for high frame rate common test condition:

- Encourage to solicit high frame rate contents (not current 100/120fps but several hundred fps)

Test condition

Sequences => Agreed

- SVT

  - OldTownCross_1920x1080_50_16bit_444.rgb

  - CrowdRun_1920x1080_50_16bit_444.rgb

  - ParkJoy_1920x1080_50_16bit_444.rgb

  - DucksTakeOff_1920x1080_50_16bit_444.rgb

  - InToTree_1920x1080_50_16bit_444.rgb

All above SVT sequences are to be used. Agreed

It was questioned whether 16bit for 12bit test is good thing?

The response by AHG chairs are

- We want to use the same contents for 12bit and 16bit

- Remove LSB should be ok

One participant prefers 12bit RGB content rather than reduced bit depth content. However as mentioned above it is agreed to use SVT 16 bit for testing.

- PQ

  - BalloonFestival_1920x1080p_24_12b_pq_709_ct2020_422/444

  - Market3Clip4000r2_1920x1080p_50_12b_pq_709_ct2020_422/444

  - EBU_04_Hurdles_1920x1080p_50_12b_pq_709_ct2020_422/444

  - EBU_06_Starting_1920x1080p_50_12b_pq_709_ct2020_422/444

On FireEater2Clip. it is mentioned that FireEater2Clip is a particular sequence. e,g, very noisy, more than 100% gain in chroma by CCLM. Another expert mentioned it is not familiar content for experts. Yet Another participant mentioned it is noisy but it generated a lot of small coefficients compared to other contents, which could be interesting.

Remove FireEater2Clip.   Agreed

At the first review, replacement of FireEater2Clip was also suggested as well as removal. In a later revisit, it was agreed to remove it and keep other four contents since it can be good thing to use the same number for contents in HLG (which becomes four) since equal number of sequences brings equal weight for PQ and HLG.

- HLG

  - DayStreet_3840x2160_60p_12bit_444/422_hlg

  - PeopleInShoppingCenter_3840x2160_60p_12bit_444/422_hlg

  - NightStreet_3840x2160_60p_12bit_444/422_hlg

  - StainedGlass_3840x2160_60p_12bit_444/422_hlg

As for HLG, it is mentioned encoding time increase should be expected. There’s some concerns.

Several experts express support for HLG inclusion. Agreed

Which HLG contents should be included was investigated using JVET-T0082 slide and data. Proponent mentioned original capture is 16bitRAW and converted to 12bit YUV444. 12 bit YUV422 contents are converted from 12 bit YUV444. It was mentioned that 444 to 422 process could be improved by luma adjustment.

Is mentioned that SeaSurface may not be typical sequence. If remove SeaSurface, EncT should  be reduced. It is also mentioned Nightstreet is noisy and show high bitrate but keep it original noisy should be important and one noisy sequences should be ok

DayStreet, PeopleInShoppingCenter, NightStreet and StainedGlass should be included in CTC. Agreed

- Other sequences

Synthesis contents are also suggested, which include full range. It could be very helpful whether VTM has some potential bug.

| Synthesised RGB HDR | EBURainFruits_1920x1080_50_10bit_444.rgb | 50 | 50 | 10 | 12,14,16 |
| --- | --- | --- | --- | --- | --- |
|  | Kimono1_1920x1080_24_10bit_444.rgb | 60 | - | 10 | 12,14,16 |

It was mentioned by other expert that it sounds important but it also seems unclear whether this test provides meaning result. More evidence or some history are suggested. Further study

Bitdepth

- Bitdepth Agreed

  - 12bit => Agreed

  - 16bit => Agreed

Is there any 14 bit sequences? Not at this moment. 12bit IBDI or 16bit ReducedBitdepth would be used for testing 14bit . IBDI shift might not be good conversion. It is mentioned that if better conversion process exists, it should be used. It is mentioned 14 bit YUV would be better than 14 bit RGB if source is 16 bit RGB and conversion process is done.

It is also mentioned only 12 bit and 16 bit testing creates some vacancy. 14 bit can provide data in between.

In HEVC, there’s no 14bit profile. The reason seems not known.

In the second meeting, no 14 bit in CTC was confirmed.

Note:Of course interesting party can study 14bit.

- ColourSpace

  - RGB

    - SVT

  - YUV

    - PQ

    - HLG

Agreed

- ColourSampling

  - RGB

    - 444

  - YUV

    - 444

    - 422

422 testing was discussed. Several express support for 422. Agreed.

It was mentioned about 422 burden, one possible way to reduce burden, 422 can be optional for some configuration. It was later agreed in RA and LDB.

QPs

| QPs | 12 bit | 14 bit (not used at this moment) | 16 bit |
| --- | --- | --- | --- |
| -33 |  |  | -33 |
| -28 |  |  | -28 |
| -23 |  | -23 | -23 |
| -18 |  | -18 | -18 |
| -13 | -13 | -13 | -13 |
| -8 | -8 | -8 | -8 |
| -3 | -3 | -3 |  |
| 2 | 2 | 2 |  |
| 7 | 7 |  |  |
| 12 | 12 |  |  |

An AHG chair reported that these QP has been selected based on the results where PSNR become around 60dB for 12bit, around 70dB for 14bit around 80dB for 16bit. These SN range should be considered useful in these bit depth. It is also reported that the above range is similar to used ranges in HEVC high-bit depth coding development as in JVET-P1006.

No opposition was mentioned.

Use the above QP ranges.  Agreed

Coding configuration

| Test name | Class | Internal bit-depth | QP range | AI | RA | LDB |
| --- | --- | --- | --- | --- | --- | --- |
| 12bit | SVT, PQ422, PQ444 HLG422 (new) HLG444 (new) | 12 | 12, 7, 2, -3, -8, -13 | M | M 422 becomes optional | M 422 becomes optional |
| 16bit | SVT | 16 | -8, -13, -18, -23, -28, -33 | M | O | M |
| Lossless 12bit | PQ422, PQ444 HLG422 HLG444 | 12 |  | O | O | O |
| Lossless 16bit | SVT | 16 |  | O |  | O |

In preliminary common test condition suggested in JVET-T0047, RA is optional since it showed very large encoding time.

There was several opinion on that.

100frm is not good number for RA. 3or 4 chunks or something can be suggested.

Difference number for difference sequence is suggested.

Check configuration for speedup. Closed GOP is also suggested (GOP boundary care is not needed then)

Shorter GOP can be also used. GOP 8 is also suggested.

It is pointed that in VTM, lossless might have bug in software. If there are bugs, decision is difficult or not reliable. Issues of some tools can be found by testing these sequences.

It was mentioned that it seems 16 bit testing can be not sufficient compared to 12bit testing.

RA mandatory is also suggested if some way to reduce encoding time is found.

It is suggested 422 optional in RA, LDB but mandatory in AI. Agreed

Encoded frame count (length)

| Class | Test sequence filename | Encoded frame count |  |  | Input bit-depth | Internal bit-depth |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | AI | RA | LDB |  |  |
| SVT 16 bit from 10 bit source (intermediate processing) | OldTownCross_1920x1080_50_16bit_444.rgb | 100 | 97 | 50 | 16 | 12 and 16 |
|  | CrowdRun_1920x1080_50_16bit_444.rgb | 100 | 97 | 50 | 16 | 12 and 16 |
|  | ParkJoy_1920x1080_50_16bit_444.rgb | 100 |  |  | 16 | 12 and 16 |
|  | DucksTakeOff_1920x1080_50_16bit_444.rgb | 100 |  |  | 16 | 12 and 16 |
|  | InToTree_1920x1080_50_16bit_444.rgb | 100 |  |  | 16 | 12 and 16 |
| PQ444 12 bit from half float exr | BalloonFestival_1920x1080p_24_12b_pq_709_ct2020_422 | 100 | 97 | 50 | 12 | 12 |
|  | Market3Clip4000r2_1920x1080p_50_12b_pq_709_ct2020_422 | 100 | 97 |  | 12 | 12 |
|  | EBU_04_Hurdles_1920x1080p_50_12b_pq_709_ct2020_422 | 100 | 97 | 50 | 12 | 12 |
|  | EBU_06_Starting_1920x1080p_50_12b_pq_709_ct2020_422 | 100 |  |  | 12 | 12 |
| PQ422 12 bit from half float exr | BalloonFestival_1920x1080p_24_12b_pq_709_ct2020_444 | 100 | 97 | 50 | 12 | 12 |
|  | Market3Clip4000r2_1920x1080p_50_12b_pq_709_ct2020_444 | 100 | 197 |  | 12 | 12 |
|  | EBU_04_Hurdles_1920x1080p_50_12b_pq_709_ct2020_444 | 100 |  | 50 | 12 | 12 |
|  | EBU_06_Starting_1920x1080p_50_12b_pq_709_ct2020_444 | 100 |  |  | 12 | 12 |
| HLG444 | DayStreet_3840x2160_60p_12bit_444_hlg | 100 | 97 | 25 | 12 | 12 |
|  | PeopleInShoppingCenter_3840x2160_60p_12bit_444_hlg | 100 | 97 | 25 | 12 | 12 |
|  | NightStreet_3840x2160_60p_12bit_444_hlg | 100 |  |  | 12 | 12 |
|  | StainedGlass_3840x2160_60p_12bit_444_hlg | 100 |  |  | 12 | 12 |
| HLG422 | DayStreet_3840x2160_60p_12bit_444_hlg | 100 | 97 | 25 | 12 | 12 |
|  | PeopleInShoppingCenter_3840x2160_60p_12bit_444_hlg | 100 | 97 | 25 | 12 | 12 |
|  | NightStreet_3840x2160_60p_12bit_444_hlg | 100 |  |  | 12 | 12 |
|  | StainedGlass_3840x2160_60p_12bit_444_hlg | 100 |  |  | 12 | 12 |

In RA and LDB, two sequences are used for testing. Which sequences should be used was discussed.

For SVT. OldTownCross and Croundrun was suggested to be used as used in JCTVC-P1006

For PQ. BalloonFestival and Hurdles was suggested to be used as these used different frame rate and cameras.

For HLG. DayStreet and PeopleInShoppingCenter should be used as these has been used in HEVC version1 development.

The above selection.  Agreed.

It was mentioned that the preliminary setting could be ok in terms of level of length but new sequences and total encoding time with new sequences should be considered.

It was questioned how many hours HLG LDB encoding takes for 50 frames?  It tooks 141 hours.

(it considered that 7days for 50frames). Proponent suggested that keep 50 frames for all cases

Another expert suggested 3.5 days could be ok by using 25 frames and 25 frames are suggested for collaboration work.

New HLG sequences. 25 frames for LDB. Agreed

Several experts suggested reduced intra period can be used for speed up RA. Further Study

A participant suggested that reduce intra period to 8 for all sequences to get more efficiency.

Another expert expressed that considering intra frame bits vs inter frame bits ratio, 8 can effectively increase intra weight, that can be not appropriate. But the participant said that 16 would be ok.

Other expert suggested that substracting intra frame result can be possible in reporting stage while another expert said it would complicate testing procedure and that is already complicated.

It was conformed that intra period should be configured using frame rate (as normal CTC do).

A participant suggested 65 frames for RA and GOP size can be equal to 32 for all class. On the other hand, it was commented if longest time is unchanged, RA encoding time can be accepted.

Several participant support 97 frames for RA. Agreed

It was reported by an expert that with fast configuration (maxmtt16), the estimated longest running time in LDB would be around 40 hours in UHD HLG 444. In case of RA, 65 frames could be longer.

Encoder configuration

Some encoder configurations of JVET-T0047 was discussed.

- RGB

  - formatRGB.cfg

ColorTransform : 1

Log2MaxTbSize : 5

DualITree: 0

LMCSEnable : 0

- YUV444,YUV420

  - yuv444.cfg

BDPCM: 1

It was questioned why BDPCM is on for camera captured content.

It was also mentioned that no sufficient information  on BDOCM and it is necessary to consider future contents.So there’s no reason for disabling BDPCM.

Keep BDPCM for YUV content: Agreed

A participant expressed that LMCS should be on and HDR Metris should be used for PQ and HLG contents in low bitrate. Another expert expressed there’s no perfect metrics. e.g WPSNR or something has issues.

It is agreed LMCS should be on in PQ and HLG but not in RGB as SDR CTC.

Keep LMCS for YUV content.  Agreed (not RGB)

- LFNST  0.47% loss Enc 10% reduced in AI

It is suggested to disable LFNST. Agreed

- Maxmtt16 in JVET-T0087

  - Loss: 0.05%, 0.08%, 0.19% and 0.08% in AI-12, AI-16, LDB-12, LDB-16

  - EncT: 75%, 61%, 61% and 64% in AI-12, AI-16, LDB-12, LDB-16

  - Max encoding time reduced from 3.6 day to 1.7 day in LDB

--MaxBTLumaISlice=16 --MaxBTNonISlice=16 --MaxTTLumaISlice=16 --MaxTTNonISlice=16

It is suggested to use this maxmtt16 at least in RA. Agreed for all configurations.

Extended precision flag, several expert support the following setting.

- 0 for 12 bit

- 1 for 16 bit

Agreed

Evaluation criteria

The following measurement should be used for quality accessment.

- PSNR and wPSNR for PQ

- PSNR otherwise

Agreed

It was mentioned combination of (average of different) class bdrate is not reasonable. Overall average should not be used for evaluation criteria . For spread sheet, we keep overall part but the column should be in gray to indicate it is not relevant value.

Agreed

Complexity uses the EncT and DecT as usual

High frame rate

It seems we have several high frame rate sequences.

- EBU 100frames/sec

- Drums 100frames

But there’s no content with high bit rate and high bit depth in our space.

It was commented that high frame rate could be independent of high bit depth.

It was asked what problem we would solve here. Conform no problem for high bit frame in the VVC design. e.g.coding efficiency is sufficient or not can be.

It was commented many hundred frame content could be interesting. But 100frames content could be similar to 50/100 fps content.

For output document, it can be said not mature to set test condition. Agreed

Encourage people for submit high bit rate test material

CE consideration

CE is related to rice parameter. T0072, T0085, T0089 and T0105.  Agreed

The initial CE document was quickly reviewed and several opinions were expressed.

It was commented if proposal changes 8bit- 10bit operation, usual CTC should be also evaluated.

It was commented 8bit and 10bit no change should be also considered.