Joint Video Experts Team (JVET)

of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29                       Document: JVET-T0128

20th Meeting, by teleconference, 7 – 16 Oct. 2020

| Title: | [DNNVC] Preliminary results of Neural Network Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Information to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Zhao Wang, Ru-Ling Liao, Changyue Ma, Yan Ye (Alibaba Group). Shanshe Wang, Siwei Ma (Peking University) | Tel: Email: | baixiu.wz@alibaba-inc.com |
| Source: | Alibaba |  |  |

_____________________________

Introduction

The most advanced Versatile Video Coding (VVC) standard achieves up to 40% bit-rate reduction while keep the same quality compared with High Efficiency Video Coding (HEVC). It is also well adapted to videos with high resolutions and different video formats. On the other hand, since VVC still employs the block based coding and quantization structure, various types of distortions still exist such as blocking artifacts, ringing, noise, blurring, etc. Although these distortions are irreversible and cannot be completely eliminated, it can be reduced by specific filters. Therefore, in-loop filtering plays a significant role in restoring these degradations and improving the quality of video frames before used as reference frames, such as Deblocking, Sample Adaptive Offset (SAO), Adaptive Loop Filter (ALF), etc [1].

Recently, inspired by the great success in high-level vision tasks such as image classification and object detection, deep neural network (DNN) has also been adopted for low-level vision tasks to find a nonlinear mapping function from the degraded image to the desired one, such as super-resolution, image restoration, image quality enhancement, image de-noising, etc. Since DNN has powerful nonlinear fitting ability, it can  perform well for some image restoration tasks.

There are also a few literatures to develop a DNN based loop filter in traditional video coding standard to reduce the artifacts and improve the quality of reconstructed frames [2-6]. In this document, we provide preliminary results for a neural network based loop filter (NNLF). Preliminary results show that our NNLF can achieve 5.57%, 12.55% and 13.62% coding gain over VVC for Y, U and V components under RA configuration for HD sequences.

NNLF

Neural Network Architecture

For the reconstructed frame before NNLF, it will first be converted into YUV444 by up-sampling the chroma components if the original color planes don’t own the same size. Neighboring samples are copied directly to simplify the up-sampling process. During inference,  only the top-left sample of each 2x2 chroma block is saved.

ResNet [7] is adopted as the network architecture (Figure 1), which consists of some residual blocks (resblock) and the number of resblock is set as a hyper-parameter. Resblock represents the residual learning, as shown in Figure 1 (right), aiming to learn the residual mapping between the current layer and the next layer. As for the activation function, rectified linear units (ReLU) [8] or others can be considered.

Figure 1. Illustration of the proposed network architecture in NNLF (act: activation function).

Training

The Vimeo90K dataset [9] is used in our training. The raw videos are compressed by VTM9.3 under RA and the reconstructed frames are obtained as the training dataset. The QP range is from 20 to 45. In our implementation, 32 resblocks are used and the channel number of each convolutional layer is set to 64. For the convolutional process, 3x3 kernel is used. Therefore, there is about 2.4 million model parameters in total.

During the training, the loaded frame will first be cropped to 128x128 size, aiming to increase the data variety and meanwhile reduce the complexity. The batch size is set to 32 and the model is trained about 384000 iterations.

Experimental Results and Future Work

By incorporating the NNLF between SAO and ALF in VTM9.3, the experimental results for Class B under QPs (27, 32, 37, 42) are shown in Table 1. It is observed that about 5.57%, 12.55% and 13.62% coding gain for Y, U and V components can be achieved.

Though promising, we consider such results preliminary. Future work includes expanding the test set and reducing complexity. In our opinion, the latter is especially critical for NNLF to become mature enough for consideration of standardization.

Table 1. The experimental results of NNLF in VTM9.3 under RA configuration.

|  | Y | U | V |
| --- | --- | --- | --- |
| MarketPlace | -4.65% | -13.59% | -19.15% |
| RitualDance | -7.45% | -14.42% | -14.14% |
| Cactus | -5.47% | -14.47% | -12.50% |
| BasketballDrive | -5.96% | -12.30% | -13.11% |
| BQTerrace | -4.31% | -7.97% | -9.20% |
| Overall | -5.57% | -12.55% | -13.62% |

References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding draft 7,” JVET-P2001, October 2019.

Y. Dai, D. Liu, F. Wu. “A convolutional neural network approach for post-processing in HEVC intra coding,” in International Conference on Multimedia Modeling, 2017.

C. Jia, S. Wang, X. Zhang, J. Liu, S. Pu, S. Wang, S. Ma. “Content-Aware Convolutional Neural Network for In-loop Filterin in High Efficiency Video Coding,” IEEE Trans. on Image Processing, 2019.

Li C, Song L, Xie R, et al. CNN based post-processing to improve HEVC[C]//2017 IEEE International Conference on Image Processing (ICIP). IEEE, 2017: 4577-4580.

Wang M Z, Wan S, Gong H, et al. Attention-based dual-scale CNN in-loop filter for Versatile Video Coding[J]. IEEE Access, 2019, 7: 145214-145226.

Jia C, Wang S, Zhang X, et al. Spatial-temporal residue network based in-loop filter for video coding[C]//2017 IEEE Visual Communications and Image Processing (VCIP). IEEE, 2017: 1-4.

He K, Zhang X, Ren S, et al. Deep residual learning for image recognition[C]. Proceedings of the IEEE conference on computer vision and pattern recognition. 2016: 770-778.

Nair, V., et al. Rectified linear units improve restricted Boltzmann machines. In Proceedings of the 27th international conference on machine learning (ICML-10) (pp. 807-814).

Xue T, Chen B, Wu J, et al. Video enhancement with task-oriented flow[J]. International Journal of Computer Vision, 2019, 127(8): 1106-1125.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).