| J oint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting: by teleconference, 7 Oct – 16 Oct 2020 | Document: JVET-T2018 |
| --- | --- |

| Title: | Common test conditions for high bit depth and high bit rate video coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document from JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Adrian Browne Tomohiro Ikai D. Rusanovskyy Xiaoyu Xiu | Email: | adrian.browne@sony.com ikai.tomohiro@sharp.co.jp dmytror@qti.qualcomm.com xiaoyuxiu@kwai.com |
| Source: | JVET |  |  |

_____________________________

# Abstract

This document proposes common test conditions (CTC) and software reference configurations to be used in the context of high bit depth and high bit rate video coding experiments after the 20th JVET meeting. These common test conditions are recommended for use in technical contributions to the 21st and following JVET meetings, as applicable.

# Introduction

Common test conditions (CTC) are desirable to conduct experiments in a well-defined environment and to ease the comparison of the outcome of experiments.

This document defines three test conditions for high bit depth and high bit rate content, reflecting intra-only, random-access, and low-delay settings:

- Intra, 12 and 16 bit

- Random access, 12 and 16 bit

- Low delay, 12 and 16 bit

A subset of these test conditions might be used for a particular experiment. For example, when testing an intra coding tool, only intra configurations might be used.

The test conditions define tests at bit depths of 12 and 16 bits. For 12 bit tests, PQ, HLG and RGB sequences have been defined. For 16 bit tests, only RGB sequences are used.

Version 11.0 or later of the VTM software is expected to be used for most experiments. More recent versions are encouraged where applicable.

Any input contributions to JVET related to high bit depth, high bit rate or high frame rate video coding should provide a set of results using these test conditions that is as complete as possible. If an input contribution has any effect on 8 or 10 bit processing further results should be provided using the appropriate test conditions, e.g. JVET-T2010[1] or JVET-T2011[2].

The following sections define the software configuration, test sequences, quantization parameter values, evaluation processes, and encoder configuration files to be used.

# Software configuration

Version 11.0 (or later if available) of VTM shall be configured for high bit depth processing by defining the following macro in the source code (TypeDef.h)

#define JVET_R0351_HIGH_BIT_DEPTH_ENABLED			1

In addition, VTM shall be enabled with HDRLib functionality. For more information see the HDR CTC[2].

# Test sequences

Tables 1, 2 and 3 define the sets of test sequences to be used for intra, random-access, and low-delay in both lossy and lossless experiment conditions. All frames (as defined by frame count in the table) shall be encoded for all sequences in the random-access and low-delay test cases described below. For the intra configuration, the first of every eight frames shall be encoded starting with the first frame in the sequence for all sequences, and bit rates shall be calculated using the sequence frame rate (as defined by frame rate in the table) divided by eight. Please note that temporal sub-sampling may be enabled in the VTM software using the parameter TemporalSubsampleRatio. For random access and low delay configurations, if there is no number in the “encoded frame count” column for the configuration, the test case is not requested in the CTC.

| Class | Sequence | Frame rate | Encoded frame count |  |  | Input bit depth | Internal bit depth |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | AI | RA | LDB |  |  |
| SVT (HD) | OldTownCross | 50 | 100 | 97 | 50 | 16 | 12 and 16 |
|  | CrowdRun | 50 | 100 | 97 | 50 | 16 | 12 and 16 |
|  | ParkJoy | 50 | 100 |  |  | 16 | 12 and 16 |
|  | DucksTakeOff | 50 | 100 |  |  | 16 | 12 and 16 |
|  | InToTree | 50 | 100 |  |  | 16 | 12 and 16 |

Table 1. RGB 4:4:4 content

RGB sequences are mandatory in both 12 and 16 bit tests.

| Class | Sequence | Frame rate | Encoded frame count |  |  | Input bit depth | Internal bit depth |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | AI | RA | LDB |  |  |
| PQ (HD) | BallonFestival | 24 | 100 | 97 | 50 | 12 | 12 |
|  | EBU_04_Hurdles | 50 | 100 | 97 | 50 | 12 | 12 |
|  | Market3Clip4000 | 50 | 100 |  |  | 12 | 12 |
|  | EBU_06_Starting | 50 | 100 |  |  | 12 | 12 |

Table 2. PQ YUV 4:4:4 and 4:2:2 content

PQ sequences are in both YUV 4:4:4 and YUV 4:2:2 format and are mandatory in 12 bit tests.

| Class | Sequence | Frame rate | Encoded frame count |  |  | Input bit depth | Internal bit depth |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | AI | RA | LDB |  |  |
| HLG (4K) | DayStreet | 60 | 100 | 97 | 25 | 12 | 12 |
|  | PeopleInShoppingCenter | 60 | 100 | 97 | 25 | 12 | 12 |
|  | NightStreet | 60 | 100 |  |  | 12 | 12 |
|  | StainedGlass | 60 | 100 |  |  | 12 | 12 |

Table 3. HLG YUV 4:4:4 and 4:2:2 content

HLG sequences are in both YUV 4:4:4 and YUV 4:2:2 format and are mandatory in 12 bit tests.

Tables 4 indicates which test configurations are required for each type of content. The column corresponding to each configuration is interpreted as follows:

- “M” indicates that the test sequence is mandatory in the CTC for the given configuration

- “O” indicates that the test sequence is optional (but encouraged) in the CTC for the given configuration

- “-” indicates that the test sequence is not requested in the CTC for the given configuration

| Class | Chroma sub-sampling | 12 bit |  |  | 16 bit |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AI | RA | LDB | AI | RA | LDB |
| SVT | 4:4:4 | M | M | M | M | O | M |
| PQ | 4:4:4 | M | M | M | - | - | - |
|  | 4:2:2 | M | O | O | - | - | - |
| HLG | 4:4:4 | M | M | M | - | - | - |
|  | 4:2:2 | M | O | O | - | - | - |

Table 4. Required test conditions for lossy coding.

| Class | Chroma sub-sampling | 12 bit |  |  | 16 bit |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AI | RA | LDB | AI | RA | LDB |
| SVT | 4:4:4 | O | O | O | O | O | O |
| PQ | 4:4:4 | O | O | O | - | - | - |
|  | 4:2:2 | O | O | O | - | - | - |
| HLG | 4:4:4 | O | O | O | - | - | - |
|  | 4:2:2 | O | O | O | - | - | - |

Table 5. Required test conditions for lossless coding.

Original versions of the test sequences in Table 1, 2 and 3 are available on ftp://jvet@ftp.ient.rwth-aachen.de/ctc/hbd/.  (Properly qualified participants of JVET may please contact the JVET chairs for login information).

# Quantization parameter values

Results shall be provided using two sets of six quantization parameter values, one for 12 bit test cases and the other for 16 bit test cases as follows:

- 12 bit: 12, 7, 2, -3, -8 and -13

- 16 bit: -8, -13, -18, -23, -28, -33

These values define the initial QP values that are used to derive respective QP for I- and B-frames in a sequence.

The method for calculating six point BD-rates described in JVET-H0030[3] should be used.

# Configuration

Configuration files for the three requested test conditions are provided with the VTM software in the cfg directory and shall be used. The three configurations provided are as follows:

- “All Intra” (AI): encoder_intra_vtm.cfg

- “Random access” (RA): encoder_randomaccess_vtm.cfg

- “Low-delay B” (LDB): encoder_lowdelay_vtm.cfg

In addition to the three test conditions, format and class specific parameter files are also provided with the VTM software and shall be used as follows:

SVT: cfg/per-class/formatRGB.cfg

PQ (both 4:4:4 and 4:2:2): cfg/444/yuv444.cfg and cfg/per-class/classH1.cfg

HLG (both 4:4:4 and 4:2:2): cfg/444/yuv444.cfg and cfg/per-class/classH2.cfg

To reduce simulation times, the high bit depth and high bit rate CTC also introduce some configuration changes to disable certain tools and limit the maximum size of transform blocks as follows:

- MaxBTLumaISlice=16

- MaxBTNonISlice=16

- MaxTTLumaISlice=16

- MaxTTNonISlice=16

- LFNST=0

- Profile=none

These changes are included in the file cfg/hbd/hbd.cfg which shall be used. This file also enables BDPCM for all content.

Additional configuration is required separately for 12 bit and 16 bit test cases. For 12 bit tests, extended precision shall be disabled and the internal bit depth shall be set to 12 at least when processing 16 bit SVT sequences. For 16 bit tests, extended precision shall be enabled.

The specific changes for different bit depths described here are included in two configuration files cfg/hbd/12-bit.cfg and cfg/hbd/16-bit.cfg and shall be used for 12 and 16 bit test cases respectively.

Finally, sequence specific parameters are provided with the VTM software in the cfg/per-sequence-HBD directory and shall used.

# HM Anchor

For the purposes of comparison an HM anchor can be created using the latest version of HM (16.22 or later) built with the following macro defined:

#define RExt__HIGH_BIT_DEPTH_SUPPORT		1

When creating an HM anchor the following configuration files shall be used for each test case:

- “All Intra” (AI): encoder_intra_main_rext.cfg

- “Random access” (RA): encoder_randomaccess_main_rext.cfg

- “Low-delay B” (LDB): encoder_lowdelay_main_rext_vtm.cfg

These files shall be used in conjuction with the files cfg/12-bit.cfg and cfg/16-bit.cfg for 12 and 16 bit test cases respectively.

# Evaluation

Results reported for PQ sequences shall include the following metrics:

- PSNR for Y′, Cb and Cr

- wPSNR for Y’, Cb and Cr (for more information see the HDR CTC[2])

Results reported for the HLG and SVT sequences class shall include the following metrics:

- PSNR for Y’, Cb and Cr

An Excel template for reporting test results is provided alongside this document.

# MD5

## SVT

6004ff26cafcf2b066faa00c935a7621  CrowdRun_1920x1080_50_16bit_444.rgb

2ab943d7b1c6e3b26a2c6da416b69828  ParkJoy_1920x1080_50_16bit_444.rgb

d882efad39ae3d360896ec09d9213424  DucksTakeOff_1920x1080_50_16bit_444.rgb

6f7d413472f856741e95543217f9795d  InToTree_1920x1080_50_16bit_444.rgb

ae5bab60c3dfb16399971b9afff120da  OldTownCross_1920x1080_50_16bit_444.rgb

## PQ

### PQ 4:2:2

b029dace9f10af78aa6c3363531b1768  BalloonFestival_1920x1080p_24_12b_pq_709_ct2020_422.yuv

716c1ec421a72c46011f31754077eec4  Market3Clip4000r2_1920x1080p_50_12b_pq _709_ct2020_422.yuv

b9f22e80fa8eb33e3c00633e6824844f  EBU_04_Hurdles_1920x1080p_50_12b_pq_709_ct2020_422.yuv

a480ed6fc9e9f4e6b84b1ab6e7bc4f1b  EBU_06_Starting_1920x1080p_50_12b_pq_709_ct2020_422.yuv

### PQ 4:4:4

1664a322cf91934872afd07035819a05  BalloonFestival_1920x1080p_24_12b_pq_709_ct2020_444.yuv

564904c4001957472483a3d79b55f17d  Market3Clip4000r2_1920x1080p_50_12b_pq_709_ct2020_444.yuv

544b1b8816d0003e69ce6a28bbd01810  EBU_04_Hurdles_1920x1080p_50_12b_pq_709_ct2020_444.yuv

f6761039392e76336493effdc257bf1a  EBU_06_Starting_1920x1080p_50_12b_pq_709_ct2020_444.yuv

## HLG

### HLG 4:2:2

136cbeb9cea1fd6969366b5979b89eb8  DayStreet_3840x2160_60p_12bit_422_hlg.yuv

8fc91ff312f3a729bf6c5e149bbaaf5e  PeopleInShoppingCenter_3840x2160_60p_12bit_422_hlg.yuv

a9773f3df122061379888a5268d0d50e  NightStreet_3840x2160_60p_12bit_422_hlg.yuv

5cf45f6880652eb9b9ddac84f3d20e66  StainedGlass_3840x2160_60p_12bit_422_hlg.yuv

### HLG 4:4:4

22d84b5024ff0477a729d5a16b29bf61  DayStreet_3840x2160_60p_12bit_444_hlg.yuv

dcdc82fcbf693c0bb0dcf16abad64c75  PeopleInShoppingCenter_3840x2160_60p_12bit_444_hlg.yuv

da10d72731df64910269f7a193021cae  NightStreet_3840x2160_60p_12bit_444_hlg.yuv

cc6f6f8c1d5c7143210e602c3d9b7471  StainedGlass_3840x2160_60p_12bit_444_hlg.yuv

# References

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-T2010, JVET, 20th Meeting: by teleconference, 7-16 Oct. 2020

[2] A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy , “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-T2011, JVET, 20th Meeting: by teleconference, 7-16 Oct. 2020

[3] A. M. Tourapis, D. Singer, Y. Su, K. Mammou, “BD-Rate/BD-PSNR Excel extensions”, JVET-H0030, JVET, 8th Meeting: Macao, CN, 18–25 Oct. 2017