| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 April 2021 | Document: JVET-V0007-v1 |
| --- | --- |

| Title: | JVET AHG report: Coding of HDR/WCG Material (AHG7) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | A. Segall E. Francois W. Husak S. Iwamura D. Rusanovskyy | Email: | asegall@sharplabs.com Edouard.Francois@InterDigital.com WJH@dolby.com iwamura.s-gc@nhk.or.jp dmytror@qti.qualcomm.com |
| Source: | AHG7 |  |  |

_____________________________

# Abstract

This document summarizes the activity of AHG7: Coding of HDR/WCG Material between the 21st meeting (6 – 15 Jan. 2021) and the 22nd meeting (20 – 28 April 2021), both held by teleconference.

# Mandates

The AHG was established with the following mandates:

- Study and evaluate available HDR/WCG test content.

- Study objective metrics for quality assessment of HDR/WCG material, including investigation of the correlation between subjective and objective results.

- Compare the performance of the VTM and HM for HDR/WCG content.

- Generate CTC anchors for the VTM according to JVET.

- Study the luma/chroma bit allocation in the HDR CTC, especially for HLG content.

- Coordinate with AHG 8 on preparing HDR material in 12 bit 4:2:0 format, and support in generating the HDR anchors for the CE.

- Coordinate with AHG4 in preparation for verification testing for HDR video content.

- Study additional aspects of coding HDR/WCG content.

# Activities

The primary activity of the AhG was related to the mandates of (i) comparing the performance of the VTM for HDR/WCG content, (ii) study of objective metrics, (iii) coordinating with AHG8 on preparing HDR material in 12 bit 4:2:0 format, and (iv) coordinating with AHG4 in preparation for verification testing for HDR video content. This work is described in the following subsection.

## Anchor Generation

The AhG generated CTC anchors for the VTM according to JVET-T2011.  A summary of the performance is provided below, and more detailed information may be found in the included XLS data.

### VTM 12.0 versus VTM 11.0

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class H2 |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 98% | 95% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 96% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM-10.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 8.27% | 6.16% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class H2 |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Overall | 8.27% | 6.16% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 99% |

## Objective Metrics

During the interim period, it was determined that there was an error in the calculation of the HDR metrics when the temporal prefilter was enabled.  This effected the calculation of DE100 and PSNR-L100 in VTM11.0.

A fix was created and integrated into VTM-12.0 to address this error, and the effect can be seen in the results above.  More information can be found at https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/1452.

## Coordinating with AHG8

The AHG coordinated with AHG8 to review the configuration files used to generated the 12-bit HLG 4:2:0 content.

## Coordinating with AHG4

The AHG coordinated with AHG4 to prepare for the HDR verification test dry run.  It is specifically highlighted that AHG4 received and studied three, new HDR sequences during the AHG period.  AHG7 provided support in preparing and converting the sequences.

# Contributions

There are six contributions related to HDR video coding.

| JVET-V0007 | JVET AHG report: Coding of HDR/WCG material (AHG7) | A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy |
| --- | --- | --- |
| JVET-V0043 | AHG4: Status Report on HDR Video Verification Test Preparation | A. Segall, M. Wien, V. Baroncini, K. Andersson |
| JVET-V0044 | AHG4: Agenda and report of the AHG meeting on the HDR verification test preparation on 2021-03-25 | A. Segall, M. Wien, V. Baroncini (AHG coordinators) |
| JVET-V0045 | Aerial 4K HDR (HLG) Test Data | Pankaj Topiwala (FastVDO) |
| JVET-V0078 | AHG10: QP control for very smooth blocks | K. Andersson, J. Enhorn, J. Ström (Ericsson) |
| JVET-V0107 | Editorial suggestion for JVET CTC on HDR/WCG | D. Rusanovskyy (Qualcomm), T.Hashimoto (Sharp) |

# Recommendations

The AHG recommends the following:

- Review all input contributions