| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0008 |
| --- | --- |

| Title: | JVET AHG Report: High bit depth, high bit rate, and high frame rate coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Adrian Browne Tomohiro Ikai Dmytro Rusanovskyy Mohammed Golam Sarwer Xiaoyu Xiu | Email: | adrian.browne@sony.com ikai.tomohiro@sharp.co.jp dmytror@qti.qualcomm.com m.sarwer@alibaba-inc.com xiaoyuxiu@kwai.com |
| Source: | AHG8 |  |  |

_____________________________

# Abstract

This document summarizes the activity of AHG8: High bit depth, high bit rate, and high frame rate coding between the 21st meeting by teleconference (6–15 January 2021) and the 22nd meeting by teleconference (20-28 April 2021).

# Mandates

The AHG was established with the following mandates:

- Study the benefits and characteristics of VVC coding tools for high bit depth, high bit rate, and high frame rate coding.

- Study lossless coding characteristics of VVC.

- Identify technologies for future extension of VVC to support such application usage.

- Discuss and refine the JVET-U2018 testing conditions for high bit depth, high bit rate, and high frame rate coding.

- Finalize, conduct and coordinate the work on the core experiment JVET-U2022.

- Identify suitable test material for testing of high bit depth, high bit rate, and high frame rate coding in coordination with AHG4 and AHG7.

- Study VVC entropy decoding throughput in the cases of high bit depth, high bit rate, and high frame rate coding.

# Activities

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, with [AHG8] in message headers. A message was sent to the reflector with respect to the conversion of HLG content within the CTC on 27th January, a CE change notification on 25th March, and a formal kick-off message was sent on 29th March. In addition to the messages on the reflector, the the co-chairs received a question about the calendar for establishing a 12-bit profile for VVC.

The primary activities of the AhG were the updates to the high bit depth CTC (JVET-U2018) and the core experiment on entropy coding for high bit depth and high bit rate coding (JVET-V0022).

As a part of refining the CTC, AhG conducted HDR 12bit test material preparation. This included generation of HDR PQ/HDR HLG test sequences in 4:2:0 format and agreed conversion UHD to HD resolution for HLG sequences. AhG8 would like to thank coordinators of AhG7 for discussion on HDR conversion practices and on HDR metric specification and implementation.

In total, there are 45 high bit depth related contributions, but none related to high frame rate.  The following section lists these contributions.

# Contributions

The contributions can be split into CE and CE-related (28 contributions), and others (17 contributions).

## CE and CE-related

JVET-V0022, “CE: Summary Report on Entropy Coding for High Bit Depth and High Bit Rate Coding”, A.Browne, T. Hashimoto, H. Jhu, D. Rusanovskyy, K. Kawamura, T. Zhou (CE co-ordinators)

JVET-V0046, “CE-1.7: Rice parameter derivation for high bit-depth coding”, T. Hashimoto, T. Ikai (Sharp)

JVET-V0047, “CE-3.1 and CE-3.2: Transform coefficients range extension for high bit-depth coding”, T. Zhou, T. Chujoh, T. Ikai (Sharp)

JVET-V0048, “CE-4.1: Combination of CE-3.1, CE-1.5 and CE-2.1”, T. Zhou, T. Chujoh, T. Ikai (Sharp)

JVET-V0049, “CE-4.2: Combination of CE-3.2, CE-1.5 and CE-2.1”, T. Zhou, T. Chujoh, T. Ikai (Sharp)

JVET-V0050, “CE-1.5, CE-1.6, CE-2.2 and CE-2.3: Rice parameter selection for high bit depths”, A. Browne, S. Keating, K. Sharman (Sony)

JVET-V0051, “CE-4.3 and CE-4.4: Combinations of CE-1.6 and CE-2.3 with CE-3.2”, A. Browne, S. Keating, K. Sharman (Sony)

JVET-V0052, “CE-1.1, CE-1.2 and CE-1.4: On the Rice parameter derivation for high bit-depth coding”, D. Rusanovskyy, M. Karczewicz, L. P. Van, M. Coban (Qualcomm)

JVET-V0053, “CE-4.5, CE-4.6 and CE-4.7: Combinations of RRC tests CE-1.1, CE-1.2 and CE-1.4 with CE-3.1/CE-3.2”, D. Rusanovskyy, M. Karczewicz, L. P. Van, M. Coban (Qualcomm)

JVET-V0054, “CE-2.1: Slice based Rice parameter selection for transform skip residual coding”, H.-J. Jhu, X. Xiu, Y.-W. Chen, W. Chen, C.-W. Kuo, X. Wang (Kwai Inc.)

JVET-V0084, “CE-related: On Rice parameter selection for regular residual coding (RRC) at high bit depths”, A. Browne, S. Keating, K. Sharman (Sony)

JVET-V0085, “CE-related: On Rice parameter selection for transform skip residual coding (TSRC) at high bit depths”, A. Browne, S. Keating, K. Sharman (Sony)

JVET-V0106, “CE-related: On history-enhanced method of Rice parameter derivation for regular residual coding (RRC) at high bit depths”, D. Rusanovskyy, L. Pham Van, M. Coban, M. Karczewicz (Qualcomm)

JVET-V0123, “CE-related: On prefix code length of remaining level coding for high bit depth and high bit rate coding”, Y. Yu, Z. Xie, F. Wang, H. Yu, D. Wang (OPPO)

JVET-V0134, “Crosscheck of CE-1.4 from JVET-V0052 (CE-1.1, CE-1.2 and CE-1.4: On the Rice parameter derivation for high bit-depth coding)”, A. Browne (Sony)

JVET-V0135, “Crosscheck of CE-4.5 and CE-4.7 from JVET-V0053 (CE-4.5, CE-4.6 and CE-4.7: Combinations of RRC tests CE-1.1, CE-1.2 and CE-1.4 with CE-3.1/CE-3.2)”, A. Browne (Sony)

JVET-V0136, “Crosscheck of JVET-V0047 (CE-3.1 and CE-3.2: Transform coefficients range extension for high bit-depth coding)”, A. Browne (Sony)

JVET-V0138, “Crosscheck of CE-1.2 from JVET-V0052 (CE-1.1, CE-1.2 and CE-1.4: On the Rice parameter derivation for high bit-depth coding)”, T. Hashimoto (Sharp)

JVET-V0139, “Cross-check report for CE1.5 and CE1.6 of JVET-V0050 and CE4.3 of JVET-V0051”, D.Rusanovskyy (Qualcomm)

JVET-V0140, “Cross-check report for CE-1.7 of JVET-V0046 and CE-4.2 of JVET-V0049”, D.Rusanovskyy (Qualcomm)

JVET-V0142, “Crosscheck of JVET-V0048 (CE-4.1: Combination of CE-3.1, CE-1.5 and CE-2.1)”, H.-J. Jhu (Kwai Inc.)

JVET-V0143, “Crosscheck of CE-2.2 and CE-2.3 from JVET-V0050 (CE-1.5, CE-1.6, CE-2.2 and CE-2.3: Rice parameter selection for high bit depths)”, H.-J. Jhu (Kwai Inc.)

JVET-V0144, “Crosscheck of CE-4.4 from JVET-V0051 (CE-4.3 and CE-4.4: Combinations of CE-1.6 and CE-2.3 with CE-3.2)”, H.-J. Jhu (Kwai Inc.)

JVET-V0145, “Crosscheck of CE-4.6 from JVET-V0053 (CE-4.5, CE-4.6 and CE-4.7: Combinations of RRC tests CE-1.1, CE-1.2 and CE-1.4 with CE-3.1/CE-3.2)”, T. Zhou (Sharp)

JVET-V0155, “Crosscheck of CE-1.1 from JVET-V0052 (CE-1.1, CE-1.2 and CE-1.4: On the Rice parameter derivation for high bit-depth coding)”, M. G. Sarwer (Alibaba)

JVET-V0156, “Crosscheck of JVET-V0106 (CE-related: On history-enhanced method of Rice parameter derivation for regular residual coding (RRC) at high bit depths)”, M. G. Sarwer (Alibaba)

JVET-V0157, “Crosscheck of JVET-V0084 (CE-related: On Rice parameter selection for regular residual coding (RRC) at high bit depths)”, M. G. Sarwer (Alibaba)

JVET-V0161, “Crosscheck of JVET-V0123 (CE-related: On prefix code length of remaining level coding for high bit depth and high bit rate coding)”, H.-J. Jhu (Kwai Inc.)

## Other contributions

JVET-V0059, “AHG8: CABAC-bypass alignment for high bit-depth coding”, M.G. Sarwer, J. Chen, Y. Ye, R. -L. Liao (Alibaba)

JVET-V0066, “AHG8: Encoder improvements to palette coding for high bit depth”, T. Tsukuba, M. Ikeda, T. Suzuki (Sony)

JVET-V0067, “AHG8: A constraint of max transform size for high bit depth and high bit rate coding”, K. Kondo, M. Ikeda (Sony)

JVET-V0068, “AHG8: A constraint of max CTU size for high bit depth and high bit rate coding”, K. Kondo, M. Ikeda (Sony)

JVET-V0121, “AHG8: on coding of last significant coefficient position for high bit depth and high bit rate extensions”, F. Wang, L. Xu, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-V0122, “AHG8: a full-bypass mode in residual coding for high bit depth and high bit rate extensions”, F. Wang, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-V0124, “AHG8: Weighted Prediction for VVC High Bit Depth Extension”, Y. Yu, H. Yu, Z. Xie, F. Wang, D. Wang (OPPO)

JVET-V0131, “AHG8: Fixing the forward transform matrices for high bit depth coding”, K. Naser, F. Galpin, F. F. Le Léannec, P. De Lagrange (InterDigital)

JVET-V0133, “AHG8: Content Adaptive Transform Precision for High Bit Depth Coding”, K. Naser, F. Galpin, F. Le Léannec, P. De Lagrange (InterDigital)

JVET-V0141, “Cross-check report of AHG8: CABAC-bypass alignment for high bit-depth coding (JVET-V0059)”, D.Rusanovskyy (Qualcomm)

JVET-V0150, “AHG8: A study on bin rate of VTM-12.0 for high bit depth coding”, T. Tsukuba, M. Ikeda, T. Suzuki (Sony)

JVET-V0158, “Crosscheck of JVET-V0121 (AHG8: on coding of last significant coefficient position for high bit depth and high bit rate extensions)”, M. G. Sarwer (Alibaba)

JVET-V0159, “Crosscheck of JVET-V0067 (AHG8: A constraint of max transform size for high bit depth and high bit rate coding)”, T. Hashimoto (Sharp)

JVET-V0160, “Crosscheck of JVET-V0068 (AHG8: A constraint of max CTU size for high bit depth and high bit rate coding)”, T. Hashimoto (Sharp)

JVET-V0162, “Crosscheck of JVET-V0066 (AHG8: Encoder improvements to palette coding for high bit depth)”, H.-J. Jhu (Kwai Inc.)

JVET-V0163, “Crosscheck of JVET-V0122 (AHG8: a full-bypass mode in residual coding for high bit depth and high bit rate extensions)”, A. Browne (Sony)

JVET-V0166, “Cross-check report on AHG8: Content Adaptive Transform Precision for High Bit Depth Coding (JVET-V0133)”, D. Rusanovskyy (Qualcomm)

# Benchmarks

The benchmarks for this section were generated using the high bit depth, high bit rate CTC (JVET-U2018). The HM anchor was generated using the RExt lossy and lossless CTC (JVET-U1100) with content and QP settings taken from JVET-U2018.

## VTM 12.0 versus HM16.23

### Low QP range

| HDR PQ |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM16.23 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -5.42% | -6.08% | -6.69% | 2996% | 175% |
| PQ422 | -8.17% | -12.30% | -12.82% | 2550% | 169% |
| Overall | -6.79% | -9.19% | -9.75% | 2773% | 172% |
|  |  |  | LDB |  |  |
|  |  |  | Over HM16.23 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -6.00% | -4.18% | -5.75% | 300% | 164% |
| PQ422 | -7.21% | -11.17% | -11.80% | 394% | 164% |
| Overall | -6.61% | -7.67% | -8.77% | 347% | 164% |
|  |  |  | RA |  |  |
|  |  |  | Over HM16.23 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -6.14% | -4.32% | -5.56% | 343% | 164% |
| PQ422 | -7.47% | -11.50% | -11.72% | 474% | 164% |
| Overall | -6.81% | -7.91% | -8.64% | 409% | 164% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM16.23 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -3.39% | -5.40% | -5.68% | 3198% | 171% |
| HLG422 | -3.97% | -6.73% | -6.70% | 2943% | 167% |
| Overall | -3.68% | -6.06% | -6.19% | 3070% | 169% |
|  |  |  | LDB |  |  |
|  |  |  | Over HM16.23 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -5.36% | -7.49% | -6.79% | 401% | 158% |
| HLG422 | -4.92% | -8.86% | -7.23% | 495% | 162% |
| Overall | -5.14% | -8.18% | -7.01% | 448% | 160% |
|  |  |  | RA |  |  |
|  |  |  | Over HM16.23 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -5.92% | -8.08% | -7.19% | 449% | 157% |
| HLG422 | -5.51% | -9.19% | -7.44% | 612% | 162% |
| Overall | -5.72% | -8.64% | -7.32% | 531% | 160% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM16.23 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 36.32% | 34.27% | 34.20% | 4934% | 144% |
| SVT12 | 0.13% | 0.68% | 0.58% | 5230% | 138% |
| Overall | 18.22% | 17.48% | 17.39% | 5082% | 141% |
|  |  |  | LDB |  |  |
|  |  |  | Over HM16.23 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 20.21% | 21.49% | 21.55% | 584% | 144% |
| SVT12 | -2.91% | -0.57% | -0.55% | 629% | 141% |
| Overall | 8.65% | 10.46% | 10.50% | 606% | 143% |
|  |  |  | RA |  |  |
|  |  |  | Over HM16.23 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 17.97% | 20.35% | 20.33% | 722% | 144% |
| SVT12 | -3.26% | -0.83% | -0.82% | 784% | 140% |
| Overall | 7.36% | 9.76% | 9.76% | 753% | 142% |

### Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | HM16.23 | VTM12.0 |  | HM16.23 | VTM12.0 |  | HM16.23 | VTM12.0 |  |
| PQ444 | 2.5 | 2.5 | 3.01% | 3.1 | 3.0 | 4.21% | 3.1 | 3.0 | 3.83% |
| PQ422 | 2.3 | 2.2 | 2.86% | 2.9 | 2.8 | 3.13% | 2.9 | 2.8 | 2.78% |
| Overall | 2.4 | 2.4 | 2.93% | 3.0 | 2.9 | 3.67% | 3.0 | 2.9 | 3.30% |
| Enc Time[%] | 7413% |  |  | 878% |  |  | 867% |  |  |
| Dec Time[%] | 158% |  |  | 139% |  |  | 140% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | HM16.23 | VTM12.0 |  | HM16.23 | VTM12.0 |  | HM16.23 | VTM12.0 |  |
| HLG444 | 1.8 | 1.7 | 6.56% | 1.9 | 1.9 | 2.10% | 2.0 | 1.9 | 1.87% |
| HLG422 | 1.7 | 1.6 | 7.55% | 1.9 | 1.9 | 2.08% | 1.9 | 1.9 | 1.83% |
| Overall | 1.7 | 1.6 | 7.05% | 1.9 | 1.9 | 2.09% | 1.9 | 1.9 | 1.85% |
| Enc Time[%] | 8301% |  |  | 765% |  |  | 722% |  |  |
| Dec Time[%] | 138% |  |  | 116% |  |  | 117% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | HM16.23 | VTM12.0 |  | HM16.23 | VTM12.0 |  | HM16.23 | VTM12.0 |  |
| SVT16 | 1.2 | 0.7 | 73.47% | 1.2 | 0.7 | 80.20% | 1.2 | 0.7 | 80.25% |
| SVT12 | 1.3 | 1.0 | 31.02% | 1.3 | 1.0 | 29.88% | 1.3 | 1.0 | 29.30% |
| Overall | 1.2 | 0.8 | 52.24% | 1.3 | 0.8 | 55.04% | 1.3 | 0.8 | 54.78% |
| Enc Time[%] | 7727% |  |  | 831% |  |  | 802% |  |  |
| Dec Time[%] | 133% |  |  | 151% |  |  | 152% |  |  |

## VTM CE anchor versus HM16.23

The VTM CE anchor consisted of VTM12.0 with the RRC and TSRC rice coding fixes in the initial versions of CE-1.1 and CE-2.1 respectively.

### Low QP range

| HDR PQ |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM16.23 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -5.82% | -6.50% | -7.13% | 3068% | 175% |
| PQ422 | -8.66% | -12.78% | -13.32% | 2562% | 169% |
| Overall | -7.24% | -9.64% | -10.23% | 2815% | 172% |
|  |  |  | LDB |  |  |
|  |  |  | Over HM16.23 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -6.06% | -4.23% | -5.79% | 301% | 165% |
| PQ422 | -7.30% | -11.23% | -11.86% | 397% | 164% |
| Overall | -6.68% | -7.73% | -8.82% | 349% | 164% |
|  |  |  | RA |  |  |
|  |  |  | Over HM16.23 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -6.26% | -4.44% | -5.69% | 347% | 165% |
| PQ422 | -7.61% | -11.64% | -11.85% | 482% | 164% |
| Overall | -6.93% | -8.04% | -8.77% | 414% | 164% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM16.23 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -3.75% | -5.67% | -5.96% | 3188% | 171% |
| HLG422 | -4.44% | -7.11% | -7.10% | 2973% | 167% |
| Overall | -4.09% | -6.39% | -6.53% | 3080% | 169% |
|  |  |  | LDB |  |  |
|  |  |  | Over HM16.23 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -5.34% | -7.53% | -6.84% | 406% | 159% |
| HLG422 | -4.90% | -8.88% | -7.29% | 503% | 164% |
| Overall | -5.12% | -8.20% | -7.06% | 454% | 162% |
|  |  |  | RA |  |  |
|  |  |  | Over HM16.23 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -5.93% | -8.12% | -7.24% | 455% | 157% |
| HLG422 | -5.54% | -9.24% | -7.51% | 616% | 162% |
| Overall | -5.73% | -8.68% | -7.37% | 535% | 159% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM16.23 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.70% | 0.82% | 0.79% | 4842% | 119% |
| SVT12 | -2.54% | -1.95% | -2.03% | 5252% | 137% |
| Overall | -0.92% | -0.57% | -0.62% | 5047% | 128% |
|  |  |  | LDB |  |  |
|  |  |  | Over HM16.23 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.52% | 1.13% | 1.13% | 572% | 132% |
| SVT12 | -3.31% | -1.05% | -1.02% | 647% | 142% |
| Overall | -1.91% | 0.04% | 0.05% | 609% | 137% |
|  |  |  | RA |  |  |
|  |  |  | Over HM16.23 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.86% | 0.91% | 0.91% | 714% | 132% |
| SVT12 | -3.72% | -1.32% | -1.29% | 802% | 142% |
| Overall | -2.29% | -0.21% | -0.19% | 758% | 137% |

### Lossless

| PQ | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | HM16.23 | VTM_U_CE |  | HM16.23 | VTM_U_CE |  | HM16.23 | VTM_U_CE |  |
| PQ444 | 2.5 | 2.5 | 0.89% | 3.1 | 3.0 | 4.80% | 3.1 | 3.0 | 4.47% |
| PQ422 | 2.3 | 2.3 | -0.18% | 2.9 | 2.8 | 3.40% | 2.9 | 2.8 | 3.13% |
| Overall | 2.4 | 2.4 | 0.36% | 3.0 | 2.9 | 4.10% | 3.0 | 2.9 | 3.80% |
| Enc Time[%] | 7206% |  |  | 976% |  |  | 983% |  |  |
| Dec Time[%] | 152% |  |  | 147% |  |  | 149% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | HM16.23 | VTM_U_CE |  | HM16.23 | VTM_U_CE |  | HM16.23 | VTM_U_CE |  |
| HLG444 | 1.8 | 1.7 | 3.27% | 1.9 | 1.9 | 4.25% | 2.0 | 1.9 | 4.08% |
| HLG422 | 1.7 | 1.6 | 2.37% | 1.9 | 1.9 | 3.66% | 1.9 | 1.9 | 3.49% |
| Overall | 1.7 | 1.7 | 2.82% | 1.9 | 1.9 | 3.96% | 1.9 | 1.9 | 3.79% |
| Enc Time[%] | 7782% |  |  | 817% |  |  | 767% |  |  |
| Dec Time[%] | 132% |  |  | 128% |  |  | 126% |  |  |
| SVT | All Intra |  |  | Low delay B |  |  | Random Access |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | HM16.23 | VTM_U_CE |  | HM16.23 | VTM_U_CE |  | HM16.23 | VTM_U_CE |  |
| SVT16 | 1.2 | 1.2 | 1.46% | 1.2 | 1.2 | 1.18% | 1.2 | 1.2 | 1.11% |
| SVT12 | 1.3 | 1.2 | 3.12% | 1.3 | 1.3 | 2.71% | 1.3 | 1.3 | 2.55% |
| Overall | 1.2 | 1.2 | 2.29% | 1.3 | 1.2 | 1.95% | 1.3 | 1.2 | 1.83% |

- Recommendations

The AHG recommends the following:

- To review all related contributions;

- To continue high bit depth, high bit rate, and high frame rate studies.

- To consider the timeline for high bit depth profile development in VVC version 2.