| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0056-v3 |
| --- | --- |

| Title: | [AHG10] GOP-based temporal filter improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | P. Wennersten C. Hollmann J. Ström | Email: | per.wennersten@ericsson.com christopher.hollmann@ericsson.com jacob.strom@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes a set of changes to the GOP-based temporal filter in VTM that aim at improving the BD-rate performance. These changes were first proposed in JVET-U0056.

The average Y/U/V BD-rates for VTM-12 CTC are reported to be −1.4%/−1.0%/−1.1% for RA. All BDR numbers were computed using unfiltered source sequences as reference.

In addition, the same changes were implemented in HM where the BD-rates are -1.0%/-1.0%/-1.0%.

Visual inspection of the resulting sequences was reportedly carried out with no problems reported.

It is proposed to adopt the proposed changes into the VTM and HM software.

Version 2 of this document adds results for HDR.

Version 3 of this document adds information about a patch for VTM-11.

# Proposal

The VTM proposal itself is unchanged since JVET-U0056 [2], except for fixing a bug regarding motion compensation of blocks along the edge of a picture. The code for the original proposal has been available since 2021-01-22 on the VTM Github.

Subjective viewing has been carried out, where the anchor and test where found to have similar quality for all viewed sequences.

Finally, an HM version of the algorithm has been implemented, with similar performance. While the algorithm is the same for the HM implementation, the multiplications for the weights have been slightly modified:

- If the block error is less than 50, the base weight is multiplied by 1.2 and if the block error is larger than 100, the base weight is multiplied by 0.8.

- If the block error is less than 50, the sigma weight is multiplied by 1.3.

- If the frequency metric is 25 or larger, the base weight is multiplied by 1.2.

- If the frequency metric is less than 25, the sigma weight is multiplied by 1.3, otherwise by 0.8.

The code for the HM implementation is also available on Github.

The code on top of VTM-11 can be found on Github. Alternatively, a patch is attached to this contribution.

# Results

The proposal was implemented on top of VTM-12.0 [1]. The method is applied for RA coding only. The BD-rate result relative to VTM-12.0 is reported below.

|  | Over VTM-12.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.83% | -1.86% | -2.55% | 93% | 95% |
| Class A2 | -1.07% | -0.78% | -0.79% | 97% | 104% |
| Class B | -1.40% | -0.95% | -0.98% | 95% | 93% |
| Class C | -1.19% | -0.57% | -0.30% | 98% | 109% |
| Class E |  |  |  |  |  |
| Overall | -1.36% | -1.00% | -1.08% | 96% | 100% |
| Class D | -1.48% | -0.09% | -0.15% | 99% | 106% |
| Class F | 0.00% | 0.00% | 0.00% | 96% | 119% |

In addition, these are the results when compared to VTM-12.0 without temporal filtering:

|  | Over VTM-12.0 with MCTF = 0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -4.54% | -8.89% | -8.66% | 90% | 81% |
| Class A2 | -6.21% | -10.83% | -10.59% | 92% | 88% |
| Class B | -7.66% | -9.17% | -9.39% | 94% | 86% |
| Class C | -2.82% | -4.37% | -3.86% | 98% | 95% |
| Class E |  |  |  |  |  |
| Overall | -5.45% | -8.17% | -8.01% | 94% | 88% |
| Class D | -0.81% | -3.53% | -3.47% | 98% | 92% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 100% |

Finally, the results in HM-16.23 compared to the CTC:

|  | Random Access Main 10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A1 | -1.4% | -1.5% | -1.8% |
| Class A2 | -1.1% | -1.1% | -1.2% |
| Class B | -1.6% | -1.8% | -1.7% |
| Class C | -1.0% | -0.9% | -1.0% |
| Class D | -1.0% | -0.8% | -0.6% |
| Class E |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% |
| Overall | -1.0% | -1.0% | -1.0% |
|  | -1.0% | -1.0% | -1.0% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 100% |  |  |

We also tested the proposed software on the HDR common test conditions:

|  |  | Over VTM-12.0 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0,40% | -0,66% | -0,68% | -0,50% | -0,69% | -0,61% | -0,40% | -0,74% | 88% | 96% |
| Class H2 |  |  |  |  |  | -0,60% | -0,33% | -0,36% | 85% | 93% |
| Overall | -0,40% | -0,66% | -0,68% | -0,50% | -0,69% | -0,61% | -0,37% | -0,60% | 87% | 95% |

# Conclusion

In this document further investigation has been done on the temporal filter changes first introduced in JVET-U0056. We conclude that the subjective performance is similar, and present results for an HM implementation.

We propose adopting the changes into VTM and HM.

# References

[1] VTM-12.0 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.git

[2] P. Wennersten, C. Hollmann, J. Ström. [AHG10] GOP-based temporal filter improvements. JVET-U0056, January 2021

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).