| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0067 |
| --- | --- |

| Title: | AHG8: A constraint of max transform size for high bit depth and high bit rate |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo Masaru Ikeda Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-70-7598-7407 Kenji.Kondo@sony.com Masaru.Ikeda@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to restrict max transform size for high bit depth. This constraint helps to reduce memory size in hardware architecture. To investigate the impact of coding efficiency, experiments to restrict max transform size to 32 was carried out. In the experiment, an impact of coding efficiency is reported as below. The numbers in the bracket mean BD rates for Y/G, U/B and V/R.

- Low QP range

  - {0.01%, 0.02%, 0.01%} for wPSNR and PQ contents

  - {0.00%, 0.02%, 0.01%} for PSNR and PQ contents

  - {0.00%, 0.01%, 0.00%} for HLG contents

  - {0.00%, 0.00%, 0.00%} for SVT contents.

- Normal QP range

  - {0.35%	, 4.45%, 4.64%} overall in RA case

  - {0.13%, 6.08%, 6.43%} overall in AI case

Introduction

In the last meeting, JVET-U meeting, we proposed to restrict max transform size to 32 for high bit depth [1]. Further study was encouraged after re-defining the CTC for higher QP. As shown in [2], a common test condition for normal QP range was newly added. This contribution report those test results on new CTC.

In the [1], it was pointed out that larger transform is not so efficient for high bitrate. Generally, if we encode contents with high bit depth, lower QPs will be often used to keep high accuracy of input signal

On the other hand, if we focus on a hardware architecture, the max size of transform affects LSI cost. And it’s desirable for LSI cost to make the max transform size smaller.

For complexity analysis, A block diagram is shown in Figure 1. Three blocks in red color mean buffer memory. For memory 2 and 3, the bit depth relates to internal bit depth. If the internal bit depth is increased from 10-bit to 12-bit or 16-bit, the memory size increases.

Figure 1: a block diagram around transformation

In [1], complexity analyses were introduced for the memory 2 and 3, and additional costs to support version-2 were shown as following.

- For memory 2,

  - 16-bit input

    - 32 x 64 x 23 bit (+43%) Additional cost to use 64-point transform and HBD.

  - 12-bit input

    - 32 x 64 x 16 bit (+0%)

- For memory 3,

  - 16-bit input

    - 64 x 64 x 17 bit (+54%) Additional cost to use 64-point transform and HBD.

  - 12-bit input

    - 64 x 64 x 13 bit (+18%) Additional cost to use 64-point transform and HBD.

If the max transform size is restricted to 32, above costs can be saved.

In a specification of VVC version-1 [3], the max transform size is 64-point. And, the 64-point transformation process uses a technique so called zero-out. This zero-out can reduce a computational cost, but it forces high frequency coefficients to zero. However, it is not desirable for high bit depth and low QP because quantization step is small at low QP, and it often make high frequency coefficients non-zero.

Since above situations, for complexity reduction for high bit depth coding, this contribution proposes to restrict max transform size.

Experiment

To investigate an impact of coding efficiency, we tested to prohibit a 64-point transform by setting Log2MaxTbSize to 5 (max transform size: 32) in VTM software (VTM-12.0). In this test, a common test condition for high bit depth [2] was followed.

Experimental results

The test results are shown in Table 1 and Table 2 for low QP range and normal QP range respectively.

Low QP range

Table 1: Summary of results for low QP range

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.01% | 0.03% | 0.03% | 0.01% | 0.03% | 0.03% | 101% | 101% |
| PQ422 | 0.01% | 0.08% | 0.04% | 0.01% | 0.08% | 0.04% | 101% | 100% |
| Overall | 0.01% | 0.06% | 0.03% | 0.01% | 0.05% | 0.04% | 101% | 100% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| PQ422 | 0.01% | 0.01% | 0.00% | 0.00% | 0.00% | 0.00% | 103% | 99% |
| Overall | 0.00% | 0.01% | 0.00% | 0.00% | 0.00% | 0.00% | 102% | 100% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM12.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.00% | 0.01% | -0.01% | 0.00% | 0.01% | -0.01% | 100% | 100% |
| PQ422 | 0.01% | 0.01% | 0.01% | 0.00% | 0.00% | 0.01% | 102% | 100% |
| Overall | 0.01% | 0.01% | 0.00% | 0.00% | 0.01% | 0.00% | 101% | 100% |
| Overall PQ | 0.01% | 0.02% | 0.01% | 0.00% | 0.02% | 0.01% | 101% | 100% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | 0.00% | 0.00% | 103% | 100% |
| HLG422 | 0.01% | 0.00% | 0.00% | 102% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.01% | 0.00% | 0.00% | 101% | 100% |
| HLG422 | 0.00% | 0.02% | 0.00% | 102% | 99% |
| Overall | 0.00% | 0.01% | 0.00% | 102% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.01% | 0.01% | 0.01% | 104% | 100% |
| HLG422 | 0.00% | 0.00% | -0.02% | 102% | 100% |
| Overall | 0.00% | 0.01% | -0.01% | 103% | 100% |
| Overall HLG | 0.00% | 0.01% | 0.00% | 102% | 100% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| SVT12 | 0.00% | 0.00% | 0.00% | 97% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 106% | 100% |
| SVT12 | 0.00% | 0.00% | 0.00% | 105% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 105% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM12.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 104% | 100% |
| SVT12 | 0.00% | 0.00% | 0.00% | 103% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 104% | 100% |
| Overall RGB | 0.00% | 0.00% | 0.00% | 103% | 100% |

Normal QP range

Table 2: Summary of test results for normal QP range

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-12.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 8.16% | 0.63% | 0.26% | 5.87% | 5.90% | 0.35% | 4.45% | 4.64% | 102% | 102% |
| Class H2 |  |  |  |  |  | 0.47% | 3.51% | 3.89% | 96% | 103% |
| Overall | 8.16% | 0.63% | 0.26% | 5.87% | 5.90% | 0.35% | 4.45% | 4.64% | 99% | 102% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM-12.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 9.68% | 0.58% | 0.13% | 9.22% | 9.44% | 0.13% | 6.08% | 6.43% | 102% | 102% |
| Class H2 |  |  |  |  |  | 0.15% | 3.69% | 4.42% | 103% | 101% |
| Overall | 9.68% | 0.58% | 0.13% | 9.22% | 9.44% | 0.13% | 6.08% | 6.43% | 103% | 101% |

Consideration

If we see the result in Table 1, as we can see, the impact is negligible for low QP range.

BD rate were {0.01%, 0.02%, 0.01%} for wPSNR and overall PQ contents, {0.00%, 0.02%, 0.01%} for PSNR and overall PQ contents, and {0.00%, 0.01%, 0.00%} for HLG contents and {0.00%, 0.00%, 0.00%} for SVT contents. Where, the numbers in bracket mean BD rates for Y, U and V.

For SVT contents, the simulations results are the same as an anchor because Log2MaxTbSize is set to 5 based on CTC.

On the other hand, for normal QP range, the coding efficiency loss was confirmed.

For coding efficiency loss, {0.35%	, 4.45%, 4.64%} in RA case, {0.13%, 6.08%, 6.43%} in AI case are observed.

As shown in above results, the proposed constraint is beneficial to save implementation cost for low QP range or high bitrate. If profile or level for high bitrate was defined, this proposed constraint works well.

Conclusion

In this contribution, a constraint (max transform size 32) was proposed to reduce hardware cost for high bit depth and high bitrate application. In the experiment, it was confirmed that the impact of coding efficiency is negligible for low QP range while loss was confirmed at normal QP range.

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

References

- Kenji Kondo, Masaru Ikeda, “A constraint of max transform size for high bit depth”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, JVET-U0063, Jan. 2021

- Adrian Browne, Tomohiro Ikai, Dmytro Rusanovskyy, Mohammed Golam Sarwer, Xiaoyu Xiu, “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018, oint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, Jan 2021

- Benjamin Bross, Jianle Chen, Shan Liu, Ye-Kui Wang, “Versatile Video Coding Editorial Refinements on Draft 10”, JVET-T2001-v2, Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, Oct 2020