| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0071 |
| --- | --- |

| Title: | AHG9: Temporal sublayer information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Martin Pettersson Mitra Damghanian Jacob Ström | Email: | rickard.sjoberg@ericsson.com martin.m.pettersson@ericsson.com mitra.damghanian@ericsson.com jacob.strom@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution is a follow-up on JVET-U0045 and proposes that a new temporal sublayer information SEI message is included in the VSEI version 2 specification and to be supported by VVC version 2.

It is claimed that it is useful to have an SEI message for providing detailed temporal sublayer information. It is also claimed that the proposed SEI message is useful for providing spatial, SNR and multiview scalability support for VVC decoders that support Main 10 (4:2:0 or 4:4:4) but do not support any of the Multilayer Main 10 (4:2:0 or 4:4:4) profiles. Scalability in this proposal is supported by treating temporal sublayers as scalability layers.

The proposed SEI message contains information for temporal sublayers and is described by the proponents to contain the following main elements:

- Direct reference sublayer flags. These flags describe which sublayers are used as direct references for a sublayer. The flags are similar in spirit to the vps_direct_ref_layer_flag syntax element in VVC but describes sublayers rather than layers.

- Output sublayer sets (OSSs). Similar to output layer sets (OLSs), these describes output sets, but instead of basing them on layers as in VVC, this proposal bases them on sublayers. A syntax element for indicating a preferred OSS is also included in the proposal.

- Picture output width and height. To assist spatial scalability, signaling of an intended picture output width and height for each sublayer is included in the SEI message. The signaling is optional, and when used it also indicates that the bitstream is a spatial scalability bitstream and that the output picture width and height is kept constant within sublayers.

- View ID and aux ID. To support multiview scalability and auxiliary pictures, view ID and aux ID values may be signalled for each sublayer. This is similar to the Scalability dimension information SEI message, but in this proposal the information is signalled per sublayer.

The proponents point out that the SEI message can be viewed as an SEI message that contains VPS layer information, but instead of layer information the SEI message carries sublayer information.

The persistence of the SEI message is claimed to be similar to the design in the Scalability dimension information SEI message using a bitstreamInScope.

The proponents claim that the required level for scalable use-cases does not need to be higher compared to the level needed for the Multilayer Main 10 profile.

# Introduction

In JVET-U0045, an idea for enabling scalability for the VVC Main 10 profiles was introduced. The Main 10 profiles do not have multilayer support, but they do support temporal sublayers and reference picture resampling. Using spatial scalability as an example, JVET-U0045 suggests that pictures that would be in different scalability layers are instead put in different temporal sublayers. All pictures of one temporal sublayer would have the same spatial output resolution and inter prediction across temporal sublayers would be done using reference picture resampling (RPR). Pictures in temporal sublayer 0 may for instance be HD pictures and pictures in temporal sublayer 1 may be 4K pictures.

SNR scalability and multiview scalability would also be supported by using different fidelity or different views in different temporal sublayers. These two types of scalability using temporal sublayers would also work for the Main and Main 10 profile in HEVC, but since HEVC does not include RPR, spatial scalability using different picture sizes would not be possible.

In JVET-U0045, a Picture output suppression SEI message was proposed that could be used to indicate for each temporal sublayer whether that sublayer should be output or not. For the HD/4K spatial scalability example above, the JVET-U0045 SEI could be used to suppress output of the sublayer containing HD pictures to only output the highest sublayer containing the 4K pictures.

In this proposal, two output sublayer sets would be defined for the HD/4K example. One output sublayer set that includes temporal sublayer 0 containing HD pictures and one output sublayer set that includes both temporal sublayer 0 and 1 where sublayer 1 contains 4K pictures. In the latter sublayer set, only temporal sublayer 1 would be an output sublayer. In addition, the intended output sizes for the temporal sublayers would be signaled to indicate that sublayer 0 is an HD sublayer and sublayer 1 is a 4K sublayer.

# Proposal

We propose to add a temporal sublayer information SEI message to the VSEI version 2 specification and to support it in VVC version 2. The idea for the SEI message is to provide sublayer information in general as well as provide indicators that the bitstream contains sublayers intended for scalability.

The proposed SEI message can be viewed as an SEI message that contains VPS layer information, but instead of layer information the SEI message carries sublayer information.

The temporal sublayer information SEI message is proposed to contain the following main elements:

- Direct reference sublayer flags. These flags describe which sublayers are used as direct references for a sublayer. The flags are similar in spirit to the vps_direct_ref_layer_flag syntax element in VVC but describes sublayers rather than layers. The proposed direct reference sublayer flags also include signaling for each sublayer whether it references pictures in its own sublayer or not.

- Output sublayer sets (OSSs). Similar to output layer sets (OLSs), these describes output sets, but instead of basing them on layers as in VVC, we propose to base them on sublayers. In the VVC design, the output layer of an OLS is signaled and the full OLS is derived using direct reference layer information. For sublayers we propose a simpler design using one flag to indicate inclusion of each sublayer in the full OSS and one second flag to indicate whether the included sublayer is an output sublayer or not. A syntax element for indicating a preferred OSS is also included in the proposal.

- Picture width and height. To assist spatial scalability, we propose to signal an intended picture output width and height used for each sublayer. The signaling is optional, and when used it also indicates that the bitstream is a spatial scalability bitstream and that the picture width and height is kept constant within sublayers.

- View ID and aux ID.
To support multiview scalability and auxiliary pictures, we suggest signaling view ID and aux ID values for the sublayers. This is similar to the Scalability dimension information SEI message, but again we signal the information per sublayer instead of per layer.

## Proposed syntax and semantics

Proposed syntax and semantics for VSEI version 2 is shown below:

8.xx	Temporal sublayer information SEI message

8.xx.1	Temporal sublayer information SEI message syntax

| temporal_sublayer_information( payloadSize ) { | Descriptor |
| --- | --- |
| tsi_max_sublayers_minus1 | u(3) |
| tsi_sublayer_spatial_scalability_info_flag | u(1) |
| tsi_sublayer_multiview_info_flag | u(1) |
| tsi_sublayer_auxiliary_info_flag | u(1) |
| tsi_direct_ref_sublayer_info_present_flag | u(1) |
| tsi_oss_info_present_flag | u(1) |
| if(tsi_direct_ref_sublayer_info_present_flag) { |  |
| for( i = 0; i <= tsi_max_sublayers_minus1; i++ ) |  |
| for( j = 0; j <= i; j++ ) |  |
| tsi_direct_ref_sublayer_flag[ i ][ j ] | u(1) |
| } |  |
| if( tsi_oss_info_present_flag ) { |  |
| tsi_num_output_sublayer_sets_minus1 | u(4) |
| for( i = 0; i <= tsi_num_output_sublayer_sets_minus1; i ++ ) { |  |
| for( j = 0; j <= tsi_max_sublayers_minus1; j++ ) { |  |
| if( j > 0 ) |  |
| tsi_oss_sublayer_flag[ i ][ j ] | u(1) |
| if( tsi_oss_sublayer_flag[ i ][ j ] ) |  |
| tsi_oss_output_sublayer_flag[ i ][ j ] | u(1) |
| } |  |
| } |  |
| tsi_oss_idx_plus1 | ue(v) |
| } |  |
| if (tsi_sublayer_spatial_scalability_info_flag) { |  |
| for( i = 0; i <= tsi_max_sublayers_minus1; i++ ) { |  |
| tsi_pic_output_width_in_luma_samples[ i ] | ue(v) |
| tsi_pic_output_height_in_luma_samples[ i ] | ue(v) |
| } |  |
| } |  |
| if( tsi_sublayer_multiview_info_flag ¦ ¦ tsi_sublayer_auxiliary_info_flag ) { |  |
| if(tsi_sublayer_multiview_info_flag ) |  |
| tsi_view_id_len | u(4) |
| for( i = 0; i <= tsi_max_sublayers_minus1; i++ ) { |  |
| if( tsi_sublayer_multiview_info_flag ) |  |
| tsi_view_id_val[ i ] | u(v) |
| if( tsi_sublayer_auxiliary_info_flag ) |  |
| tsi_aux_id[ i ] | u(8) |
| } |  |
| } |  |
| } |  |

8.xx.2	Temporal sublayer information SEI message semantics

The temporal sublayer information SEI message provides information for each temporal sublayer in bitstreamInScope (defined below), such as 1) which temporal sublayers that may be referenced by other temporal sublayers 2) output sublayer sets (OSSs) where each OSS contains one or more temporal sublayers and provides an intended output alternative 3) intended picture output width and height for each temporal sublayer, when applicable, and 4) the intended view ID and aux ID of each temporal sublayer, when applicable.

The bitstreamInScope is a sequence of AUs that consists, in decoding order, of the AU containing the current temporal sublayer information SEI message, followed by zero or more AUs of the same layer as the temporal sublayer information SEI message, including all subsequent AUs up to but not including any subsequent AU that contains a temporal sublayer information SEI message of the same layer.

The presence of a temporal sublayer information SEI message indicates that the layer with the layer_id value of the temporal sublayer information SEI message is an independent layer.

tsi_max_sublayers_minus1 plus 1 indicates the maximum number of temporal sublayers that may be present in the bitstreamInScope. The value of tsi_max_sublayers_minus1 shall be in the range of 0 to 6, inclusive.

tsi_sublayer_spatial_scalability_info_flag equal to 1 indicates that the bitstreamInScope may be a spatial scalability bitstream using temporal sublayers. When equal to 1, the intended picture output width and height for each temporal sublayer is indicated. tsi_sublayer_spatial_scalability_info_flag equal to 0 indicates that bitstreamInScope is not a spatial scalability bitstream using temporal sublayers.

NOTE – Practical spatial scalability using temporal sublayers requires support of using different spatial picture sizes for different temporal sublayers. VVC supports this by reference picture resampling.

tsi_sublayer_multiview_info_flag equal to 1 indicates that bitstreamInScope may be a multiview bitstream using temporal sublayers. When equal to 1, information for associating temporal sublayers with view ID values is present in the SEI. tsi_sublayer_multiview_info_flag equal to 0 indicates that bitstreamInScope is not a multiview bitstream.

tsi_sublayer_auxiliary_info_flag equal to 1 indicates that there may be auxiliary information carried by one or more temporal sublayers in bitstreamInScope and the tsi_aux_id[ ] syntax elements are present. tsi_sublayer_auxiliary_info_flag equal to 0 indicates that there is no auxiliary information carried by one or more temporal sublayers in bitstreamInScope and the tsi_aux_id[ ] syntax elements are not present.

tsi_direct_ref_sublayer_info_present_flag equal to 1 specifies that tsi_direct_ref_sublayer_flag[ i ][ j ] syntax elements are present in the SEI message. tsi_direct_ref_sublayer_info_present_flag equal to 0 specifies that tsi_direct_ref_sublayer_flag[ i ][ j ] syntax elements are not present in the SEI message

tsi_oss_info_present_flag equal to 1 specifies that OSS information is present in the SEI. tss_oss_info_present_flag equal to 0 specifies that no OSS is specified by the SEI message.

tsi_direct_ref_sublayer_flag[ i ][ j ] equal to 0 indicates that there is no picture in bitstreamInScope with TemporalId equal to j that is used as a reference picture for any picture in bitstreamInScope with TemporalId equal to i. tsi_direct_ref_sublayer_flag[ i ][ j ] equal to 1 does not impose such a constraint.

When tsi_direct_ref_sublayer_info_present_flag is equal to 1, the variable dependencyFlag [ i ][ j ] is derived as follows:

for( i = 0; i  <=  tsi_max_sublayers_minus1; i++ ) {
	for( j = 0; j  <  i; j++ ) {
		dependencyFlag[ i ][ j ] = tsi_direct_ref_sublayer_flag[ i ][ j ]
		for( k = j+1; k < i; k++ )
			if( tsi_direct_ref_sublayer_flag[ i ][ k ]  &&  dependencyFlag[ k ][ j ] )
				dependencyFlag[ i ][ j ] = 1
	}
}

tsi_num_output_sublayer_sets_minus1 plus 1 specifies the total number of OSSs specified by the SEI message. When not present, tsi_num_output_sublayer_sets_minus1 is inferred to be equal to 0.

tsi_oss_sublayer_flag[ i ][ j ] equal to 1 specifies that temporal sublayer j is included in the i-th OSS. tsi_oss_sublayer_flag[ i ][ j ] equal to 0 specifies that temporal sublayer j is not included in the i-th OSS. When not present and tsi_oss_info_present_flag is equal to 1 and i is in the range of 0 to tsi_num_output_sublayer_sets_minus1, inclusive, tsi_oss_sublayer_flag[ i ][ 0 ] is inferred to be equal to 1.

When tsi_oss_info_present_flag is equal to 1, each temporal sublayer shall be included in at least one OSS specified by the SEI message. In other words: When tsi_oss_info_present_flag is equal to 1, then for any value j, in the range of 0 to tsi_max_sublayers_minus1, inclusive, there shall be a value i in the range of 0 to tsi_num_output_sublayer_sets_minus1, inclusive, for which tsi_oss_sublayer_flag[ i ][ j ] is equal to 1.

For each OSS specified by the SEI message, discarding all temporal sublayers that are not included in the OSS shall result in a conforming bitstream.

If tsi_direct_ref_sublayer_info_present_flag and tsi_oss_info_present_flag are both equal to 1 and the i-th OSS includes the j-th temporal sublayer, the i-th OSS shall also include all temporal sublayers k for which dependencyFlag[ j ][ k ] is equal to 1.

tsi_oss_output_sublayer_flag[ i ][ j ] equal to 1 specifies that temporal sublayer j is an output sublayer of the i-th OSS. tsi_oss_output_sublayer_flag[ i ][ j ] equal to 0 specifies that temporal sublayer j is not an output sublayer of the i-th OSS. When not present, the value of tsi_oss_output_sublayer_flag[ i ][ j ] is inferred to be equal to 0.

For each OSS specified by the SEI message, there shall be at least one temporal sublayer that is an output sublayer. In other words: When tsi_oss_info_present_flag is equal to 1, then for any value of i in the range of 0 to tsi_num_output_sublayer_sets_minus1, inclusive, there shall be a value j in the range of 0 to tsi_max_sublayers_minus1, inclusive, such that tsi_oss_output_sublayer_flag[ i ][ j ] is equal to 1.

tsi_oss_idx_plus1 minus 1 indicates a preferred OSS in bitstreamInScope. tsi_oss_idx_plus1 equal to 0 indicates that no OSS in bitstreamInScope is indicated to be a preferred OSS. When tsi_oss_idx_plus1 is larger than 0, a system may choose to output only the pictures that belong to a temporal sublayer j for which tsi_oss_output_sublayer_flag[ tsi_oss_idx_plus1 – 1 ][ j ] is equal to 1. The value of tsi_oss_idx_plus1  shall not be greater than tsi_num_output_sublayer_sets_minus1 + 1.

tsi_pic_output_width_in_luma_samples[ i ] and tsi_pic_output_height_in_luma_samples[ i ] specifies an intended picture output size for all pictures within the bitstreamInScope that belong the i-th temporal sublayer.

tsi_view_id_len specifies the length, in bits, of the tsi_view_id_val[ i ] syntax element.

tsi_view_id_val[ i ] specifies the view ID of the i-th temporal sublayer in bitstreamInScope. The length of the tsi_view_id_val[ i ] syntax element is tsi_view_id_len bits. When not present, the value of tsi_view_id_val[ i ] is inferred to be equal to 0.

tsi_aux_id equal to 0 indicates that the i-th temporal sublayer in bitstreamInScope does not contain auxiliary pictures. tsi_aux_id[ i ] greater than 0 indicates the type of auxiliary pictures in the i-th temporal sublayer in bitstreamInScope as specified in Table X.

Table X – Mapping of tsi_aux_id[ i ] to the type of auxiliary pictures

| tsi_aux_id[ i ] | Name | Type of auxiliary pictures |
| --- | --- | --- |
| 1 | AUX_ALPHA | Alpha plane |
| 2 | AUX_DEPTH | Depth picture |
| 3..127 |  | Reserved |
| 128..159 |  | Unspecified |
| 160..255 |  | Reserved |

NOTE – The interpretation of auxiliary pictures associated with tsi_aux_id in the range of 128 to 159, inclusive, is specified through means other than the tsi_aux_id value.

tsi_aux_id[ i ] shall be in the range of 0 to 2, inclusive, or 128 to 159, inclusive, for bitstreams conforming to this version of this Specification. Although the value of tsi_aux_id[ i ] shall be in the range of 0 to 2, inclusive, or 128 to 159, inclusive, in this version of this Specification, decoders shall allow values of tsi_aux_id[ i ] in the range of 0 to 255, inclusive.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).