| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0086-v3 |
| --- | --- |

| Title: | EE2-related: OBMC fixes and updates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal, |  |  |
| Author(s) or Contact(s): | A. Robert, F. Galpin, F. Le Leannec T. Poirier | Email: | antoine.robert@interdigital.com franck.galpin@interdigital.com fabrice.leleannec@interdigital.com tangi.poirier@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes some OBMC fixes and updates on top of JVET-U0100. This includes:

- Fixing the AMVR index for OBMC processing,

- Limiting the number of lines/columns processed at top and left borders in case of subblock mode,

- Considering as non subblock translational affine CUs,

- Using SIF and BCW in the OBMC process,

- Subtracting OBMC of the original signal in the AMVP, MMVD and CIIP modes.

It is reported that tool-on test achieves averagely 0.05% BD rate saving in RA while saving 1% encoding and 5% decoding time, and 0.06% BD rate saving in LDB while saving 1% encoding and 3% decoding time. Tool-off test is achieving averagely 0.06% BD rate saving in RA while encoding time remains the same and saving 1% decoding time, and 0.05% BD rate saving in LDB while keeping encoding time and saving 2% decoding time

# Introduction

In JVET-U0100 [1] reintroduces the OBMC process from JVET-L0101. When OBMC is applied, top and left boundary pixels of a CU are refined using neighboring block’s motion information with a weighted prediction.

Conditions of not applying OBMC are as follows:

- When OBMC is disabled at SPS level

- When current block has intra mode or IBC mode

- When current block applies LIC

- When current luma block area is smaller or equal to 32

A subblock-boundary OBMC is further proposed to apply the same blending to the top, left, bottom, and right subblock boundary pixels using neighboring subblock’s motion information. It is enabled for the subblock based coding tools:

- Affine AMVP modes;

- Affine merge modes and subblock-based temporal motion vector prediction (SbTMVP);

- Subblock-based bilateral matching.

# Proposal

This proposal aims at fixing and updating OBMC on several points while keeping or reducing the whole complexity.

## AMVR fix

In the current JVET-U0100 implementation of OBMC, several CU information are saved prior to the OBMC process, set to default values, and restored at the end to avoid impacting the OBMC results. It includes: the BCW index and the affine and GEO flags.

However, there exist a missed CU information, the AMVR index. Indeed, during the OBMC motion compensations using neighboring information, the current AMVR index can be wrongly used. It is then proposed to apply the same process of saving, setting to default, and restoring as for the other CU information.

## Subblock limitation

In the current JVET-U0100 implementation of OBMC, when a sub-block CU is processed by OBMC the same number of lines/columns are filtered along the top and left borders as for non subblock CUs while internal subblock boundaries will be also filtered.

It is proposed to limit the number of lines/columns filtered along the top and left borders to only 3 for subblock CUs.

## Translational affine CUs

When the number of lines/columns filtered along the top and left borders is reduced for sub-block CUs, and to save some complexity regarding the internal subblock boundaries filtering, it is proposed to consider the affine CUs that have a translational motion model as non subblock ones.

## SIF and BCW usage

Both BCW and AMVR indexes can modify the predicted samples in the OBMC motion compensations. It is proposed to use them in the OBMC process to better fit the real predicted signal.

When processing the top and left borders, the neighboring BCW index and SIF flag are used in addition to the motion vectors and reference frames. The neighboring SIF flag allows setting the AMVR index to half-pel when it is on, otherwise the default AMVR index is used. For the internal subblock boundaries, the BCW and AMVR indexes of the current CU are also used.

## OBMC subtraction

In older OBMC, the original signal was compensated with OBMC so that it is taken into account during the motion estimation of the AMVP mode. To do so, a predicted part of the current original CU using the neighboring motion information is subtracted from the original signal.

It is proposed to reintroduce such an OBMC subtraction in AMVP modes so that OBMC is considered in the motion estimation without applying it on each step. Furthermore, as OBMC is only applied in the full RDO search of merge modes and avoided from the fast merge pass, it is also possible to do the same subtraction of OBMC for merge modes not using subblock CUs during the fast merge pass. For that, and to overcome the problem of LIC flag predictor dependency, two versions of the original signal are stored: one without OBMC and one with the OBMC subtraction. Then in MMVD and CIIP fast merge pass, when the considered predictor uses LIC then the current signal is the original one, otherwise when it does not use LIC, the current CU is the original signal with the OBMC subtraction.

# Experimental results

## Tool-on tests

Tool-on tests have been performed using JVET-U0100 with only OBMC and tool-on configuration files as anchor compared to the proposed modifications of OBMC in the same configuration.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JVET-U0100-OBMConly |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.15% | -0.12% | 99% | 96% |
| Class A2 | 0.00% | -0.01% | -0.13% | 100% | 99% |
| Class B | -0.04% | -0.10% | -0.07% | 99% | 93% |
| Class C | -0.10% | -0.14% | -0.14% | 99% | 94% |
| Overall | -0.05% | -0.10% | -0.11% | 99% | 95% |
| Class D | -0.09% | -0.25% | -0.08% | 102% | 97% |
| Class F (optional) | 0.00% | -0.02% | -0.07% | 98% | 88% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JVET-U0100-OBMConly |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.03% | -0.53% | 0.01% | 100% | 100% |
| Class C | -0.07% | 0.06% | 0.04% | 96% | 93% |
| Class E | -0.09% | 0.01% | -0.30% | 100% | 100% |
| Overall | -0.06% | -0.20% | -0.06% | 99% | 97% |
| Class D | -0.19% | 0.95% | -0.07% | 100% | 102% |
| Class F (optional) | -0.16% | -0.20% | -1.18% | 95% | 95% |

## Tool-off tests

Tool-off tests of the proposed methods have been performed on top of the JVET-U0100 as described in the EE2 document JVET-U2024 [2].

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JVET-U0100 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.02% | -0.07% | 101% | 99% |
| Class A2 | -0.09% | -0.19% | -0.21% | 101% | 99% |
| Class B | -0.06% | -0.25% | -0.33% | 99% | 99% |
| Class C | -0.09% | -0.05% | -0.14% | 100% | 98% |
| Overall | -0.06% | -0.13% | -0.20% | 100% | 99% |
| Class D | -0.13% | -0.10% | -0.38% | 101% | 100% |
| Class F (optional) | -0.20% | -0.93% | -0.99% | 101% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JVET-U0100 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.02% | -0.19% | -0.42% | 100% | 97% |
| Class C | -0.09% | -0.12% | 0.02% | 100% | 100% |
| Class E | -0.03% | 0.50% | 0.22% | 100% | 99% |
| Overall | -0.05% | 0.01% | -0.11% | 100% | 98% |
| Class D | -0.17% | -0.04% | -0.27% | 100% | 98% |
| Class F (optional) | -0.11% | -0.30% | -0.75% | 100% | 98% |

# Conclusion

In this proposal, some parts of the JVET-U0100 OBMC are fixed, and some simple updates are also proposed. It is shown that this can achieve some BD rate saving as well as some encoding and decoding time saving for both tool-on and tool-off tests. It is proposed to incorporate the OBMC fixes into the EE2 software and to investigate the updates in the scope of EE2.

# References

- “Compression efficiency methods beyond VVC”, JVET-U0100, Y-J. Chang, C-C. Chen, J. Chen, J. Dong, H. E. Egilmez, N. Hu, H. Huang, M. Karczewicz, J. Li, B. Ray, K. Reuze, V. Seregin, N. Shlyakhov, L. Pham Van, H. Wang, Y. Zhang, Z. Zhang.

- “Exploration Experiment on Enhanced Compression beyond VVC capability”, JVET-U2024, V. Seregin, J. Chen, S. Esenlik, F. Le Leannec, L. Li, M. Winken, J. Strom, X. Xiu, K. Zhang.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).