| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0091-v2 |
| --- | --- |

| Title: | AHG9/AHG11: SEI messages for carriage of neural network information for post-filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi, Zeqiang Li, Wei Wang, Wei Jiang, Xiaozhong Xu, Stephan Wenger, Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes an updated SEI message design for carriage of a neural network (NN) topology and parameters that are utilized for post filtering with neural network models. In the update design, two SEI messages are proposed; the first SEI specifies the internal or external carriage of a NN topology and its parameters, and the second one specifies the organization of multiple NN models, which are carried by the first SEI messages, for the post-processing. The second SEI message contains the information on which model is applied to each picture or block among multiple candidate models. The proposed high-level syntax structure is applicable to SEI messages for post-filtering as well as other NN-based EE proposals.

# Introduction

This contribution proposes the updated high-level syntax design of JVET-U0091[1] that contains one or more neural network topologies and their trained parameters, which defines network inference processes designed for post/in-loop filtering, super-resolution processing, virtual reference generation or any other NN-based processing for VVC or HEVC. Although the information on network topology and parameters can be explicitly specified with fixed parameters in SEI messages, parameter sets or in the specification documents, having a more flexible mechanism of network representation with trained parameters of users, which can be used for post-processing or side-operation to improve the quality of output video, should be useful for various applications, without any harm on the video coding core design.

The basic idea is to have two mechanisms in terms of signaling neural network model information; The first one is signaling explicitly one or more topology information as well as the corresponding parameters trained with specific syntax elements defined in VSEI, the other one is providing an external linkage information which indicates where the corresponding topology information and network parameters are present.

For signaling network topologies and parameters, referencing the existing formats that have been developed for representation of network. One good example could be Neural Network Exchange Format (NNEF) that is a generalized neural network exchanging format developed by Khronos [2]. Other possible examples are Open Neural Network Exchange (ONNX)[3] and MPEG NNR[4], which is the new format for coded representation of neural network.

Ideally, any neural network models can be exported to NNEF and other formats, and network accelerator and libraries can consume data in the formats without compatibility issue with any network framework. As a simplest and practical method, it would be good to directly reference outside files or bitstream with URI information. However, it is also desired to have a lightweight syntax design to represent video coding specific network for VVC or HEVC-extension with novel neural-network based video coding tools, because a generic representation of network model may be bulky to be used for the compressed video format. Since mostly used network models for video compression are based on convolutional neural network (CNN), having a compact representation of CNN in SEI message is expected to be helpful to reduce the total bitrate as well as to enable easy access to the network model data.

On top of JVET-U0091, an additional SEI message is proposed to organize multiple CNN models for the post-processing. Various post-processing methods utilize multiple neural network models. For example, JVET-V0090 employs two NN models; video-based processing and image-based processing. In case of multiple NNs are employed for the NN processing, the new SEI message provides the information on how and where the NN models are applied for the post-processing. In the proposed syntax, a unique identifier (NN ID) is assigned to each model for identification. In picture/block level, a flag indicates whether the specified NN-processing is applied or not. When the flag of a particular picture or block is equal to 1, the best model of the picture/block is indicated by a NN ID among candidates.

# Things to be specified

Generally, a neural network model can be represented by a computational graph, which is a directed graph with multiple nodes. The nodes consist of operation nodes and data, such as tensor. According to applications, various network topologies are designed and used. However, for post-/in-loop filtering for video processing, CNN-based simple models are popularly adopted. In the case, a simple multi-layered feedforward network like CNN can be represented by a linear graph that starts from the input data, where each operation node in a layer produces an intermediately processed data. Finally, the output data is generated through multiple layers.

Figure 1. An example of computational graph for convolutional neural network.

Fig.1 illustrate an example of linear computational graph for CNN. When the input tensor data are fed into an operation node, the operation node processes the input data with and the pretrained constants and/or variables and outputs the intermediate tensor data. When the operation nodes are executed, actual data is consumed by operation node. Usually for CNN, the weighted summation of input data with trained constants and/or updated variables is the output of each operation node.

In terms of specification of the network graph, once a specific operation node is specified as a single step, the same operation node can be used iteratively. We believe such simple network topologies can be described by some well-designed syntax elements in SEI message. In case that more complicated model designs are required, external formats like NNEF or ONNX can be used. 
For delivering network parameters, usually the bit size of trained network parameters is too huge to be contained in an SEI message. To reduce the data size, MPEG-NNR format can be used for compressing parameters and can be partitioned into multiple data chunk. Each chunk of compressed parameters can be contained in an SEI message or a separate data file, which is potentially delivered in the same bitstream or stored in a remote server. When decoded, all concatenated chunk data in SEI messages for representation of neural network are spliced and consumed by neural network library or decoders.

Figure 2. An example of block-level selection of NN-inference outputs.

In some cases, a bitstream may need to carry multiple NN topologies and their parameters, where the bitstream has multiple applications that utilizes NN information, or an application utilizes multiple NNs. JVET-U0087 combines three different NN models for the NN inference process, while JVET-V0090 uses a different NN model for each NN processing stage. To deal with various cases using multiple NN models, in this proposal, it is allowed that a bitstream contains multiple (neural_network_topology_parameter_info ) SEI messages that carry NN topologies and their parameters. A set of NN topologies and parameters can be contained in band, while other NN topologies and parameters are transmitted out of band. An NN identifiers (NN ID) can be used for indication of the associated NNs for each application.

Beyond that, most post-/inloop-filtering methods utilize multiple NN models, so that the best NN model can be selectively applied to each picture or block. Fig. 2 illustrates an example that the picture is hypothetically divided into multiple blocks, and each block is processed by a different NN inference process. To provide such a picture/block-level adaptation information of multiple NNs, another SEI message (neural_network_inference_process_info) is proposed. The proposed neural_network_inference_process_info SEI message contains the information on where a specific NN inference process is applied. The NN ID is assigned to each picture or block to identify the corresponding NN model, where a unique value of NN ID is signaled in each neural_network_topology_parameter_info SEI message.

# Proposed SEI messages for carriage of NN information

## Neural network topology and parameter SEI message

### Neural network topology and parameter SEI message syntax

| neural_network_topology_parameter_info ( payloadSize ) { | Descriptor |
| --- | --- |
| nn_id | u(8) |
| nn_partition_flag | u(1) |
| nn_topology_info_external_present_flag | u(1) |
| nn_parameter_info_external_present_flag | u(1) |
| nn_input_pic_format_present_flag | u(1) |
| nn_output_pic_format_present_flag | u(1) |
| nn_postfilter_type_idc | u(3) |
| num_nn_input_ref_pic | ue(v) |
| if( nn_partition_flag ) { |  |
| num_partitioned_nn_sei_messages | ue(v) |
| nn_sei_message_idx | ue(v) |
| } |  |
| if( network_topology_info_external_present_flag ) { |  |
| external_nn_topology_info_format_idc | u(4) |
| num_bytes_external_nn_topology_uri_info | ue(v) |
| external_nn_topology_uri_info | u(v) |
| } |  |
| else { |  |
| network_topology_info( input ) |  |
| } |  |
| if( network_parameter_info_external_present_flag ) { |  |
| external_network_parameter_info_format_idc | u(4) |
| num_bytes_external_network_parameter_uri_info | ue(v) |
| external_network_parameter_uri_info | u(v) |
| } |  |
| else { |  |
| network_parameter_info( input ) |  |
| } |  |
| if( network _input_pic_format_present_flag ) { |  |
| nn_input_chroma_format_idc | u(2) |
| nn_input_bitdepth_minus8 | ue(v) |
| nn_input_pic_width | ue(v) |
| nn_input_pic_height | ue(v) |
| nn_patch_size_present_flag | u(1) |
| if( nn_patch_size_present_flag ) { |  |
| nn_input_patch_width | ue(v) |
| nn_input_patch_height | ue(v) |
| nn_boundary_padding_idc | u(2) |
| } |  |
| } |  |
| if( num_network_input_ref_pic > 0 ) { |  |
| num_fwd_ref_pics_as_input | u(4) |
| if ( NumFwdRefPics > 0 ) { |  |
| nearest_fwd_ref_pics_used_flag | u(1) |
| for( i = 0; i < NumFwdRefPics && !nearest_fwd_ref_pics_used_flag; i++ ) { |  |
| poc_dist_fwd_ref_pic[ i ] | ue(v) |
| } |  |
| } |  |
| if ( NumBwdRefPics > 0 ) { |  |
| nearest_bwd_ref_pics_used_flag | u(1) |
| for( i = 0; i < NumBwdRefPics && !nearest_bwd_ref_pics_used_flag; i++ ) { |  |
| poc_dist_bwd_ref_pic[ i ] | u(6) |
| } |  |
| } |  |
| } |  |
| } |  |

| network_topology_info ( input ) { | Descriptor |
| --- | --- |
| nn_topology_storage_format_idc | u(4) |
| nn_topology_compression_format_idc | u(4) |
| num_bytes_topology_data | ue(v) |
| if( nn_topology_format_idc > 0 ) { |  |
| for( i = 0; i < num_bytes_topology_data; i++ ) { |  |
| nn_topology_data_byte[ i ] | b(8) |
| } |  |
| } |  |
| else { |  |
| num_variables | ue(v) |
| for( i = 0; i < num_variables; i++ ) |  |
| define_variable( i ) |  |
| num_operation_nodes_types | ue(v) |
| for( i = 0; i < num_operation_nodes_types; i++ ) |  |
| define_operation_node( i ) |  |
| num_operation_node_executions | ue(v) |
| for( i = 0; i < num_operation_node_executions; i++ ) |  |
| operation_node_excution( i ) |  |
| } |  |
| } |  |

| define_variable ( i ) { | Descriptor |
| --- | --- |
| nn_variable_class_idc[ i ] | u(4) |
| nn_variable_type_idc [ i ] | u(4) |
| nn_variable_dimensions[ i ] | ue(v) |
| for( j = 0; j < num_bytes_topology_data; j++ ) { |  |
| nn_variable_dimension_size[ i ][ j ] | ue(v) |
| } |  |
| … |  |
| } |  |

| define_operation_node ( i ) { | Descriptor |
| --- | --- |
| nn_operatoin_class_idc[ i ] | u(8) |
| nn_operation_function_idc[ i ] | u(8) |
| num_input_variables[ i ] | ue(v) |
| num_output_variables[ i ] | ue(v) |
| … |  |
| } |  |

| operation_node_execution ( i ) { | Descriptor |
| --- | --- |
| nn_op_node_idx[ i ] | u(v) |
| for( j = 0; j < num_input_variables[ nn_op_node_idx[ i ] ]; j++ ) { |  |
| nn_input_variable_idx[ i ][ j ] | u(v) |
| } |  |
| for( j = 0; j < num_output_variables[ nn_op_node_idx[ i ] ]; j++ ) { |  |
| nn_output_variable_idx[ i ][ j ] | u(v) |
| } |  |
| … |  |
| } |  |

| network_parameter_info ( input ) { | Descriptor |
| --- | --- |
| nn_parameter_type_idc | u(4) |
| nn_parameter_storage_format_idc | u(4) |
| nn_parameter_compression_format_idc | u(4) |
| num_bytes_parameter_data | ue(v) |
| for( i = 0; i < num_bytes_topology_data; i++ ) { |  |
| nn_parameter_data_byte[ i ] | b(8) |
| } |  |
| … |  |
| } |  |

### Neural-network based post-filtering SEI message semantics

nn_id provides an identifier for the neural network.

nn_partition_flag equal to 0 specifies that all data to represent the neural network topology and the trained parameters are included in an SEI message. nn_partition_flag equal to 1 specifies that the data to represent the network topology and the trained parameters are partitioned into multiple SEI messages.

nn_topology_info_external_present_flag equal to 0 specifies that the data of neural network topology representation is contrained in the SEI message. nn_topology_info_external_present_flag equal to 1 specifies that the data of neural network topology representation may be externally present and the SEI message contains the external linkage information only.

nn_parameter_info_external_present_flag equal to 0 specifies that the data of neural network parameters is contrained in the SEI message. nn_parameter_info_external_present_flag equal to 1 specifies that the data of neural network parameters may be externally present and the SEI message contains the external linkage information only.

nn_input_pic_format_present_flag equal to 0 specifies that the syntax elements indicating the input picture format are not present in the SEI message and the input picture format of the neural network inference process is identical to the output picture format of decoder. nn_input_pic_format_present_flag equal to 1 specifies that the syntax elements indicating the input picture format are present in the SEI message.

nn_output_pic_format_present_flag equal to 0 specifies that the syntax elements indicating the output picture format are not present in the SEI message and the output picture format of the neural network inference process is identical to the output picture format of decoder. nn_output_pic_format_present_flag equal to 1 specifies that the syntax elements indicating the output picture format are present in the SEI message.

nn_postfilter_type_idc specifies the post-filter type of the neural network represented by the SEI message, as specified by the below table:

Table 1 –  Neural network post filter type.

| nn_postfilter_type_idc | Post filtering type |
| --- | --- |
| 0 | Visual quality improvement with single input picture |
| 1 | Visual quality improvement with multiple input pictures |
| 2 | Super resolution with single input picture |
| 3 | Super resolution with multiple input pictures |
| 4..15 | Reserved |

num_nn_input_ref_pic specfies the number of input reference pictures. num_nn_input_ref_pic equal to 0 specifies that the current output picture of decoder is the only input data of the neural network. num_nn_input_ref_pic greater than 0 specifies that the number of reference pictures, which are used as input data of the neural network, is num_nn_input_ref_pic – 1.

num_partitioned_nn_sei_messages specifies the number of neural network based post-filtering SEI messages to represent the entire neural network topology with the corresponding parameters. When not present, the value of num_partitioned_nn_sei_messages is inferred to be equal to 1.

nn_sei_message_idx specifies the index of the partial neural network data carried in SEI message. When not present, the value of nn_sei_message_idx is inferred to be equal to 0.

external_nn_topology_info_format_idc specifies the external storage format of the neural network topology representation, as specified by the below table:

Table 2 –  External neural network topology storage format identifier.

| external_nn_topology_info_format_idc | Storage format |
| --- | --- |
| 0 | Unrecognized storage format |
| 1 | NNEF [2] |
| 2 | ONNX [3] |
| 3..15 | Reserved |

num_bytes_external_nn_topology_uri_info specifies the number of bytes of the syntax element external_nn_topology_uri_info.

external_nn_topology_uri_info specifies the URI information of the external neural network topology information, as defined by RFC 3986. The length of the syntax element is Ceil( Log2( num_bytes_external_nn_topology_uri_info ) ) bytes.

external_network_parameter_info_format_idc specifies the external storage format of the neural network parameters, as specified by the below table:

Table 3 –  External neural network parameter storage format identifier.

| external_network_parameter_info_format_idc | Storage format |
| --- | --- |
| 0 | Unrecognized storage format |
| 1 | NNEF [2] |
| 2 | ONNX [3] |
| 3 | MPEG-NNR [4] |
| 4..15 | Reserved |

num_bytes_external_network_parameter_uri_info specifies the number of bytes of the syntax element external_network_parameter_uri_info.

external_network_parameter _topology_uri_info specifies the URI information of the external neural network parameters, as defined by RFC 3986. The length of the syntax element is Ceil( Log2( num_bytes_external_network_parameteruri_info ) ) bytes.

nn_input_chroma_format_idc specifies the chroma sampling relative to the luma sampling.

Table 4 –  chroma format identifier

| nn_input_chroma_format_idc | Chroma format |
| --- | --- |
| 0 | Monochrome |
| 1 | 4:2:0 |
| 2 | 4:2:2 |
| 3 | 4:4:4 |

nn_input_bitdepth_minus8 plus 8 specifies the bit depth of the luma and chroma samples in the input picture of neural network.

nn_input_pic_width specifies the width of the input picture.

nn_input_pic_height specifies the height of the input picture.

nn_patch_size_present_flag equal to 0 specifies that the patch size is equal to the input picture size. nn_patch_size_present_flag equal to 1 specifies that the patch size is explicitly signaled.

nn_input_patch_width specifies the width of patch for neural network inference process.

nn_input_patch_height specifies the height of patch for neural network inference process.

nn_boundary_padding_idc specifies the padding method appied to the boundary of patch, when the patch size is different from the input pictur size.

Table 5 –  boundary padding identifier

| nn_boundary_padding_idc | Padding type |
| --- | --- |
| 0 | Nearest neighbor copy |
| 1..3 | Reserved |

num_fwd_ref_pics_as_input specifies the number of forward reference pictures that are used as input data of neural network.

if ( num_nn_input_ref_pic > 0 ) 
	NumFwdRefPics = num_fwd_ref_pics_as_input
else
	NumFwdRefPics = 0

nearest_fwd_ref_pics_used_flag specifies the nearest forward reference pictures, which have the smallest picture order count distance from the current picture, are used as input data of the neural network.

poc_dist_fwd_ref_pic[ i ] specifies the picture order count value of the i-th forward refere picture that is used as input data of the nueral network. The picture order count value of the i-th forward refere picture is equal to the picture order count value of the current picture minus poc_dist_fwd_ref_pic[ i ].

num_bwd_ref_pics_as_input specifies the number of backward reference pictures that are used as input data of neural network.

if ( num_nn_input_ref_pic > 0 ) 
	NumBwdRefPics = num_bwd_ref_pics_as_input
else
	NumBwdRefPics = 0

nearest_bwd_ref_pics_used_flag specifies the nearest backward reference pictures, which have the smallest picture order count distance from the current picture, are used as input data of the neural network.

poc_dist_bwd_ref_pic[ i ] specifies the picture order count value of the i-th backward refere picture that is used as input data of the nueral network. The picture order count value of the i-th backward refere picture is equal to the picture order count value of the current picture plus poc_dist_bwd_ref_pic[ i ].

nn_topology_storage_format_idc specifies the storage format of the neural network topology representation, as specified by the below table:

Table 6 –  Neural network topology storage format identifiers.

| nn_parameter_storage_format | Storage format |
| --- | --- |
| 0 | Unrecognized storage format |
| 1 | NNEF [2] |
| 2 | ONNX [3] |
| 3..15 | Reserved |

nn_topology_compression_format_idc specifies the compression format of the neural network topology, as specified by the below table:

Table 7 –  Neural network topology compression format identifier.

| nn_topology_compression_format_idc | Compression format |
| --- | --- |
| 0 | Uncompressed |
| 1 | ZLIB compressed data format (RFC 1950 [5]) |
| 2..15 | Reserved |

num_bytes_topology_data specifies the number of bytes of the neural network topology payload that is contained in this SEI message.

nn_topology_data_byte[ i ] specifies the i-th byte of neural network topology payload.

num_variables specfies the number of variables that may be used for the execution of operation nodes in the neural network specified by this SEI message.

num_operation_nodes_types specifies the number of operantion node types that may be used for the execution of operatoin nodes in the neural network specified by this SEI message.

num_operation_node_executions specifies the number of operation node executions with the input variables by the neural network specified by this SEI message.

nn_variable_class_idc[ i ] specifies the variable class of the i-th variable in the neural network, as specified by the below table:

Table 8 –   Neural network variable classes

| nn_variable_class_idc | Variable type |
| --- | --- |
| 0 | Unspecified |
| 1 | Input |
| 2 | Output |
| 3 | Intermediate variable |
| 4 | Kernel |
| 5 | Bias |
| 6 | Constant |
| 7..15 | Reserved |

When nn_variable_class_idc is equal to 1, the variable is the input data of the neural network. When nn_variable_class_idc is equal to 2, the variable is the output data of the neural network. When nn_variable_class_idc is equal to 3, the variable is the intermediate data between operation nodes. When nn_variable_class_idc is equal to 4, the variable is the pretrained or predefined constant data.

nn_variable_type_idc[ i ] specifies the variable type of the i-th variable in the neural network, as specified by the below table:

Table 9 –  Neural network variable types

| nn_parameter_type_idc | Parameter type |
| --- | --- |
| 0 | Integer |
| 1 | Float |
| 2..15 | Reserved |

nn_variable_dimensions[ i ] specifies the number of dimensions of the i-th variable.

nn_variable_dimension_size[ i ][ j ] specifies the size of the j-th dimension of the i-th variable.

Note – For example, when the i-th variable is the input data whose the number of color component, the width and the height are 3, 1920 and 1080 respectively,  nn_variable_class_idc[ i ] is equal to 1, nn_variable_dimensions[ i ] is equal to 3, nn_variable_dimension_size[ i ][ 0 ] is equal to 3, nn_variable_dimension_size[ i ][ 1 ] is equal to 1920 and nn_variable_dimension_size[ i ][ 2 ] is equal to 1080, respectively.

nn_operation_class_idc[ i ] specifies the class of the i-th operation node, as specified by th below table:

Table 10 –  Neural network operation function

| nn_operation_class_idc | Operation class |
| --- | --- |
| 1 | Convolution/deconvolution |
| 2 | Filtering |
| 3 | Up and Down sampling |
| 4 | Pooling |
| 5 | Element-wise operation |
| 6 | Reshaping |
| 7 | Activation |
| 8 | Quantization |
| 9..255 | Reserved |

nn_operation_function_idc[ i ] specifies the fuction of the i-th operation node, as specified by th below table:

Below is the example table of nn_operation_class_idc equal to 7 (activation fuction).

Table 11 –  Neural network operation function

| nn_operation_function_idc (nn_operation_class_idc equal to 7) | Function |
| --- | --- |
| 1 | Sigmoid |
| 2 | Relu |
| 3 | Leaky_relu |
| 4 | Tanh |
| 5 | Softmax |
| 6..255 | Reserved |

num_input_variables[ i ] specifies the number of input varibles of the i-th operation node.

num_output_variables[ i ] specifies the number of output varibles of the i-th operation node.

nn_op_node_idx[ i ] specifies the index of the operation node for the i-th operation node execution. The nn_op_node_idx[ i ]-th operation node is used for this execution.

nn_input_variable_idx[ i ][ j ] specifies the variable index of the j-th input variable of the i-th operation node execution.

nn_output_variable_idx[ i ][ j ] specifies the variable index of the j-th output variable of the i-th operation node execution.

nn_parameter_type_idc specifies the data payload type of the neural network parameters, as specified by the below table:

Table 12 –  Neural network parameter payload types

| nn_parameter_type_idc | Parameter type |
| --- | --- |
| 0 | Integer |
| 1 | Float |
| 2..15 | Reserved |

nn_parameter_storage_format_idc specifies the storage format of the neural network parameters, as specified by the below table:

Table 13 –  Neural network parameter storage format identifier.

| nn_parameter_storage_format_idc | Storage format |
| --- | --- |
| 0 | Unrecognized storage format |
| 1 | NNEF [2] |
| 2 | ONNX [3] |
| 3 | MPEG-NNR [4] |
| 4..15 | Reserved |

nn_parameter_compression_format_idcs specifies the compression format of the neural network parameters, as specified by the below table:

Table 14 –  Neural network topology compression format identifier.

| topology_compression_format_idc | Compression format |
| --- | --- |
| 0 | Uncompressed |
| 1 | ZLIB compressed data format (RFC 1950 [5]) |
| 2..15 | Reserved |

num_bytes_parameter_data specifies the number of bytes of the neural network parameter payload that is contained in this SEI message.

nn_parameter_data_byte[ i ] specifies the i-th byte of neural network parameter payload.

## Neural network inference process SEI message

### Neural network inference process SEI message syntax

| neural_network_inference_process_info ( payloadSize ) { | Descriptor |
| --- | --- |
| num_nn_models_minius1 | ue(v) |
| for( i = 0; i <= num_nn_models_minius1; i++ ) { |  |
| nn_used_id[ i ] |  |
| } |  |
| nn_pic_width_in_luma_samples | ue(v) |
| nn_pic_height_in_luma_samples | ue(v) |
| nn_num_block_columns_minus1 | ue(v) |
| nn_num_block_ rows_minus1 | ue(v) |
| for( i = 0; i <= nn_num_block_columns_minus1; i++ ) |  |
| nn_block_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= nn_num_block_rows_minus1; i++ ) |  |
| nn_block_row_height_minus1[ i ] | ue(v) |
| for( i = 0; i < (nn_num_block_columns_minus1+1)*(nn_num_block_rows_minus1+1); i++ ) |  |
| nn_block_inference_enabled_flag[ i ] | u(1) |
| if( nn_inference_enabled_flag && num_nn_models_minius1 == 0 ) |  |
| nn_block_model_index[ i ] | u(v) |
| } |  |
| } |  |

### Neural network inference process SEI message semantics

num_nn_models_minus1 plus 1 specifies the number of neural network models that are used for the picture.

nn_used_id[i] indicates the identifier of the i-th neural network model that is used for the picture.

nn_pic_width_in_luma_samples specifies the width of each decoded picture associated with this SEI message in units of luma samples.

nn_pic_height_in_luma_samples specifies the height of each decoded picture associated with this SEI message in units of luma samples.

nn_num_block_rows_minus1 plus 1 specifies the number of block row height. The value of nn_num_block_rows_minus1 shall be in the range of 0 to nn_pic_height_in_luma_samples − 1, inclusive.

nn_num_block_columns_minus1 plus 1 specifies the number of block column widths. The value of nn_num_block_columns_minus1 shall be in the range of 0 to nn_pic_width_in_luma_samples − 1, inclusive.

nn_block_column_width_minus1[ i ] plus 1 specifies the width of the i-th block in units of luma samples. The value of nn_block_column_width_minus1[ i ] shall be in the range of 0 to nn_pic_width_in_luma_samples − 1, inclusive.

nn_block_row_height_minus1[ i ] plus 1 specifies the height of the i-th block in units of luma samples. The value of nn_block_row_height_minus1[ i ] shall be in the range of 0 to nn_pic_height_in_luma_samples − 1, inclusive.

nn_block_inference_enabled_flag[ i ] equal to 0 indicates neural network inference process is not applied to the i-th block. nn_block_inference_enabled_flag[ i ] equal to 1 indicates the neural network inference process associated with nn_block_model_index[ i ] is applied to the i-th block.

nn_block_model_index[ i ] indicates that the neural network model associated with nn_used_id[ nn_block_model_index[ i ] ] is applied to the i-th block. The length of the syntax element is Ceil( Log2( num_nn_models_minius1 + 1 ) ) bits.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] Byeongdoo Choi, Zeqiang Li, Wei Wang, Wei Jiang, Xiaozhong Xu, Stephan Wenger, Shan Liu, "AHG9/AHG11: SEI message for carriage of neural network information for post filtering", JVET-U0091, Jan 2021.

[2] Neural Network Exchange Format, The Khronos NNEF Working Group, Version 1.0.3, 2020-06-12 	
      (https://www.khronos.org/registry/NNEF/specs/1.0/nnef-1.0.3.pdf)

[3] Open Neural Network Exchange, VERSION 6, 2019-09-19     
      (https://github.com/onnx/onnx/blob/master/onnx/onnx.proto)

[4] “Text of ISO/IEC DIS 15938-17 Compression of Neural Networks for Multimedia Content Description and Analysis”, ISO/IEC JTC 1/SC29/WG11 Documents, MDS19764, Online, Oct 2020.

[5] IETF RFC 1950, ZLIB Compressed Data Format Specification version 3.3, 1996.