| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0093-v3 |
| --- | --- |

| Title: | AHG9: Film grain estimation and film grain synthesis for VVC Film grain characteristics SEI message, film grain estimation and film grain synthesis modules |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miloš Radosavljević, Edouard François (InterDigital), Wassim Hamidouche, Thomas Amestoy, Guillaume Gautier (INSA) | Tel: Email: | {milos.radosavljevic,edouard.francois}@interdigital.com {wassim.hamidouche,thomas.amestoy,guillaume.gautier}@insa-rennes.fr |
| Source: | InterDigital, INSA |  |  |

_____________________________

# Abstract

This contribution proposes a complete VTM software implementation of the film grain feature, including an estimation module at encoder side and a synthesis module at decoder side. The implementation is based on the SMPTE RDD 5 Film Grain Technology specification, with some adaptations.

The software implementation illustrates the use of the film grain for VVC. Specifically, the software illustrates:

- Film grain characteristics SEI message for VVC specified in H.SEI | ISO/IEC 23002-7;

- Film grain synthesis module based on frequency filtering model specified in SMPTE RDD 5;

- Film grain analysis (parameter estimation) module used to estimate film grain parameters according to SMPTE RDD 5 model.

It is reported that the visual quality benefits from the synthesized film grain and the algorithmic simplicity for both estimation and synthesis part makes this approach good candidate for future use in VVC and VTM reference software.

# Introduction

Film grain is a desirable feature in video production, creating a natural appearance and contributing to the expression of creative intent. Moreover, it has been demonstrated that film grain can be used as a tool to mask compression artifacts. Pleasant film grain can be used to slightly re-noise a frame, and thus mask certain compression artifacts, leading to increased subjective video quality.

Information on film grain can be communicated through the film grain characteristics (FGC) SEI message specified by Versatile Supplemental Enhancement Information (VSEI) [1]. Film grain parameters estimated at encoder side can be signaled to the decoder by the SEI, and the modeled film grain can be synthesized on the decoder side from the SEI metadata. Synthesis of simulated film grain on the decoded images for the display process is optional and does not affect the decoding process specified in the draft VVC specification.

Due to the benefits of film grain, this contribution proposes a complete VTM software implementation, including an estimation module at encoder side and a synthesis module at decoder side. It uses a more precise specification of the film grain synthesis process than what is described in the VSEI specification. In particular, the proposed methods are based on the SMPTE RDD 5 that includes a detailed description of the film grain synthesis process, with some additional adaptations. One of the main proposed adaptations is to use the VVC’s core DCT-II transform instead of the transform proposed in SMPTE RDD 5.

# Illustration of FGC SEI message based on SMPTE RDD 5

Specifically, the software illustrates the example of film grain analysis (estimation) and synthesis in a manner consistent with SMPTE RDD 5 [2], and signalling of the corresponding model parameters values within the VSEI FGC SEI. The software illustrates the methods of SMPTE RDD 5 specification which is based on frequency filtering approach. The method of film grain analysis (parameter estimation) and film grain synthesis is given in the following.

## SMPTE RDD 5 General Workflow

A general framework of film grain analysis and synthesis is given in Figure 1.

Figure 1: General framework of film grain analysis and synthesis in the context of video compression.

The SMPTE RDD 5 specification defines its own DCT-2 transform. To ensure consistency with the VVC specification, it is proposed instead to use the VVC core 64x64 DCT-2 transform in the presented implementation.

The proposed modification does not affect standardized decoder. In addition, both analysis (estimation) and synthesis parts are optional and complete framework is in conformance with standardized syntax.

In addition, complete process is bit-exact.

## Film Grain Analysis Workflow

It consists of pre-processing block and film grain estimation block. It is to note that this automatic estimation step is not mandatory, and manually tuned parameters can be provided to the encoder based on the input from configuration file.

Pre-processing step consists of low-pass filtering of the input frame. Filtered frame is used in film grain estimation part. MCTF can be re-used for this purpose, as we propose in our software. After filtering, film grain estimate (film grain image) is calculated as a difference between original input frame and MCTF filtered frame. In case for Qp<17, film grain analysis is disabled, however FGC SEI messages still can be encoded. Also, we created additional instance of MCTF, which gives us flexibility to test MCTF with different parameters without interfering with initial MCTF used during encoding process. Also MCTF filtering for film grain analysis uses internal buffer to store filtered frame for film grain analysis module. It means that we chose to keep all encoding processes unchanged - bitrate and decoded frames are the same as without film grain analysis.

In addition, film grain analysis consists of creating a map that indicates the flat regions of the input frame, since edges and complex texture can interfere analysis process [3]. To create a map, this contribution uses Canny edge detector plus additional morphological operations. Film grain parameters are estimated only on flat regions (more precisely 64x64 blocks that belong to the flat regions) as indicated with map provided in previous step. Film grain image and map are produced in pre-processing step, and they are afterwards provided to the estimation module. The complete analysis workflow is illustrated in Figure 2.

Figure 2: Film Grain Analysis workflow.

Afterwards, 64x64 blocks are extracted from film grain image, based on map that indicates flat regions of an image. Average variance over all blocks that belong to the flat regions of a frame is calculated. It corresponds to the comp_model_value[ c ][ i ][ 0 ] parameter in the SMPTE RDD 5.

Cut-off frequencies estimation part consists of applying forward transform on the film grain blocks to transfer it to the frequency domain. Transformed blocks are then analyzed in order to calculate film grain parameters (cut-off frequencies). First, average 64x64 transformed block Bavg is computed over all available blocks Bi (pixel-wise):

Bavg(x,y) = 1Ki=0K-1Bi(x,y)

After, average vector by columns Bc and by rows Br are calculated:

Bc(y)=164i=064-1Bavg(i,y) and Br(x)=164i=064-1Bavg(x,i)

Average vectors are represented as a curve and its intersection points with average value (avg=164*64i=064-1j=064-1Bavg(i,j)) are computed. Based on the analysis of the intersection point(s), cut-off frequencies are obtained (comp_model_value[ c ][ i ][ 1 ] and comp_model_value[ c ][ i ][ 2 ] in SMPTE RDD 5).

## Film grain Synthesis Workflow

Decoded samples

FGC-SEI Params

Video stream

VVC video decoder

Decoded frame

Video Output

8x8 grain block selection

Film grain Database

Uniform pseudo-Random Number generator

De-blocking

Compute 8x8 block average

Previous 8x8 block

The film grain simulation and blending workflow for SMPTE RDD 5 is illustrated in Figure 3. SMPTE RDD 5 film grain simulation is accomplished by specifying: a database of film grain patterns; a uniform pseudo-random number generator; and a precise sequence of operations, all of which are specified in SMPTE RDD 5 [2].

FGC-SEI Params

Decoded samples

Video stream

VVC video decoder

Decoded frame

Video Output

Select Film grain params

8x8 grain block selection

Film grain Database

Uniform pseudo-Random Number generator

De-blocking

Compute 8x8 block average

Previous 8x8 block

Figure 3: Film grain synthesis and blending workflow for SMPTE RDD 5.

Figure 4: Film grain synthesis and blending workflow for SMPTE RDD 5.

In the proposed implementation, film grain database is created off-line for all combinations of cut-off frequencies, even if this is not strict requirement (film-grain pattern can be calculated on-fly). In addition, in the proposed implementation it is created by using VVC’s core transform (DCT-2 64x64, same as in analysis part) instead of RDD 5 internal transform.

# Syntax settings

For the SMPTE RDD 5 example, the parameters conveyed as part of the FGC SEI message are tabulated in Table 1.

Table 1: FGC SEI syntax parameter values for SMPTE RDD 5.

| Parameter | Supported value / Range | Explanation |
| --- | --- | --- |
| film_grain_model_id | 0 | Frequency filtering model |
| separate_colour_description_present_flag | 0 | Film grain colour space same as encoding colour space |
| blending_mode_id | 0 | Additive blending |
| log2_scale_factor | 2-7 | Ensures simulation compatible with 16-bit signed integer math |
| num_model_values_minus1[ c ] | 0-2 | Bandpass and cross-colour correlation not supported |
| comp_model_value[ c ][ i ][ 0 ] | 0-(2Bitdepth – 1) | See Note |
| comp_model_value[ c ][ i ][ 1 ] | 2-14 | Indicates horizontal cut-off frequency |
| comp_model_value[ c ][ i ][ 2 ] | 2-14 | Indicates vertical cut-off frequency |
| film_grain_characteristics_persistence_flag | 0 | FGC SEI message applied to the current decoded frame only |

Note: SMPTE-RDD 5 is specified for 8-bits but the example software implementation we provide extends RDD 5 to higher bit depths, which allows comp_model_value[c][i][0] to exceed 255 for higher bit depths, with a maximum supported value of (2Bitdepth – 1).

# Software description

The software provided is based on the latest version of VVC reference software VTM-12.0.

The film grain software in defined under several macros:

- #define ID_FILM_GRAIN_ENABLE 1

It defines plain FGC SEI for film grain plus SMPTE RDD 5 based film grain model parameters. It will perform encoding of the FGC SEI message.

- #define ID_FILM_GRAIN_SYNTHESIS 1

It enables film grain synthesis on decoder side (SMPTE RDD 5 based film grain synthesis enabled).

- #define ID_FILM_GRAIN_ANALYSIS 1

It enables adaptive film grain parameter estimation on the encoder side (SMPTE RDD 5 based film grain analysis enabled). In addition, SEIFGCAnalysisEnabled parameter in configuration file should be set to 1 to enable film grain analysis at the encoder.

- #define ID_FG_IMPROVEMENTS 1

It uses VVC’s core transform (DCT-2 64x64) instead of the custom RDD 5 transform for film grain analysis and film grain synthesis.

SEIFGCEnabled : 1 is used to enable FGC SEI encoding. If other parameters are not provided, encoder will use default film grain parameters.

As one option, film_grain_characteristics.cfg is updated by adding new used-defined configuration parameters. It is able to specify the FGC parameters for each component: num_intensity_intervals_minus1, num_model_values_minus1, intensity_interval_lower_bound, intensity_interval_higher_bound and comp_model_value. Other parameters, that are defined with FGC SEI message format (Section 2), can be provided in configuration file.

Example encoding command:

EncoderApp.exe –c encoder_randomaccess_vtm.cfg –c input.cfg –c film_grain_characterstics.cfg

Example decoding command line:

DecoderApp.exe –b str.bin –o decoded.yuv --SEIFGSFilename=decoded_FGS.yuv

# Simulations

In our software, film grain analysis is performed only on intra pictures, meaning that for intra configuration it is estimated on each frame, for low delay only on a first frame, and for randomaccess configuration estimation is performed per I-period. To estimate film grain only on I frames is implementation decision of the authors. There is no obstacle to perform analysis in a different way. SEI message is inserted in the same manner, however it can be inserted once per picture as well. We provided SEIFGCPerPictureSEI parameter in configuration file to control the frequency of FGC SEI messages.

We tested various settings to verify the correctness of the software. We used one VVC CTC clip ArenaOfValor 1080p 4:2:0. ArenaOfValor is a gaming sequence captured from screen, so it is clean and easier to observe the effect of simulated film grain.

We also used three sequences that are noisy, and originally posses film grain. We use those sequences to test if analysis part is able to reproduce similar appearance of film grain at the decoder side after the film grain parameters are estimated. We used among others the well-known CrowdRun, InToTree and OldTownCross clips to test the presented software. Our results show that analysis part is able to estimate close-to-original look of film grain, however there is still space for improvement.

In addition, we provided one artificially created sequence for sanity check purpose that can be used in development process. Sequence consists of 35 identical frames, where film grain is present in Y and Cr component and not in Cb component. Snapshot is provided in Figure 4, and sequence is provided as supplementary material to this contribution. In these examples, QP 37 was used.

We also provide snapshot of CrowdRun sequence to illustrate the reconstructed film grain (Figure 5 and Figure 6).

We have provided several configuration files for testing. One default configuration (film_grain_characteristics_test_default.cfg) only enables FGC SEI message (SEIFGCEnabled is set to 1) and it uses default values that are hardcoded.

Configurations film_grain_characteristics_test.cfg - film_grain_characteristics_test5.cfg are used to test software with manually tuned parameters (analysis part is not enabled, SEIFGCAnalysisEnabled is set to 0). They simulate film grain either on Y component only, or on all three components. They differ in film grain parameters such as cut-off frequencies and film grain variance. Different values are used to show the performance of the code, even such simulation in many cases do not reflect real world use-case.

For example, film_grain_characteristics_test.cfg and film_grain_characteristics_test5.cfg are used to simulate film grain on all three channels. It uses only one intensity interval.

film_grain_characteristics_test1.cfg - film_grain_characteristics_test3.cfg are used to simulate film grain only on luma component, and it still uses one intensity interval. Different pair of cut-off frequencies and different film grain variance are used.

film_grain_characteristics_test4.cfg is used to simulate different intensity intervals (4 of them) with different film grain variance. Film grain is only added on luma component.

Figure 4: Original, decoded (top), and decoded + film grain synthesis (bottom) snapshots. Film grain synthesis is based on estimated parameters. Qp=37 and randomaccess is used.

We also provide snapshot of CrowdRun sequence to illustrate the reconstructed film grain (Figure 5 and Figure 6).

We have provided several configuration files for testing. One default configuration only enables FGC SEI message (SEIFGCEnabled is set to 1) and it used default values that are hardcoded. Other configurations simulate film grain either on Y component only, or on all three components. They differ in film grain parameters such as cut-off frequencies and film grain variance. Different values are used to show the performance of the code, even such simulation in many cases do not reflect real world use-case.

For example, film_grain_characteristics_test.cfg and film_grain_characteristics_test5.cfg are used to simulate film grain on all three channels. It uses only one intensity interval.

film_grain_characteristics_test1.cfg - film_grain_characteristics_test3.cfg are used to simulate film grain only on luma component, and it still uses one intensity interval. Different pair of cut-off frequencies and different film grain variance are used.

film_grain_characteristics_test4.cfg is used to simulate different intensity intervals (4 of them) with different film grain variance. Film grain is only added on luma component.

If analysis part is enabled (macro ID_FILM_GRAIN_ANALYSIS is set to 1), we suggest using film_grain_characteristics_test_analysis.cfg file. It sets SEIFGCAnalysisEnabled parameter to 1. It also provides only a subset of FGC SEI parameters, since there is no need to specify all parameters that are re-calculated (re-set) in the film grain analysis process (in order to avoid confusion  which may results from a misinterpretation of the configuration file). However, note that SEIFGCCompModelPresentComp0, SEIFGCCompModelPresentComp1 and SEIFGCCompModelPresentComp2 parameters are still present. Reason behind is that manual control to disable the film grain synthesis on individual components is still highly desirable. For example, even if film grain may be detected in chroma components during analysis process, SEIFGCCompModelPresentComp flag from configuration file will have higher importance if it is set to 0. It means that even if analysis module detects film grain in chroma, it is not coded in SEI, and SEIFGCCompModelPresentComp flag remains 0. On the contrary, if SEIFGCCompModelPresentComp is 1 within the configuration file, the software uses analysis results on film grain. It means that if analysis part did not find film grain, for example in chroma component, it will reset the SEIFGCCompModelPresentComp flag to 0 even if it was 1 in configuration file. Reason behind this is that the purpose of the analysis part is to estimate film grain as close as possible to the original one. In this case SEIFGCCompModelPresentComp flags can be interpreted as a flag that indicate if analysis is performed on particular component or not. This approach is only implementation oriented, and do not require any changes in SEI message format and SMPTE RDD 5 specification.

Note, to use film grain estimation feature, current implementation requires MCTF to be applied. However, for low delay and intra configurations, MCTF is applied but it will not change original frames and encoding performance will not change (we use internal buffers for film grain MCTF in order to be consistent with previous decisions on coding configurations).

Lastly, we test film grain sizes by modifying horizontal/vertical high-cut frequency. The film grain size gets smaller as the high-cut frequency gets larger. Also, a setting with multiple intensity intervals is provided. In order to see the effect of film grain simulation clearly, we encourage to purposely selected a value of simulated film grain variance larger than would be sensible for real applications.

It was observed that the example software implementation and FGC SEI message work as expected.

Figure 5: CrowdRun sequence, original (top), decoded (middle) and decoded +film grain (bottom). Qp=37 and randomaccess is used.

Figure 6: Zoom part of CrowdRun sequence. Original (top), decoded + film grain estimation and synthesis (middle), decoded only (bottom). Qp=37 and randomaccess is used.

# Conclusion

As film grain is becoming particularly interesting for video compression community, we propose to include the presented software and frequency filtering model for further use in VTM. Note that the method is simple but effective, and both estimation and synthesis can be done in an efficient manner (for example, by re-using MCTF or VVC’s core transform). Also, important aspect is that this method is bit-exact. During our simulations it was observed that simulated film grain can hide compression artefacts.

Thus, we conclude:

- Film grain turns out to be a very important tool for subjective quality enhancement of compressed video;

- It may be relevant to make the FG synthesis and blending process more precisely specified, in order to be guaranty reproducibility of the process;

- It may be relevant to make the FG synthesis and blending process mandatory, when the FG SEI is inserted in the bitstream;

- We propose to include the software in the fork of VTM for further study.

# Acknowledgment

# This work has been achieved in the context of the 3EMS project funded by the “Region Bretagne”. We would like to thank Dolby Inc. for cross-checking the software. Also, the code provided in this contribution related with synthesis part based on RDD 5 model and SEI message for film grain is based on previous Dolby’s implementation [4].

# References

- J. Boyce, V. Drugeon, G. Sullivan, Y.-K. Wang, “Supplemental enhancement information for coded video bitstreams (Draft 3)”, JVET Q2007-v6, Brussels, BE, Jan 2020

- “RDD 5:2006 - SMPTE Registered Disclosure Doc - Film Grain Technology — Specifications for H.264 | MPEG-4 AVC Bitstreams,” RDD 5:2006, pp. 1–18, Mar. 2006, doi: 10.5594/SMPTE.RDD5.2006.

- “Film Grain Synthesis for AV1 Video Codec”, Andrey Norkin and Neil Birkbeck, 2018 Data Compression Conference.

- Sean McCarthy, Fangjun Pu, Taoran Lu, Peng Yin, Walt Husak, Tao Chen, “AHG17: Illustration of the film grain characteristics SEI message for VVC”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, document JVET-R0359-v2, teleconference, April 2020.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).