| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0097-v3 |
| --- | --- |

| Title: | AHG12: Unsymmetric partitioning methods in video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Zhipin Deng Na Zhang, Yang Wang 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution presents two unsymmetric partitioning methods denoted as unsymmetric binary-tree (UBT) partitioning and unsymmetric quad-tree (UQT) partitioning. UBT and UQT can be applied for blocks with dimensions no greater than 64×64. On top of the EE code of JVET-U0100, simulation results of the proposed method are reported as below:

Tool-on test (VTM-11 as the anchor):

AI: -0.68%, 190%, 104%; RA: -1.48%, 204%, 102%; LB: -1.46%, 206%, 108%.

Tool-off test (EE code as the anchor):

AI: -0.61%, 187%, 103%; RA: -1.23%, 201%, 101%; LB: -1.25%, 207%, 101%.

# Introduction

In VVC, three kinds of partitioning methods, known as quad-tree (QT), binary-tree (BT) and ternary-tree (TT), can be selectively applied to split a coding unit (CU). All the partitioning methods supported in VVC are symmetric, i.e., a CU can only be horizontally or vertically partitioned into child-CUs. For QT and BT, the four or two child-CUs are with the same size. While for the TT which splits one CU to three child-CUs, the two smaller ones are with the same size. It is observed that symmetric partitioning methods may not always be efficient. During the development of VVC, several unsymmetric partitioning methods have been proposed [1][2].

# Proposed methods

In this proposal, two unsymmetric partitioning methods denoted as unsymmetric binary-tree (UBT) partitioning and unsymmetric quad-tree (UQT) partitioning are proposed, as additional partitioning choices together with QT, BT and TT.

## UBT

Four types of UBT are proposed as shown in Fig. 1. UBT-H1 and UBT-H2 split a CU with dimension W×H into two child-CUs with dimensions {W×H/4, W×3H/4} and {W×3H/4, W×H/4}, respectively. UBT-V1 and UBT-V2 split a CU with dimension W×H into two child-CUs with dimensions {W/4×H, 3W/4×H} and {3W/4×H, W/4×H}, respectively.

With UBT, the width or height of a CU or TU may be a non-dyadic number (not in a form of 2N). Some coding tools like ISP, MIP and SBT are disabled in those CUs. Besides, 6-point, 12-point, 24-point, and 48-point transforms/inverse-transforms are introduced to code residual blocks with non-dyadic dimensions.

## UQT

Four types of UQT are proposed as shown in Fig. 2. UQT-H1 and UQT-H2 split a CU with dimension W×H into four child-CUs with dimensions {W×H/8, W×H/2, W×H/4, W×H/8} and {W×H/8, W×H/4, W×H/2, W×H/8}, respectively. UQT-V1 and UQT-V2 split a CU with dimension W×H into four child-CUs with dimensions {W/8×H, W/2×H, W/4×H, W/8×H} and {W/8×H, W/4×H, W/2×H, W/8×H}, respectively.

| UBT-H1 | UBT-H2 |
| --- | --- |
| UBT-V1 | UBT-V2 |

Fig. 1 UBTs.

| UQT-H1 | UQT-H2 |
| --- | --- |
| UQT-V1 | UQT-V2 |

Fig. 2 UQTs.

## Signaling of partitioning methods

Whether to apply vertical or horizontal UBT/UQT partitioning is indicated by mtt_split_cu_vertical_flag in VVC.

An mtt_split_cu_ubt_flag is signaled before mtt_split_cu_binary_flag to indicate whether UBT is applied. If UBT is signaled to be applied, a second flag is further signaled to indicate whether type-1 or type-2 UBT is applied.

To avoid redundant represenation, UQT is signaled in a two-step manner as demonstrated in Fig. 3. First, an mtt_split_cu_uqt_flag is signaled when mtt_split_cu_binary_flag is equal to 0. If mtt_split_cu_uqt_flag is equal to 0, regular TT is applied. Otherwise, if mtt_split_cu_uqt_flag is equal to 1, the CU is firstly split into three parts as {1/8, 3/4, 1/8}, and then the central part is further split into to two parts as {2/3, 1/3} or {1/3, 2/3}, indicated by a third flag. Combining the two steps, a CU is split into four parts as {1/8, 1/2, 1/4, 1/8} or {1/8, 1/4, 1/2, 1/8}.

Fig. 3 A two-step method to signal UQTs.

## Normative constraints on partitioning

Some normative constrains are imposed on partitioning to reduce redundancy and complexity.

- QT is not allowed for a CU if any of its ancestor CUs is split from UBT or UQT.

- UBT and UQT can only be applied to CUs no greater than 64×64.

- The width and height on luma component must be in a form of 4N after any partitioning.

- UBT and UQT are not used on chroma components for dual-tree coding structure.

- Local dual-tree is disabled when UBT and UQT are used.

- For a CU with dimension W×H:

  - If W is a non-dyadic number, no vertical partitioning is allowed on the CU.

  - If H is a non-dyadic number, no horizontal partitioning is allowed on the CU.

# Experimental results

The proposed unsymmetric partitioning methods are tested on top of the EE code of JVET-U0100 [3], according to the common test conditions defined in [4]. The results of tool-on test (based on the EE code with the same configuration as VVC, VTM-11 as the anchor) and tool-off test (based on the EE code with all new features on, EE code as the anchor) are tabulated in Table 1 and Table 2, respectively. The average results are reported as below:

Tool-on test (VTM-11 as the anchor):

AI: -0.68%, 190%, 104%; RA: -1.48%, 204%, 102%; LB: -1.46%, 206%, 108%.

Tool-off test (EE code as the anchor):

AI: -0.61%, 187%, 103%; RA: -1.23%, 201%, 101%; LB: -1.25%, 207%, 101%.

Table 1: Performance of the tool-on test

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.51% | -0.55% | -0.33% | 181% | 102% | -1.24% | -1.53% | -1.78% | 193% | 103% |  |  |  |  |  |
| Class A2 | -0.68% | -0.73% | -0.76% | 189% | 104% | -1.45% | -1.88% | -1.90% | 198% | 103% |  |  |  |  |  |
| Class B | -0.69% | -0.63% | -0.83% | 198% | 103% | -1.44% | -1.86% | -1.82% | 201% | 102% | -1.43% | -2.45% | -2.43% | 208% | 105% |
| Class C | -0.69% | -0.96% | -0.97% | 190% | 105% | -1.74% | -1.86% | -1.93% | 223% | 102% | -1.80% | -2.21% | -2.45% | 238% | 111% |
| Class E | -0.83% | -0.51% | -0.47% | 191% | 104% |  |  |  |  |  | -1.06% | -1.07% | -0.97% | 168% | 108% |
| Overall | -0.68% | -0.69% | -0.71% | 190% | 104% | -1.48% | -1.80% | -1.86% | 204% | 102% | -1.46% | -2.02% | -2.07% | 206% | 108% |
| Class D | -0.56% | -1.12% | -0.98% | 186% | 107% | -1.69% | -2.57% | -2.39% | 219% | 94% | -1.93% | -2.71% | -3.58% | 237% | 110% |

Table 2: Performance of the tool-off test

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.49% | -0.26% | -0.23% | 181% | 102% | -1.11% | -1.27% | -1.46% | 186% | 100% |  |  |  |  |  |
| Class A2 | -0.57% | -0.27% | -0.29% | 191% | 105% | -1.19% | -1.43% | -1.29% | 197% | 100% |  |  |  |  |  |
| Class B | -0.62% | -0.38% | -0.30% | 197% | 102% | -1.27% | -1.53% | -1.70% | 203% | 100% | -1.26% | -2.21% | -2.35% | 205% | 102% |
| Class C | -0.58% | -0.30% | -0.37% | 180% | 104% | -1.29% | -1.57% | -1.71% | 213% | 102% | -1.36% | -2.29% | -2.08% | 235% | 100% |
| Class E | -0.77% | -0.31% | -0.53% | 184% | 101% |  |  |  |  |  | -1.09% | 0.17% | -1.26% | 179% | 101% |
| Overall | -0.61% | -0.31% | -0.34% | 187% | 103% | -1.23% | -1.47% | -1.57% | 201% | 101% | -1.25% | -1.64% | -1.99% | 207% | 101% |
| Class D | -0.45% | -0.26% | -0.33% | 179% | 98% | -1.07% | -1.41% | -1.58% | 207% | 104% | -1.56% | -2.15% | -1.64% | 232% | 105% |

# Conclusions

This contribution presents two unsymmetric partitioning methods UBT and UQT. The proposed methods contribute promising coding gains. It is proposed to further study the new partitioning methods on the video coding research platform beyond VVC.

# References

- F. Le Léannec, T. Poirier, F. Galpin, “CE1: Asymmetric Binary Tree (tests 1.0.1, 1.0.2, 1.0.3, 1.0.4, 8.0.1 and 8.0.2)”, JVET-K0197, July 2018.

- K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, “Non-CE: Unsymmetrical quadtree partitioning,” JVET-N0421, March 2019.

- Y. -J. Chang, C. -C. Chen, J. Chen, J. Dong, etc. “Compression efficiency methods beyond VVC,” JVET-U0100, Jan. 2021.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-U2017, Jan. 2021.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).