| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0098-v2 |
| --- | --- |

| Title: | EE2-related: Template-based intra mode derivation using MPMs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang1, Li Zhang2, Kai Zhang2 Zhipin Deng1, Na Zhang1 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | wangyang.cs@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution proposes a template-based intra mode derivation (TIMD) method using most probable modes (MPMs). For each intra prediction mode in MPMs, the sum of absolute transformed differences (SATD) between the prediction and reconstruction samples of a template is calculated. The intra prediction mode with the minimum SATD is selected as the TIMD mode and used for intra prediction of current block. On top of the EE2 code of JVET-U0100, simulation results of the proposed method are reported as below:

Tool-on test (Anchor: VTM-11.0): AI: {-0.41%, 117%, 111%}; RA: {-0.23%, 109%, 107%}.

Tool-off test (Anchor: EE2 code): AI: {-0.27%, 118%, 109%}; RA: {-0.11%, 104%, 101%}.

Introduction

A decoder-side intra mode derivation (DIMD) approach was proposed in JVET-C0061 [1]. The intra prediction mode of a CU was derived with a template based method at both encoder and decoder, instead of being signalled to the decoder. As shown in Figure 1, the prediction samples of the template is generated using the reference samples of the template for each candidate mode. A cost is calculated as the SATD between the prediction and the reconstruction samples of the template. The intra prediction mode with the minimum cost is selected as the DIMD mode and used for intra prediction of the CU. The candidate modes may be 67 intra prediction modes as in VVC or extended to 131 intra prediction modes. A progressive mode search method was proposed to select the DIMD mode from all allowed intra prediction modes, in which the search interval was reduced to half in each step.

Figure 1. Template and its reference samples used in DIMD

Although this progressive search method can reduce the number of the modes to be checked, the starting modes are always the same for all CUs. Different regions in the video have different characteristics. Generally most probable modes (MPMs) can provide a clue to indicate the directional information of a CU. Thus, it is proposed to derive the intra prediction mode of current CU from the MPMs.

Proposed method

This contribution proposes a template-based intra mode derivation (TIMD) method using MPMs, in which a TIMD mode is derived from MPMs using the neighbouring template. The TIMD mode is used as an additional intra prediction method for a CU.

TIMD mode derivation

For each intra prediction mode in MPMs, The SATD between the prediction and reconstruction samples of the template is calculated. The intra prediction mode with the minimum SATD is selected as the TIMD mode and used for intra prediction of current CU. Position dependent intra prediction combination (PDPC) is included in the derivation of the TIMD mode.

TIMD signalling

A flag is signalled in sequence parameter set (SPS) to enable/disable the proposed method. When the flag is true, a CU level flag is signalled to indicate whether the proposed TIMD method is used. The TIMD flag is signalled right after the MIP flag. If the TIMD flag is equal to true, the remaining syntax elements related to luma intra prediction mode, including MRL, ISP, and normal parsing stage for luma intra prediction modes, are all skipped.

Interaction with new coding tools in JVET-U0100

A DIMD method with prediction fusion using Planar [2] was integrated in EE2. When EE2 DIMD flag is equal to true, the proposed TIMD flag is not signalled and set equal to false.

Similar to PDPC, Gradient PDPC is also included in the derivation of the TIMD mode.

When secondary MPM is enabled, both the primary MPMs and the secondary MPMs are used to derive the TIMD mode.

6-tap interpolation filter is not used in the derivation of the TIMD mode.

Modification of MPM list construction in the derivation of TIMD mode

During the construction of MPM list, intra prediction mode of a neighbouring block is derived as Planar when it is inter-coded. To improve the accuracy of MPM list, when a neighbouring block is inter-coded, a propagated intra prediction mode [3] is derived using the motion vector and reference picture and used in the construction of MPM list. This modification is only applied to the derivation of the TIMD mode.

Simulation results

The proposed method was implemented on top of the EE2 code of JVET-U0100 [4]. Simulations were performed following the common test conditions [5]. More detailed results could be found in the attached excel sheets.

The results of tool-on test (based on the EE2 code with the same configuration as VVC) compared to VTM-11.0 and tool-off test (based on the EE2 code with all new features on) are shown in Table 1 and Table 2.

Table 1: Coding performance compared to VTM-11.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.16% | -0.19% | 115% | 106% |
| Class A2 | -0.34% | -0.37% | -0.37% | 117% | 105% |
| Class B | -0.41% | -0.28% | -0.37% | 117% | 112% |
| Class C | -0.37% | -0.26% | -0.26% | 118% | 116% |
| Class E | -0.78% | -0.72% | -0.83% | 118% | 115% |
| Overall | -0.41% | -0.35% | -0.39% | 117% | 111% |
| Class D | -0.25% | -0.22% | -0.20% | 116% | 115% |
| Class F (optional) | -0.33% | -0.36% | -0.37% | 109% | 109% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.05% | -0.23% | 109% | 103% |
| Class A2 | -0.27% | -0.25% | -0.26% | 108% | 106% |
| Class B | -0.24% | -0.20% | -0.09% | 110% | 107% |
| Class C | -0.25% | -0.04% | 0.10% | 110% | 110% |
| Class E |  |  |  |  |  |
| Overall | -0.23% | -0.14% | -0.10% | 109% | 107% |
| Class D | -0.12% | 0.14% | 0.01% | 109% | 108% |
| Class F (optional) | -0.15% | -0.30% | -0.21% | 109% | 107% |

Table 2: Coding performance compared to EE2

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EE2 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.06% | -0.16% | 117% | 104% |
| Class A2 | -0.22% | -0.22% | -0.24% | 117% | 109% |
| Class B | -0.28% | -0.17% | -0.13% | 119% | 111% |
| Class C | -0.24% | -0.15% | -0.15% | 117% | 113% |
| Class E | -0.49% | -0.45% | -0.43% | 118% | 107% |
| Overall | -0.27% | -0.20% | -0.21% | 118% | 109% |
| Class D | -0.23% | -0.07% | -0.13% | 117% | 113% |
| Class F (optional) | -0.10% | -0.12% | -0.12% | 112% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over EE2 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.08% | -0.13% | 103% | 101% |
| Class A2 | -0.14% | -0.12% | -0.12% | 103% | 101% |
| Class B | -0.10% | -0.08% | -0.21% | 103% | 101% |
| Class C | -0.09% | -0.05% | -0.07% | 105% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.08% | -0.14% | 104% | 101% |
| Class D | -0.10% | -0.13% | -0.10% | 107% | 101% |
| Class F (optional) | -0.12% | 0.12% | -0.13% | 106% | 101% |

Conclusions

In this contribution, a template-based intra mode derivation using MPMs is proposed. It is recommended to further study the proposed method on the video coding research platform beyond VVC.

Reference

- X. Xiu, Y. He, and Y. Ye, “Decoder-side intra mode derivation,” JVET-C0061, Geneva, June 2016.

- M. Abdoli, T. Guionnet, E. Mora, M. Raulet, S. Blasi, A. S. Dias, and G. Kulupana, “Non-CE3: Decoder-side Intra Mode Derivation with Prediction Fusion Using Planar,” Document of Joint Video Experts Team, JVET-O0449, Gothenburg, July 2019.

- K. Zhang, L. Zhang, W. Chien and M. Karczewicz, "Intra-Prediction Mode Propagation for Video Coding," in IEEE Journal on Emerging and Selected Topics in Circuits and Systems, vol. 9, no. 1, pp. 110-121, March 2019.

- Y.-J. Chang, C.-C. Chen, J. Chen, J. Dong, etc., “Compression efficiency methods beyond VVC,” Document of Joint Video Experts Team, JVET-U0100, by teleconference, January 2021.

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-T2010, by teleconference, October 2020.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).