| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting: Teleconference, 20–28 April 2021 | Document: JVET-V0104-v3 |
| --- | --- |

| Title: | EE2-related: TU-level adaptive self-guided filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Li Zhang2, Yang Wang1, Hongbin Liu1 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1(858)999-7093 yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com wangyang.cs@bytedance.com liuhongbin.01@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution presents a TU-level adaptive self-guided filter (TU-ASGF) with five taps and a cross-shape. TU-ASGF is applied on the reconstruction samples of a TU. The filtering strength is based on the TU size, QP, and statistical information adaptively.

On top of the EE code of JVET-U0100, simulation results of the proposed method are reported as below:

Tool-on test (VTM-11 as the anchor):

AI: -0.42%, 122%, 123%; RA: -0.32%, 112%, 111%; LB: -0.41%, 108%, 113%.

Tool-off test (EE code as the anchor):

AI: -0.43%, 115%, 110%; RA: -0.24%, 104%, 101%; LB: -0.28%, 104%, 102%.

On top of the EE code of JVET-V0094 (bilateral filter), simulation results of the proposed method are reported as below:

Tool-on test (VTM-11 as the anchor):

AI: -0.71%, 126%, 122%; RA: -0.55%, 110%, 108%; LB: -0.64%, 107%, 110%.

Tool-off test (EE-bilateral filter code as the anchor):

AI: -0.22%, 115%, 114%; RA: -0.09%, 106%, 104%; LB: -0.11%, 103%, 103%.

# Introduction

It is well-known that quantized transform blocks may bring some artifacts (e.g., ringing artifact) around edges inside a block, both in still images and videos. Applying a self-guided filter could significantly reduce the ringing artifacts. Inspired by [1], this contribution presents a TU-level adaptive self-guided filter (TU-ASGF) directly applied on reconstructed samples of a TU.

# Proposed schemes

In this section, the filtering process and how to control the enabling of filtering process are described.

## Filtering process

With TU-ASGF, each sample in the reconstructed TU is substituted by a weighted summation of itself and its neighbouring samples. The weights are calculated based on the statistical information (e.g., variance and mean) within the filtering window which as shown in Figure 1.

Figure 1. Example of an 8x8 TU block and the filter aperture for the sample located at (1,1)

In a formulation way, the filtering result of a sample located at (i,j) is computed as a function of the sample to be filtered and the mean value within the filtering window, such as follows:

Ifilteredi,j=a×Iunfilteredi,j+b×meanwini,j

where I(i,j) stands for the intensity value of sample located as (i,j) and meanwini,j stands for the mean value within the filtering window that centered at (i,j). a and b are two weights that dominate the filtering strength for each sample, derived as below:

a= varwinvarwin+eps

b=1-a

where eps is a parameter that is used to control the filtering strength.

After the weights of each position are calculated, a refinement of weights is performed to further improve the performance of the proposed filter. The filtering weights a and b are refined as follows:

airefine= i=0i=Stotoal-1aistotal

birefine= i=0i=Stotal-1bistotal

where ai and bi stand for the specific weights within a filtering window, and stotal stands for the total sample number inside the filtering window. The window used here may have the same shape and size with the window used in computation of window variance and mean value.

Figure 2. Framework of the proposed guided filter.

## Application of the filtering process

First of all, the TU-ASGF is only performed on the LUMA component. For a LUMA TU, when QP of it is less than 18, or min of width and height of it is larger than 16, or max of the width and height of is larger than 64, or the corresponding CU is IBC mode, the TU-ASGF will be disabled.

# Experimental results

The following tables show the overall results for AI, RA and LDB. The simulations were performed following the the common test conditions [2]. The tests were implemented on top of JVET-U0100 [3] and JVET-V0094 [4] (bilateral filter). Compared to the VTM-11.0, the performance of the proposed filter is summarized in Table 1 (tool-on test). On top of the EE code of JVET-U0100, the performance of the proposed filter is summarized in Table 2 (tool-off test). On top of the EE code of JVET-V0094 (bilateral filter), the performance of the proposed filter is summarized in Table 3 (tool-on test and VTM-11.0 as anchor) and Table 4 (tool-off test).

It is noticed that the simulations are based on our preliminary try wherein the code hasn’t been optimized, such as SIMD optimization hasn’t been implemented.

Table 1. Performance of the proposed filter on top of JVET-U0100 (tool-on test).

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | 0.12% | 0.15% | 124% | 120% |
| Class A2 | -0.39% | 0.05% | 0.03% | 121% | 117% |
| Class B | -0.40% | 0.14% | 0.11% | 123% | 123% |
| Class C | -0.56% | -0.04% | -0.03% | 122% | 127% |
| Class E | -0.47% | -0.15% | 0.04% | 122% | 126% |
| Overall | -0.42% | 0.03% | 0.06% | 122% | 123% |
| Class D | -0.47% | -0.08% | 0.09% | 123% | 135% |
| Class F | -0.43% | -0.05% | -0.16% | 112% | 119% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | 0.12% | -0.04% | 111% | 107% |
| Class A2 | -0.34% | -0.05% | 0.04% | 109% | 104% |
| Class B | -0.32% | -0.01% | 0.28% | 113% | 111% |
| Class C | -0.40% | 0.09% | 0.19% | 113% | 120% |
| Class E |  |  |  |  |  |
| Overall | -0.32% | 0.04% | 0.15% | 112% | 111% |
| Class D | -0.38% | 0.34% | 0.11% | 112% | 125% |
| Class F | -0.29% | -0.01% | 0.00% | 110% | 113% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.32% | 0.35% | 0.23% | 107% | 110% |
| Class C | -0.43% | 0.33% | 0.07% | 109% | 117% |
| Class E | -0.53% | 0.71% | 1.26% | 106% | 111% |
| Overall | -0.41% | 0.43% | 0.43% | 108% | 113% |
| Class D | -0.54% | 0.90% | 0.32% | 110% | 121% |
| Class F | -0.35% | 0.44% | 0.55% | 107% | 113% |

Table 2. Performance of the proposed filter on top of JVET-U0100 (tool-off test).

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over U100base1tools1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | 0.12% | 0.08% | 117% | 108% |
| Class A2 | -0.42% | -0.04% | 0.03% | 115% | 109% |
| Class B | -0.41% | 0.08% | 0.09% | 116% | 111% |
| Class C | -0.57% | 0.04% | 0.04% | 113% | 112% |
| Class E | -0.48% | 0.01% | 0.06% | 114% | 108% |
| Overall | -0.43% | 0.05% | 0.06% | 115% | 110% |
| Class D | -0.47% | 0.12% | 0.12% | 112% | 111% |
| Class F | -0.45% | -0.09% | 0.09% | 107% | 108% |
|  | Random access Main10 |  |  |  |  |
|  | Over U100base1tools1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | 0.07% | 0.07% | 104% | 101% |
| Class A2 | -0.33% | 0.12% | 0.06% | 104% | 100% |
| Class B | -0.23% | 0.07% | 0.05% | 105% | 101% |
| Class C | -0.26% | -0.04% | 0.01% | 104% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.24% | 0.05% | 0.04% | 104% | 101% |
| Class D | -0.27% | 0.28% | 0.08% | 104% | 102% |
| Class F | -0.27% | -0.15% | -0.18% | 104% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over U100base1tools1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.25% | 0.01% | 0.07% | 105% | 102% |
| Class C | -0.21% | 0.28% | 0.85% | 103% | 103% |
| Class E | -0.40% | 0.00% | 0.28% | 103% | 102% |
| Overall | -0.28% | 0.09% | 0.38% | 104% | 102% |
| Class D | -0.52% | 0.98% | 1.00% | 104% | 104% |
| Class F | -0.21% | -0.08% | 0.05% | 103% | 102% |

Table 3. Performance of the proposed filter on top of JVET-V0094 (bilateral filter) (tool-on test).

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.48% | 0.45% | 0.31% | 128% | 124% |
| Class A2 | -0.69% | 0.31% | 0.23% | 125% | 120% |
| Class B | -0.59% | 0.43% | 0.44% | 127% | 124% |
| Class C | -0.99% | 0.08% | 0.10% | 125% | 121% |
| Class E | -0.80% | 0.22% | 0.16% | 126% | 122% |
| Overall | -0.71% | 0.30% | 0.26% | 126% | 122% |
| Class D | -0.81% | 0.19% | 0.31% | 124% | 123% |
| Class F | -0.98% | -0.07% | 0.00% | 110% | 118% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.43% | 0.26% | -0.04% | 110% | 109% |
| Class A2 | -0.53% | 0.25% | 0.16% | 109% | 106% |
| Class B | -0.49% | 0.20% | 0.40% | 111% | 108% |
| Class C | -0.72% | 0.21% | 0.54% | 110% | 110% |
| Class E |  |  |  |  |  |
| Overall | -0.55% | 0.22% | 0.30% | 110% | 108% |
| Class D | -0.65% | 0.30% | 0.29% | 109% | 108% |
| Class F | -0.72% | -0.07% | 0.07% | 107% | 106% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.52% | 0.67% | 0.61% | 108% | 110% |
| Class C | -0.74% | 0.58% | 0.46% | 108% | 113% |
| Class E | -0.72% | 0.73% | 0.95% | 103% | 107% |
| Overall | -0.64% | 0.66% | 0.65% | 107% | 110% |
| Class D | -1.01% | 1.24% | 0.86% | 109% | 113% |
| Class F | -0.89% | 0.10% | 0.52% | 103% | 107% |

Table 4. Performance of the proposed filter on top of JVET-V0094 (bilateral filter) (tool-off test).

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EE2-bilateral filter |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | 0.17% | 0.00% | 117% | 111% |
| Class A2 | -0.24% | 0.04% | -0.05% | 114% | 110% |
| Class B | -0.19% | 0.03% | 0.06% | 116% | 114% |
| Class C | -0.32% | 0.06% | -0.08% | 114% | 120% |
| Class E | -0.24% | 0.03% | 0.16% | 116% | 116% |
| Overall | -0.22% | 0.06% | 0.02% | 115% | 114% |
| Class D | -0.25% | -0.01% | 0.12% | 111% | 126% |
| Class F | -0.28% | -0.27% | -0.01% | 111% | 113% |
|  | Random access Main10 |  |  |  |  |
|  | Over EE2-bilateral filter |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.09% | 0.01% | 104% | 101% |
| Class A2 | -0.12% | -0.10% | 0.00% | 102% | 100% |
| Class B | -0.07% | -0.22% | -0.09% | 108% | 105% |
| Class C | -0.10% | 0.01% | -0.16% | 108% | 109% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.07% | -0.07% | 106% | 104% |
| Class D | -0.12% | 0.13% | 0.67% | 105% | 110% |
| Class F | -0.11% | -0.07% | 0.07% | 106% | 107% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over EE2-bilateral filter |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | 0.05% | 0.42% | 102% | 103% |
| Class C | -0.07% | -0.11% | 0.48% | 104% | 103% |
| Class E | -0.27% | -0.38% | -0.39% | 103% | 103% |
| Overall | -0.11% | -0.11% | 0.24% | 103% | 103% |
| Class D | -0.08% | -0.90% | -0.62% | 105% | 111% |
| Class F | -0.16% | 0.16% | 0.08% | 104% | 104% |

The proposed method can also be implemented after the deblocking stage, out of the prediction loop. The result (tool-off test) of performing the proposed filter after deblocking is shown in Table 5. The anchor is JVET-V0094(bilateral filter).

Table 5. Performance of the proposed filter after deblocking on top of JVET-V0094 (bilateral filter) .

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EE2-bilateral filter |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.17% | 0.09% | 0.03% | 112% | 111% |
| Class E | -0.12% | 0.16% | 0.14% | 116% | 113% |
| Overall |  |  |  |  |  |
| Class D | -0.12% | 0.07% | 0.16% | 112% | 115% |
| Class F |  |  |  |  |  |

# Conclusion

This contribution presents a TU-ASGF method. It is asserted that the proposed filter achieves gains in terms of coding efficiency with a reasonable increase in encoder and decoder complexity. It is recommended to further study this method in the common code base platform.

- References

[1] F. Kou, W. Chen, C. Wen and Z. Li “Gradient Domain Guided Image Filtering”, IEEE Transactions on Image Processing, vol. 24, no. 11, Nov 2015.

[2] F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, Oct 2020.

[3] Y. Chang, C, Chen, J. Chen, J. Dong and et al “Compression efficiency methods beyond VVC”, JVET-U0100, Jan 2021.

[4] J. Ström, P. Wennersten, K. Andersson, R. Sjöberg, S. Ikonin and E. Alshina “EE2: Bilateral filter in VTM, EE2 and VVenC”, JVET-V0094, Apr 2021.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).