| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0111-v1 |
| --- | --- |

| Title: | On Decoding Unit Information for VVC Version 1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Following is proposed as bug-fixes for asserted issues related to dui_dpb_output_du_delay:

- It is proposed to fix the inference rule for the dui_dpb_output_du_delay when not present.

- It is proposed to add a bitstream conformance constraint related to dui_dpb_output_du_delay_present_flag.

- It is proposed to remove the note in C3.3.

- It is proposed to clarify the condition for the existing constraint on the value of dui_dpb_output_du_delay.

# Introduction

VVC specification text [1] includes a design of HRD including a decoding unit information SEI message.

In JVET-T2001-v2, according to the semantics of dui_dpb_output_du_delay:

“When not present, the value of dui_dpb_output_du_delay is inferred to be equal to pt_dpb_output_du_delay.”

One condition under which dui_dpb_output_du_delay is not present is when bp_du_dpb_params_in_pic_timing_sei_flag is equal to 0 and dui_dpb_output_du_delay_present_flag is equal to 0. However, in this case since bp_du_dpb_params_in_pic_timing_sei_flag is equal to 0, pt_dpb_output_du_delay is not present, and it does not have an inference rule when not present. Thus, in this case pt_dpb_output_du_delay does not have a defined value and inferring dui_dpb_output_du_delay to be equal to pt_dpb_output_du_delay means dui_dpb_output_du_delay has undefined value. It is asserted that the value of dui_dpb_output_du_delay is used to set the value of dpbOutputDuDelay when bp_du_dpb_params_in_pic_timing_sei_flag is equal to 0 and is thus used for the operation of HRD.

Another issue is that there is no constraint which disallows setting bp_du_dpb_params_in_pic_timing_sei_flag equal to 0 and dui_dpb_output_du_delay equal to 0 in each DUI SEI of an AU. In fact the note in C.3.3 includes this possibility and refers to the inference mentioned above (and asserted to be problematic): When the syntax element dui_dpb_output_du_delay is not present in any DUI SEI message associated with AU n, the value is inferred to be equal to pt_dpb_output_du_delay in the PT SEI message associated with AU n.

It is asserted that the design goal and intent of the decoding unit information SEI message has been that when  bp_du_dpb_params_in_pic_timing_sei_flag is equal to 0, the syntax element dui_dpb_output_du_delay may be signalled in one or more DU information SEI messages for one or more DUs of an AU. For example, the dui_dpb_output_du_delay may only be signalled in the last DU of the AU and thus the value of dui_dpb_output_du_delay_present_flag is set to 1 for the last DUI SEI and is set to 0 for the other DUI SEIs for the AU. This is the reason there is a flag (dui_dpb_output_du_delay_present_flag) to control the presence of dui_dpb_output_du_delay, when bp_du_dpb_params_in_pic_timing_sei_flag is equal to 0.

Following is proposed as bug-fixes:

- It is proposed to fix the inference rule for the dui_dpb_output_du_delay when not present.

- It is proposed to add a bitstream conformance constraint related to dui_dpb_output_du_delay_present_flag.

- It is proposed to remove the note in C3.3.

- It is proposed to clarify the condition for the existing constraint on the value of dui_dpb_output_du_delay.

# Proposal

The proposed modifications are shown highlighted below compared to [1].

1) It is proposed to fix the inference rule for the dui_dpb_output_du_delay when not present.

Two alternative options are proposed for this aspect:

Option 1: In this option the inference rule for dui_dpb_output_du_delay is proposed to be fixed.

dui_dpb_output_du_delay is used to compute the DPB output time of the AU when DecodingUnitHrdFlag is equal to 1 and bp_du_dpb_params_in_pic_timing_sei_flag is equal to 0. It specifies how many sub clock ticks to wait after removal of the last DU in an AU from the CPB before the decoded pictures of the AU are output from the DPB. When not present and when bp_du_dpb_params_in_pic_timing_sei_flag is equal to 0, the value of dui_dpb_output_du_delay is inferred to be equal to dui_dpb_output_du_delay from any DU belonging to the same AU for which  dui_dpb_output_du_delay_present_flag  is equal to 1 pt_dpb_output_du_delay. The length of the syntax element dui_dpb_output_du_delay is given in bits by bp_dpb_output_delay_du_length_minus1 + 1.

Option 2: In this option the inference rule for dui_dpb_output_du_delay is removed and the text in HRD operation is modified.

dui_dpb_output_du_delay is used to compute the DPB output time of the AU when DecodingUnitHrdFlag is equal to 1 and bp_du_dpb_params_in_pic_timing_sei_flag is equal to 0. It specifies how many sub clock ticks to wait after removal of the last DU in an AU from the CPB before the decoded pictures of the AU are output from the DPB. When not present, the value of dui_dpb_output_du_delay is inferred to be equal to pt_dpb_output_du_delay. The length of the syntax element dui_dpb_output_du_delay is given in bits by bp_dpb_output_delay_du_length_minus1 + 1.

..

C 3.3 Picture output

The processes specified in this clause happen instantaneously at the CPB removal time of AU n, AuCpbRemovalTime[ n ].

When picture n has PictureOutputFlag equal to 1, its DPB output time DpbOutputTime[ n ] is derived as follows, where the variable firstPicInBufferingPeriodFlag is equal to 1 if AU n is the first AU of a BP and 0 otherwise:

if( !DecodingUnitHrdFlag ) {
	DpbOutputTime[ n ] = AuCpbRemovalTime[ n ] + ClockTick * ( pt_dpb_output_delay –	(1591)
				AuDpbOutputDelta[ Htid ] )
	if( firstPicInBufferingPeriodFlag )
		DpbOutputTime[ n ]  −=  ClockTick * DpbDelayOffset
} else
	DpbOutputTime[ n ] = AuCpbRemovalTime[ n ] + ClockSubTick * dpbOutputDuDelay

where AuDpbOutputDelta[ Htid ] is derived according to pt_cpb_removal_delay_minus1[ Htid ] and pt_cpb_removal_delay_delta_idx[ Htid ] in the PT SEI message associated with AU n and bp_cpb_removal_delay_delta_val[ pt_cpb_removal_delay_delta_idx[ Htid ] ] and bp_dpb_output_tid_offset[ Htid ] in the BP SEI message associated with AU n, and dpbOutputDuDelay is the value of dui_dpb_output_du_delay in the DUI SEI messages associated with AU n when bp_du_dpb_params_in_pic_timing_sei_flag is equal to 0 and dui_dpb_output_du_delay_present_flag is equal to 1, or the value of pt_dpb_output_du_delay in the PT SEI message associated with AU n when bp_du_dpb_params_in_pic_timing_sei_flag is equal top 1.

2) Bitstream conformance constraint related to dui_dpb_output_du_delay_present_flag:

It is proposed to add a bitstream conformance constraint to signal dui_dpb_output_du_delay in at least one DUI SEI message for an AU when bp_du_dpb_params_in_pic_timing_sei_flag is equal to 0.

dui_dpb_output_du_delay_present_flag equal to 1 specifies the presence of the dui_dpb_output_du_delay syntax element in the DUI SEI message. dui_dpb_output_du_delay_present_flag equal to 0 specifies the absence of the dui_dpb_output_du_delay syntax element in the DUI SEI message. When not present, the value of dui_dpb_output_du_delay_present_flag is inferred to be equal to 0.

When bp_du_dpb_params_in_pic_timing_sei_flag is equal to 0, at least one DUI SEI message associated with the DUs of an AU shall have dui_dpb_output_du_delay_present_flag equal to 1.

Alternatively, a less preferable approach (since the flag dui_dpb_output_du_delay_present_flag, which is only signalled when bp_du_dpb_params_in_pic_timing_sei_flag  is equal to 0, should not even have been needed in the syntax if it was always required to be 1 when signalled) is to require following constraint:

If bp_du_dpb_params_in_pic_timing_sei_flag  is equal to 0, dui_dpb_output_du_delay_present_flag   shall be equal to 1.

3) It is proposed to remove the note in C3.3.

C3.3 Picture output

The processes specified in this clause happen instantaneously at the CPB removal time of AU n, AuCpbRemovalTime[ n ].

When picture n has PictureOutputFlag equal to 1, its DPB output time DpbOutputTime[ n ] is derived as follows, where the variable firstPicInBufferingPeriodFlag is equal to 1 if AU n is the first AU of a BP and 0 otherwise:

if( !DecodingUnitHrdFlag ) {
	DpbOutputTime[ n ] = AuCpbRemovalTime[ n ] + ClockTick * ( pt_dpb_output_delay –	(1591)
				AuDpbOutputDelta[ Htid ] )
	if( firstPicInBufferingPeriodFlag )
		DpbOutputTime[ n ]  −=  ClockTick * DpbDelayOffset
} else
	DpbOutputTime[ n ] = AuCpbRemovalTime[ n ] + ClockSubTick * dpbOutputDuDelay

where AuDpbOutputDelta[ Htid ] is derived according to pt_cpb_removal_delay_minus1[ Htid ] and pt_cpb_removal_delay_delta_idx[ Htid ] in the PT SEI message associated with AU n and bp_cpb_removal_delay_delta_val[ pt_cpb_removal_delay_delta_idx[ Htid ] ] and bp_dpb_output_tid_offset[ Htid ] in the BP SEI message associated with AU n, and dpbOutputDuDelay is the value of dui_dpb_output_du_delay in the DUI SEI messages associated with AU n when bp_du_dpb_params_in_pic_timing_sei_flag is equal to 0, or the value of pt_dpb_output_du_delay in the PT SEI message associated with AU n when bp_du_dpb_params_in_pic_timing_sei_flag is equal top 1.

NOTE – When the syntax element dui_dpb_output_du_delay is not present in any DUI SEI message associated with AU n, the value is inferred to be equal to pt_dpb_output_du_delay in the PT SEI message associated with AU n.

4) It is proposed to clarify the condition for the existing constraint on the value of dui_dpb_output_du_delay.

It is a requirement of bitstream conformance that all DUI SEI messages that are associated with the same AU, apply to the same operation point, and have bp_du_dpb_params_in_pic_timing_sei_flag  equal to 0 and dui_dpb_output_du_delay_present_flag equal to 1 shall have the same value of dui_dpb_output_du_delay.

# Conclusion

It is proposed to include the proposed modifications to VVC Errata text.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-T2001-v2, Versatile Video Coding Editorial Refinements on Draft 10, October 2020