| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0112-v1 |
| --- | --- |

| Title: | On Bitstream Properties Signalling for Decoder Initialization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

It is proposed to signal picture storage and picture format related information about CVSs in the bitstream to help the decoder initialization.

# Introduction

It is proposed to signal picture storage and picture format related information about CVSs in the bitstream to help the decoder initialization. In this document this information is called bitstream properties information.

It is asserted that signaling information which provides maximum required decoder resources for decoding a bitstream allows a decoder to allocate enough resources (e.g. memory/ DPB space) which can be used to decode the entire bitstream. Thus, need to deallocate and reallocate memory when decoding each CVS of a bitstream may be avoided. This can also result in lowering the decoder delay when switching decoding from one CVS to another CVS, including in streaming environments. Additionally, this may result in requiring allocation of less memory compared to the maximum required from the signalled profile-tier-level. JVET-U0083 also provides a summary of issues related to decoder initialization and additionally refers to the related discussions in systems, including in Systems for Video AHG.

Two alternative options are proposed in this document for helping decoder initialization:

- Option 1: It is proposed to signal following information in Decoding Capability Information (DCI) (or in a new SEI):

- Maximum number of luma samples for a picture

- Maximum number of (luma and chroma) samples for a picture

- Maximum number of luma samples multiplied by bit depth for a picture

- Maximum number of (luma and chroma) samples multiplied by bit depth for a picture

- Maximum value for - number of luma samples for a picture multiplied by maximum required DPB size

- Maximum value for - number of (luma and chroma) samples for a picture multiplied by maximum required DPB size

- Maximum value for - number of luma samples for a picture multiplied by maximum required DPB size multiplied by bit depth

- Maximum value for - number of (luma and chroma) samples for a picture multiplied by maximum required DPB size multiplied by bit depth

- Maximum picture width

- Maximum picture height

- Maximum value of required DPB size

- Maximum bit depth (minus 8)

- Maximum chroma format

- Option 2: A list of unique sets of storage and format information is specified from all the OLSs in the bitstream.

# Proposal

Two alternative options proposed for signaling picture storage and picture format related information about CVSs in the bitstream to help the decoder initialization are described below.

The text below specifies signaling the proposed information in DCI extension. This is asserted to be suitable and aligned with the design goal of DCI NAL unit. Alternatively, this information could be signalled in a new SEI message.

## Option 1

In this option it is proposed to signal following information in Decoding Capability Information (DCI) (or in a new SEI):

- Maximum number of luma samples for a picture

- Maximum number of (luma and chroma) samples for a picture

- Maximum number of luma samples multiplied by bit depth for a picture

- Maximum number of (luma and chroma) samples multiplied by bit depth for a picture

- Maximum value for - number of luma samples for a picture multiplied by maximum required DPB size

- Maximum value for - number of (luma and chroma) samples for a picture multiplied by maximum required DPB size

- Maximum value for - number of luma samples for a picture multiplied by maximum required DPB size multiplied by bit depth

- Maximum value for - number of (luma and chroma) samples for a picture multiplied by maximum required DPB size multiplied by bit depth

- Maximum picture width

- Maximum picture height

- Maximum value of required DPB size

- Maximum bit depth (minus 8)

- Maximum chroma format

It is asserted that knowing information about maximum number of samples or maximum number of samples multiplied by maximum bit depth or maximum number of samples multiplied by maximum bit depth multiplied by maximum DPB size for the bitstream can help decoder allocate DPB memory optimally for the decoding of entire bitstream compared to only knowing the maximum picture width and maximum picture height for the bitstream. This is because the bitstream may contain some CVSs which have maximum (or large) picture width and narrow height, and some CVSs which have maximum (or large) picture height and narrow picture width.

The proposed modifications are shown highlighted below compared to [1].

| decoding_capability_information_rbsp( ) { | Descriptor |
| --- | --- |
| dci_reserved_zero_4bits | u(4) |
| dci_num_ptls_minus1 | u(4) |
| for( i = 0; i <= dci_num_ptls_minus1; i++ ) |  |
| profile_tier_level( 1, 0 ) |  |
| dci_extension_flag | u(1) |
| if( dci_extension_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| dci_extension_alignment_bit_equal_to_one | u(1) |
| bitstream_max_info( ) |  |
| dci_extension2_flag | u(1) |
| if( dci_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| dci_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

dci_extension_flag equal to 0 specifies that no dci_extension_data_flag syntax elements are bitstream_max_info( ) syntax structure is present in the DCI RBSP syntax structure. dci_extension_flag equal to 1 specifies that dci_extension_data_flag syntax elements might be the bitstream_max_info( ) syntax structure is present in the DCI RBSP syntax structure. dci_extension_flag shall be equal to 0 in bitstreams conforming to this version of this Specification. However, some use of dci_extension_flag equal to 1 could be specified in some future version of this Specification, and decoders conforming to this version of this Specification shall allow the value of dci_extension_flag equal to 1 to appear in the syntax.

dci_extension_alignment_bit_equal_to_one shall be equal to 1.

dci_extension2_flag equal to 0 specifies that no dci_extension_data_flag syntax elements are present in the DCI RBSP syntax structure. dci_extension2_flag equal to 1 specifies that dci_extension_data_flag syntax elements might be present in the DCI RBSP syntax structure. dci_extension2_flag shall be equal to 0 in bitstreams conforming to this version of this Specification. However, some use of dci_extension2_flag equal to 1 could be specified in some future version of this Specification, and decoders conforming to this version of this Specification shall allow the value of dci_extension2_flag equal to 1 to appear in the syntax and shall ignore all data that follow the value 1 for dci_extension2_flag in a DCI RBSP.

| bitstream_max_info( ) { | Descriptor |
| --- | --- |
| picsize_max_luma_samples | u(32) |
| picsize_max_samples | u(32) |
| picsize_max_luma_samples_bits | u(32) |
| picsize_max_samples_bits | u(32) |
| dpbsize_max_luma_samples | u(32) |
| dpbsize_max_samples | u(32) |
| dpbsize_max_luma_samples_bits | u(33) |
| reserved_zero_7bits | u(7) |
| dpbsize_max_samples_bits | u(34) |
| reserved_zero_6bits | u(6) |
| max_pic_width | u(16) |
| max_pic_height | u(16) |
| max_dec_pic_minus1 | u(4) |
| max_bitdepth_minus8 | u(4) |
| max_chroma_format | u(2) |
| reserved_zero_6bits | u(6) |
| } |  |

Semantics:

picsize_max_luma_samples specifies the maximum value of PicSizeMaxInSamplesY of a decoded picture when decoding the bitstream.

picsize_max_samples specifies the maximum value of (PicSizeMaxInSamplesY*MaxChromaFactor) of a decoded picture when decoding the bitstream.

picsize_max_luma_samples_bits specifies the maximum value of (PicSizeMaxInSamplesY*MaxBitDepth) of a decoded picture when decoding the bitstream.

picsize_max_samples_bits specifies the maximum value of (PicSizeMaxInSamplesY*MaxChromaFactor*MaxBitDepth) of a decoded picture when decoding the bitstream.

dpbsize_max_luma_samples specifies the maximum value of (PicSizeMaxInSamplesY *(dpb_max_dec_pic_buffering_minus1[ MaxSubLayersMinus1 ] + 1)) of a decoded picture when decoding the bitstream.

dpbsize_max_samples specifies the maximum value of (PicSizeMaxInSamplesY*MaxChromaFactor *(dpb_max_dec_pic_buffering_minus1[ MaxSubLayersMinus1 ] + 1)) of a decoded picture when decoding the bitstream.

dpbsize_max_luma_samples_bits specifies the maximum value of (PicSizeMaxInSamplesY *MaxBitDepth*(dpb_max_dec_pic_buffering_minus1[ MaxSubLayersMinus1 ] + 1)) of a decoded picture when decoding the bitstream.

reserved_zero_7bits shall be equal to 0 in bitstreams conforming to this version of this Specification. The values greater than 0 for reserved_zero_7bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall allow values greater than 0 for reserved_zero_7bits to appear in the bitstream and shall ignore the value of reserved_zero_7bits.

dpbsize_max_samples_bits specifies the maximum value of (PicSizeMaxInSamplesY*MaxChromaFactor *MaxBitDepth*(dpb_max_dec_pic_buffering_minus1[ MaxSubLayersMinus1 ] + 1)) of a decoded picture when decoding the bitstream.

reserved_zero_6bits shall be equal to 0 in bitstreams conforming to this version of this Specification. The values greater than 0 for reserved_zero_6bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall allow values greater than 0 for reserved_zero_6bits to appear in the bitstream and shall ignore the value of reserved_zero_6bits.

max_pic_width indicates the maximum width, in units of luma samples, of a decoded picture when decoding the bitstream.

max_pic_height indicates the maximum height, in units of luma samples, of a decoded picture when decoding the bitstream.

max_dec_pic_minus1 plus 1 indicates the maximum number of decoded pictures stored in the DPB at any moment when decoding the bitstream.

max_chroma_format indicates the maximum value of sps_chroma_format_idc of the SPS referenced by a picture when decoding the bitstream.

max_bitdepth_minus8 plus 8 indicates the maximum bit depth, in units of bits, of each colour component of a decoded picture when decoding the bitstream.

Additionally following highlighted modification is added to Annex A, to calculate variables MaxChromaDepth and MaxBitDepth.

Annex A:

- For an OLS with OLS index TargetOlsIdx, the variables PicWidthMaxInSamplesY, PicHeightMaxInSamplesY, and PicSizeMaxInSamplesY, MaxChromaFactor, MaxBitDepth and the applicable dpb_parameters( ) syntax structure are derived as follows:

  - If NumLayersInOls[ TargetOlsIdx ] is equal to 1, PicWidthMaxInSamplesY is set equal to sps_pic_width_max_in_luma_samples, PicHeightMaxInSamplesY is set equal to sps_pic_height_max_in_luma_samples, and PicSizeMaxInSamplesY is set equal to PicWidthMaxInSamplesY * PicHeightMaxInSamplesY, MaxChromaFactor is set equal to 1 if sps_chroma_format_idc is equal to 0, MaxChromaFactor is set equal to 1.5 if sps_chroma_format_idc is equal to 1, MaxChromaFactor is set equal to 2 if sps_chroma_format_idc is equal to 2, MaxChromaFactor is set equal to 3 if sps_chroma_format_idc is equal to 3,  and MaxBitDepth is set equal to (sps_bitdepth_minus8+8)    where  sps_pic_width_max_in_luma_samples and sps_pic_height_max_in_luma_samples, sps_chroma_format_idc and sps_bitdepth_minus8 are found in the SPS referred to by the layer in the OLS, and the applicable dpb_parameters( ) syntax structure is also found in that SPS.

  - Otherwise (NumLayersInOls[ TargetOlsIdx ] is greater than 1), PicWidthMaxInSamplesY is set equal to vps_ols_dpb_pic_width[ MultiLayerOlsIdx[ TargetOlsIdx ] ], PicHeightMaxInSamplesY is set equal to vps_ols_dpb_pic_height[ MultiLayerOlsIdx[ TargetOlsIdx ] ], PicSizeMaxInSamplesY is set equal to PicWidthMaxInSamplesY * PicHeightMaxInSamplesY, MaxChromaFactor and MaxBitDepth is set equal to 1 if vps_ols_dpb_chroma_format[ MultiLayerOlsIdx[ TargetOlsIdx ] ] is equal to 0, MaxChromaFactor is set equal to 1.5 if vps_ols_dpb_chroma_format[ MultiLayerOlsIdx[ TargetOlsIdx ] ] is equal to 1, MaxChromaFactor is set equal to 2 if vps_ols_dpb_chroma_format[ MultiLayerOlsIdx[ TargetOlsIdx ] ] is equal to 2, MaxChromaFactor is set equal to 3 if vps_ols_dpb_chroma_format[ MultiLayerOlsIdx[ TargetOlsIdx ] ] is equal to 3 and MaxBitDepth is set equal to (vps_ols_dpb_bitdepth_minus8[ MultiLayerOlsIdx[ TargetOlsIdx ] ]+8) and the applicable dpb_parameters( ) syntax structure is identified by vps_ols_dpb_params_idx[ MultiLayerOlsIdx[ TargetOlsIdx ] ] found in the VPS.

## Option 2

In this option a list of unique sets of storage and format information is specified from all the OLSs in the bitstream. This allows a decoder to know in advance the picture storage requirements for decoding any OLS in the entire bitstream and to allocate DPB memory optimally for the decoding of entire bitstream. It avoids need to locate and parse each individual SPS and VPS in the bitstream to obtain this information.

The proposed modifications are shown highlighted below compared to [1].

| decoding_capability_information_rbsp( ) { | Descriptor |
| --- | --- |
| dci_reserved_zero_4bits | u(4) |
| dci_num_ptls_minus1 | u(4) |
| for( i = 0; i <= dci_num_ptls_minus1; i++ ) |  |
| profile_tier_level( 1, 0 ) |  |
| dci_extension_flag | u(1) |
| if( dci_extension_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| dci_extension_alignment_bit_equal_to_one | u(1) |
| cvs_ols_format_info( ) |  |
| dci_extension2_flag | u(1) |
| if( dci_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| dci_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

Semantics of dci_extension_flag, dci_extension_alignment_bit_equal_to_one,  and dci_extension2_flag are same as above for option 1.

| cvs_ols_fomats_info( ) { | Descriptor |
| --- | --- |
| num_formats | u(16) |
| for(i=0; i<num_formats;i++) { |  |
| ols_pic_width[ i ] | u(16) |
| ols_pic_height[ i ] | u(16) |
| ols_max_dec_pic_minus1[ i ] | u(4) |
| ols_bitdepth_minus8[ i ] | u(4) |
| ols_chroma_format[ i ] | u(2) |
| reserved_zero_6bits | u(6) |
| } |  |
| } |  |

num_formats specifies the number of format value entries signalled in the for loop.

ols_pic_width[ i ] specifies the width, in units of luma samples, of each picture storage buffer for i-th OLS in a CVS in the bitstream.

Each ols_pic_width[ i ] shall be equal to either a vps_ols_dpb_pic_width[ j ] (for some value of j) from a VPS in the bitstream (for a multilayer OLS) or sps_pic_width_max_in_luma_samples from a SPS in the bitstream (for a single layer OLS).

ols _pic_height[ i ] specifies the height, in units of luma samples, of each picture storage buffer for i-th OLS in a CVS in the bitstream.

Each ols_pic_height[ i ] shall be equal to either a vps_ols_dpb_pic_height[ j ] (for some value of j) from a VPS in the bitstream (for a multilayer OLS) or sps_pic_height_max_in_luma_samples from a SPS in the bitstream (for a single layer OLS).

ols _max_dec_pic_minus1[ i ] plus 1 indicates the maximum number of decoded pictures stored in the DPB for i-th OLS in a CVS in the bitstream.

Each ols_ max_dec_pic_minus1[ i ] shall be equal to dpb_max_dec_pic_buffering_minus1[ MaxSubLayersMinus1 ] from dpb_parameters() structure corresponding to vps_ols_dpb_params_idx[ j ] (for some value of j) from a VPS in the bitstream or from dpb_parameters() structure in SPS in the bitstream (for a single layer OLS).

ols_bitdepth_minus8[ i ] plus 8 specifies the maximum value of bit depth of the samples of the luma and chroma array for i-th OLS in a CVS in the bitstream.

Each ols_bitdepth_minus8[ i ] shall be equal to either a vps_ols_dpb_bitdepth_minus8[ j ] (for some value of j) from a VPS in the bitstream (for a multilayer OLS) or sps_bitdepth_minus8 from a SPS in the bitstream (for a single layer OLS).

ols _chroma_format[ i ] specifies the maximum value of chroma sampling relative to the luma sampling for i-th OLS in a CVS in the bitstream.

Each ols_chroma_format[ i ] shall be equal to either a vps_ols_dpb_chroma_format[ j ] (for some value of j) from a VPS in the bitstream (for a multilayer OLS) or sps_chroma_format_idc from a SPS in the bitstream (for a single layer OLS).

reserved_zero_6bits shall be equal to 0 in bitstreams conforming to this version of this Specification. The values greater than 0 for reserved_zero_6bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall allow values greater than 0 for reserved_zero_6bits to appear in the bitstream and shall ignore the value of reserved_zero_6bits.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-T2001-v2, Versatile Video Coding Editorial Refinements on Draft 10, October 2020